3GPP TSG-RAN WG4 Meeting #115                                                               R4-2507480

Malta, MT, 19th – 23rd May, 2025

Source:	Samsung

Title:	Study on testing issues for CSI compression two-sided models

Agenda item:			7.22.3

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two use cases (i.e., beam management and positioning accuracy enhancements) [2].

Furthermore, the study objectives with corresponding checkpoints in RAN#105 (Sep ‘24) particularly on the two sub-use cases of CSI feedback enhancements are provided [2]. From RAN4 perspective, it is tasked to further analyze the various testing options for two-sided models, i.e., particularly related to the use case of CSI compression among all (sub-)use cases discussed in Rel-18 study item.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

As per the discussions in RAN#105, the exploration of testability and interoperability challenges related to AI-driven CSI compression is set to continue within the remaining timeframe of Release 19, while the relevant objectives are included in a newly set-up study item [RP-250308]. In this contribution, we aim to further the discussion on testing issues for CSI compression two-sided models.

# 2. Simulation analysis for option 3 and 4

## 2.1 General description for dataset

For dataset, the following endorsed preliminary observations are obtained in RAN4#114bis:

<Way forward in RAN4#114bis> Endorsed preliminary observations - Datasets consist of separate training data and separate test data. - All companies have produced datasets based on the same simulation assumptions, however differences can be observed between performance with different companies datasets. - The “mixed dataset” provides much more consistent results than training or testing using different individual company datasets - The exercise demonstrates that in order to agree on test decoder(s) and or reference encoder(s), some effort is needed to align dataset(s).

Accordingly, the following text proposal is drafted to TR 38.843.

Proposal 1: For general part of test encoder/decoder for 2-sided model, the following text proposal to TR 38.843 is adopted to elaborate dataset:

========================== Text Proposal Start ============================

#### 7.3.2.4	Test encoder/decoder for 2-sided model

(Unchanged text omited)

Other testing options are not precluded and different options might be required after RAN4 performs additional studies.

During the investigation on test decoder standardization for Option 3 and 4, the following observations are obtained for the dataset used for training:

- Datasets consist of separate training data and separate test data.

-	All companies have produced datasets based on the same simulation assumption, however differences can be observed between performance with different companies datasets.

-	The “mixed dataset” provides much more consistent results than training or testing using different individual company datasets

-	The exercise demonstrates that in order to agree on test decoder(s) and or reference encoder(s), some effort is needed to align dataset(s).

========================== Text Proposal End =============================

## 2.2 Standardization for test decoder Option 3

As per the discussions in RAN4#110bis, RAN4 is examining the feasibility of this approach in terms of standardization processes. As further discussed in RAN4#112, RAN4 agreed the way forward [8] on how to proceed the evaluation on test decoder Option 3 (test decoder specified in RAN4 spec.), with the detailed procedure for Option 3 next steps captured as:

Issue 4-1: Option 3 next steps Agreement: - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

Based on RAN4 discussion during/after RAN4#114, based on the reference decoder provided by some companies, most companies provided the trained test encoder based on different assumptions of track-1 and track 2 per requested in RAN4#113.

<Agreement in RAN4#113> Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Proposal 2: For test decoder Option 3, the following text proposal to TR 38.843 is adopted:

========================== Text Proposal Start ============================

7.3.2.4.1	Test decoder Option 3

The target of test decoder Option 3 is that a single decoder defined in the specifications for at least a single test for any DUTs. By assuming the test decoder and reference encoder AI/ML model structures can be agreed in the work item phase, it is agreed that the following steps can be followed in the standardization process to derive test decoder Option 3:

Step-1:	Based on the test decoder and reference encoder AI/ML model structure, companies individually jointly train the reference encoder and test decoder based on a dataset. Check on performance alignment, i.e., to see simulation results from contributing companies

- The dataset used for jointly training can be company specific training dataset or mixed training dataset

- repeat simulations until good alignment is achieved

- move to next step after alignment

Step-2:	Share models(encoder or decoder or both)/datasets (training/testing/inference)

- framework to share data among companies(dataset/model) to be discussed

Step-3:	Select one or more decoder for further analysis

- selection criteria to be discussed

Step-4:	Each company brings results for training of “own encoder” with selected decoder(s)

- performance alignment to be checked/discussed

- using “own” data or data shared by other companies or mixed test dataset

Step-5:	Conclude on the test decoder(s) to be specfied.

During the investigation on test decoder standardization for Option 3, differentiated by the dataset used, there are two tracks identified:

Track 1 (Decoder trained over company own dataset): Select 3 decoders, which are trained in step-1 with companies’ own training dataset at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

- The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Track 2 (Decoder trained over mixed dataset): Create a mixed dataset at RAN4#113. By RAN4#114, cCompanies train decoders based on the mixed dataset in Step-1. At RAN4#114, oOne or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own or the mixed dataset checked against decoder(s).

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

========================== Text Proposal End =============================

### 2.2.1 Option 3 Track-1 (Decoder trained over company own dataset)

Based on the practice for Option 3 Track 1, the following endorsed preliminary observations are obtained in RAN4#114bis, and there is no expectation for further study on Track-1:

<Way forward in RAN4#114bis> Endorsed preliminary observations - The investigation on option 3 is based on using the agreed structure for the decoder and encoder - It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional - The performance of the encoder is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testing dataset - The variation in the performance depending on the training datasets and test datasets can be as much as [20%] in SGCS. Further details are available in the results spreadsheet. o Preliminary observation; further results may be added/refined and the observation updated (for example, using the mixed dataset as in track 2) - For option 3, track 1 to be useful, careful attention would be needed to the alignment of datasets - Up to now, the same encoder structure is considered as used when creating the reference decoder; the impact of different encoder structure is not yet considered

Accordingly, the following text proposal is drafted to TR 38.843.

Proposal 3: For Option 3 Track 1, the following text proposal to TR 38.843 is adopted:

========================== Text Proposal Start ============================

Test decoder Option 3 Track-1

During the investigation on test decoder standardization for Option 3 Track-1, the following observations are obtained for the dataset used for training:

- The investigation on option 3 is based on using the agreed structure for the decoder and encoder

- It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional

- The performance of the encoder is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testing dataset

- The variation in the performance depending on the training datasets and test datasets can be as much as [20%] in SGCS. Further details are available in the results spreadsheet.

  - Preliminary observation; further results may be added/refined and the observation updated (for example, using the mixed dataset as in track 2)

- For option 3, track 1 to be useful, careful attention would be needed to the alignment of datasets

- Up to now, the same encoder structure is considered as used when creating the reference decoder; the impact of different encoder structure is not yet considered

========================== Text Proposal End =============================

By fixing the decoder (i.e., Samsung decoder in ONNX) and performing further training on different datasets provided by different companies, the corresponding encoders are provided with performance evaluated, from which the performance is provided for reference.

| Case | Training Method | Dataset | Encoder trained by | Decoder | SGCS of CNN with quantization | Source for SGCS results |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Joint en/decoder training | Samsung | Joint-trained by Samsung |  | 0.6459 | Reference purpose |
| 1 | Joint en/decoder training | Ericsson | Joint-trained by Ericsson |  | 0.6977 | R4-2416314 |
| 2 | Encoder-only training | Ericsson | Samsung | Samsung (ONNX) | 0.6762(-3.08%) | Evaluated by Samsung |
| 3 | Joint en/decoder training | Nokia | Joint-trained by Nokia |  | 0.705 | R4-2419178 |
| 4 | Encoder-only training | Nokia | Samsung | Samsung (ONNX) | 0.7027(-0.33%) | Evaluated by Samsung |
| 5 | Joint en/decoder training | OPPO | Joint-trained by OPPO |  | 0.717 | R4-2417618 |
| 6 | Encoder-only training | OPPO | Samsung | Samsung (ONNX) | 0.7160(-0.14%) | Evaluated by Samsung |
| 7 | Joint en/decoder training | vivo | Joint-trained by vivo |  | 0.732 | R4-2418748 |
| 8 | Encoder-only training | vivo | Samsung | Samsung (ONNX) | 0.7266(-0.74%) | Evaluated by Samsung |

Specifically, by adopting Samsung decoder to perform training on Ericsson, Nokia, OPPO and vivo provided datasets (accessible in https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/
CSI_compression/Datasets/R4_113), we compared the SGCS performance (for CNN model with quantization) by

- Training method-1: Joint encoder/decoder training for Case 1, 3, 5, 7

- Training method-2: Only encoder training for Case 2, 4, 6, 8.

For a given dataset, by comparing the two training methods, the performance degradation percentage values of training method-2 (encoder-only training) from training method-1 (Joint en/decoder training) are provided.

Observation 1: In Option 3 Track-1 (private encoder-only training), over different datasets, the SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is minor, i.e., 0.14%, 0.33%, 0.74%, 3.08% depending on different datasets.

Based on our experience of data training for CSI compression, as long as the SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is within [1]% over a certain dataset, the selected reference decoder shall be regarded as “containing the same feature” as this dataset.

Proposal 4: Based on the experience of AI-CSI compression with CNN-based neural network, the criteria to determine “whether the selected reference decoder contains the same feature as the specific dataset” is:

- The SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is within [1]% over a the specific dataset.

For the method of reference decoder (Option 3 decoder) to be used for private encoder-only training (i.e., fixing reference decoder, and performing training for encoder in a private company-A dataset), based on the above results, we can reach the following conclusion.

Proposal 5: For Option 3, the feasibility of Track-1 “Decoder trained over company own dataset” can be confirmed under the following conditions:

- The selected reference decoder shall be generalized by performing the training over the dataset with enough feature contained.

- The dataset used for private training should have the feature distribution aligned with the one used for reference decoder, as much as possible.

### 2.2.2 Option 3 Track-2 (Decoder trained over mixed dataset)

As agreed in RAN4#113, RAN4 has created a mixed dataset, which consisting of samples from Ericsson, Mediatek, Nokia, Oppo and Vivo’s datasets, in https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/
NR_AIML_air/CSI_compression/Datasets/R4_114/MIXEDR4_114dsei00.zip.

Based on the investigation on test decoder for Option 3 Track-2, the following preliminary observations and the next step way forward are provided:

<Way forward in RAN4#114bis> Further simulation activity for option 3 track 2 - Definitions - Training Dataset 1 is the dataset used for training of the frozen decoder. - Training Dataset 2 is the dataset used for training of the new encoder based on the frozen decoder. - For the existing results above, Training dataset 1 = training dataset 2 = mixed training dataset. Test dataset = mixed test dataset. - Additional results 1 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test Dataset = Mixed test dataset. - Additional results 2 for RAN4#115: - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = company specific training dataset. Test dataset = company specific test dataset. - Additional results 3 for RAN4#115: Further results using a lower complexity CNN or transformer encoder - Frozen decoder = Samsung decoder trained using the mixed dataset. (i.e., training dataset 1 = mixed dataset) - For the new encoder: Training dataset 2 = mixed. Test Dataset = Mixed test dataset. - Encoder structure: As suggested in [R4-2505156] or company specific selection. Structure and complexity should be reported.

Based the selected frozen decoder, i.e., Samsung decoder trained using the mixed dataset (i.e., training dataset 1 = mixed dataset), the additional results are further provided here.

For additional results 1 and 2 for RAN4#115, there are three options for the company specific training dataset used for training dataset 2, as follows:

| Samsung dataset | Samsung private dataset |
| --- | --- |
| Combined-dataset-Large (abbv. as Comb-L) | The dataset is constructed by containing all the 500k samples from the mixed dataset and 600k samples from Samsung private dataset. |
| Combined-dataset-Small (abbv. as Comb-S) | The dataset is constructed by containing the 300k samples from Mixed dataset and 300k samples of Samsung dataset. |
| Note: For both Combined-dataset-L and Combined-dataset-S, the samples are proportionally divided into training subset and validation subset. The validation subset contains samples from both Mixed dataset and Samsung dataset. |  |

The above three kinds of datasets are used for training for additional results 1&2, in which the encoder is trained by being initialized with random weights. Accordingly, the SGCS performances for additional results are provided as below. The SGCS performances of joint en/decoder training on Samsung dataset are also provided as the benchmark.

For additional results 3 for RAN4#115, two encoders with lower complexities are used to evaluate the impact of different encoder structures.

One lightweight Transformer-based encoder adopts the same structure as suggested in R4-2505156, except using d_head=8 rather than d_head=64.

| Model type | Transformer |
| --- | --- |
| d_model | 64 |
| d_feedforward | 256 |
| N_head | 8 |
| d_head | 64 8 |
| N_TF | 4 |
| dropout | 0.1 |
| FLOPs | 3.63M 1.81M |
| Params | 0.16M |
| Model size | 1.6Mbyte 930Kbyte |

One lightweight CNN-based encoder adopts the same structure as suggested in R4-2505156

| Model type | CNN (Resnet like design) |
| --- | --- |
| FLOPs | (4.14M) 4.11M |
| Model size | (92Kbyte) 107Kbyte |
| Params | 0.02M |
| Number of Resnet blocks | 2 |
| Layer after Resnet blocks | 2 layers (one FC layer, one Conv layer) |
| Layer before Resnet blocks | 1 layer (one Conv layer) |
| Layer within a resnet block | 2 Conv layer and direct combination |
| CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
| Kernel/Channel size within a resnet block | (3,3,16) |
| Size of message (feature map) passed across layers | (13,32) |

In Option 3 Track-2 (Decoder trained over mixed dataset), the following additional results are provided.

| Option 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Joint training |  | New encoder training with (Samsung’s) frozen decoder |  |  |  |
|  |  | Testing on mixed |  | Testing on mixed |  |  | Testing on own |
|  |  | training on mixed | (2nd priority) training on own | training on mixed | Additional results 1 training on own | (2nd priority) training on mixed & own | Additional results 2 training on own |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs, 0.13M Params, 531Kbyte) | Samsung | 0.7018 | - | 0.7016 | 0.6942(-1.08%) |  | 0.6278(-2.80%) |
| Encoder-2 (Transformer with 1.81M FLOPs, 0.16M Params, 930Kbyte Model size) | Samsung |  |  | 0.6977(-0.55%) |  |  |  |
| Encoder-3 (CNN with 4.11M FLOPs, 0.02M Params, 107Kbyte Model size) | Samsung |  |  | 0.6827(-2.69%) |  |  |  |

Observation 2: In additional result 1, -1.08% loss arises from the differences between feature distribution of Samsung training dataset learned by new decoder and feature distribution of mixed training dataset learned by the joined training encoder using mixed dataset.

Observation 3: In additional result 2, -2.80% loss (compared to aforementioned SGCS 0.6459) stems from the differences between feature distribution of Samsung training dataset learned by the joined training decoder using Samsung dataset and feature distribution of mixed training dataset learned by the joined training encoder using mixed dataset.

Observation 4: In additional results 3, compared to the agreed encoder, lightweight Transformer-based encoder suffers -0.55% loss and lightweight CNN-based encoder suffers -2.69% loss.

## 2.3 Standardization for test decoder Option 4

For test decoder Option 4, the following sub-options are agreed in RAN4#112 [13]:

-	4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors

FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder)

-	4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors

FFS whether training dataset (channel information) needs to be specified

-	4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

To make the progress, the following refinement of both Option 4a and 4b in RAN4#114 [18]:

The following update to the procedure description to option 4 is agreed (change marks show delta to WF from RAN4#113) Option 4a(Dataset based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more Eigenvalue dataset(s) for further analysis based option 3 - Selection criteria: select the dataset(s) generated from the selected encoder and decoder pair(s) from Option 3 track 1 - Or mixed dataset from track 2 - If multiple datasets are selected (as was the case of Option 3 track 1), the subsequent steps of the feasibility procedure will be applied on each one of the selected datasets separately - Step 5: Label selected dataset with encoder input/output using encoder corresponding to the selected test decoder from option 3. - Step 6: Companies bring results for training of “own encoder(s) and decoder(s)” with selected dataset(s) - Performance alignment to be checked/discussed - Step 7: Conclude on overall feasibility of Option 4a - feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset Option 4b (encoder based) for 2-sided model - Step 1-3: Reuse results of Option 3 - Step 4: Select one or more encoder(s) for further analysis based on option 3 - Selection criteria: select the encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 1 - Or encoder(s) from the selected encoder and decoder pair(s) of Option 3 track 2 - Step 5: Company brings results for training of “own decoder(s)” with selected encoder(s) - Performance alignment to be checked/discussed - FFS what is the dataset assumed for the training - Step 6: Company trains “own encoder(s)” with “own decoder(s)” from step 5 - FFS what is the dataset assumed for the training - Step 7: Conclude on overall feasibility of Option 4b - Feasibility criteria to be discussed (e.g. Perform testing of all the UE encoders using all test decoders (replicating the process of testing UE’s against RAN4 requirements with different TE vendor decoders, others given in R4-2415376, etc ). - Dataset for feasibility evaluation to be discussed; may be common test dataset or own test dataset Encoder in step 5 should at least have the agreed structure. Companies can also bring analysis/results with other encoders (using different structures)

For Option 4, the following preliminary observations and the next step way forward are provided:

<Way forward in RAN4#114bis> Endorsed preliminary observations - For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used - For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b) Further simulation activity for option 4 - All interested companies are requested to upload their “own” decoders to the server as soon as possible. - “Own” decoder means a decoder trained using the Samsung Frozen encoder and the mixed training dataset. - Definitions - Dataset 1 = Dataset used for training the frozen encoder - Dataset 2 = Dataset used for training the own decoder (using the frozen encoder) - Dataset 3 = Dataset used for training the own encoder (using the own decoder) - First results priority results for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your company’s “own” decoder (this is a test for interoperability between encoders and test decoders) - Second priority results for RAN4#115: - Dataset 1 = Dataset 2 = Mixed training dataset - Dataset 3 = Company own dataset - Test Dataset: Mixed Dataset, also consider own test dataset if interested. - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - Results for consideration of lower complexity encoder for RAN4#115: - Dataset 1 = Dataset 2 = Dataset 3 = Mixed training dataset - Test dataset = Mixed test dataset - Encoder: Lower complexity “own” encoder, examples are provided in [R4-2505156] - Provide results for - Your company’s “own” decoder and “own encoder” - Other companies uploaded “own” decoders and your companies “own” decoder (this is a test for interoperability between encoders and test decoders) - For a future meeting, a further consideration, for option 4b is to consider the frozen encoder with a company own dataset; i.e. Dataset 1 = Mixed training dataset, Dataset 2 = Dataset 3 = Company specific dataset.

### 2.3.1 Results for Option 4

Based the selected frozen encoder, i.e., Samsung encoder trained using the mixed dataset (i.e., training dataset 1 = mixed dataset), the additional results are further provided here.

For first priority results for RAN4#115, Samsung’s “own encoder” is respectively concatenated with Samsung’s “own decoder” and four other companies’ “own decoders” to test the interoperability between encoders and test decoders. Additionally, Samsung’s “own encoders” with lower complexities are used to perform the same model inferences. Samsung’s “own encoders” with lower complexities adopt the same model structures as displayed in Option 3 Track-2. The following table shows the results.

| Option 4 Provide results for - ‘own encoder’ with ‘own decoder’ - ‘own encoder’ with other companies’ decoder |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st priority Dataset1=Dataset2=Dataset3=mixed, Testdataset=mixed |  |  |  |  |  |  |  |  |  |  |
|  |  | Source of 'own' decoder (retrained with the frozen decoder) |  |  |  |  |  |  |  |  |  |  |
|  |  | Samsung | Qualcomm | MediaTek | Apple | ZTE | VIVO | Nokia | OPPO | Huawei, Hisilicon | Ericsson | CATT |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Samsung | 0.6959 | 0.6964 | 0.6850 |  |  |  | 0.6847 |  | 0.7023 |  |  |
| Encoder-2 (Transformer with 1.81M FLOPs, 0.16M Params, 930Kbyte Model size) | Samsung | 0.6936 | 0.6938 | 0.6846 |  |  |  | 0.6840 |  | 0.6984 |  |  |
| Encoder-3 (CNN with 4.11M FLOPs, 0.02M Params, 107Kbyte Model size) | Samsung | 0.6765 | 0.6768 | 0.6670 |  |  |  | 0.6664 |  | 0.6829 |  |  |

Observation 5: In first priority results, for the three encoders of different complexities, companies provided “own decoders” have quite similar SGCS performance, with standard deviation less than 0.01 (0.0077, 0.0063, 0.0071 respectively for encoder-1, encoder-2 and encoder-3).

For second priority results for RAN4#115, Samsung’s “own encoder” is respectively concatenated with Samsung’s “own decoder” and four other companies’ “own decoders”. The results of both mixed test dataset and Samsung own test dataset are compared. The following table shows the results.

| Option 4 Provide results for - ‘own encoder’ with ‘own decoder’ - ‘own encoder’ with other companies’ decoder |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2nd priority Dataset1=Dataset2=mixed, Dataset3=own, Testdataset=mixed |  |  |  |  |  |  |  |  |  |  |
|  |  | Source of 'own decoder’ (retrained with the frozen encoder) |  |  |  |  |  |  |  |  |  |  |
|  |  | Samsung | Qualcomm | MediaTek | Apple | ZTE | VIVO | Nokia | OPPO | Huawei, Hisilicon | Ericsson | CATT |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Samsung | 0.6881 | 0.6895 | 0.6785 |  |  |  | 0.6778 |  | 0.6948 |  |  |

|  |  | 2nd priority Dataset1=Dataset2=mixed, Dataset=own, Testdataset=Samsung own dataset |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Source of 'own decoder’ (retrained with the frozen encoder) |  |  |  |  |  |  |  |  |  |  |
|  |  | Samsung | Qualcomm | MediaTek | Apple | ZTE | VIVO | Nokia | OPPO | Huawei, Hisilicon | Ericsson | CATT |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | Samsung | 0.6224 | 0.6235 | 0.6138 |  |  |  | 0.6129 |  | 0.6284 |  |  |

Observation 6: In second priority results, for both mixed test dataset and Samsung own test dataset, companies provided “own decoders” have quite similar SGCS performance, with standard deviation less than 0.01 (0.0074, 0.0067 respectively for mixed dataset and Samsung dataset).

# 3. Non-simulation issues

In this section, we provided our viewpoints on the non-simulation issues for CSI compression.

## 3.1 What needs to be captured or specified for Option 3

For the content to be captured or specified for Option 3, the following agreement and future discussion points are provided in RAN4#114bis:

## What needs to be captured or specified for Option 3 Agreements - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) For further discussion: The following topics should be further discussed: - For the encoder, the TBD and FFS above; i.e., where the encoder is stored/referred to and whether several encoders would be captured. - Whether it is agreeable to assume that the training dataset for the standardized test decoder is stored within 3GPP, if available - For example in case UE or network vendors could find access to the dataset used for the test encoder useful for their developments - Whether a test dataset is needed in the specification, or some other alternative (such as a channel model etc.)

For Option 3, it is obvious that the test decoder shall be specified, for which the RAN4 specification could refer to a certain 3GPP database location storing the test decoder. The open issues are how the encoder and whether/how training/test dataset should be stored in 3GPP.

Based on our understanding, RAN4 reach the agreement to store the reference encoder since it could facilitate UE encoder development, e.g., to use the reference encoder directly or finetuning on top of the reference encoder. However, we expect one reference encoder is generated at most, if it is joint training. If it is encoder-only training based on the frozen test decoder, based on different model structure, we expect multiple reference encoders can be obtained.

Proposal 6: For Option 3, the encoder that corresponds to the standardized test decoder is stored within 3GPP:

- If this encoder is not provided as RAN1 reference encoder, it could be stored in TR as standardization intermediate outcome, which can further refer to a certain 3GPP database location storing the reference encoder;

- If this encoder and the corresponding standardized test decoder are joint-trained:

-- Only one reference encoder is expected;

- If this encoder is obtained by encoder-only training with frozen test decoder:

-- More than one reference encoders shall be allowed.

Based on our experience, in most cases, the joint-training of the encoder and test decoder can perform better than encoder-only training, therefore, we expect the reference encoder stored here should be obtained by joint-training.

Proposal 7: For Option 3, the encoder that corresponds to the standardized test decoder is stored within 3GPP:

- It should be based on joint-training of this encoder and corresponding standardized test decoder.

For the corresponding datasets for training and test, based on the experience till now, the mixed dataset is better when generating the test decoder with reasonable generalization, therefore, it is reasonable to store the training dataset and corresponding test dataset, which could be used for UE encoder development and TE environment development.

Proposal 8: For Option 3, for dataset used for training and testing:

- Training dataset for the standardized test decoder: it should be stored in 3GPP database “permanently”, which should be accessible to UE or network vendors to facilitate their development.

- Testing dataset which corresponding to the above-mentioned training dataset: it should also be stored in 3GPP dataset, which could be referred to by RAN5 for test case development.

## 3.2 What needs to be captured or specified for Option 4

For the content to be captured or specified for Option 4, the following agreement and future discussion points are provided in RAN4#114bis:

provided in RAN4#114bis:

## What needs to be captured or specified for Option 4 Agreement: - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. For further discussion: The above FFS; i.e. - Whether a dataset is captured for option 4b - Whether a reference encoder is specified for option 4a

For Option 4a, we should firstly clarify the dataset format mentioned in the existing agreement, which is still not clear now. Furthermore, we see no reason to also specify reference encoder in Option 4a, if so the boundary of Option 4a and 4b are confusing.

Proposal 9: For Option 4a, the specified dataset should be the labelled dataset used for deriving test decoder, and there is no need to specify reference encoder.

For Option 4b, by assuming the reference encoder should be specified which can be used to derive the test decoder, the detailed test decoder derived by TE vendors is still dependent on the training dataset.

Proposal 10: For Option 4b, RAN4 should firstly clarify which dataset is considered to be specified, by considering the following four datasets used for Option 4b:

- Dataset 1 = Dataset used for training the frozen encoder

- Dataset 2 = Dataset used for training the own decoder (using the frozen encoder)

- Dataset 3 = Dataset used for training the own encoder (using the own decoder)

- Dataset 4 = Dataset used for conformance testing

Proposal 11: For Option 4b, whether a dataset is captured:

- Dataset 1 can be captured as part of the standardization intermediate outcome;

- Dataset 2 needs to be specified, which can be used to generate the aligned test decoders by different TE vendors, otherwise the Option 4b will be broken to generate a test decoder.

- There no reason to specify Dataset 3, because own encoder is not supposed to be specified.

- Dataset 4 should also be stored in 3GPP dataset, which could be referred to by RAN5 for test case development.

## 3.3 Relation between RAN1 option 3 and RAN1 inter-vendor collaboration direction C

It should be noted that in RAN1 study, the following direction A, B and C are agreed in RAN1#118:

Agreement RAN1#118 Continue the study of following directions - on-device operation and UE side offline engineering - Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) … - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b), including at least the following issues … - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1), including at least the following issues … Agreement RAN1#118bis For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Based on the current progress in RAN1, Direction A and C can be confirmed as feasible as the study item outcome, and for Direction A, the Option 4-1 and 3a-1 are considered as candidates.

RAN4 has discussed the relationship between RAN4 Option 3 and RAN1 inter-vendor collaboration direction C, while no agreement achieved yet:

## Relation between RAN1 option 3 and RAN1 inter-vendor collaboration direction C Agreements None For further discussion: - From the online discussion: FFS on the relation between RAN4 test decoder options and RAN1 inter-vendor interoperability directions. The relation between the RAN4 options and RAN1 inter-vendor interoperability directions should be further clarified.

Considering the above progress in RAN1, we see the similarity of Option 3 compared with Direction C. However, at least the following difference should also be noted:

Proposal 12: Despite the similarity between Option 3 in RAN4 and Direction C in RAN1, the following difference should be noted:

- In inter-vendor collaboration Direction C (in RAN1), both reference encoder/decoder can be specified, while Option 3 (in RAN4) only consider specifying test decoder while the corresponding encoder can be captured in 3GPP for information.

Furthermore, based on our understanding, if RAN1 can confirm it will follow the Direction C in the work item phase, it is straightforward to adopt one or more reference decoder(s) to be used for test decoder.

Proposal 13: If RAN1 can confirm it will follow the Direction C in the work item phase, it is straightforward to adopt one or more reference decoder(s) to be used for test decoder.

# 4. Conclusion

In this contribution, we provided our viewpoints on the on the interoperability and testability aspects for AI-CSI compression use case, accordingly the following observations and proposals are obtained:

Simulation analysis for Option 3 and 4

Proposal 1: For general part of test encoder/decoder for 2-sided model, the following text proposal to TR 38.843 is adopted to elaborate dataset:

========================== Text Proposal Start ============================

#### 7.3.2.4	Test encoder/decoder for 2-sided model

(Unchanged text omited)

Other testing options are not precluded and different options might be required after RAN4 performs additional studies.

During the investigation on test decoder standardization for Option 3 and 4, the following observations are obtained for the dataset used for training:

- Datasets consist of separate training data and separate test data.

-	All companies have produced datasets based on the same simulation assumption, however differences can be observed between performance with different companies datasets.

-	The “mixed dataset” provides much more consistent results than training or testing using different individual company datasets

-	The exercise demonstrates that in order to agree on test decoder(s) and or reference encoder(s), some effort is needed to align dataset(s).

========================== Text Proposal End =============================

Proposal 2: For test decoder Option 3, the following text proposal to TR 38.843 is adopted:

========================== Text Proposal Start ============================

7.3.2.4.1	Test decoder Option 3

The target of test decoder Option 3 is that a single decoder defined in the specifications for at least a single test for any DUTs. By assuming the test decoder and reference encoder AI/ML model structures can be agreed in the work item phase, it is agreed that the following steps can be followed in the standardization process to derive test decoder Option 3:

Step-1:	Based on the test decoder and reference encoder AI/ML model structure, companies individually jointly train the reference encoder and test decoder based on a dataset. Check on performance alignment, i.e., to see simulation results from contributing companies

- The dataset used for jointly training can be company specific training dataset or mixed training dataset

- repeat simulations until good alignment is achieved

- move to next step after alignment

Step-2:	Share models(encoder or decoder or both)/datasets (training/testing/inference)

- framework to share data among companies(dataset/model) to be discussed

Step-3:	Select one or more decoder for further analysis

- selection criteria to be discussed

Step-4:	Each company brings results for training of “own encoder” with selected decoder(s)

- performance alignment to be checked/discussed

- using “own” data or data shared by other companies or mixed test dataset

Step-5:	Conclude on the test decoder(s) to be specfied.

During the investigation on test decoder standardization for Option 3, differentiated by the dataset used, there are two tracks identified:

Track 1 (Decoder trained over company own dataset): Select 3 decoders, which are trained in step-1 with companies’ own training dataset at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

- The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

Track 2 (Decoder trained over mixed dataset): Create a mixed dataset at RAN4#113. By RAN4#114, cCompanies train decoders based on the mixed dataset in Step-1. At RAN4#114, oOne or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own or the mixed dataset checked against decoder(s).

- At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

========================== Text Proposal End =============================

Proposal 3: For Option 3 Track 1, the following text proposal to TR 38.843 is adopted:

========================== Text Proposal Start ============================

Test decoder Option 3 Track-1

During the investigation on test decoder standardization for Option 3 Track-1, the following observations are obtained for the dataset used for training:

- The investigation on option 3 is based on using the agreed structure for the decoder and encoder

- It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional

- The performance of the encoder is very dependent on the alignment of the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testing dataset

- The variation in the performance depending on the training datasets and test datasets can be as much as [20%] in SGCS. Further details are available in the results spreadsheet.

  - Preliminary observation; further results may be added/refined and the observation updated (for example, using the mixed dataset as in track 2)

- For option 3, track 1 to be useful, careful attention would be needed to the alignment of datasets

- Up to now, the same encoder structure is considered as used when creating the reference decoder; the impact of different encoder structure is not yet considered

========================== Text Proposal End =============================

Observation 1: In Option 3 Track-1 (private encoder-only training), over different datasets, the SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is minor, i.e., 0.14%, 0.33%, 0.74%, 3.08% depending on different datasets.

Proposal 4: Based on the experience of AI-CSI compression with CNN-based neural network, the criteria to determine “whether the selected reference decoder contains the same feature as the specific dataset” is:

- The SGCS performance degradation of training method-2 (encoder-only training) from training method-1 (Joint en/decoder trainings) is within [1]% over a the specific dataset.

Proposal 5: For Option 3, the feasibility of Track-1 “Decoder trained over company own dataset” can be confirmed under the following conditions:

- The selected reference decoder shall be generalized by performing the training over the dataset with enough feature contained.

- The dataset used for private training should have the feature distribution aligned with the one used for reference decoder, as much as possible.

Observation 2: In additional result 1, -1.08% loss arises from the differences between feature distribution of Samsung training dataset learned by new decoder and feature distribution of mixed training dataset learned by the joined training encoder using mixed dataset.

Observation 3: In additional result 2, -2.80% loss (compared to aforementioned SGCS 0.6459) stems from the differences between feature distribution of Samsung training dataset learned by the joined training decoder using Samsung dataset and feature distribution of mixed training dataset learned by the joined training encoder using mixed dataset.

Observation 4: In additional results 3, compared to the agreed encoder, lightweight Transformer-based encoder suffers -0.55% loss and lightweight CNN-based encoder suffers -2.69% loss.

Observation 5: In first priority results, for the three encoders of different complexities, companies provided “own decoders” have quite similar SGCS performance, with standard deviation less than 0.01 (0.0077, 0.0063, 0.0071 respectively for encoder-1, encoder-2 and encoder-3).

Observation 6: In second priority results, for both mixed test dataset and Samsung own test dataset, companies provided “own decoders” have quite similar SGCS performance, with standard deviation less than 0.01 (0.0074, 0.0067 respectively for mixed dataset and Samsung dataset).

Non-simulation issues

Proposal 6: For Option 3, the encoder that corresponds to the standardized test decoder is stored within 3GPP:

- If this encoder is not provided as RAN1 reference encoder, it could be stored in TR as standardization intermediate outcome, which can further refer to a certain 3GPP database location storing the reference encoder;

- If this encoder and the corresponding standardized test decoder are joint-trained:

-- Only one reference encoder is expected;

- If this encoder is obtained by encoder-only training with frozen test decoder:

-- More than one reference encoders shall be allowed.

Proposal 7: For Option 3, the encoder that corresponds to the standardized test decoder is stored within 3GPP:

- It should be based on joint-training of this encoder and corresponding standardized test decoder.

Proposal 8: For Option 3, for dataset used for training and testing:

- Training dataset for the standardized test decoder: it should be stored in 3GPP database “permanently”, which should be accessible to UE or network vendors to facilitate their development.

- Testing dataset which corresponding to the above-mentioned training dataset: it should also be stored in 3GPP dataset, which could be referred to by RAN5 for test case development.

Proposal 9: For Option 4a, the specified dataset should be the labelled dataset used for deriving test decoder, and there is no need to specify reference encoder.

Proposal 10: For Option 4b, RAN4 should firstly clarify which dataset is considered to be specified, by considering the following four datasets used for Option 4b:

- Dataset 1 = Dataset used for training the frozen encoder

- Dataset 2 = Dataset used for training the own decoder (using the frozen encoder)

- Dataset 3 = Dataset used for training the own encoder (using the own decoder)

- Dataset 4 = Dataset used for conformance testing

Proposal 11: For Option 4b, whether a dataset is captured:

- Dataset 1 can be captured as part of the standardization intermediate outcome;

- Dataset 2 needs to be specified, which can be used to generate the aligned test decoders by different TE vendors, otherwise the Option 4b will be broken to generate a test decoder.

- There no reason to specify Dataset 3, because own encoder is not supposed to be specified.

- Dataset 4 should also be stored in 3GPP dataset, which could be referred to by RAN5 for test case development.

Proposal 12: Despite the similarity between Option 3 in RAN4 and Direction C in RAN1, the following difference should be noted:

- In inter-vendor collaboration Direction C (in RAN1), both reference encoder/decoder can be specified, while Option 3 (in RAN4) only consider specifying test decoder while the corresponding encoder can be captured in 3GPP for information.

Proposal 13: If RAN1 can confirm it will follow the Direction C in the work item phase, it is straightforward to adopt one or more reference decoder(s) to be used for test decoder.

# 5. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402390, “Discussion on testability and interoperability issues for AI-CSI”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] R4-2405653, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#110bis, April 2024.

[6] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[7] R4-2409762, “Further study on testability and interoperability issues for AI-CSI”, Samsung, RAN#111, May 2024.

[8] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[9] https://en.wikipedia.org/wiki/Multiply%E2%80%93accumulate_operatio

[10] R4-2411636, “Further study on testability and interoperability issues for AI-CSI”, RAN#112, Aug. 2024.

[11] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[12] R4-2416495, “Further study on testability and interoperability issues for AI-CSI compression”, Samsung, RAN4#112bis, Oct. 2024.

[13] R4-2417207, “WF on data sharing for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[15] R4-2418095, “Further study on testability and interoperability issues for AI-CSI compression”, Samsung, RAN4#113, Nov. 2024.

[16] R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, RAN4#113, Nov. 2024.

[17] R4-2501966, “Study on testing issues for CSI compression two-sided models”, Samsung, RAN4#114, Feb. 2025.

[18] R4-2502953, “WF on AI/ML air interface study”, Ericsson, RAN4#114, Feb. 2025.

[19] R4-2504217, “Study on testing issues for CSI compression two-sided models”, Samsung, RAN4#114bis, Apr. 2025.

[20] R4-2505107, “WF on AI/ML air interface study”, Ericsson, RAN4#114bis, Apr. 2025.

[21] R4-2505156, “Simulation assumptions on AI/ML CSI compression”, Huawei, RAN4#114bis, Apr. 2025.