3GPP TSG-RAN WG4 Meeting # 115						R4-2507496

St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			A-IoT general overview

Agenda item:		7.26.2

Document for:		Approval

# Introduction

In this paper, we present our thoughts on general aspects for A-IoT WI [1].

# Discussion

## UE feature list

- Ambient_IoT_Solutions

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63.Ambient_IoT_Solutions | 63-1 |  |  |  |  |  |  |  |  |  |  |  |  |

Usually the feature or UE capability would be defined for a new feature from RF perspective. For example, the output power and band support as UE capability.  For device 1, as the simple device so if it would support multiple band, it may not be necessary to report to network.  If the device 1 roam into a area where other band than the device supported band is deployed, the device will not respond to the network and it is the operator responsibility to deploy the “right” device with needed band support.

No UE capability is defined for A-IoT from RF perspective.

## System parameter

In WID [1], both in-band and standalone are scoped and this is quoted below:

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

## Topic 2-3: Channel raster

Issue 2-3-1: R2D channel raster

Agreement:

- Consider following options for R2D channel raster

  - Option 1: Define 10KHz as R2D channel raster (i.e. same as enhanced channel raster)

  - Option 2: Define channel raster considering the RB orthogonality between macro BS and A-IoT BS

  - Option 3: Define 100KHz as R2D channel raster

  - Other options are not precluded.

For standalone operation, the A-IoT BS should have the same NR channel raster as NR system so the spectrum can be utilized in a efficient way. As the enhanced channel raster is also specified in 38.104 for legacy BS. It should be understood that following the legacy channel raster would be more flexible than only 10kHz raster. For in-band operation, some discussion will be needed as in this case, the A-IoT BS may be configured in a way that RBs allocated to A-IoT BS should be orthogonal and aligned with RB grid of a macro BS.

For in-band operation, the orthogonality between RBs in A-IoT BS and RBs in macro NR BS should be guaranteed.

For standalone operation, the legacy NR channel raster should be followed

To derive the channel raster for A-IoT operating in-band within a macro NR BS, some basic concepts should be considered:

- The RF reference frequency defined by this channel raster identifies the RF channel position depending on the number of specified RBs (NRB) by the spectrum utilization for a given SCS and channel bandwidth as shown in Figure 5.2.1.3-1 [3] quoted below.

If this number is even, then it points to the subcarrier #0 (first one) of RB index #0,

If this number is odd, it points to the subcarrier #6 of RB index #0,

Figure 5.2.1.3-1 [3]: NR RF reference frequency position

- The center frequency of the A-IoT BS can be either center of one RB (frequency position between tone 5 and 6 for 1 RB, 3 RB transmission bandwidth) or the center of the two RB. It is assumed that A-IoT BS center frequency use the same definition of the NR BS.

The RB for A-IoT BS has no NB-IoT channel raster restrictions (e.g. follow the LTE DC subcarrier, 100kHz channel raster only) and as such, the channel raster can be expressed with following equations:

- NR 1channel raster and center frequency of NR channel (less than 3 GHz, so ΔF is 5kHz as shown below) is

fNR=FREF+ΔF∙(NREF-NREF0)=FREF+ΔF∙(NREFstart-NREF0)+ΔF∙Step∙m

Where:

        - m being an integer identifying NR RF reference frequency, e.g. ΔF=5kHz, Step=20 for 100kHz raster

  - # RB for A-IoT is odd and maximum # of RB of NR BS is odd, ± means the A-IoT center frequency is either on the left (-) or right (+):

fR2D=fNR±180p

Where:

        - p being an integer representing the RB index from NR reference frequency (p = 0 for the NR RB hosting the NR Reference frequency

  - # RB for A-IoT is even and maximum # of RB of NR BS is even, ± means the A-IoT center frequency is either on the left (-) or right (+):

fR2D=fNR±180p

  - # RB for A-IoT is odd and maximum # of RB of NR BS is even, there will be 6 subcarrier (90kHz) offset between A-IoT center frequency and NR BS center frequency, ± means the A-IoT centre frequency is either on the left (-) or right (+)

fR2D=fNR±(180p+90)

  - # RB for A-IoT is even and maximum # of RB of NR BS is odd, there will be 6 subcarrier (90kHz) offset between A-IoT center frequency and NR BS center frequency

fR2D=fNR±(180p-90)

It can be observed that if modular 2 of maximum # of RB of A-IoT and modular 2 of the maximum # of RB of NR BS is the same, there will be integer of the 180kHz between A-IoT channel raster and NR raster. Otherwise, if modular 2 of maximum # of RB of A-IoT and modular 2 of the maximum # of RB of NR BS is different, the frequency offset between A-IoT channel raster and NR channel raster will be integer number of 180 kHz plus or minus 90 kHz.

As the 10kHz raster is specified in 38.104, it is up to BS to support it and therefore, there is no need to specify the additional channel raster as the 10kHz raster has finer granularity and divisible by the (180p-90). Reusing 10kHz enhanced channel raster meet the orthogonality of A-IoT BS RB and NR BS RB for inband operation.

Depending on whether modular 2 of maximum # of RB of A-IoT and modular 2 of the maximum # of RB of NR BS is the same or not, there will be either integer number of 180 kHz or integer number of 180 kHz plus or minus 90 kHz for the frequency offset between A-IoT channel raster and NR channel raster

For in-band operation, 10kHz enhanced channel raster can be configured to make sure the orthogonality between A-IoT BS and NR BS.

For in-band operation, reuse the 10kHz enhanced channel raster for A-IoT BS.

Issue 2-2-1: D2R bandwidth

Agreement:

- Define D2R channel bandwidth for A-IoT device:

- Based on 2SB transmission,

- Consider SFO impact

- Consider small frequency shift defined in RAN1

- D2R transmission bandwidth (2 times of 1SB), consider bit rate defined in RAN1

With the defined the D2R channel bandwidth, unwanted emission and occupied bandwidth should be met. We test one criterion of the occupied bandwidth RF requirement. Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. Figure 3 illustrates the SFS (small frequency shift) generation.

With the OBW and different Tb and Tc in combination, The OBW is illustrated in Figure 1. It can be observed that OBW is different with different Tb and Tc combination. As RAN1 still down selection the combination, RAN4 can wait further RAN1 progress to decide on the OBW. With the 10% SFO the final D2R bandwidth can be set as OBW(Tb,Tc)+0.2*1/Tc + guard band.

Use the occupied bandwidth requirement to quantify the channel bandwidth considering the data rate and other imperfection.

Wait Ran1 further down selection of Tb, Tc combination.

Set the D2R channel bandwidth to OBW(Tb,Tc)+0.2*1/Tc + guard band

Figure 1: The OBW of different Tb and Tc combination

Figure 2: OOK D2R signal in frequency domain

Figure 3:Time domain waveform to generate SFS (repetition number = 4)

RAN1 has latest agreement as quoted below to support the analysis above.

Agreement

The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

- FFS further down-selections of Tchip, Tb, and R to be supported

- Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4.

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
|  | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
|  | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
|  | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
|  | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
|  | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
|  | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
|  | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
|  | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
|  | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
|  | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
|  | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
|  | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
|  | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |

# Conclusions

In this contribution, we present our overview on the A-IoT general issues with below observation and proposals:

Observation 1 For in-band operation, the orthogonality between RBs in A-IoT BS and RBs in macro NR BS should be guaranteed.

Proposal-1: No UE capability is defined for A-IoT from RF perspective.

Proposal-2: For standalone operation, the legacy NR channel raster should be followed

Observation 2 Depending on whether modular 2 of maximum # of RB of A-IoT and modular 2 of the maximum # of RB of NR BS is the same or not, there will be either integer number of 180 kHz or integer number of 180 kHz plus or minus 90 kHz for the frequency offset between A-IoT channel raster and NR channel raster

Observation 3 For in-band operation, 10kHz enhanced channel raster can be configured to make sure the orthogonality between A-IoT BS and NR BS.

Proposal-3: For in-band operation, reuse the 10kHz enhanced channel raster for A-IoT BS.

Proposal-4: Use the occupied bandwidth requirement to quantify the channel bandwidth considering the data rate and other imperfection.

Proposal-5: Wait Ran1 further down selection of Tb, Tc combination.

Proposal-6: Set the D2R channel bandwidth to OBW(Tb,Tc)+0.2*1/Tc + guard band

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2502858, WF on requirements for system parameters and A-IOT device, CMCC

- TR 37.824