# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507498

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			CW node RF impact overview

Agenda item:		7.26.3.3

Document for:		Approval

# Introduction

In this paper, we present our overview for RF requirements for a CW node.

# Discussion

Below is the recap of the RAN agreement relating to CW in general.

RAN#103:

Proposal 3v2 (endorsed) - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

From the RAN1 and RAN agreements, it can be observed that the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant and RAN4 can continue to specify the RF characteristic of CW signal based on these basic assumptions.

CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

CW transmission’s timing, power, bandwidth, spectrum is controlled.

### Issue 3-2: ON/OFF power

- Agreement:

  - FFS for OFF power

The same OFF power as A-IOT BS can be specified which is aligned with A-IoT BS

- Transmitter OFF power spectral density shall be less than [-85] dBm/MHz per antenna connector

Specify the -85 dBm/MHz for CWT node OFF power.

### Issue 3-4: Phase noise

- Agreement:

  - FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI.

  - Encourage companies provide phase noise profile

The phase noise model can be referred to TR 38.803 and the phase noise is scaled from 30 GHz phase model with 20 log(f/fc) factor. The phase noise figure is illustrated in Figure 1 with the original phase noise profile of 30GHz for comparison.

Figure 1: Phase noise for 1GHz

As discussed in our companion paper for BS RF requirement, the received CW signal power is either -15.5 dBm or -25.5 dBm. The phase noise in absolute PSD is illustrated in Figure 2.

Figure 2: phase noise power received at BS with different CWT power

To evaluate the noise power within the signal bandwidth, it is illustrated in Figure 3 for R=1 case with a frequency shift equals to 10kHz and 100kHz. It can be observed that noise power is around 10 dB higher for received CW power of -15.5 dBm than noise power for received CW power of -25.5 dBm. This is in general irrespective the frequency shift factor. Though from the noise power perspective, it is desired to have a lower received CW signal, the wanted signal level will be proportional to the CW transmit power.  The received SNR does not change when CWT transmits either 23 dBm or 33 dBm.

Received SNR does not change when CWT transmit either 23 dBm or 33 dBm considering the noise contribution from phase noise only.

As such, the SNR can be only improved when noise power from phase noise would be reduced. This has nothing to do with the CWT received power as explained above. To reduce the noise power, one way is to increase the frequency shift of the modulated signal. This is illustrated in Figure 4. It can be observed that the noise power for Tb=266.67 can be reduced from -96 dBm (R=1) to – 104 dBm (frequency shift= 10 kHz) and can be improved further to -107 dBm (frequency shift = 100kHz).

Another way to improve the SNR is to tighten the backscatter loss. This is to increase the wanted signal with fixed CW power. In our companion paper for device requirement discussion, the backscatter loss is 40 dB when calculated with a geometry with min/max distance. However, such high backscatter loss will not work when considering the phase noise impact. As illustrated in Figure 4, the backscatter loss of 30 dB can only provide around 2 m reader range for R=1 and frequency shift of 10kHz.  One thing should mention that even backscatter loss is 15 dB, the reader range is around 4 m, therefore, R=1 should be excluded from the specification due to the higher noise power and limited coverage range.

R=1 limit the reader range significantly due to the close-in phase noise.

For the phase noise profile, as this leads to other critical RF parameter setting (frequency shift and backscatter loss for device), it will be good to specify a mask for it. When the bandwidth of CWT is decided, so phase noise profile mask may cross in-band and out of band.

Specify the phase noise profile, to be decided whether in-band or out of band mask.

It is observed that a certain frequency shift can help to reduce the noise power. RAN4 can further decide a proper frequency offset where the phase noise profile starts to apply. The frequency offset as the start of the frequency offset for phase noise profile should be decided and RAN4 can discuss this further.

Use frequency offset = X kHz as the start of the frequency offset for the phase noise profile.

Figure 3: noise power within different signal BW (1/Tb) with R=1

Figure 4: noise power within ½ signal BW (1/Tb) with frequency shift = 10kHz and 100kHz

Figure 4: Received SNR with different backscatter loss and frequency shift

### Issue 3-5: Unwanted emission

- Agreement:

  - FFS on unwanted emission boundary and corresponding values:

  - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth

  - Option 2: FFS whether or not to define channel bandwidth for CW

  - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4.

Table 2.2-4: CW node spectrum emission mask

| Δf (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0-90 | N/A |  |
| 90-270 | -15 | 30 kHz |
| 270-450 | -20 | 30 kHz |
| 450- fmax | -27 | 30 kHz |

  - Other options are not precluded.

For unwanted emission, the first question is the CWT channel bandwidth. As the legacy unwanted emission specification, the boundary of unwanted emission is from the channel edge. Though CW signal is unmodulated signal, it still occupies some bandwidth measured in OBW. The output power should be measured within a certain bandwidth where 99% emitted power is confined. OBW is in some region To study the needed bandwidth, a simulation with CW signal with phase noise profile in TR 38.803 is illustrated in Error! Reference source not found. (using the phase noise profile in Figure 1 ) and it can be observed that CW occupied bandwidth is around 66Hz. OBW is a regulatory requirement in some regions. In FCC 47 CFR 2.202, the definition of occupied bandwidth is defined:

§ 2.202 Bandwidths.

(a) Occupied bandwidth. The frequency bandwidth such that, below its lower and above its upper frequency limits, the mean powers radiated are each equal to 0.5 percent of the total mean power radiated by a given emission. In some cases, for example multichannel frequency-division systems, the percentage of 0.5 percent may lead to certain difficulties in the practical application of the definitions of occupied and necessary bandwidth; in such cases a different percentage may prove useful.

In ETSI harmonized standard (e.g EN 300 220-2), the OBW is also required. The requirements refer to EN 300 220-1 and quoted as below:

5.6.2 Reference limits The Operating Channel shall be declared and shall reside entirely within the Operational Frequency Band. The Maximum Occupied Bandwidth at 99 % shall reside entirely within the Operating Channel defined by Flow and Fhigh.

OBW is defined within channel bandwidth and is a regulatory requirement

As the occupied bandwidth measured in simulated signal with phase noise impact without PA impact is around 66 Hz, it seems to declare such a small bandwidth is not necessary as BS defines the channel bandwidth and CWT node can be measured within the same channel bandwidth as the BS. As such, we suggest using the min channel BW defined in BS for channel bandwidth for A-IoT. It is not necessary to define a CWT specific channel bandwidth for the OBW.

Define the min channel BW for D2R for CWT.

Suppose the CWT channel bandwidth is X Hz.  And the frequency offset for phase noise starts from Y Hz (we propose Y=10kHz in previous section). And most likely X > Y, so there is a need to specify the in-channel emission mask and out of channel emission mask.

For out of channel emission mask , the option 3 could be used and this is absolute value as the same as the legacy requirement.

For in-channel emission mask, this is mainly for phase noise profile. Phase noise mask may cross the in-channel and out of channel but as it is relative so it can be two set of requirements in the end.

Option 3 is fine for out of band emission requirement.

Define a relative phase noise mask starting from the frequency offset Y Hz to Z Hz in addition to the option 3.

The channel bandwidth is under discussion, as CW always transmit a CW tone, a min channel bandwidth can be chosen for CW channel bandwidth.

A bandwidth should be defined together with CW output power so that the power can be measured with OCC bandwidth.

A minimum channel bandwidth can be defined for CW node.

We also studied the ACLR for the simulated waveform. The ACLR1 is around 70 dBc measured using the waveform in Figure 5.  As it is sinusoid and unmodulated signal, the PAPR is low and therefore, the PA impact can be ignored. In this sense, there is no need to specify the ACLR for CWT node.

No need to specify the ACLR for CW node.

Figure 5: CW signal OBW

### Issue 3-8: CW others

- Agreement:

  - CW transmission and A-IoT BS downlink data transmission are non-concurrent.

    - FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

As per RAN plenary, the CW transmission power ON /OFF and timing to turn ON/OFF is controlled. Therefore, it can be expected that the CW transmission does not overlap with the R2D signal transmission from BS. If this would occur, for device near the CW node, the received power of R2D can be more than 20 dB below the received power of CW as illustrated in Figure 6. Though it is assumed 10 MHz RF bandwidth for device 1 during study phase, the antenna selectivity at stop band may be around 20 dB for printed filtering dipole antenna [3]. Therefore, the device 1 may not decode the R2D when it is located near the CW node and far from A-IoT BS. In this case, the CW node transmission should be ON after the R2D signal transmission and colliding with R2D transmission in time domain should be avoided.

The attenuation at stop band for device antenna may not be good enough for D1T1-B and CW and R2D simultaneously transmission may cause outage of the R2D receiving at device.

In TR 38.769, there is also a RF BPF in device 1 architecture and this can enable the out of band blocking performance of device 1. The OOB for device 1 is not considered so far. If the CW transmission can occur simultaneously with R2D, RAN4 needs to discuss OOB for device 1 also.

Figure 6: Received power at antenna of device and BS with ISD of 20 meters

A relevant issue for CW ON/OFF mask is the synchronization accuracy between the CW node and BS. As illustrated in Figure 7, if the CW node is switched on too early or switched off too late, the R2D signal may be corrupted at device. Therefore, a reasonable synchronization accuracy between CW node and BS should be expected. For TDD network, 3 us is a known synchronization requirement between the TDD node. We propose to reuse such requirement for CW node and A-IoT BS. As discussed above, if CW and R2D operate simultaneously, the OOB requirement needs to be specified for device 1. We think maybe it will be better network guarantee the CW and R2D not be transmitted simultaneously rather than adding device complexity as passive device is sensitive to the power consumption and cost.

Introduce the synchronization requirement of 3 us between CW node and A-IoT BS.

OOB performance of device 1 needs to be specified if CW and R2D should be transmitted simultaneously at network

Figure 7: CW node switch too early or too late

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

Observation 2 CW transmission’s timing, power, bandwidth, spectrum is controlled.

Proposal-1: Specify the -85 dBm/MHz for CWT node OFF power.

Observation 3 Received SNR does not change when CWT transmit either 23 dBm or 33 dBm considering the noise contribution from phase noise only.

Observation 4 R=1 limit the reader range significantly due to the close-in phase noise.

Proposal-2: Specify the phase noise profile, to be decided whether in-band or out of band mask.

Proposal-3: Use frequency offset = X kHz as the start of the frequency offset for the phase noise profile.

Observation 5 OBW is defined within channel bandwidth and is a regulatory requirement

Proposal-4: CW transmission’s timing, power, bandwidth, spectrum is controlled.

Proposal-5: Option 3 is fine for out of band emission requirement.

Proposal-6: Define a relative phase noise mask starting from the frequency offset Y Hz to Z Hz in addition to the option 3.

Proposal-7: A bandwidth should be defined together with CW output power so that the power can be measured with OCC bandwidth.

Proposal-8: A minimum channel bandwidth can be defined for CW node.

Observation 6 The attenuation at stop band for device antenna may not be good enough for D1T1-B and CW and R2D simultaneously transmission may cause outage of the R2D receiving at device.

Proposal-9: No need to specify the ACLR for CW node.

Proposal-10:Introduce the synchronization requirement of 3 us between CW node and A-IoT BS.

Observation 7OOB performance of device 1 needs to be specified if CW and R2D should be transmitted simultaneously at network

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei

- “Printed Filtering Dipole Antenna With Compact Size and High Selectivity” Xiyao Liu; Ken Ning etc, IEEE Transactions on Antennas and Propagation (Volume: 72, Issue: 3, March 2024)