# 3GPP TSG-RAN WG4 Meeting # 115						R4-2507502

# St Julian’s, Malta, 19th – 25th May 2025

Source:		Ericsson

Title:			RF imact for TDM DMRS for IoT NTN

Agenda item:		7.32.2

Document for:		Approval

# Introduction

In this paper, we present our view on the RF impact for TDM DMRS feature specified in RAN1.

# Discussion

In LS [1], RAN1 ask the RAN4 the phase continuity issue for TDM DMRS feature and RAN4 will confirm their working assumption below:

Working assumption

For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.

Send LS to RAN4 asking whether there would be any issue (e.g. phase continuity) for supporting such TDM DMRS for IoT NTN.

Firstly, the TDM DMRS introduced in RAN1 relating to the LS statement is illustrated in below,

Figure 1: First two slots for 2 UEs using TDM DMRS over 4 slots, where UE2 blanked the first two DMRS symbols in first two slots

Figure 2: Next two slots for 2 UEs using TDM DMRS over 4 slots, where UE1 blanked the two DMRS symbols in second two slots

The slot is defined as picture below in TS 36.211 as 2ms duration for 3.75kHz SCS.

For the TDM DMRS, there is  a question on the OCC2 performance over the slots where DMRS symbols are blanked. In 36.102, the measurement duration is defined in frequency error requirement clause and quoted below:

“The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency.”

6 ms

This means the CFO is measured over 2ms slot for 3.75kHz SCS. For the slot where the DMRS symbol is blanked, the CFO estimation will be up to the bundled time slot over 6ms where the two DMRS symbols between the 6ms slots can be used to estimate the CFO.

Figure 3: The largest time gap of 6ms between two DMRS symbol

Assuming the two adjacent different DMRS symbols are used for the CFO estimation at base station, and assume the UE has 200Hz frequency error, phase rotation between two DMRS symbols are:

200Hz * 6ms * 360 degree = 432 degree.

The phase rotation over 360 degree will not be detected at the base station and as such a frequency estimation error of 166.7Hz will be created (base station will see 72 degree instead of 432 degree in this case).

200Hz -  72/360/6ms = 200Hz- 33.3Hz = 166.7Hz.

Such frequency error will generate the phase rotation for two OCC symbols:

166.7 Hz * 2* 275 us* 360 = 30 degree

Some company mentioned last meeting that the above phase wrapping issue can be avoided if network can estimate the two slots before the DMRS-muting slots and two lots after the DMRS-muting slot. If there is no frequency change on these two measurements, network can assume the same frequency error in the time slot where DMRSs are muted. This is one case it works assuming UE not updating the frequency in the slots where DMRS is muted. In another case, where there are dramatic change between these two measurement (when pre-compensation of doppler applying new value), there would be a question how to estimate the frequency error in the time slots where DMRS are muted.

We think it should be appropriate to use the DMRS beside the two DMRS-Muted slots to measure the frequency error instead. This would have the impact on the frequency error requirement. UE should keep frequency not changed during the time slots where DMRS are muted, this including two time slot beside the DMRS-muted time slots where DRMS is present. The measurement time can be 6ms instead of 2ms for existing measurement time for 3.75kHz SCS.

Considering the 30 degree phase error introduced by 6 ms measurement time at network, it should not impact the OCC2 performance. Currently it shows in our companion paper that the DMRS bundling requirement may be fine to use for OCC2/OCC4.

Another aspects is for the test, current measurement time is 2 ms for 3.75kHz and it is not possible to measure the frequency error in DMRS-muted time slots. In this sense, RAN4 can consider whether to introduce a measurement time relaxation for TDM DMRS feature.

Evaluate whether to introduce the 6ms measurement time relaxation for the frequency error for the TDM DMRS feature in specification.

# Conclusions

In this contribution, we present our general overview for LS [1]

Observation 1 The largest time gap of 6ms between two adjacent DMRS symbols may introduce 30 degree phase offset between two OCC2 code when UE frequency error is 200Hz.

Proposal-1: Evaluate whether to introduce the 6ms measurement time relaxation for the frequency error for the TDM DMRS feature in specification.

# References

- R1-2501622,	LS on TDM DMRS for 3.75kHz SCS OCC,