3GPP TSG-RAN WG4 Meeting#115                                R4-2507508

Malta , MT, 19th – 23th May, 2025

Agenda item:	7.26.3.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for CW node

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

- Issue 3-1: CW Output power - Agreement: - CW output power <=33dBm, and can be declared by manufacture. - Issue 3-2-1: ON/OFF power - Agreement: - Define CW OFF power - Value TBD - Issue 3-4: Phase noise - Agreement: - FFS on whether to define phase noise requirements or leave to implementation in Rel-19 WI. - Encourage companies provide phase noise profile - Issue 3-5: Unwanted emission - Agreement: - FFS on unwanted emission boundary and corresponding values: - Option 1: The unwanted emission of CW is defined from minimum small frequency shift ± D2R transmission bandwidth/2 to the boundary of D2R channel bandwidth - Option 2: FFS whether or not to define channel bandwidth for CW - Option 3: The spectrum emission mask of the CW node applies to frequencies (Δf) starting from the assigned transmission frequency. The spectrum emission mask for CW node is defined as in Table 2.2-4. Table 2.2-4: CW node spectrum emission mask Δf (kHz); Emission limit (dBm); Measurement bandwidth. 0-90; N/A. 90-270; -15; 30 kHz. 270-450; -20; 30 kHz. 450- fmax; -27; 30 kHz. - Other options are not precluded. - Issue 3-6: Spurious emissions - Agreement: - FFS on spurious emissions boundary and corresponding values: - Option 1: The spurious emission of CW starts from the boundary of D2R channel bandwidth. - Option 2: The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz - Other options are not precluded. - Issue 3-8: CW others - Agreement: - CW transmission and A-IoT BS downlink data transmission are non-concurrent. - FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

Discussion

- Issue 3-2: ON/OFF power

- Agreement:

  - FFS for OFF power

- In the last RAN4 meeting, there were some discussion how to define ON/OFF power for CW node since the CW transmission is not always on or contiguous transmission, therefore some ON/OFF power might be needed. From our understanding, the UE OFF power as defined in the 38.101-1 could be updated as following:

|  | Channel bandwidth / Transmit OFF power / Measurement bandwidth |
| --- | --- |
|  | 0.18MHz |
| Transmit OFF power | -50 dBm |
| MBW | 0.18MHz |

- Proposal 1: propose to define the OFF power for CW node transmission as -50dBm/0.18MHz.

- Issue 3-4: Phase noise

- In the previous RAN4 meetings, there were some discussions on the phase noise requirement and also its impacts on D2R reception. In the following figure, some initial simulation results under the AWGN channel are provided for further discussion. It could be observed that phase noise -70dBc/15KHz still have limited impacts e.g. less than 1dB@BLER 10%. If phase noise is improved as -80dBc/15KHz or -90dBc/15KHz, then the impacts on D2R reception will further limited and almost negligible.

Figure 1. phase noise impacts on D2R reception under the AWGN channel

Regarding how to define the phase noise of CW node, if necessary, we could derive the necessary phase noise profile by considering other phase noise cancellation capability e..g. space isolation, antenna isolation, analog filtering or by-pass filtering e.g.

- Step 1: D2R REFSENS= -174+10*log10(15k)+NF(10dB)+SNR(-6dB@BLER 10%)+IM;

- Step 2: Acceptable phase noise profile at D2R reception: -174+NF(10dB)+SNR(-6dB@BLER 10%)+IM- [70dBc]

- Step 3: Acceptable phase noise from the CW transmitter perspective: -174+NF(10dB)+SNR(-6dB@BLER 10%)+IM- [70dBc]+CW/Phase noise cancellation capability.

Proposal 2: for phase noise profile for CW node if necessary, RAN4 need to further discuss the acceptable degradation of phase noise on D2R reception and also phase noise cancellation technique (e.g. space isolation, antenna isolation, analog filtering or by-pass filtering etc)

Proposal 3: if the phase noise requirement of CW node is defined from the CW transmitter, then unwanted emission requirement could be not defined.

- Issue 3-8: CW others

- Agreement:

  - CW transmission and A-IoT BS downlink data transmission are non-concurrent.

- FFS whether or not introduce the synchronization requirement of 3 us between CW node and A-IoT BS

During the last RAN4 meeting, there was one proposal to specify the 3us timing error requirement between A-IoT BS and CW node. From our understanding, this is quite similar as network sync error requirement as defined in RRM spec, we propose not to discuss this requirement in RF session unless there are any request from RRM requirement definition perspective.

Proposal 4: propose not to discuss this requirement in RF session since this is RRM requirement.

Conclusions

In this contribution, we shared some initial views on the RF requirement for CW node in D1T1-B and proposals are made as following:

Proposal 1: propose to define the OFF power for CW node transmission as -50dBm/0.18MHz.

Proposal 2: for phase noise profile for CW node if necessary, RAN4 need to further discuss the acceptable degradation of phase noise on D2R reception and also phase noise cancellation technique (e.g. space isolation, antenna isolation, analog filtering or by-pass filtering etc)

Proposal 3: if the phase noise requirement of CW node is defined from the CW transmitter, then unwanted emission requirement could be not defined.

Proposal 4: propose not to discuss this requirement in RF session since this is RRM requirement.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282	RF requirements for Ambient IoT BS, Huawei,

- R4-2500782, RF requirements for A-IoT BS, Huawei, Noted.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.

- R4-2501863, RF requirements for CW, Huawei, approved.

- R4-2505097, WF on A-IoT BS and CW requirements, Huawei, approved.