3GPP TSG-RAN WG4 Meeting#115                                R4-2507509

Malta, MT, 19th – 23th May, 2025

Agenda item:	7.12.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for Inter-band CA for ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we would like to share some views on the inter-band CA related issues for ATG UE.

Issue 1-3: MSD for CA_n3A-n39A with n3 UL Agreement: - MSD requirements will be decided based on the average values among companies’ simulation results. - Encourage companies to provide results next meeting.

Discussion

- 2.2. RF requirements for Inter-band CA with band n3 UL transmission

For CA_n3-n39 with uplink transmission as n3, MSD analysis with UL transmission power ranging from 23dBm to 40dBm is provided as following:

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n3, dBm |  | 23 | 23 |
| Tx band n3 BW(MHz, Lcrb@15kHz) | 25 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 receival singal at ANT port, dBm |  | 23 | 3 |
| n39 filter rejection at 1710~1785MHz, dB | 40 |  |  |
| signal After n3 filter, dBm |  | -21 | -41 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -92 | -132 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -88 | -128 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n39 BW(MHz, NRB) | 28.8 |  |  |
| Thermal noise, dBm |  | -90.41 | -90.41 |
| ACLR1, dB | 30 |  |  |
| ACLR3, dB | 60 |  |  |
| n3 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -50.98 |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -36.39 |  |
| n3 Tx filter rejection at 1880~1920MHz, dB | 30 |  |  |
| n3 PA noise power at Rx n39 ant port at 1880~1920MHz, dBm |  | -66.39 | -86.39 |
| Total noise level at ANT port |  | -66.34 | -84.94 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(30MHz BW) |  | -67.34 | -85.94 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(30MHz BW), dBm | -83.50 |  |  |
| MSD | 5.37 |  |  |

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n3, dBm |  | 40 | 40 |
| Tx band n3 BW(MHz, Lcrb@15kHz) | 25 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 receival singal at ANT port, dBm |  | 40 | 20 |
| n39 filter rejection at 1710~1785MHz, dB | 40 |  |  |
| signal After n3 filter, dBm |  | -4 | -24 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -58 | -98 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -54 | -94 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n39 BW(MHz, NRB) | 28.8 |  |  |
| Thermal noise, dBm |  | -90.41 | -90.41 |
| ACLR1, dB | 30 |  |  |
| ACLR3, dB | 60 |  |  |
| n3 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -33.98 |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -19.39 |  |
| n3 Tx filter rejection at 1880~1920MHz, dB | 30 |  |  |
| n3 PA noise power at Rx n39 ant port at 1880~1920MHz, dBm |  | -49.39 | -69.39 |
| Total noise level at ANT port |  | -48.10 | -69.34 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(30MHz BW) |  | -49.10 | -70.34 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(30MHz BW), dBm | -67.87 |  |  |
| MSD | 20.99 |  |  |

Based on the above analysis, MSD for 23dBm to 40dBm transmission power would be ranging from 5.37dB to 20.99dB which will bring the significant performance degradation of band n3 DL reception.

Proposal 1: for the inter-band CA CA_n3-n39 with uplink transmission as n39, MSD requirement could defined as following:

| CA_n3-n39 with UL n3 | MSD [dB] |
| --- | --- |
| 23 | 5.37 |
| 24 | 6.11 |
| 25 | 6.89 |
| 26 | 7.71 |
| 27 | 8.56 |
| 28 | 9.44 |
| 29 | 10.34 |
| 30 | 11.26 |
| 31 | 12.20 |
| 32 | 13.15 |
| 33 | 14.11 |
| 34 | 15.07 |
| 35 | 16.05 |
| 36 | 17.03 |
| 37 | 18.01 |
| 38 | 19.00 |
| 39 | 20.00 |
| 40 | 20.99 |

Conclusions

In this contribution, we shared some initial views on the RF requirements for ATG UE supporting inter-band DL CA in Rel-19 and proposals are made as following:

Observation 1:MSD for 23dBm to 40dBm transmission power would be ranging from 5.37dB to 20.99dB which will bring the significant performance degradation of band n39 DL reception when the uplink power from band n3 is high.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.

- R4-2410687,WF on Rel-19 ATG UE RF requirements, Approved.

- TR 38.718-02-01

- R4-2417121, WF on UE RF for ATG enhancement, CMCC, approved.

- R4-2502952,WF on UE RF requirements for ATG, CMCC, approved.