3GPP TSG-RAN WG4 Meeting#115                                R4-2507513

Malta, MT, 19th – 23th May, 2025

Agenda item:	7.13.3.3

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on Coexistence spurious emission requirements

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In the RAN#106 meeting, the following objective is added for the reply LS to ETSI TFES. In this contribution, we would like to share our initial understanding on this work and its corresponding work plan for it.

Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

Discussion

## 2.1. Antenna pattern evaluation of spurious emission at 6.4GHz

Based on the agreed coexistence scenario, aggressive system is at 2GHz or 3.5GHz and victim system is at 6.4GHz.

Spurious emission of 2GHz and 3.5GHz will falling into the receiver bandwidth of both BS or UE. One key question how to model this antenna pattern for this spurious emission at 6.4GHz is quite critical, therefore in the following section, some initial simulation results are provided for further discussion in RAN4.

Way forward: - Aggressor carrier frequencies: prioritize 2 GHz, 3.5 GHz, - Victim frequency bands: prioritize lower bound frequency of band n104, i.e., 6425 MHz.

In the following section, radiation pattern at 2GHz and U6GHz of 2GHz antenna element and 3.5GHz antenna elements are provided to check the performance of wanted signal and spurious emission signal.

The simulation results for 2GHz antenna element are summarized in the following table 2.1-1:

Table 2.1-1. summarized simulation results for 2G antenna element

|  | 2G | U6G |
| --- | --- | --- |
| Operating frequency | 1710-2170MHz | 6425-7125MHz |
| Maximum Antenna gain | 8.1dB | 6.22dB |
| VSWR | <2 | 2.2-4.4 |

Figure 2.1-1. antenna gain diagram at 2GHz antenna element [2GHz left, U6GHz right]

The simulation results for 2GHz antenna sub-array with 1x4 configuration are summarized in the following table 2.1-2:

Table 2.1-2. summarized simulation results for 2GHz antenna sub-array with 1x4 configuration

|  | 2G | U6G |
| --- | --- | --- |
| Operating frequency | 1710-2170MHz | 6425-7125MHz |
| Maximum Antenna gain | 13.4dBi | 10.4dBi |
| VSWR | <1.7 | <4.56 |

Figure 2.1-3. antenna gain diagram at 2GHz antenna element [2GHz left, U6GHz right]

The simulation results for 3.5GHz antenna element are summarized in the following table 2.1-3:

Table 2.1-3. summarized simulation results for 3.5G antenna element

|  | 3.5G | U6G |
| --- | --- | --- |
| Operating frequency | 3300-3700MHz | 6425-7125MHz |
| Maximum Antenna gain | 8.7dB | -4.1dB |
| VSWR | <1.7 | 2.1-8.3 |

Figure 2.1-4. antenna gain diagram at 3.5GHz antenna element [3.5GHz left, U6GHz right]

Observation 1a: if the spurious emission signal is not the harmonic spurious signal, then beamforming gain of spurious emission cannot been assumed at 6.4GHz.

Observation 1b: if the spurious emission signal is the harmonic spurious signal, RAN4 needs more evaluations whether beamforming gain could assumed or not.

## 2.2. Simulation results for BS2BS coupling loss

In the last RAN4 meeting, either deterministic way or system level simulation could be allowed to do the statistical analysis for BS2BS or BS2UE coupling loss analysis. In the following section, some simulation results of BS2BS MCL under the different grid shit are provided in Figure 2.2-1/3/4/5 for further discussions.

Figure 2.2-1. the network deployment layout [100% grid shift] for BS2BS coupling loss

Figure 2.2-2. simulation results for BS2BS coupling loss at 6.4GHz, 100% grid shift

Figure 2.2-3. simulation results for BS2BS coupling loss at 6.4GHz, 50% grid shift

Figure 2.2-4. simulation results for BS2BS coupling loss at 6.4GHz, 20% grid shift

Figure 2.2-5. simulation results for BS2BS coupling loss at 6.4GHz, 10% grid shift

Note: antenna elements considered for transmitter spurious emission and sub-array considered from the receiver perspective which is applicable for non-harmonic spurious emission signal at least.

Based on the simulation results in Figure Figure 2.2-1/2/3/4/5, the BS2BS MCL values are summarized in the following table 1.

Observation 2: the BS2BS MCL under the different grid shift are summarized as following:

Table 1. the summarized BS2BS MCL under the different grid shift

|  | MCL |  |  |
| --- | --- | --- | --- |
| Grid shift | 99% | 99.998% | Baseline |
| 100% | -95 | -75.7 | -67 |
| 50% | -67 | -67.3 | -67 |
| 20% | -78.07 | -58.6 | -67 |
| 10% | -72.7 | -51.01 | -67 |

## 2.3. Simulation results for BS2UE coupling loss

In the following section, some simulation results of BS2UE MCL are provided in Figure 6 for further discussions.

Note: antenna elements considered for transmitter spurious emission and 0dBi considered from the receiver perspective which is applicable for non-harmonic spurious emission signal at least.

Figure 6. simulation results for BS2UEcoupling loss at 6.4GHz, 100% grid shift

Based on the simulation results in Figure 6 the BS2UE MCL values are summarized in the following table 1.

Observation 3: the BS2UE MCL values are summarized as following:

Table 2. the summarized BS2UE MCL

|  | MCL |  |  |
| --- | --- | --- | --- |
| Grid shift | 99% | 99.965% | Baseline |
| 100% | -71 | -65.7 | -53 |

## 2.4. Co-existence spurious emission requirement

If we consider the 99% tile of MCL for both BS2BS and BS2UE, then around 18dBc relaxation could be allowed.

If we consider the 99.9% tile of MCL for both BS2BS and BS2UE, then around 12dBc relaxation could be allowed.

|  | BS2BS MCL |  |  |
| --- | --- | --- | --- |
| Grid shift | 99% | 99.998% | Baseline |
| 100% | -95 | -75.7 | -67 |
|  | BS2 UE MCL |  |  |
| Grid shift | 99% | 99.965% | Baseline |
| 100% | -71 | -65.7 | -53 |

Observation 4:

If the 99% tile of MCL is considered for both BS2BS and BS2UE, then around 18dBc relaxation could be allowed.

If the 99.9% tile of MCL is considered for both BS2BS and BS2UE, then around 12dBc relaxation could be allowed.

Conclusions

In this contribution, we shared some initial views on the applicability improvement of TRP in Annex in RAN4 and proposals are made as following:

Observation 1a: if the spurious emission signal is not the harmonic spurious signal, then beamforming gain of spurious emission cannot been assumed at 6.4GHz.

Observation 1b: if the spurious emission signal is the harmonic spurious signal, RAN4 needs more evaluations whether beamforming gain could assumed or not.

Observation 2: the BS2BS MCL under the different grid shift are summarized as following:

Table 1. the summarized BS2BS MCL under the different grid shift

|  | MCL |  |  |
| --- | --- | --- | --- |
| Grid shift | 99% | 99.998% | Baseline |
| 100% | -95 | -75.7 | -67 |
| 50% | -67 | -67.3 | -67 |
| 20% | -78.07 | -58.6 | -67 |
| 10% | -72.7 | -51.01 | -67 |

Observation 3: the BS2UE MCL values are summarized as following:

Table 2. the summarized BS2UE MCL

|  | MCL |  |  |
| --- | --- | --- | --- |
| Grid shift | 99% | 99.965% | Baseline |
| 100% | -71 | -65.7 | -53 |

Observation 4:

If the 99% tile of MCL is considered for both BS2BS and BS2UE, then around 18dBc relaxation could be allowed.

If the 99.9% tile of MCL is considered for both BS2BS and BS2UE, then around 12dBc relaxation could be allowed.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2419797	Way Forward for [113][303] NR_BS_RF_Part2_CLTA

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP

- R4-2504769	Way Forward for Tx spurious emissions co-existence limits re-evaluation, Huawei, approved.

- R4-2504298   Huawei, HiSilicon

- R4-2504267,   Ericsson