3GPP TSG-RAN WG4 Meeting #115	R4-2507557

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.1.1

Source:	Moderator (Huawei)

Title:	Topic summary for [115][118] NR_ENDC_RF_Ph4_part3

Document for:	Information

Introduction

This topic summary document Rel-19 NR_ENDC_RF_Ph4-Core WI 6Rx UE topics:

- Topic #1: Reference sensitivity requirements (AI 7.1.2.3.1)

- Topic #2: MIMO layer evaluation for 6Rx UE (AI 7.1.2.3.2)

- Topic #3: SRS antenna switching requirements (AI 7.1.2.3.3)

- Topic #4: SRS IL imbalance (AI 7.1.2.3.4)

- Topic #5: feature list (R4-2505587)

| Topic title | WI | Topic areas | AI covered in the topic thread |
| --- | --- | --- | --- |
| [115][118] NR_ENDC_RF_Ph4_part3 | NR_ENDC_RF_Ph4-Core | 7.1.2.3 | 6Rx UE |

Topic #1: Reference sensitivity requirements

Companies’ contributions summary

The following tdocs submitted under AI 7.1.1.3.1/3 are handled in this summary document. The list of proposals are summarized in Annex.

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R4-2505586 | On leftover issues for 6Rx | Huawei, HiSilicon | 7.1.2.3.1 |
| R4-2505649 | Discussion on the usage of the terminology “FWA form factor” and its application for 6Rx, 4Rx and 8Rx requirements | Anritsu Limited | 7.1.2.3.1 |
| R4-2506867 | Discussion on general aspects of 6Rx requirements | vivo | 7.1.2.3.1 |
| R4-2506381 | Draft CR to TS38.101-1: Introduction of ?TRxSRS requirements for 6Rx | ZTE Corporation, Sanechips, Huawei, HiSilicon, vivo | 7.1.2.3.1 |
| R4-2505436 | Draft CR for TS 38.101-1 to introduce delta_TRxSRS requirements for 6Rx | Apple | 7.1.2.3.3 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: FWA terminology

Sub-topic description: The sub-topic covers one sub-topic for general part, i.e., FWA terminology

Agreements and open issues in the previous meeting

Issue 1-1-2: FWA terminology Agreement: Further discuss possible clarifications and improvements on the use of FWA terminology in TS 38.101-1

Issue 1-1: FWA terminology

- Proposals

  - Option 1: Unify the wording as “FWA form factor” for the notes in the corresponding tables for 4Rx, 6Rx and 8Rx. (Huawei)

  - Option 2: No update of wording in the specifications for the corresponding tables for 4Rx, 6Rx and 8Rx, but add the definition of CPE which is the same as that in LTE specification, i.e., CPE, Customer Premises Equipment. (Huawei)

  - Option 3: Remove “form factor” and add the specific list of UE types supported for the corresponding tables for 4Rx, 6Rx and 8Rx. (Anritsu)

    - Add the definition of “FWA UE” in sub-section 3.1, where CPE can be mentioned.

    - Add the definition of “Industrial (devices) UE” in sub-section 3.1.

  - Option 4: It is suggested to use ‘FWA form factor’ for 6Rx when applying ΔRIB,6R requirement for FWA, since only two UE types for 6Rx, i.e., handheld or FWA, are clearly stated in WID.

- Recommended WF

  - Remove “form factor” from the notes in the corresponding tables for 4Rx, 6Rx and 8Rx, and use “FWA UE” for ΔRIB,6R requirement in the note of the table.

    - Add the definition of FWA UE, CPE, and industrial (devices) UE in the specifications

Sub-topic 1-2: Draft CRs

Issue 1-2: Draft CRs

- Draft CRs

  - R4-2506381 (ZTE Corporation, Sanechips, Huawei, HiSilicon, vivo)

  - R4-2505436 (Apple)

- Recommended WF

  - Check whether the changes of R4-2506381 are agreeable except for the changes under section 6.2.4.

  - Further discuss the changes of R4-2506381 together with R4-2505436 under issue topic 3.

  - Revise R4-2506381 as the join contribution to capture all the agreements and mark R4-2505436 as “merged”.

Topic #2: MIMO layer evaluation for 6Rx UE

Companies’ contributions summary

The following tdocs submitted under AI 7.1.1.3.2/3/4 and AI 7.1.4 are handled in this summary document. The list of proposals are summarized in Annex.

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R4-2506379 | On release independent for 6Rx RF requirement | ZTE Corporation, Sanechips | 7.1.2.3.1 |
| R4-2506867 | Discussion on general aspects of 6Rx requirements | vivo | 7.1.2.3.1 |
| R4-2505384 | Discussion on remaining issues on 6 MIMO layer evaluation for 6Rx UE | China Telecom | 7.1.2.3.2 |
| R4-2505853 | On Core Requirements for 6Rx UEs | Apple | 7.1.2.3.2 |
| R4-2506222 | Discussions on release independence for 6Rx devices | QUALCOMM Europe Inc. - Spain | 7.1.2.3.2 |
| R4-2506917 | R19 release independent of 6Rx | OPPO | 7.1.2.3.2 |
| R4-2507528 | Views on remaining issues for 6Rx UE | Samsung | 7.1.2.3.2 |
| R4-2506918 | R19 release independent of 3T6R SRS antenna switching | OPPO | 7.1.2.3.3 |
| R4-2505437 | On release independence for 6Rx RF requirements | Apple | 7.1.2.3.4 |
| R4-2505499 | Discussion on 6 Layer MIMO Specification Impact | Nokia | 7.1.2.3.4 |
| R4-2507030 | Discussion on 6Rx UE performance requirements | Huawei, HiSilicon | 7.1.4 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: Release independency for 6Rx UE (being related to MIMO layer)

Sub-topic description: The sub-topic covers aspects related to 6Rx UE release independency.

Agreements and open issues in the previous meeting

Issue 1-1-1: Release independence of 6Rx RF requirements Agreement: FFS whether to define 6Rx RF requirements in a release independent manner

Issue 2-1: Release independency for 6Rx UE

- Proposals

  - Option 1: Apply all the 6Rx requirements to both handheld UE and FWA UE from the earlier release than Rel-19

    - Option 1a: from Rel-17 (ZTE)

    - Option 1b: from Rel-18 (Qualcomm)

  - Option 2: Apply the features/requirements related to 6Rx from different releases

    - Option 2a: (OPPO)

      - For 6Layer, due to new signaling introduced, apply it at least from Rel-19, whether can support from earlier releases (e.g. Rel-18) needs to check with RAN2.

      - For 6Rx with 4Layer+2Div,

        - if new signaling is introduced it can be applied from Rel-19, whether can support from earlier releases (e.g. Rel-18) needs to check with RAN2;

        - if no new signaling is introduced then no restriction on the applicable release and can be supported from Rel-15.

      - The 3T6R SRS antenna switching is supported from Rel-19.

    - Option 2b: (Apple)

      - 6Rx RF requirements for handheld UEs can be release independent from Rel-19

      - 6Rx RF requirements for FWA UEs can be release independent from Rel-18

      - Do not define 6Rx UE (demodulation requirements) with 4 and 6 MIMO layers as a release independent feature.

  - Option 3: Apply 6Rx RF and demodulation requirements from Rel-19, i.e., no release independency applies for 6Rx UE requirements (vivo, Samsung,  Huawei)

- Recommended WF

  - Companies’ views are diverge. Please provide the feedback or clarifications for the following reasons behind the above options before discussing the potential compromise.

    - The reasons behind Option 1 from companies supportive to release independency

      - RF requirements for 8Rx UE is release independent from Rel-17.

      - It is not harmful that this 6Rx feature for both handheld UE and FWA can be release independent from Rel-17.

      - Due to the srs-AntennaSwitchingBeyond4RX-r17 capability is an optional capability, so we think it should be decoupled with 6Rx feature although ΔTSRSRx requirements for t3r6 are defined. Of course, the release independent for t3r6 cannot be earlier than the release which 6Rx feature can be release independent from.

    - The reasons behind Option 2b

      - For 6Rx REFSENS, RAN4 has agreed to have separate requirements for handheld UEs and FWA UEs.

      - The ΔTRxSRS requirements for 6Rx REFSENS include t3r6 antenna configuration. Single-carrier 3Tx is a Rel-19 feature for handheld UEs while 4Tx is a Rel-18 feature for FWA UEs. (?)

    - The reasons behind Option 3 from companies not in favour of the release independency for 6Rx UE

      - For 6Rx UE requirements, new capability for maximum 6 MIMO layer will be introduced by RAN3, which would be only used from Rel-19 (Potential NBC issue if release independent)

      - 6Rx RF requirements mainly include 6Rx REFSENS and SRS-AS IL ΔTRxSRS. The ΔTRxSRS requirement involves many new UE capabilities, e.g. t3r6 SRS antenna switching, which are introduced from R19.

      - Other reasons like “there is no evidence showing that handheld UEs are widely equipped with 6Rx at early release”, “too aggressive.

  - Moderator observations:

    - There is a package of features and requirements for 6Rx UE.

    - It seems that companies might be on the same page that at least the sub-features of 6 MIMO layer and the ΔTSRSRx requirements for t3r6 are the Rel-19 new features which could not be release independent from the earlier release than Rel-19.

Sub-topic 2-2: MIMO layers for 6Rx UE

Issue 2-2: MIMO layer related

- Proposals (Nokia)

  - Observation 1: 8 Layer MIMO is already intrinsically defined for FWA-type devices only.

  - Observation 2: For other MIMO layer definitions in 38.306 or 38.331, it is not defined whether the UE is handheld or FWA.

- Recommended WF

  - Can the proponent clarify what agreements they want to reach by proposing observation 1 and 2 in RAN4?

Topic #3: SRS antenna switching requirements

Companies’ contributions summary

The following tdocs submitted under AI 7.1.1.3.3/1 are handled in this summary document. The list of proposals are summarized in Annex.

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R4-2505436 | Draft CR for TS 38.101-1 to introduce delta_TRxSRS requirements for 6Rx | Apple | 7.1.2.3.3 |
| R4-2506381 | Draft CR to TS38.101-1: Introduction of ?TRxSRS requirements for 6Rx | ZTE Corporation, Sanechips, Huawei, HiSilicon, vivo | 7.1.2.3.1 |
| R4-2506868 | Further discussion on SRS antenna switching requirements for 6Rx | vivo | 7.1.2.3.3 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: UE capability signalling related to 3t6r

Sub-topic description: The sub-topic covers aspects relate to UE 3t6r capability

Agreements and open issues in the previous meeting

Issue 3-1-1: UE capability signalling updates for SRS antenna switching Agreement - Send LS to RAN2 to introduce UE capability signalling for t3r6 SRS antenna switching after the release independent for 3Tx in single band and 6Rx is clear.

Issue 3-1: UE capability signalling related to SRS antenna switching 3t6r

- Proposal (vivo)

  - Proposal 1: It is not necessary for RAN4 to send LS to RAN2 to introduce UE capability signalling for t3r6 SRS antenna switching.

- Recommended WF

  - Make conclusion as vivo’s proposal below, since RAN1 has already sent the LS [2] on UE feature list to RAN2 and asked them to design corresponding UE capability. 3T6R Antenna Switching was introduced in NR_MIMO_Ph5.

    - It is not necessary for RAN4 to send LS to RAN2 to introduce UE capability signalling for t3r6 SRS antenna switching.

Sub-topic 3-2: Draft CRs

Issue 3-2: Draft CRs

- Draft CRs

  - R4-2506381 (ZTE Corporation, Sanechips, Huawei, HiSilicon, vivo)

  - R4-2505436 (Apple)

- Proposal (vivo)

  - Proposal 2: Put brackets on the condition when the t3r6 SRS-AS IL requirement applies when preparing CR for 6Rx RF requirements.

- Recommended WF

  - Check if RAN4 can agree on

    - Put brackets on the condition when the t3r6 SRS-AS IL requirement applies when preparing CR for 6Rx RF requirements.

  - If no other comments, merge 5436 to 6381 and have further offline discussions on the wording of draft CR.

Topic #4: SRS IL imbalance

Companies’ contributions summary

The following tdocs submitted under AI 7.1.1.3.4 are handled in this summary document. The list of proposals are summarized in Annex.

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R4-2505438 | On SRS Antenna Switching IL Imbalance Solution for 6Rx | Apple | 7.1.2.3.4 |
| R4-2505587 | On SRS IL imbalance issue | Huawei, HiSilicon | 7.1.2.3.4 |
| R4-2505674 | UE assistance for SRS imbalance solution considerations | Nokia | 7.1.2.3.4 |
| R4-2506226 | Discussion on remaining issues on SRS IL imbalance | China Telecom | 7.1.2.3.4 |
| R4-2506380 | On 6Rx SRS antenna IL | ZTE Corporation, Sanechips | 7.1.2.3.4 |
| R4-2506865 | Discussion on SRS IL imbalance reporting | vivo | 7.1.2.3.4 |
| R4-2506888 | Views on SRS insertion loss compensation and reporting enhancements | Intel Corporation | 7.1.2.3.4 |
| R4-2506916 | R19 SRS IL indication | OPPO | 7.1.2.3.4 |
| R4-2506998 | Discussion on SRS IL imbalance for 6Rx UE | Google Korea LLC | 7.1.2.3.4 |
| R4-2507073 | SRS IL imbalance indication | Qualcomm France | 7.1.2.3.4 |
| R4-2507529 | Views on SRS IL imbalance issue | Samsung | 7.1.2.3.4 |
| R4-2507772 | SRS Power Imbalance Handling and PHR Reporting | Lenovo | 7.1.2.3.4 |
| R4-2507782 | On the SRS IL imbalance reporting for the SRS-AS | Ericsson | 7.1.2.3.4 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1: Solutions for SRS IL imbalance issue

Sub-topic description: The sub-topic covers aspects related to solutions for SRS IL imbalance issue

Agreements and open issues in the previous meeting

Way forward - RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2. - Companies are encouraged to provide views on additional aspects including - SRS output power testing - Enhancements to PCmax equations to improve SRS IL compensation Issue 4-2-1: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behavior for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behavior are needed - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications) Issue 4-2-2: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

Issue 4-1-1: SRS IL imbalance UE assistance and reporting

- Proposals:

  - Option 1: follow Solution 1 “Introduce UE capability indication on whether UE performs SRS IL self-compensation” with necessary updates (Apple, Huawei, ZTE

    - Option 1a: Further clarifications on the candidate solution 1 (Huawei)

      - UE is allowed to indicate its capability on handling SRS IL, which includes two states and the UE shall not indicate them simultaneously

        - State 1: the applied ∆TRxSRS shall be smaller than the respective minimum requirements

        - State 2: the applied ∆TRxSRS equals to the respective minimum requirements

  - Option 2: follow Solution 2 “Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility” with necessary updates or changes (Ericsson, Nokia)

    - Option 2a: (Ericsson)

      - The following SRS IL imbalance reporting solution should be specified:

        - UE provides dynamic assistance to the network on SRS IL compensation:

          - Per SRS resource power headroom reporting

          - Configured maximum output power per SRS resource reporting

          - ΔPPowerClass reporting

        - Note: UE performs self-compensation of SRS IL up to its configured maximum output power (as in 38.213)

      - The PH reporting with the proposed MAC-CE could be periodic (triggered by the expiration of timers, including a “prohibit” timer” to reduce reporting frequency) or by any modification of the SRS configuration.

    - Option 2b: For UE SRS imbalance assistance, UE should report network only SRS resource specific PCMAX_SRS,f,c(i) (Nokia)

  - Option 3: Need both Solution 1 and Solution 2 (China Telecom)

    - Option 4a: Consider introducing UE indication and Type 3 PHR together for SRS IL imbalance improvement (China Telecom)

      - The indication should be dynamic and triggered when the UE performing self-compensation.

      - Introduce a new IE to convey the indication from UE to NW

      - Inform RAN2 to add Type 3 PHR to single cell scenario.

      - Send LS to RAN1 and ask whether Type 3 PHR could be enhanced for PUSCH cell in this feature while controlled by signaling.

  - Option 5: Neither Solution 1 nor Solution 2 is preferred in Rel-19 basically (vivo, Samsung, Google, Qualcomm

    - Option 5a: It is proposed not to introduce any solution for SRS-IL imbalance issue for 6Rx UE in Rel-19 (Google, Qualcomm)

    - Option 5b: But solution 1 can be considered as a compromise, if quick agreement can be reached. The additional solution can be considered in 6G. (vivo, Samsung)

  - Option 6: Report UE specific SRS IL values with capability signalling, which can be called as Solution 3 (Intel, OPPO,

    - Option 6a: Agree on the following UE assistance and reporting framework (Intel)

      - Introduce UE capability indication on whether UE performs SRS IL self-compensation

      - Introduce semi-static reporting of UE-specific SRS IL value

      - Extend Type 3 SRS PHR reporting to carriers with PUSCH

    - Option 6b:  (OPPO)

      - UE compensation issue and SRS IL changes due to hand blocking need to be considered if SRS IL reporting scheme is pursued in RAN4.

      - If SRS IL is to be reported, dynamic reporting shall be considered

      - To simplify dynamic reporting complexity, gNB request based SRS IL reporting might be a way to go. For example, before gNB to do the SRS IL compensation in channel estimation, it asks UE to report which SRS IL is applied.

    - Option 6c: (Lenovo)

      - In order to assist the gNB in correctly estimating the downlink channel from the received uplink SRS transmissions, the UE should indicate whether or not it UE compensates for the SRS insertion losses at power levels below PCMAX,f,c

      - The UE should signal the actual insertion loss differences between the SRS port with the least insertion loss and the remaining SRS ports.  That is, if there are N SRS ports and port 1 is defined as the SRS port with the smallest insertion loss, then the UE should signal

δ1,j=∆TRxSRS,jA-∆TRxSRS,1A  for  2≤j≤N .

      - If the UE compensates SRS insertion losses, and if the power transmitted on SRS port 1 pt,1 satisfies

pt,1>miniPCMAX,f,ci ,

the transmit power on at least one SRS ports will be less than the transmit power on the first SRS port.  In this case, the UE sends a PHR report only for port 1 that includes the optional field configuredMaxTxPower. From this transmit power for an arbitrary SRS port j can be determined as

pt,j=pt,1pt,1≤PCMAX,f,c1-δ1,jPCMAX,f,c1-δ1,jpt,1>PCMAX,f,c1-δ1,j .

otherwise, if

pt,1≤miniPCMAX,f,ci ,

the power transmitted on the SRS ports is equal, and no power headroom report is needed.

    - Option 6d (ZTE)

      - Introduce UE capability indication on whether UE performs SRS IL self-compensation, and it should be per SRS resource reporting.

      - If dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern is considered, and several thresholds associated with capability class for the actual SRS IL reporting can be considered.

- Recommended WF

  - Suggest to drop Option 6 as well as Option6x following the previous agreements which has narrowed down to two “may” solutions, unless there would be a quick agreement on the direction of “Solution 3”.

    - Arguments from Ericsson

      - In general, the Tx-Rx imbalance is unpredictable and, in most cases, non-reciprocal. In most cases, the Tx imbalance is larger than the Rx imbalance, even though there are some UEs in the field for which the imbalance is reciprocal.

      - There would be a very limited benefit from reporting of the actual Tx insertion losses – the reporting of the SRS output power would be more beneficial, where the benefit of such reporting is the largest when the Tx imbalance is much larger than the Rx imbalance.

  - Focus on options based on Solution 1, Solution 2 and both

    - The proponents of Solution 1 should reply the following arguments against Solution 1:

      - Argument #1 (vivo):

        - Having expectation that Rel-19 UE would do compensation for SRS IL is already a large step for UE. (clarification of UE behaviour seems enough)

      - Argument #2 (Qualcomm, Ericsson): Self-compensation being mandatory

        - Adding a UE capability to indicate whether UE performs self-compensation or not was proposed by some companies. It would be very weird to add such capability indication without asking RAN1 whether TS38.213 mandates UE self-compensation or not; If it does, then there is no point adding the bit as UE’s not indicating that would be incompliant as per RAN1. If it does not, then RAN4 assumption in 4-2-1 was wrong and all things should be re-considered. (Qualcomm)

        - Solution 1 does not represent an acceptable compromise since it proposes to introduce UE capability indication on whether the UE performs SRS IL self-compensation, a behavior which is already mandatory according to the RAN1 specifications. (Ericsson)

      - Argument #3 (Google) Factory RF calibration, OTA impacts

        - Since RAN4 assumptions on Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power for SRS transmission, SRS-IL imbalance can be self-compensated well by factory RF calibration process from conductive perspective.

        - Also from OTA perspective, SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance.

      - Argument #4 (Samsung) unclear benefit for network

        - Solution 1 still remains unclear how the network can get the benefit from the UE indication.

        - Additional UE capability indication for the network with UE self-compensation would not help improving the downlink channel estimation from the legacy SRS IL imbalance issue.

    - The proponents of Solution 2 should reply the following arguments against Solution 2:

      - Argument #1: (Huawei) Triggering event & SRS-Tx mapping changes

        - The trigger event for reusing PHR type 3 to resolve SRS IL reporting is hardly to be defined without specification impact because at least following factors should be considered:

          - The timing (or duration) whenever partial or all antenna ports for AS-SRS transmission become MOP limited

          - Changing on the mapping between SRS port and Tx chain due to e.g. hand blocking

          - The specification impact for defining the trigger event under candidate solution 2 i.e. reusing PHR type 3 shall be clarified.

      - Argument #2 (Qualcomm, vivo) Complexity, no simulation showing gains

        - Proposed SRS IL indication scheme would not work for all cases.

        - Proposed SRS IL indication scheme would introduce quite a bit of extra complexity.

        - Even for Solution 1, whether UE performs SRS IL self-compensation may be still controversial in details. More complicated “Solution 2” cannot be accepted by most companies in previous discussion.

        - There have been no simulation results to show system level gains for the cases where UE compensates when it has power to do so and does not compensate when there is no power remaining.

      - Argument #3 (Google) Factory RF calibration, OTA impacts

        - Since RAN4 assumptions on Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power for SRS transmission, SRS-IL imbalance can be self-compensated well by factory RF calibration process from conductive perspective.

        - Also from OTA perspective, SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance.

      - Argument #4 (Samsung)  no RAN1 impact

        - RAN4 agreed to perform UE self-compensation of SRS IL up to its configured maximum output power for SRS transmissions without RAN1/4 specification impacts.

        - Solution 2 to change Type 3 PHR triggering condition is nothing to do with the UE self-compensation, but has RAN1 specification impact, which is totally up to RAN1 discussion.

  - Further discussions during the meeting. If no consensus was reached in this meeting, it would be suggested to go with Option 5a

    - It is proposed not to introduce any solution for SRS-IL imbalance issue for 6Rx UE in Rel-19, since there is no consensus.

Issue 4-1-2: Clarifications or updates related to SRS IL imbalance UE self-compensation

- Proposals:

  - Option 1: Clarify that UE self-compensation is conducted on the condition that UE has the power headroom relative to the configured maximum Tx power (China Telecom, ZTE, Intel, Lenovo)

    - China Telecom’s Proposal2: If the self-compensation is totally determined at UE side, it should be triggered only if the actual PCMAX,f,c with IL influence is smaller than the necessary SRS power.

    - ZTE’s Proposal 1: UE SRS IL power compensation is up to UE implementation and optional, and if UE hasn’t reached its Tx power upper limit, UE should indicate the power compensation behaviour to NW once the NW requests such information.

    - Intel’s Proposal #1: Specify solution for UE to perform SRS IL pre-compensation based on the following principles

      - For SRS transmissions Rel-19 UE shall perform self-compensation of SRS IL up to its configured maximum output power

        - UE shall perform full or partial self-compensation of the SRS IL on every SRS resource (SRS antenna port) whenever it has power headroom relative to the configured maximum Tx power.

        - The compensation shall be performed for any SRS transmission power level in TS 38.213.

        - No RAN1/4 specification impacts

    - Lenovo’s observations:

      - Observation 1: Since the power control equation for the SRS port does not allow transmit power relaxations below the configured maximum power PCMAX,f,c(i), the UE should compensate the SRS transmit power for the insertion loss ∆TRxSRS at power levels below PCMAX,f,c(i).

      - Observation 2: At some future point in time, it would be beneficial to define a requirement to ensure that the UE compensates for the SRS insertion losses at power levels below PCMAX,f,c(i).

  - Option 2: No clarification is needed for UE self-compensation (Qualcomm, Ericsson)

    - Qualcomm’s Proposal 1: No clarifications/changes to RAN4 specification related to UE self-compensation.

    - Ericsson’s Proposal 4: Since the UE shall perform self-compensation of the SRS IL according to the specifications, no additional clarifications on behavior are needed.

- Recommended WF

  - Companies should align the views on the specifications of 38.213 and 38.101-1.

  - Moderator’s observations: as shown in the figure below, PSRS,b,f,c applies on the reference plan where the insertion loss should be taken into account. To reach PSRS,b,f,c, UE seems have already done the proper compensation for the additional path loss for diversity path compared to main Tx chain path already. The question is whether PCMAX,f,c is also defined on the reference plane. If so, both PSRS,b,f,c and PCMAX,f,c cover the insertion loss. Not sure if the condition is needed to be added since the equation of 38.213 has already reflects option 1.

Issue 4-1-3: SRS output power testing

- Proposals:

  - Option 1: Define the test to verify UE performance of power self-compensation (Apple, Intel)

    - Apple’s Proposal3: The following test procedure is proposed to determine if the UE did indeed perform power self-compensation on each SRS port and how effective the compensation was:

      - The UE is configured for SRS antenna switching

      - The UE is configured for CSI reporting – CRI-RI-CQI

      - The TE measures SRS and reconstructs the full channel matrix

      - The TE determines the rank and CQI of the reconstructed channel

      - If the UE has effectively pre-compensated the IL, the reciprocity based CQI and RI will be within certain margin of the UE reported CQI and RI

    - Intel’s proposal 2: Introduce SRS output power testing framework to guarantee proper UE behavior based on existing specifications at least for PSRS < PCMAX.

  - Option 2: Introduce the SRS output power testing with proper UL RMC to ensure the performance of reciprocity based DL MIMO

    - Nokia’s Proposal 2 and 3:

      - Proposal 2: UL RMCs shall be added to TS 38.101-1 to verify SRS resource power at each applicable antenna when SRS power is not limited by PCMAX_SRS,f,c.

      - Proposal 3: UL RMCs shall be added to TS 38.101-1 to verify PCMAX_SRS,f,c(i) and PHSRSi at each applicable antenna are correctly reported.

    - Qualcomm’s Proposal 3: Discuss whether UL RMC’s to verify SRS power at least at max power level can be added

      - Whether UL RMC’s to verify SRS power at least at max power level are added

      - If SRS power were tested below max power, should RAN4 and RAN1 view on UE compensation before doing that?

      - From which release, if any should UL RMC’s be added to verify SRS power

    - Ericsson’s proposal 3 and observation 9:

      - Observation 9: Testing of the SRS output power is important for the performance of the reciprocity-based DL MIMO and it should be introduced in addition to the IL imbalance reporting as it doesn’t solve the same issue.

      - Proposal 3: Introduce the SRS output power testing in order to improve the performance of the reciprocity-based DL MIMO.

- Recommended WF

  - If Option 2 is acceptable in issue 4-1-2, then Option 1 in this issue can be dropped and Option 2 in this issue can be further discussed.

Issue 4-1-4: Enhancement to Pcmax equations to improve SRS IL compensation

- Proposals:

  - Option 1: Replace ∆TRxSRS by ∆TRxSRS,UE in the equation of PCMAX_L,f,c (Intel)

    - Intel’s Proposal4: TAdjust the PCMAX_L,f,c equations to include UE-specific SRS-IL component:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

  - Option 2: No changes for equation of PCMAX_L,f,c(Ericsson)

    - Ericsson’s Observation 10: Modifying the PCMAX lower bound by considering the UE-specific (or SRS port specific) SRS IL value would not solve any issue as the UE still has a freedom to configure PCMAX anywhere between the lower and the upper bound. In addition, it would make the current ∆TRxSRS requirements not needed anymore, and large specification changes would have to be made.

- Recommended WF

  - In moderator’s view, option 2 would be reasonable.

Sub-topic 4-2: Other issues related to SRS IL imbalance issue

Sub-topic description: The sub-topic covers aspects related to general consideration in the last meeting

Agreements and open issues in the previous meeting

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way forward: - No further discussion on the issue in future meetings and focus on identification of possible SRS IL imbalance issue solutions. - NOTE: This does not mean that any solution for SRS IL imbalance issue is specified in Rel-19 Issue 4-1-2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx Agreement - FFS on the potential SRS IL solution applicability to 2Rx/4Rx/8Rx UEs in addition to 6Rx UEs

Issue 4-2: SRS IL solution applicability to 2Rx/4Rx/6Rx/8Rx

- Proposals:

  - Option 1: Apply SRS IL solution to 2Rx/4Rx/6Rx/8Rx, if the solution is agreeable. (Apple, ZTE, Intel, Ericsson)

    - Apple’s Proposal2: Solution 1 can also be used to address the SRS IL imbalance issue for 2Rx/4Rx/8Rx

    - ZTE’s Proposal 4. The SRS IL imbalance reporting mechanism in 6Rx discussion should be also specified for 2Rx, 4Rx and 8Rx

    - Intel’s Proposal #5: Introduce SRS IL compensation mechanisms as an optional feature in a generic manner for 2Rx/4Rx/6Rx/8Rx UEs.

    - Ericsson’s Proposal 2: The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases.

  - Option 2: Preclude 2Rx or make applicability to 2Rx and 4Rx be optional. (Huawei, vivo)

    - Huawei’s Observation 5: 2Rx can be precluded since it should have the least imbalance thus the expected gain from network perspective is relatively negligible.

    - Vivo’s Proposal 3: The potential SRS IL solution (including implementation only) can apply to 6Rx and 8Rx. 4Rx and 2Rx need not be mandatory.

- Recommended WF

  - Option 1, since more companies support it.

Topic #5: Feature list for 6Rx UE

Companies’ contributions summary

The following tdoc is handled in this summary document submitted under AI 7.1.1.3.4.

| TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- |
| R4-2505587 | On SRS IL imbalance issue | Huawei, HiSilicon | 7.1.2.3.4 |

Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 5-1: Feature list for 6Rx UE

Sub-topic description: The sub-topic covers one sub-topic for feature list.

Issue 5-1: Feature list for 6Rx UE

- Proposals: (Huawei)

  - UE feature list for 6Rx

    - For 6Rx WI, we propose to have following UE features based on RAN4 discussions so far.

    - Proposal 3: Adopt the following UE feature list for Rel-19 WI NR_ENDC_RF_Ph4 objective 6Rx.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 46-6 | Support maximum 6 MIMO layers for DL reception | Indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA. |  | Yes | N/A | UE does not support maximum 6 MIMO layers for DL reception | Per FSPC | N/A | Yes (only FR1) | N/A |  | Optional |
|  | 46-7 | SRS antenna switching for 6Rx | Indicates supporting of t3r6 SRS antenna switching. |  | Yes | N/A | UE does not support t3r6 SRS antenna switching | Per BC | N/A | Yes (only FR1) | N/A | This capability cannot be indicated simultaneously with Tx diversity | Optional |
|  | 46-8 | [SRS IL imbalance compensation] | Indicates whether UE performs SRS IL self-compensation | 2-55 or 23-8-3 | Yes | N/A |  | Per BC | N/A | Yes (only FR1) | N/A |  | Optional |

Annex: list of proposals

Proposals for topic #1

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505586 | Huawei | Proposal 1: Suggest RAN4 to clarify whether “FWA form factor” and “FWA/CPE/Vehicle/Industrial devices form factor” have identical interpretation from RF requirement perspective. Proposal 2: RAN4 considers following two alternatives for better specification clarity. • Alt 1. Unify the wording as “FWA form factor” if no disagreement. • Alt 2. No need to unify the spec language but reproduce the definition of CPE from LTE spec. - CPE, Customer Premises Equipment. |
| R4-2505649 | Anritsu Limited | Proposal 1: Remove any use of the term “form factor” in TS 38.101-1 and replace it by the intended UE types. Only the usage of “form factor” is generic manner should be allowed like for example in “Vehicular UE does not refer to other UE form factors placed inside the vehicle.”. Proposal 2: Add the definition of “FWA UE” in sub-section 3.1, where CPE can be mentioned. Proposal 3: Add the definition of “Industrial (devices) UE” in sub-section 3.1. Proposal 4: For each NOTE 1 in the three tables: Table 7.3.2-2 (4Rx), Table 7.3.2-2a (8Rx) and the to-be-added Table 7.3.2-2b (6Rx), clearly give for respectively 4 Rx, 6 Rx and 8 Rx the specific list of UE types that do support them. |
| R4-2506867 | vivo | Unlike 8Rx, we only consider two UE types for 6Rx, Handheld or FWA, which is clearly stated in the WID. Proposal 2: It is suggested to use ‘FWA form factor’ for 6Rx when applying ΔRIB,6R requirement for FWA. |
| R4-2506381 | ZTE Corporation, Sanechips, Huawei, HiSilicon, vivo | Draft CR to TS 38.101-1 Reason for change: The ∆TRxSRS requirements for 6Rx are agreed in the WF R4-2503014, and the draft CR for the Rx related part have already been endorsed in R4-2417094. In addition, there is no consensus for PC1.5 3Tx in RAN4 in Rel-19 according to the WF R4-2505111. Summary of change: To introduce the RF requirements for 6Rx. |
| R4-2505436 | Apple | Reason for change: To capture the recently agreed requirements of ∆TRxSRS for 6Rx into the corresponding parts of the TS38.101-1 specifications. The values of ∆TRxSRS for 6Rx were agreeed in RAN4 Meeting#114 as captured in the WF R4-2503014. The table below, taken from the WF, summarizes the values for the different operating bands and for the different SRS antenna configurations |

Proposals for topic #2

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2506379 | ZTE | For 6Rx, similar as 8Rx, where RF requirements for 8Rx UE is release independent from Rel-17, we think it will be no harmful that this 6Rx feature for both handheld UE and FWA can be release independent from Rel-17, and only 4-layer is applicable for handheld UE performance requirements. Regarding the SRS antenna switching, currently t1r6, t2r6 are already included in srs-AntennaSwitchingBeyond4RX-r17 capability, and capability for t3r6 is under discussing. Due to the srs-AntennaSwitchingBeyond4RX-r17 capability is an optional capability, so we think it should be decoupled with 6Rx feature although ΔTSRSRx requirements for t3r6 are defined. Of course, the release independent for t3r6 can not be earlier than the release which 6Rx feature can be release independent from. Proposal: Release independent for 6Rx RF requirement for both handheld UE and FWA can be from Rel-17. |
| R4-2506867 | vivo | 6Rx RF requirements mainly include 6Rx REFSENS and SRS-AS IL ΔTRxSRS. The ΔTRxSRS requirement involves many new UE capabilities, e.g. t3r6 SRS antenna switching, which are introduced from R19. If release independence from earlier releases is defined, there will be forward compatibility issues; this is one reason why release independence is not recommended. Proposal 1：It is suggested 6Rx RF requirements to be release independent from Rel-19. |
| R4-2505853 | Apple | In our view, 6Rx UE is a Release 19 feature and should be treated as such. The implementation of SU-MIMO is already quite involved in terms of UE architecture and implementation, especially in RF front end, channel estimation, MIMO detection, and in 2 CW processing at the LDPC decoder stage. Hence, we prefer to maintain the working assumption that the feature is specific to Release 19 and beyond. Observation #1: The implementation of 6Rx UE is quite involved in terms of UE architecture and modem implementation in comparison to 4Rx. Proposal #1: Do not define 6Rx UE with 4 and 6 MIMO layers as a release independent feature. |
| R4-2506222 | Qualcomm | Although the signaling support is intended for Rel-19, it could be early implementable and may allow support from previous releases. Therefore, we propose that both RF and demodulation requirements for 6Rx devices be release-independent from Rel-18. Observation 1: Although the signaling support for 6Rx devices is intended for Rel-19, it could be early implementable, allowing support from previous releases. Proposal 1: Both RF and demodulation requirements for 6Rx devices shall be release-independent from Rel-18. |
| R4-2506917 | OPPO | Basically, when new signalling is introduced, it needs to be careful to be release independent from earlier releases due to the inter-operation problems that may confront for earlier BS/UE. Therefore, in our view, 6Layer can be applied since Rel-19 at least, we are also supportive to apply it to earlier releases like Rel-18 to benefit the industry if feasible from RAN2 point view. Proposal 1: For 6Layer, due to new signaling introduced, apply it at least from Rel-19, whether can support from earlier releases (e.g. Rel-18) needs to check with RAN2. For 4Layer + 2Div, this can already be supported from MIMO layer point of view today. In terms of defining new signalling to differentiate UE supporting 4Layer and UE supporting 4Layer+2Div, if is introduced, then this will be in as similar situation of 6Layer. Proposal 2: For 6Rx with 4Layer+2Div, - if new signaling is introduced it can be applied from Rel-19, whether can support from earlier releases (e.g. Rel-18) needs to check with RAN2; - if no new signaling is introduced then no restriction on the applicable release and can be supported from Rel-15. |
| R4-2507528 | Samsung | Observation 1: 8Rx reused the legacy MIMO layer capability, i.e., maxNumberMIMO-LayersPDSCH, with MIMO-LayersDL ENUMERATED values {2, 4, and 8} which allowed release independent from Rel-17. Observation 2: New capability to enable 6 MIMO layers will be introduced by RAN2, which would be only from Rel-19. Proposal 1: In order to avoid any unnatural situation, 6Rx requirements should not be applied in a release independent manner. |
| R4-2505437 | Apple | Observation #1: For 6Rx REFSENS, RAN4 has agreed to have separate requirements for handheld UEs and FWA UEs. Observation #2: The ΔTRxSRS requirements for 6Rx REFSENS include t3r6 antenna configuration. Single-carrier 3Tx is a Rel-19 feature for handheld UEs while 4Tx is a Rel-18 feature for FWA UEs. Proposal #1: We can make the following recommendations for 6Rx release independence: - 6Rx RF requirements for handheld UEs can be release independent from Rel-19 - 6Rx RF requirements for FWA UEs can be release independent from Rel-18 |
| R4-2505499 | Nokia | The following Observations and Proposals were made: Observation 1: 8 Layer MIMO is already intrinsically defined for FWA-type devices only. Observation 2: For other MIMO layer definitions in 38.306 or 38.331, it is not defined whether the UE is handheld or FWA. |
| R4-2507030 | Huawei | It’s proposed not to introduce release independence for 6Rx requirements, since 6Rx requirements target at handheld UE, which is commonly equipped with 4Rx at early release, there is no evidence showing that handheld UEs are widely equipped with 6Rx at early release. It is aggressive to introduce release independence for 6Rx requirements for handheld UE. Furthermore, regarding 6 MIMO layers, there are inter-operation issues between UE and BS to support such feature, BS needs to support the new UE capability and change the scheduling strategy by taking into max DL 6 MIMO layers on top of the current scheduling. Proposal 1: No release independent for 6Rx requirements and applicable from Rel-19. |
| R4-2506918 | OPPO | Regarding 3T6R SRS antenna switching, it depends on 3Tx, 6Rx and also related SRS antenna switching configuration. - One point needs to be clarified is whether 6Rx here is restricted to 6Layer or 4Layer+2Div also apply. In our view, 3T6R SRS antenna switching can also be applied to UE supporting 4Layer+2Div. And if new signalling is introduced, it can be applied from Rel-19, whether can support from earlier releases (e.g. Rel-18) needs to check with RAN2. If no new signalling is introduced then no restriction on the applicable release and can be supported from Rel-15. - Another point is the 3Tx here, does it restricted to 3Layer or less than 3Layer can also be used in this case. In our view, 3T6R can be applied to UE supports 3Tx antennas but less than 3Layers which is similar as the 6Rx. - Finally, in RAN1 the 3T6R SRS antenna switching related configurations are defined in Rel-19 MIMO WI. It seems this cannot be applied in earlier releases. Therefore, if 3T6R SRS antenna switching configurations defined in RAN1 can only be applied in Rel-19, then the applicable release should be no earlier than Rel-19. Proposal: The 3T6R SRS antenna switching is supported from Rel-19. |

Proposals for topic #3

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505436 | Apple | Reason for change: To capture the recently agreed requirements of ∆TRxSRS for 6Rx into the corresponding parts of the TS38.101-1 specifications. Summary of change: The values of ∆TRxSRS for 6Rx were agreeed in RAN4 Meeting#114 as captured in the WF R4-2503014. The table below, taken from the WF, summarizes the values for the different operating bands and for the different SRS antenna configurations |
| R4-2506868 | vivo | However, it is not necessary for RAN4 to send the LS to introduce UE capability for t3r6. In Wuhan meeting, RAN1 has already sent the LS [2] on UE feature list to RAN2 and asked them to design corresponding UE capability. 3T6R Antenna Switching was introduced in NR_MIMO_Ph5. Proposal 1: It is not necessary for RAN4 to send LS to RAN2 to introduce UE capability signalling for t3r6 SRS antenna switching. In the last meeting, we tried to give the wording for condition when t1r6, t2r6, t3r6 SRS-AS IL requirements apply in our draft CR. The part for t3r6 needs clear conclusion from RAN1 and RAN2. h) UE transmits SRS on the second, third, fourth, fifth, sixth SRS resources of the total 6 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't1r6', or i) UE transmits SRS from the SRS port pair on the second, third SRS resource in every configured SRS resource set consisting of two SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't2r6', or [j) UE transmits SRS from the set of SRS ports on the second, third SRS resource in every configured SRS resource set consisting of three SRS ports when the srs-AntennaSwitchingBeyond4RX-r17 capability is indicated as at least 't3r6',] For the indication of ‘t3r6’, it can be different with ‘t1r6’ or ‘t2r6’ since the support of t3r6 SRS Tx port switching is based on 4Tx port with one port disable as described in the feature list. The indication of t3r6 can wait the conclusion in RAN1. The way to capture this condition whenΔTRxSRS for t3r6 is applicable in RAN4 can wait for the conclusion in RAN1. Proposal 2: Put brackets on the condition when the t3r6 SRS-AS IL requirement applies when preparing CR for 6Rx RF requirements. |
| R4-2506381 | ZTE Corporation, Sanechips, Huawei, HiSilicon, vivo | Draft CR to TS 38.101-1 Reason for change: The ∆TRxSRS requirements for 6Rx are agreed in the WF R4-2503014, and the draft CR for the Rx related part have already been endorsed in R4-2417094. In addition, there is no consensus for PC1.5 3Tx in RAN4 in Rel-19 according to the WF R4-2505111. Summary of change: To introduce the RF requirements for 6Rx. |

Proposals for topic #4

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2505438 | Apple | Proposal 1: For UE assistance and reporting, we support Solution 1 from the previous meeting WF: Introduce UE capability indication on whether UE performs SRS IL self-compensation. Proposal 2: Solution 1 can also be used to address the SRS IL imbalance issue for 2Rx/4Rx/8Rx. Observation #3: A simple test could be introduced to confirm if the UE has been pre-compensated or not for SRS insertion loss imbalances. Proposal 3: The following test procedure is proposed to determine if the UE did indeed perform power self-compensation on each SRS port and how effective the compensation was: - The UE is configured for SRS antenna switching - The UE is configured for CSI reporting – CRI-RI-CQI - The TE measures SRS and reconstructs the full channel matrix - The TE determines the rank and CQI of the reconstructed channel - If the UE has effectively pre-compensated the IL, the reciprocity based CQI and RI will be within certain margin of the UE reported CQI and RI |
| R4-2505587 | Huawei, HiSilicon | Observation 1: For PSRS calculation as defined in TS 38.213, PCMAX is a value determined by UE according to the definition in TS 38.101-1, where ∆TRxSRS has already been counted respective to each SRS occasion, but not the range [PCMAX, L, PCMAX, H] itself. Observation 2: SRS transmission power determination can be categorised as following cases. Case #1; For all SRS resources within the set, none of PSRS equals to PCMAX; The estimated path loss could be relatively small, e.g. cell centre. Case #2; For all SRS resources within the set, some of PSRS equals to PCMAX; The estimated path loss could still be not so high while some of the diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. mildly away from cell centre. Case #3; For all SRS resources within the set, all of PSRS equals to PCMAX; The estimated path loss could be high and all diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge. Observation 3: The following scenario can be considered as the target for SRS IL reporting: - For all SRS resources within the set, all of PSRS equals to PCMAX - In this scenario, the estimated path loss could be high and all diversity branches become MOP limited due to ∆TRxSRS, or P-MPR dominated scenario (not in the scope), e.g. cell middle/edge Observation 4: The trigger event for reusing PHR type 3 to resolve SRS IL reporting is hardly to be defined without specification impact because at least following factors should be considered: - The timing (or duration) whenever partial or all antenna ports for AS-SRS transmission become MOP limited - Changing on the mapping between SRS port and Tx chain due to e.g. hand blocking Observation 5: 2Rx can be precluded since it should have the least imbalance thus the expected gain from network perspective is relatively negligible. Proposal 1: The specification impact for defining the trigger event under candidate solution 2 i.e. reusing PHR type 3 shall be clarified. Proposal 2: Further clarifications on the candidate solution 1: - UE is allowed to indicate its capability on handling SRS IL, which includes two states and the UE shall not indicate them simultaneously - State 1: the applied ∆TRxSRS shall be smaller than the respective minimum requirements - State 2: the applied ∆TRxSRS equals to the respective minimum requirements Proposal 3: Adopt the following UE feature list for Rel-19 WI NR_ENDC_RF_Ph4 objective 6Rx. |
| R4-2505674 | Nokia | In this contribution, we have made number of observations. Summarizing all the observations we can make following conclusions. Proposal 1: For UE SRS imbalance assistance, UE should report network only SRS resource specific PCMAX_SRS,f,c(i) Proposal 2: UL RMCs shall be added to TS 38.101-1 to verify SRS resource power at each applicable antenna when SRS power is not limited by PCMAX_SRS,f,c. Proposal 3: UL RMCs shall be added to TS 38.101-1 to verify PCMAX_SRS,f,c(i) and PHSRSi at each applicable antenna are correctly reported. |
| R4-2506226 | China Telecom | Proposal1: Consider introducing UE indication and Type 3 PHR together for SRS IL imbalance improvement. Proposal2: If the self-compensation is totally determined at UE side, it should be triggered only if the actual PCMAX,f,c with IL influence is smaller than the necessary SRS power. Proposal3: The indication should be dynamic and triggered when the UE performing self-compensation. Proposal4: Introduce a new IE to convey the indication from UE to NW. Proposal5: Inform RAN2 to add Type 3 PHR to single cell scenario. Proposal6: send LS to RAN1 and ask whether Type 3 PHR could be enhanced for PUSCH cell in this feature while controlled by signaling. |
| R4-2506380 | ZTE Corporation, Sanechips | In this contribution, we give some further discussions on the SRS IL imbalance related to 6Rx. The conclusions are: Proposal 1. UE SRS IL power compensation is up to UE implementation and optional, and if UE hasn’t reached its Tx power upper limit, UE should indicate the power compensation behaviour to NW once the NW requests such information. Proposal 2: Introduce UE capability indication on whether UE performs SRS IL self-compensation, and it should be per SRS resource reporting. Proposal 3. If dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern is considered, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. Proposal 4. The SRS IL imbalance reporting mechanism in 6Rx discussion should be also specified for 2Rx, 4Rx and 8Rx. |
| R4-2506865 | vivo | In this paper, the following proposals were provided: Observation 1: Having expectation that Rel-19 UE would do compensation for SRS IL is already a large step for UE. Proposal 1: It is proposed not to introduce any signalling related to for SRS IL reporting. Observation 2: Even the simpler “Solution 1” on UE capability indication on whether UE performs SRS IL self-compensation may still controversial in details, and may not be more useful compared to fully implementation dependent. Observation 3: More complicated “Solution 2” can not be accepted by most companies in previous discussion. Proposal 2: Neither solution 1 nor solution 2 is preferred in Rel-19. - Solution 1 can considered a compromise if quick convergence on details can be reached. - Solution 2 should not be considered. Proposal 3: The potential SRS IL solution (including implementation only) can apply to 6Rx and 8Rx. 4Rx and 2Rx need not be mandatory. |
| R4-2506888 | Intel Corporation | In this paper we provide views on the insertion loss imbalance across SRS ports. In summary, the following proposals are made: Proposal #1: Specify solution for UE to perform SRS IL pre-compensation based on the following principles - For SRS transmissions Rel-19 UE shall perform self-compensation of SRS IL up to its configured maximum output power - UE shall perform full or partial self-compensation of the SRS IL on every SRS resource (SRS antenna port) whenever it has power headroom relative to the configured maximum Tx power. - The compensation shall be performed for any SRS transmission power level in TS 38.213. - No RAN1/4 specification impacts Proposal #2: Introduce SRS output power testing framework to guarantee proper UE behavior based on existing specifications at least for PSRS < PCMAX. Proposal #3: Agree on the following UE assistance and reporting framework - Introduce UE capability indication on whether UE performs SRS IL self-compensation - Introduce semi-static reporting of UE-specific SRS IL value - Extend Type 3 SRS PHR reporting to carriers with PUSCH Proposal #4: Adjust the PCMAX_L,f,c equations to include UE-specific SRS-IL component: PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) } Proposal #5: Introduce SRS IL compensation mechanisms as an optional feature in a generic manner for 2Rx/4Rx/6Rx/8Rx UEs. |
| R4-2506916 | OPPO | In this contribution, SRS IL reporting issue is discussed and got the following observations and proposals. Observation 1: If UE support SRS IL reporting feature it will do the compensation, otherwise, there is no restriction. Observation 2: The mapping between SRS resource and physical antennas might change due to, e.g. hand blocking. Proposal 1: UE compensation issue and SRS IL changes due to hand blocking need to be considered if SRS IL reporting scheme is pursued in RAN4. Observation 3: Dynamic reporting is necessary to make sure gNB knows whether it needs to do compensation or which value it should be used. Proposal 2: If SRS IL is to be reported, dynamic reporting shall be considered. Proposal 3: To simplify dynamic reporting complexity, gNB request based SRS IL reporting might be a way to go. For example, before gNB to do the SRS IL compensation in channel estimation, it asks UE to report which SRS IL is applied. |
| R4-2506998 | Google Korea LLC | The proposals in this contribution are summarized in the following. Observation 1: From conducted RF antenna connector perspective, although RFFE insertion loss and PCB tracing loss may cause SRS Tx power imbalance issue when performing SRS antenna switching, the SRS-IL imbalance can be reduced a lot by factory RF calibration process for every commercial UE before shipment, which is also the common process in the industry. Hence, the SRS-IL imbalance would not be a critical issue if the device is well calibrated from conducted RF antenna connector perspective. Observation 2: From OTA perspective, SRS Tx power imbalance at the network side needs to consider not only SRS-IL imbalance but also different antenna implementation. In our understanding, the antenna performance for the MIMO Rx diversity paths would be usually sacrificed because of more protection on main Tx/PRx. Even if SRS-IL imbalance is known in advance, SRS Tx power imbalance may still become higher and the imbalance may be caused by different antenna implementation and/or different channel condition. Hence, it is straightforward that SRS-IL imbalance would not be a critical factor to impact SRS Tx power imbalance from OTA perspective, and it would be better to handle it by enhancing the network UL channel estimation performance. Proposal 1: Since RAN4 assumptions on Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power for SRS transmission, SRS-IL imbalance can be self-compensated well by factory RF calibration process from conductive perspective. Also from OTA perspective, SRS-IL imbalance may not be the critical factor to impact SRS Tx power imbalance. Hence, it is proposed not to introduce any solution for SRS-IL imbalance issue for 6Rx UE in Rel-19. |
| R4-2507073 | Qualcomm France | SRS IL indication was discussed with the following observations and proposals. Proposal 1: No clarifications/changes to RAN4 specification related to UE self-compensation. Proposal 2: Do not add UE capability to indicate support of UE SRS IL self-compensation. Observation 1: Proposed SRS IL indication schemes would not work for all cases. Observation 2: Proposed SRS IL indication schemes would introduce quite a bit of extra complexity. Observation 3: There have been no simulation results to show system level gains for the cases where UE compensates when it has power to do so and does not compensate when there is no power remaining. Proposal 3: Discuss whether UL RMC’s to verify SRS power at least at max power level can be added Proposal 4: Do not pursue option 2. |
| R4-2507529 | Samsung | In this contribution, we continued to provide our analysis and view on the proposed solutions. Following observations and proposals are provided to be considered by this meeting. Observation 1: RAN4 agreed to perform UE self-compensation of SRS IL up to its configured maximum output power for SRS transmissions without RAN1/4 specification impacts. Observation 2: RAN4 needs to discuss the topic on whether/how to improve UE self-compensation method such as UE capability introduction to indicate the self-compensation for the network as Solution 1. Observation 3: Solution 2 to change Type 3 PHR triggering condition is nothing to do with the UE self-compensation, but has RAN1 specification impact, which is totally up to RAN1 discussion. Proposal 1: Solution 2 should be excluded from RAN4 discussion to follow the agreement in Observation 1. Observation 4: Solution 1 still remains unclear how the network can get the benefit from the UE indication. Observation 5: Additional UE capability indication for the network with UE self-compensation would not help improving the downlink channel estimation from the legacy SRS IL imbalance issue. Observation 6: An optional UE capability with single bit indication can be introduced for the compromise if it still helps the network performance practically. Proposal 2: Additional solution to resolve fundamental issues of SRS, i.e., PCMAX or PHR for SRS, should be handled in the future radio discussion. |
| R4-2507772 | Lenovo | Based on the above discussion, we have the following proposals: Proposal 1: In order to assist the gNB in correctly estimating the downlink channel from the received uplink SRS transmissions, the UE should indicate whether or not it UE compensates for the SRS insertion losses at power levels below PCMAX,f,c. Proposal 2: The UE should signal the actual insertion loss differences between the SRS port with the least insertion loss and the remaining SRS ports. That is, if there are N SRS ports and port 1 is defined as the SRS port with the smallest insertion loss, then the UE should signal δ1,j=∆TRxSRS,jA-∆TRxSRS,1A for 2≤j≤N . Proposal 3: If the UE compensates SRS insertion losses, and if the power transmitted on SRS port 1 pt,1 satisfies pt,1>miniPCMAX,f,ci , the transmit power on at least one SRS ports will be less than the transmit power on the first SRS port. In this case, the UE sends a PHR report only for port 1 that includes the optional field configuredMaxTxPower. From this transmit power for an arbitrary SRS port j can be determined as pt,j=pt,1pt,1≤PCMAX,f,c1-δ1,jPCMAX,f,c1-δ1,jpt,1>PCMAX,f,c1-δ1,j . otherwise, if pt,1≤miniPCMAX,f,ci , the power transmitted on the SRS ports is equal, and no power headroom report is needed. |
| R4-2507782 | Ericsson | In the previous sections, we made the following observations: Observation 1: Based on the lab testing results of UE #1, the Tx imbalance is much larger than and non-reciprocal to the Rx imbalance. Observation 2: Based on the lab testing results of UE #1, contrary to TS38.213 and TS38.215, the total power at the antenna connectors is not split equally for multi-port SRS resources (e.g. 2T4R), and there is no specified test which verifies the expected behavior. Observation 3: Based on the lab testing results of UE #2, the Tx imbalance is slightly smaller than the Rx imbalance, but it is non-reciprocal to it due to Tx insertion losses presumably. Observation 4: Based on the lab testing results of UE #3, the Tx and the Rx imbalance are roughly the same and reciprocal. Observation 5: Based on the lab testing results of UE #4, the Tx imbalance is larger than and non-reciprocal to the Rx imbalance. Observation 6: In general, the Tx-Rx imbalance is unpredictable and, in most cases, non-reciprocal. In most cases, the Tx imbalance is larger than the Rx imbalance, even though there are some UEs in the field for which the imbalance is reciprocal. Observation 7: There would be a very limited benefit from reporting of the actual Tx insertion losses – the reporting of the SRS output power would be more beneficial, where the benefit of such reporting is the largest when the Tx imbalance is much larger than the Rx imbalance. Observation 8: The Tx-Rx imbalance issue has been demonstrated by lab testing of the UEs with 4 Rx chains. For the higher number of Rx chains, e.g. 6 Rx or 8 Rx, we could expect that the imbalance and the uncertainty can only grow and thus the recognized issue becomes even more relevant. Observation 9: Testing of the SRS output power is important for the performance of the reciprocity-based DL MIMO and it should be introduced in addition to the IL imbalance reporting as it doesn’t solve the same issue. Observation 10: Modifying the PCMAX lower bound by considering the UE-specific (or SRS port specific) SRS IL value would not solve any issue as the UE still has a freedom to configure PCMAX anywhere between the lower and the upper bound. In addition, it would make the current ∆TRxSRS requirements not needed anymore, and large specification changes would have to be made. Observation 11: Reporting of the configured maximum output power per SRS resource, the power headroom (PH) per SRS resource and the information on the power class fallback or the return to the supported power class (ΔPPowerClass) is already part of “Single Entry PHR MAC CE” and “Multiple Entry PHR MAC CE”. Observation 12: The proposed solution is beneficial for all the UEs, irrespective of whether the self-compensation of the IL is performed or not. Observation 13: The expected benefit from the proposed solution is the maximization of the accuracy of the DL CSI estimation, which is of key importance for the performance of both SU and MU MIMO. Observation 14: Solution 1 does not represent an acceptable compromise since it proposes to introduce UE capability indication on whether the UE performs SRS IL self-compensation, a behavior which is already mandatory according to the RAN1 specifications. Based on the discussion in the previous sections we propose the following: Proposal 1: Introduce reporting for the SRS insertion loss imbalance issue in Rel-19. Proposal 2: The introduced solution for the SRS insertion loss imbalance issue should be applicable to all 2Rx/4Rx/6Rx/8Rx cases. Proposal 3: Introduce the SRS output power testing in order to improve the performance of the reciprocity-based DL MIMO. Proposal 4: Since the UE shall perform self-compensation of the SRS IL according to the specifications, no additional clarifications on behavior are needed. Proposal 5: The following SRS IL imbalance reporting solution should be specified: - UE provides dynamic assistance to the network on SRS IL compensation: - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - ΔPPowerClass reporting - Note: UE performs self-compensation of SRS IL up to its configured maximum output power (as in 38.213) Proposal 6: The PH reporting with the proposed MAC-CE could be periodic (triggered by the expiration of timers, including a “prohibit” timer” to reduce reporting frequency) or by any modification of the SRS configuration. |