3GPP TSG-RAN WG4 Meeting #115 	R4-2507578

Malta, MT, May 19th – 23rd, 2025

Agenda Item: 7.7.4

Source: MediaTek inc.

Title: Discussion on low-band CA switching

Document for: Discussion

# Introduction

The low band carrier aggregation has been one of the network operators’ interests. The benefit on low band is mainly due to the large coverage compared to that using higher frequencies. Nevertheless, although RAN4 RF spec has already covered the low bands CA band combination, yet this has never been implemented in real field. This is mainly due to challenging task to implement low-band CA on handheld UE especially when there are some combinations has almost no frequency gap between UL transmission and DL reception channels. To overcome such obstacle and to better utilize the low band fragmented spectrums owned by operator, a new WI was approved in RANP#106 [1], in which, 25 companies agreed to introduce low-band CA via switching.

This contribution provides discussion on the low-band CA with the new carrier switching feature.

# Discussion

The main objectives of the work item is to introduce the physical procedures and requirements to enable low band CA via switching, detailed below:

### Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4]

The PHY procedure as described in the WID objective is that a UE is configured with two low band carriers, where the UE operates on a single carrier at a time. The switching configuration is set by RRC signalling (semi-static). The procedure can be depicted as in Figure 1 below. It should be noted that the PCell of CA is within carrier 1, while the SCell is within carrier 2.

Figure 1: UE carrier switching procedure in low band CA.

RAN4 has already discussed several RRM aspects related to the low band CA operation and the agreements have been captured in the WF [2]:

## Discussion on general issues

Issue 1-1-1: SSB assumptions for R19 LB CA via switching

Recommended WF - Can we skip this issue and discuss the requirement directly? - [moderator]: The SSB-less operation here does not mean SSB is not transmitted on SDL SCell, and that means UE may use the same operation as for SSB-less case, e.g., skip L3 measurement on SSB-less SCell. - Option 1 (OPPO, LGE, MTK, Nokia, Xiaomi, QC): RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching. - Option 2 (Apple): RAN4 to use SSB-less based operation as baseline for Rel-19 low band CA via switching.

For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Due to the limited time of this WI, RAN4 to focus on discussing and defining requirements for SDL SCell operation for SSB-based Scell, i.e. RRM to deprioritise the scenario of SSB-less Scell based.

- RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching, rather than using TRS/A-TRS based RRM.

Issue 1-1-2: MRTD assumptions for R19 LB CA via switching

- FFS: - Option 1 (OPPO, vivo, QC, ZTE, Nokia, Xiaomi, CATT, Samsung, HW, MTK, LGE): The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Option 3 (Ericsson): MRTD = CP for in LB-LB CA with switching.

MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Based on agreement from the previous meeting, RRM group down-selected the values to either 4.67us or 3us. Given that both values are within the CP length, hence there is no major difference between these two values on the 5G system performance.

- The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Issue 1-1-4: RRM requirement applicability for R19 LB CA via switching

Recommended WF - [Moderator]: check which method can be used in P1. P2 can be further discussed in the detailed requirement design. - Discuss which method (From Ericsson’s paper last meeting) can be used: - Method 1: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures. - Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching

In general from RRM spec, measurement period has been extended to compensate a missing sample. Hence, the same principle can be applied in here where extend the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the aforementioned procedures.

- Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the other procedures/measurements.

## Discussion on switching delay issues

Agreement in RAN1 #120bis For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

Issue 1-2-1A: Necessity of interruption requirement in RRM between FDD CC and SDL CC

Based on RAN1 agreement, the switching gap applied for scenario of SDL to FDD includes switching period plus UL TA. Unless RAN1 define scheduling restriction for the DL symbols of SDL carrier, DL scheduling restriction requirement is needed to apply to DL symbols during switching gap on SDL FDD.

- Unless RAN1 define scheduling restriction for the DL symbols of SDL carrier, DL scheduling restriction requirement is needed in RRM spec to apply to DL symbols during switching gap on SDL FDD.

Issue 1-2-1B: Necessity of interruption requirement in RRM when more carriers are considered

This issue is out of the current WI scope. RAN4 RRM to close this issue.

- RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

Issue 1-2-2: Interruption requirement (if needed) for carrier switching between FDD CC and SDL CC

In order to enable the switching procedures from RAN4 point of view, one of the main aspect is switching delay and location in time. RAN4 RRM looked into this issue in the previous meeting and companies are discussing interruption requirements on slot level or symbol level. It should be pointed out that slot level is too slow for such feature and might have major negative impact the system performance and hence hinder the successful implementation of such feature in real field. Therefore, it is suggested that the interruption shall be defined as within symbol level only.

- RAN4 RRM to define interruption requirements for carrier switching in R19 LB CA on a symbol level only.

## Discussion on SDL SCell related requirement

Issue 1-3-1: Activated SDL SCell L3 measurement and corresponding interruption

Recommended WF - Discuss following points: - Whether to define the requirement for activated SDL SCell L3 measurement or not? - Option 1: No (QC, vivo) - Option 2: Yes for SSB based operation (OPPO, Apple, LGE, HW, Ericsson, Samsung, ZTE, MTK, Nokia) - If Yes for above, followings are FFS - Whether to reuse existing corresponding requirement for activated SDL SCC measurement - Whether and how such measurement shall consider switching pattern, e.g., only measure SDL SCC during the period of SDL SCell ON. - Whether and how to define the corresponding interruption requirement for this measurement. - The scaling factor of CSSF and Kp.

Provided that this new feature is to enable CA band combo rather than enhancing existing SCell operation, then RAN4 shall define requirements for activated SDL SCell L3 measurement.

- RAN4 shall define requirements for activated SDL SCell L3 measurement.

Furthermore, the following issues can be addressed as:

- Whether to reuse existing corresponding requirement for activated SDL SCC measurement?

  - Discussion: Although RAN4 should try to reuse existing requirements as much as possible, yet, there is a need to extend the measurement requirements for activated SDL following the switching pattern.

- Whether and how such measurement shall consider switching pattern, e.g., only measure SDL SCC during the period of SDL SCell ON.

  - Discussion: The measurements of activated SDL SCell shall conducted during the ON period of the switching pattern.

- Whether and how to define the corresponding interruption requirement for this measurement.

  - Discussion: Since the UE measures only during the period of switching pattern ON, hence, FFS the necessity of interruption requirements?

- there is a need to extend the activated SDL SCell L3 measurement requirements for activated SDL following the switching pattern.

- The measurements of activated SDL SCell shall conducted during the ON period of the switching pattern.

Issue 1-3-2: Deactivated SDL SCell measurement and corresponding interruption

Recommended WF - Discuss following points: - Whether to define the requirement for deactivated SDL SCell L3 measurement or not? - Option 1: No (QC) - Option 2: Yes for SSB based operation (OPPO, CATT, Apple, LGE, HW, Ericsson, vivo, MTK, Nokia) - If Yes for above, followings are FFS - Whether and how such measurement shall consider switching pattern and/or meaCycleSCell, e.g., only measure SDL SCC during the period of SDL SCell ON. - Whether to reuse existing corresponding requirement for deactivated SDL SCC measurement - Option 1: Yes (Apple, MTK) - Option 2: No - Whether and how to use MG for deactivated SDL SCC measurement (LGE, Nokia). - Whether and how to define the corresponding interruption requirement for this measurement. - Option1 (Apple, HW): If SSB-based SCell operation is assumed for SDL SCell measurement, for deactivated SDL SCell measurement, interruptions on PCell or activated SCell(s) are allowed with up to 0.5% probability of missed ACK/NACK regardless of the configured measCycleSCell. - Option 2 (Ericsson): interruption on PCell or activated SCell(s) are allowed with up to X % probability of missed ACK/NACK when measCycleSCell is below threshold 1 and Y% otherwise. Y can be 0.

Provided that this new feature is to enable CA band combo rather than enhancing existing SCell operation, then RAN4 shall define requirements for deactivated SDL SCell L3 measurement. In addition, provided that the UE only applies the pattern during activated SCell, hence, during deactivated SCell the UE behavior is based on activating RF chain to perform measurements and hence the UE is expected to cause interruption during the measurements. Thus, the existing requirements can be applied for interruption before and after measurements, yet additional scheduling restrictions should be applied during the measurement time. Furthermore, regarding whether to use measurement gaps, it should be noted that MG interrupts all carriers for the UE during the measurements. Although, the current scenario is focusing on two carriers only yet there is ongoing discussion to extend the case to three carriers. Now, given that MG interrupts all carriers then when the scenario includes three carriers then all the three carriers are interrupted. Therefore, our preference is to have this feature covered by interruption and scheduling restriction during SMTC/SSB period.

- RAN4 shall define requirements for deactivated SDL SCell L3 measurement.

- Existing interruption requirements based on meaCycleSCell can be applied with additional scheduling restrictions to cover the SSB/SMTC period.

- MG interrupts all CC, while the scenario that we have in here should interrupt PCell only. Hence, there is no need to extend using MG to this scenario.

Issue 1-3-3: SDL SCell activation/deactivation

Recommended WF - Discuss following points: - Side condition for SDL SCell activation requirement - RTD between the FDD PCell and SDL SCell - RTD condition is based on issue 1-1-2 - reception power difference between the FDD PCell and SDL SCell - EPRE difference condition is reused from FR1 inter-band SSB-less SCell activation for Rel-18 NES, e.g., 12dB - The SDL SCell activation consider SSB-less operation or SSB based operation or both? - Option 1: SSB-less only (Apple, Samsung- for UE supporting SSB-less SCell operation) - Option 1a: such SSB-less operation is regardless of whether or not UE supporting SSB-less SCell feature. - Option 2: SSB based only - Option 3: both (HW, vivo) - Can existing SCell activation requirement be reused or not - Option 1: Yes (Apple, Samsung, MTK, Nokia) - Option 2: need a new requirement.(Xiaomi, CATT) - Interruption and scheduling restriction for SDL SCell activation: need FFS - Known/unknown condition for SDL Cell activation and other details of the SDL SCell activation(if needed): need FFS - The other scopes for SDL SCell activation requirements, e.g., in proposal 9: need FFS

Side condition for SDL SCell activation requirement:

  - RTD between the FDD PCell and SDL SCell <=3us

  - FFS: [reception power difference between the FDD PCell and SDL SCell]

The SDL SCell activation consider SSB-less operation or SSB based operation or both?

  - SSB based SCell activation shall be considered as baseline.

- RAN4 shall define side conditions: (i) RTD between the FDD PCell and SDL SCell <=3us, (ii) Limit the reception power difference between the FDD PCell and SDL SCell.

- SSB based SCell activation shall be considered as baseline.

Issue 1-3-3: SDL SCell activation/deactivation

Recommended WF - Discuss following points: - Whether to define requirement for BFD/CBD for SDL SCells in LB CA via switching - Option1 (QC, Apple): No, RAN4 to not define BFD and CBD requirements for SDL SCells in LB CA via switching - Option 2 (MTK): Yes, reuse the existing BFD/CBD requirement for R19 LB CA - Option 3(LGE, CATT, Ericsson): Yes, FFS on detailed requirements. - Whether to define requirement for PL-RS switching delay for SDL SCells in LB CA via switching - Option1 (Apple): No, RAN4 to not define PL-RS switch delay requirements for SDL SCells in LB CA via switching - Option 1a(vivo): No if assuming SSBs on FDD PCell and SDL SCell are QCL-ed - Option 2 (MTK): Yes, reuse the existing PL-RS switch delay requirement for R19 LB CA - Option 3: Yes, FFS on detailed requirements. - Whether to define requirement for TCI switching delay for SDL SCells in LB CA via switching - Option 1 (Apple): existing TCI switching requirement can be used for SDL SCell in R19 LB CA via switching, provided that PCell SSB is configured as the QCL source of SDL SCell channel(s); otherwise, no requirement applies. - Option 2 (MTK): reuse the existing TCI switching delay requirement for R19 LB CA. - Option 3 (Ericsson): consider to change the requirement, as in P7. - Option 4 (vivo): No requirement is needed if assuming SSBs on FDD PCell and SDL SCell are QCL-ed - Whether and how to define requirement for L1-RSRP/SINR measurement for SDL SCells in LB CA via switching - [Moderator]: check if following option 1 is agreeable or not. For other parts can be FFS after RAN1 concludes on the switching pattern. - Option 1: For SSB-less operation, UE can skip SSB/CSI-RS based L1-RSRP measurement on SDL SCell, and the L1 RSRP measurement on collocated PCell can be leveraged

RAN4 to define at least the following measurements for SDL SCells in LB CA via switching: (i) BFD/CBD, TCI switching delay requirements, (iii) L1-RSRP/SINR measurements.

- RAN4 to define at least the following measurements for SDL SCells in LB CA via switching: (i) BFD/CBD, TCI switching delay requirements, (iii) L1-RSRP/SINR measurements.

## Discussion on impact on FDD PCell RRM

Issue 1-4-1: Impact on timing requirement for PCell

Agreement in RAN1#120bis - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement in RAN1#120bis - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms.

P=40ms, RAN4 can reuse the existing timing requirement with the same applicability condition (SSB is available in last 160ms).

- RAN4 can reuse the existing timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) and RAN4 may revisit this condition if new P (e.g., P>160ms) is introduced.

# Summary

In summary:

Proposal 1: RAN4 to use SSB based operation as baseline for Rel-19 low band CA via switching, rather than using TRS/A-TRS based RRM.

Proposal 2: The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Proposal 3: Agree on the principle of extending the delay based on missed reception occasion of necessary DL signals/channels and apply this principle to all the other procedures/measurements.

Proposal 4: Unless RAN1 define scheduling restriction for the DL symbols of SDL carrier, DL scheduling restriction requirement is needed in RRM spec to apply to DL symbols during switching gap on SDL FDD.

Proposal 5: RAN4 RRM to close the issue of RRM requirements when more carriers are considered, unless it is specified in the WID.

Proposal 6: RAN4 RRM to define interruption requirements for carrier switching in R19 LB CA on a symbol level only.

Proposal 7: RAN4 shall define requirements for activated SDL SCell L3 measurement.

Proposal 8: there is a need to extend the activated SDL SCell L3 measurement requirements for activated SDL following the switching pattern.

Proposal 9:The measurements of activated SDL SCell shall conducted during the ON period of the switching pattern.

Proposal 10:RAN4 shall define requirements for deactivated SDL SCell L3 measurement.

Proposal 11:Existing interruption requirements based on meaCycleSCell can be applied with additional scheduling restrictions to cover the SSB/SMTC period.

Proposal 12:MG interrupts all CC, while the scenario that we have in here should interrupt PCell only. Hence, there is no need to extend using MG to this scenario.

Proposal 13:RAN4 shall define side conditions: (i) RTD between the FDD PCell and SDL SCell <=3us, (ii) Limit the reception power difference between the FDD PCell and SDL SCell.

Proposal 14:SSB based SCell activation shall be considered as baseline.

Proposal 15:RAN4 to define at least the following measurements for SDL SCells in LB CA via switching: (i) BFD/CBD, TCI switching delay requirements, (iii) L1-RSRP/SINR measurements.

Proposal 16:RAN4 can reuse the existing timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) and RAN4 may revisit this condition if new P (e.g., P>160ms) is introduced.

# References

- RP-243317, “New WID on low band carrier aggregation via switching”, Madrid, Spain, Dec 2024.

- R4-2504895, “WF on RRM requirements for NR_LBCA_Sw”, Apple, Wuhan, China, April 2025.

- R4-2503614, “Topic summary for [114bis][204] NR_LBCA_Sw”, Apple, Wuhan, China, April 2025.