3GPP TSG-RAN WG4 Meeting # 115	R4-2507619

# Malta, MT, 19th – 23th May, 2025

Source:	LG Electronics

Title:	Discussion on Power domain enhancement for single carrier

Agenda item:	7.1.2.2.1

Document for:	Discussion

Introduction

In 3GPP RAN4 #114-bis, WF[1] was approved. In this paper, we provide our views on Power domain enhancements for single carrier.

- Topic #1: Power domain enhancements for single carrier - Sub-topic 1-1: Approaches to enable MPR reduction - Issue 1-1-1: RB based or CBW based extension - WF - Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB - If a limited number of additional fixed values are introduced, the extension is determined by X*NRB at each side, where X < 0.5 - The extended RBs are aligned with common RB grid - Issue 1-1-2: Fixed or flexible extension - Agreement in main session: - Fixed value (0.5*CBW and/or 0.5*NRB) for Symmetrical/ Asymmetric by default for UE supporting the MPR reduction feature - In addition, consider a limited number of additional fixed values with capability - Issue 1-1-3: Derivation of new inner RB region in extended CBW - WF - FFS in next meeting - Issue 1-1-4: Guard band for extended CBW - Options - Option 1: No need to define multiple UE guard bands for initial CBW and extended CBW, UE should have one guard band per each side. All frequency segments within UE extended CBW and outside UE maximum transmission bandwidth (number of RBs in 38.101-1) act as UE total guard bands (i.e., UE initial guard band + additional guard band according to UE extended CBW NW indication), where this UE cannot be allocated and OOBE edges are the outer edges of UE total guard band. - Option 2: NRB based extension and associated guard band: - maximum bandwidth extension: The extended RB BW NRBext inherits the guard-bands (GBext) from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext. - flexible NRB-based extension: Similarly to the ceil(NRB/2) extension, the guard band mapping for the fractional NRB extension can inherit the guard band from the next corresponding existing higher CBW that uses the next higher NRB versus NRBext - Option 3: The GBs of extended UE CBW should only be retained on the side that is far away from the original CBW. The GBs of extended UE CBW reuse the size of original GB of UE CBW. - Option 4: The GBs of the extended CBW shall be the remaining portion obtained by subtracting the RB numbers from the specified channel bandwidth when the extended CBW is selected from Table 5.3.2-1 in 3GPP TS 38.101-1. - Others - WF - FFS in next meeting - Sub-topic 1-2: Applicable requirements - Issue 1-2-1: A-MPR handling - Proposals - Option 1: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. - Option 2: - If A-MPR is specified for a given RB allocation X, then the UE performs max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs max(MPR(X), A-MPR(X)) - Option 3: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function based on the specified A-MPR and the initial UE CBW MPR and thus set its backoff ignoring any MPR reduction. - Option 4: max(MPRenh(X), A-MPR(X)), where A-MPR(X) is determined based on a given RB allocation, and RB region with respect to initial CBW, WF and modulation etc, and MPRenh(X) is determined based on RB region/etc. within the extended UE CBW. - Others - WF - Only apply the power domain enhancement scheme to channels which are not subject to A-MPR in Rel-19. - Issue 1-2-2: Whether the extended IBE is applied in the GB of original UE CBW Issue1 Issue 2; Option1. Allocate the extended RBs from the GB of the original UE; Option2. Allocate the extended RBs from the edge of the UE CBW. Option1. Individual evaluation of the extended RBs & Horizontal line; Case1; Case2. Option2. Combine evaluation of the extended RBs with the non-allocated RBs of the UE CBW & Stepped line; Case3; Case4. Note: It is assumed that extended RBs are RB aligned with the UE and BS RBs for the listed cases.. - WF: - Case 1 and case 3 could be further down selected - GB of UE CBW may or may not be scheduled to other UEs - Extended IBE can be applied in the GB of UE CBW - Relaxation can be considered for the GB, based on calculation from the existing IBE formula from the last RB of original UE CBW - Issue 1-2-3: Derivation of the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - Option 2: Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs. - Option 3: Others - WF - Discuss together with issue 1-2-2, and down select the options in next meeting. - Issue 1-2-4: Relaxation of the extended IBE - Proposals - Proposal 1: For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. (Xiaomi) - Proposal 2: (vivo) - If the extended RBs are allocated from the GB of the original UE (case1/3), then the IBE in the GB should be relaxed to ensure the MPR of outer allocation reduces to the inner level: - Way 1: Skip the IBE evaluation for the extended RBs allocated in the GB. - Way2: Relax EVM in the IBE formula to that for BPSK for extended RBs. IBE_general = max(PRB - 30 dB, ) - WF - For case 1 in issue 1-2-2, use BPSK EVM in the IBE formula for all extended RBs - For case 3 in issue 1-2-2, FFS the relaxation - Sub-topic 1-3: Signaling - Issue 1-3-1: NW signaling - WF - FFS in next meeting - Issue 1-3-2: UE capability - Agreement in main session: - An optional UE capability to indicate whether it supports the CBW extension based MPR reduction feature - To discuss the following aspects - Whether to have separate UE capabilities for scenario 1 and scenario 2 - Whether to indicate multiple values of fixed extension capability

Discussion

Derivation of new inner RB region in extended CBW

At original UE CBW, inner region have decided with follow equation.

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB – RBStart,Low – LCRB

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB/2)

Each parameter at above equation mean LCRB is Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks. NRB mean Transmission bandwidth configuration, expressed in units of resource blocks. Others parameter is coming based on NRB and LCRB.

Figure1. NRB and LCRB definition

According to NRB , LCRB and RBstart, RB allocation was decided whether it is outer region or inner region or edge region. NRB , LCRB and RBstart was decided at each UE Channel. RB allocation is automatically decided with following plot.

Figure2. RB allocation example (SCS=15KHz, UE CBW=20MHz, NRB=106)

If RB extension was applied to original UE Channel, some parameters related original UE CBW is same at extension CBW case. LCRB is same with original UE CBW. It cannot extend to extended UE CBW. LCRB should be limited with LCRB≤NRB. It is only changed with NRB and RBStart. New NRB to derive the new inner region  could be starting point as follow.

NRB_extension=NRB+ RBextension_lower+RBextension_higher

Proposal 1: New NRB to derive the new inner region could be defined as follow

NRB_extension=NRB+ RBextension_lower+RBextension_higher

At last meeting, RAN4 agree with maximum RB extension value. Symmetrical/ Asymmetric for maximum extension at each side are determined by 0.5*NRB. But NRB will be decided with following table.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

In case of odd number NRB, half of odd number NRB is not matched with NRB. For example, the half of 25MHz(NRB=133) is 66.5. 0.5RB cannot be defined. So NRB is needed to revise to ceil(NRB*0.5) or floor(NRB*0.5).

Proposal 2: Maximum extension at each side should be revised to ceil(NRB*0.5) or floor(NRB*0.5)

RBextension_lower and RBextension_higher value is limited as follow range.

RBextension_lower, RBextension_higher ≤ ceil(NRB*0.5) or floor(NRB*0.5)

Until last meeting, RAN4 agree that extension channel bandwidth should be extended with RB based. Because RAN4 need to consider how network indicate the information related to extension channel bandwidth. When network indicate channel bandwidth size to UE for extension. it is necessary to discuss more how UE decide the RB allocation whether it is outer region or inner region. Network indication like (RBextension lower, NRB extension), RB allocation could be easily changed. If network indicate this information(RBextension lower, NRB extension), new RB allocation could be calculated based on RBextension left, NRB extension. It could be one of the method to convert from outer region to inner region. NRB_extension was used instead of NRB. And RBStart+RBextension_left was used instead of RBStart. Following as it was changed, (X,Y) at legacy UE CBW can be converted at the extension UE CBW dimension.

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_extension – RBStart,Low – LCRB

RBStart,Low  ≤  RBStart +RBextension_lower ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_extension/2)

Figure3. Operation based on BS indication

We consider network indication when converting outer RB allocation to inner allocation as below

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart +RBextension_lower ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_extension/2)

Proposal 3: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart +RBextension_lower ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_extension/2)

Whether the extended IBE is applied in the GB of original UE CBW

Until last meeting, RAN4 agree that IBE should be used between edges of UE CBW and extended UE CBW. For the application of IBE in the range between edges of UE CHBW and extended UE CHBW. IBE can be specified as following figure 4. IBE of extended UE CBW can be applied guard RB of original CBW. Because there are no active RBs in the extension region. and also virtual CBW will be defined. That’s why IBE of extended CBW can applied to guard RB of UE CBW.

Figure4. IBE requirement applied with new approach[1]

Equation of general requirement for in-band emissions is limited according to UE CHBW and allocated RB.

IBE requirement for extended UE CHBW can be changed from NRB to NRB_extension(NRB_extension=RBleft_extension+ RBright_extension +NRB). if IBE of extended UE CHBW was specified with above method. There is no need to define the IBW with extension region separately.

Proposal 4 : Support Case 3, Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs.

Case3

Signaling related

Considering a limited number of additional fixed values with capability. BS transmit signal to UE with parameter related MPR reduction. Bandwidth extension should be signalled by the BS to UE with extended UE CBW and lower side RB extension.

Proposal 5: Bandwidth extension should be signalled by the BS to UE (e.g., NRB_extension andRBLower_extension)

- 2. Conclusion

In this contribution, we want to share some views on Power domain enhancements for single carrier and proposals are made as following:

Proposal 1: New NRB to derive the new inner region could be defined as follow

NRB_extension=NRB+ RBextension_lower+RBextension_higher

Proposal 2: Maximum extension at each side should be revised to ceil(NRB*0.5) or floor(NRB*0.5)

Proposal 3: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart +RBextension_lower ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_extension/2)

Proposal 4 : Support Case 3, Combine the evaluation of the extended RBs with the non-allocated RBs of the UE CBW: Stepped line from the first non-allocated RB to the last extended RB, NRB’ = NRB + extended RBs.

Case3

Proposal 5: Bandwidth extension should be signalled by the BS to UE (e.g., NRB_extension andRBLower_extension)

- 3. Reference

[1] R4-2505099, “WF on Power domain enhancements”

- Annex

Requirement for supporting MPR reduction

If the UE supports [MPR reduction for BW extension], and when IE [BW extension-LowerSide] and [BW extension -Total] are both set to 1.

NRB_extension  is the number of RBs for extended Channel bandwidth. NRB_extension= NRB + RBlower_extension+ RBHigher_extension

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB_extension – RBStart,Low – LCRB

The RB allocation is an new Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart +RBlower_extension ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_extension/2)

where ceil(x) is the smallest integer greater than or equal to x.