3GPP TSG-RAN WG4 Meeting #115	R4-2507625

Malta, MT, May 19 – 23, 2025

Source:	Nokia

Title:	On Enhancements of Network Energy Savings for NR

Agenda item:	7.27.5

Document for:	Discussion

Introduction

The Work Item Description (WID) of Rel-19 Enhancements of network energy savings for NR is documented in [1]. The Objective of the Performance part of the Work Item (WI) is described as follows [1]:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Specify corresponding performance requirements and test cases for the network energy saving techniques [RAN4].

To address the WI’s Performance part, in RAN4#115 meeting, one Agenda Item is dedicated for discussions on UE and BS demodulation performance requirements.

This contribution provides our initial views on demodulation performance requirements for Rel-19 Enhancements of network energy savings for NR.

Discussion

To further assess the impact on the demodulation performance requirements of Rel-19 Enhancements of network energy savings for NR, we refer to the Objective of the Core part of the WI in [1]:

4.1 Objective of SI or Core part WI or Testing part WI The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. […] - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

As stated in [1], there are some normative works to be done in RAN4 for the core part as highlighted above. In relation to demodulation performance requirements, the adaptation of common signal/channel transmission is expected to have impact on the specifications.

Demodulation Performance

UE demodulation performance

Referring to the Objective of Core part of the WI [1], the possible impact to UE demodulation performance would come from the adaptation of SSB in time domain. Recent RAN1#120 meeting has the following agreements on the adaptation of SSB:

Agreement For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell, - The DCI is scrambled a new RNTI, - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell - the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter - the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell Agreement For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell. - Xmax=2 Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst

As shown above, the downlink information on the adaptation of SSB in time domain is carried by the DCI 2_9.

RAN4 to define requirements for PDCCH DCI format 2_9 for adaptation of SSB in time-domain.

Additionally, RAN1 is currently discussing how adaptation of SSB periodicity affects rate matching operation, as well as CSI measurements and reporting. Once rate matching impact is decided, RAN4 shall discuss the demodulation performance impact.

Once RAN1 has decided how adaptation of SSB periodicity affects rate matching operation, as well as, CSI measurements and reporting, RAN4 shall discuss the corresponding demodulation performance impact.

BS demodulation performance

The work on the core part which may impact BS demodulation performance is the adaptation of PRACH in time domain. Recent RAN1#120 meeting has the following agreements of the adaptation of PRACH in time domain for Rel-19 Enhancement of network energy savings in NR:

Agreement For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon Agreement For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling. Agreement - For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI - The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..}) Agreement For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. - Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs Agreement LS on DCI-based PRACH adaptation endorsed with the ACTION part modified compared to draft LS in R1-2503085 as follows: - ACTION: RAN1 respectfully asks RAN2 to confirm whether the use of above bit is feasible. Final LS in R1-2503086. Agreement For adaptation of PRACH in time-domain, for a connected mode UE, - One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources. Agreement For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of - PRACH association period - This PRACH mask applies to every [configured] K SSB RO association pattern period(s) Agreement For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by: - Option 1-2: Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods). For K: one from up to four candidate values {2,4,8, [1 or 16]} Agreement Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported - CB-PreamblesPerSSB

Based on the current discussions in RAN1, while there is adaptation of PRACH in time domain, nonetheless, its impact to BS demodulation performance is not expected. As there is no new preamble format nor new LRA, it is not necessary to define requirements for PRACH.	
The additional ROs are introduced to give the NW flexibility to quickly handle overload/peak situations (in a deliberately under dimensioned RACH periodicity setup). The treatment of legacy and additional ROs does not change.

RAN4 to not introduce new RACH requirements for additional ROs in this work item, as only RO scheduling is affected but not RO treatment/demodulation.

Conclusion

In this contribution, we provide our initial views on Rel-19 Enhancements of network energy savings for NR. The following Observations and Proposals were made:

Proposal 1: RAN4 to define requirements for PDCCH DCI format 2_9 for adaptation of SSB in time-domain.

Proposal 2: Once RAN1 has decided how adaptation of SSB periodicity affects rate matching operation, as well as, CSI measurements and reporting, RAN4 shall discuss the corresponding demodulation performance impact.

Proposal 3: RAN4 to not introduce new RACH requirements for additional ROs in this work item, as only RO scheduling is affected but not RO treatment/demodulation.

- References

- RP-250343, Revised WID on enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #107, Ericsson, Incheon, S. Korea, March 12-14, 2025