3GPP TSG-RAN WG4 Meeting #115	R4-2507626

Malta, May 19 – 23, 2025

Source:	Nokia

Title:	On the Evolution of NR Duplex Operation - Sub-Band Full Duplex

Agenda item:	7.25.6

Document for:	Discussion

Introduction

The Work Item Description (WID) of Rel-19 Evolution of NR duplex operation: Sub-band Full Duplex (SBFD) in [1] describes the objective of performance part as follows:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The objectives are as follows [RAN4]: - Specify BS RF conformance requirements to support SBFD operation at gNB - Specify necessary RRM performance requirements to support SBFD operation - Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation

As shown above, one of the objectives is to specify necessary BS and/or UE demodulation performance requirements to support SBFD operation. In this contribution, we provide our initial views on demodulation performance requirements for SBFD operation.

Discussion

Overview of SBFD

SBFD is being specified in Rel-19 as a new mode of operation in unpaired (TDD) spectrum. Different to non-SBFD (legacy) operation, in which the transmission within each symbol/slot can only be in one direction (i.e., either UL or DL), SBFD operation allows both UL and DL transmission to happen simultaneously in different frequency resources. Separate sets of RBs within SBFD symbols/slots are allocated for UL and DL transmissions. Figure 1 illustrates examples of two different arrangements of UL and DL resources for SBFD operation, namely, DU/UD (Fig. 1A) and DUD (Fig. 1B). As shown in Figure 1, SBFD symbols are symbols (or slots) which simultaneously allocate one UL sub-band and one (or two) DL sub-band(s) for SBFD operation.

Figure 1: SBFD resources in time and frequency domain

Different time-domain sequences were discussed by RAN1 in TR 38.858, e.g. DXXXU, XXXXX, or XXXXU, where D, U, and X refer to DL, UL and SBFD slots, respectively. One or more RBs will be used as guard-band between the UL and DL sub-bands to facilitate rejection of cross-link interference. Similarly, one or more OFDM symbols will be used as guard-time or period for the transition between different symbol/slot types.

Interference modeling

Due to different transmitter and receiver imperfections, SBFD faces challenges from cross-link interference (CLI) between UEs and CLI between gNBs. Figure 2 illustrates the CLI types studied in RAN1 and RAN4, namely:

- gNB self-interference (1),

- gNB-to-gNB inter-sector (or intra-site) interference (2),

- gNB-to-gNB inter-site interference (3),

- UE-to-UE intra-cell interference (4), and

- UE-to-UE inter-cell interference (5).

Figure 2: CLI types present in SBFD

In the following, we provide the modelling of each type of CLI assumed during the Rel-18 Duplex Evolution SI with primary focus on FR1. More details, and for FR2-1, can be found in TR 38.858 [3].

gNB Self-interference modeling

The feasibility of SBFD was assessed by breaking down the self-interference into different components, such as: the transmit power, frequency isolation capability, spatial isolation, TX beam nulling and RF interference cancellation. The assessment was based on a framework for calculating the residual self-interference calculation (RSIC) under the assumption that the UL sensitivity degradation at the gNB receiver due to the self-interference is 1 dB. Candidate values of RSI were provided by RAN4 to RAN1, so that the desense target was also considered in the performance evaluation of SBFD.

gNB Inter-sector interference

The objective of gNB inter-sector interference analysis was to identify the inter-sector spatial isolation values that would result in additional desensitization at the gNB receiver. RAN4 considered different factors in both the transmitter characteristics (e.g., tx power and beam nulling) and the BS deployment (e.g., antenna isolation and absorbent materials between the different sectors). gNB inter-sector spatial isolation values between 75 dB to 103 dB for FR1 were discussed, where the latter would also result in a negligible UL received desensitization (considering the effects of ACLR/ACS on top of the spatial isolation) even for high-power FR1 macro base station deployments.

Inter-site gNB-to-gNB interference

The co-channel inter-sub-band inter-site interference analysis considered the unwanted emission impact on the receiver selectivity. The same transmitter leakage and receiver impairment model as for investigating the gNB self-interference was used. However, the antenna isolation was replaced with the inter-site isolation which can be derived using 3GPP propagation models (based on TR 38.901 [4]). The Tx leakage baseline studied was the gNB adjacent channel leakage ration (ACLR), and the receiver impairment was the gNB adjacent channel selectivity (ACS).

UE-to-UE CLI modeling

UE-to-UE CLI exists because of the transmitter leakage and receiver selectivity. RAN1 agreed to model the UE-to-UE interference as a combination of UE-to-UE spatial isolation (based on TR 38.901 [4] propagation models), in-band emission (IBE) model (based on 38.101-1 [5]) for transmitter leakage, and a fixed selectivity value of 33 and 23 dB for FR1 and FR2-1, respectively.

Frequency range and channel bandwidth for SBFD

RAN4 BS RF have agreed on the operating bands for SBFD as listed in R4-2502310 [2] for NR TDD FR1 and FR2-1 bands. For FR1, the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39. For FR2-1, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

SBFD over narrow carrier bandwidths (e.g. 40 MHz for FR1) have the challenge of larger overhead as the number of RBs used for the guard bands are relatively large compared to the number of RBs that are used for UL and DL transmissions. At least for FR1, RAN4 should discuss whether to keep existing carrier bandwidth assumption, e.g. 40 MHz, or whether to increase it to e.g. 100 MHz given that larger bandwidths are more attractive for SBFD operation.

SBFD is specified for NR TDD bands in both FR1 and FR2-1.

For FR1 the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39.

For FR2-1, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

For FR1, current channel bandwidth assumption for UE/BS demodulation is 40 MHz, while SBFD is especially intended for large carrier bandwidths, e.g. 100 MHz, due to lower relative overhead of the guard-bands.

RAN4 to prioritize the minimum bandwidth sizes of SBFD for both FR1 (40 MHz) and FR2-1 (100 MHz) NR TDD bands for SBFD demodulation performance requirements. If additional bandwidth is required, RAN4 to consider 100 MHz for FR1.

BS Demodulation

PUSCH demodulation requirements for SBFD

RAN1 introduced the concept of UL resource muting for handling the gNB-to-gNB CLI. A UE transmitting PUSCH with UL resource muting configured shall mute, assuming a comb-2 pattern, the resource elements overlapping with the up to two UL muting symbols configured within a slot. The UE applies the muting to the whole PUSCH bandwidth and with 3 dB power boosting over the active REs within the UL muting symbol(s). UL muting is not performed if an UL muting symbol collides with the UL DMRS of the PUSCH.

Compared to the legacy TDD operation, the PUSCH demodulation in an SBFD-capable gNB is affected by its own self-interference as well as the gNB-to-gNB CLI (cf. CLI types 1, 2, 3 in Figure 2). Concerning the gNB self-interference, on how to accommodate such an interference into the link-level simulation is not yet clear. As the transmit antennas and the receive antennas are close to each other, propagation conditions as in [4] may not be used as the propagation condition of the self-interference signal. Nonetheless, referring to [3] self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

With UL muting, the UE shall apply 3 dB power boosting (over the active REs within UL muting symbols) and rate-matching around the muted REs.

In SBFD, the PUSCH demodulation at SBFD-capable gNB is affected by the gNB self-interference and the gNB-to-gNB CLI.

Referring to 38.858, self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

RAN4 to first discuss whether to follow 38.858 by treating SBFD interference signals including gNB self-interference as AWGN in the link-level simulation before agreeing on having PUSCH demodulation requirements for SBFD or not.

PRACH detection requirements

RAN1 agreed on the following for random access in SBFD operation:

Agreement A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE: - The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH. - UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols - There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH. - Other assumptions are not precluded. NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB.

As noted in the agreement, the UE can be configured with the so-called RACH configuration option 2 which can allow RACH occasions (RO) to start in a SBFD symbol and end in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration.

Figure 3: RACH occasions (ROs) across SBFD and non-SBFD symbols according to RACH Configuration option 2

While RACH configuration option 2 allows RO to start in an SBFD symbol and end in non-SBFD symbol, its impact to BS demodulation performance is not expected. As there is no new preamble format nor new LRA, it is not necessary to define requirements for PRACH.

RAN4 to discuss whether RACH configuration option 2 is expected to have differing demodulation performance. If not, then requirements for PRACH are not necessary.

UE Demodulation

UEs operating in SBFD networks will be affected by UE-to-UE CLI from nearby transmitting UEs (from the same or different cell as illustrated in Figure 2). More specifically, a UE transmitting in UL direction would generate interference to another UE receiving e.g. a PDSCH channel or performing measurements for CSI reporting.

Considering the effects of timing advance on the UL transmission, UE-to-UE CLI may be quite different to the traditional inter-cell DL cochannel interference present in legacy TDD networks. In addition to the effects of UE-to-UE CLI on UE DL reception and measurements, RAN1 has specified various enhancements to account for the two disjoint DL sub-bands in case SBFD DUD operation. For UE demodulation, some of the most relevant ones are as follows:

- For PDSCH FDRA Type 0 allocation (bitmap-based), the part of a RBG that partially overlaps with the DL sub-band, only the RBs within the DL sub-band are used by the UE for PDSCH reception.

- For PDSCH FDRA Type 1 allocation (start RB and RB length), only the RBs within the DL sub-band are used by the UE for PDSCH reception.

- For CSI-RS, only CSI-RS frequency resources within DL sub-band are valid for the UE. For CSI reporting, the following has been additionally specified by RAN1:

  - For a CSI reporting sub-band which overlaps with at least one PRB within DL PRBs and one or more PRBs outside DL PRBs, the CSI reporting sub-band includes PRB(s) within DL PRBs only.

  - For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Figure 4: RAN1 specified enhancements for SBFD DL reception and CSI-RS measurement across disjoint DL sub-bands

In 38.101-4, the minimum requirements for wideband CQI reporting with inter-cell interference are specified.

In SBFD, the UE-to-UE CLI will impact DL reception and the reported CQI.

Furthermore, in SBFD, only PDSCH and CSI-RS frequency resources within DL sub-band are valid for the UE.

RAN4 to specify CSI reporting requirements for SBFD and prioritize CQI reporting with UE-to-UE CLI.

RAN4 to specify PDSCH demodulation requirements for SBFD and take into consideration the presence of UE-to-UE CLI.

Conclusion

In this contribution, we provide our views on demodulation performance requirements for Rel-19 Evolution of NR duplex operation: Sub-band Full Duplex (SBFD). The following Observations and Proposals were made:

Observation 1: SBFD is specified for NR TDD bands in both FR1 and FR2-1.

Observation 2: For FR1 the BS channel bandwidth for SBFD operation shall be no less than 40MHz and no less than 20MHz for band n39.

Observation 3: For FR2-1, the BS channel bandwidth for SBFD operation shall be no less than 100MHz.

Observation 4: For FR1, current channel bandwidth assumption for UE/BS demodulation is 40 MHz, while SBFD is especially intended for large carrier bandwidths, e.g. 100 MHz, due to lower relative overhead of the guard-bands.

Proposal 1: RAN4 to prioritize the minimum bandwidth sizes of SBFD for both FR1 (40 MHz) and FR2-1 (100 MHz) NR TDD bands for SBFD demodulation performance requirements. If additional bandwidth is required, RAN4 to consider 100 MHz for FR1.

Observation 5: With UL muting, the UE shall apply 3 dB power boosting (over the active REs within UL muting symbols) and rate-matching around the muted REs.

Observation 6: In SBFD, the PUSCH demodulation at SBFD-capable gNB is affected by the gNB self-interference and the gNB-to-gNB CLI.

Observation 7: Referring to 38.858, self-interference can be modelled as additive white gaussian noise with fixed INR = - 6 dB targeting 1 dB desense.

Proposal 2: RAN4 to first discuss whether to follow 38.858 by treating SBFD interference signals including gNB self-interference as AWGN in the link-level simulation before agreeing on having PUSCH demodulation requirements for SBFD or not.

Proposal 3: RAN4 to discuss whether RACH configuration option 2 is expected to have differing demodulation performance. If not, then requirements for PRACH are not necessary.

Observation 8: In 38.101-4, the minimum requirements for wideband CQI reporting with inter-cell interference are specified.

Observation 9: In SBFD, the UE-to-UE CLI will impact DL reception and the reported CQI.

Observation 10: Furthermore, in SBFD, only PDSCH and CSI-RS frequency resources within DL sub-band are valid for the UE.

Proposal 4: RAN4 to specify CSI reporting requirements for SBFD and prioritize CQI reporting with UE-to-UE CLI.

Proposal 5: RAN4 to specify PDSCH demodulation requirements for SBFD and take into consideration the presence of UE-to-UE CLI.

- References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), 3GPP TSG RAN104, Huawei, Shanghai, China, June 17-20, 2024

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, 3GPP TSG RAN4#114, Athens, GR, February 17 – 21, 2025

- 3GPP TR 38.858, Study on Evolution of NR Duplex Operation, V18.2.0, 12-2024

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V18.0.0, 03-2024

- 3GPP TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.1.0 (2025-03)