3GPP TSG-RAN WG4 Meeting # 115	R4-2507632

# Malta, MT, 19th – 23th May, 2025

Source:	LG Electronics

Title:	Discussion on RF requirement for A-IoT device 1

Agenda item:	7.26.3.2

Document for:	Discussion

Introduction

In 3GPP RAN4 #114-bis, WF[1] was approved. In this paper, we provide our views on RF requirement for device 1.

Issue 3-1-1: Transmit output power Agreement: - Performance metric of transmit output power: - Backscattering loss - Backscattering output power is defined as the minimum backscattered power of Device 1 at certain input CW Tx power - Minimum Backscattering power at device antenna= Input CW Tx power at device antenna-Backscattering loss - For the Input CW Tx power to define requirements: - FFS on the whether to use one value or a range of CW Tx power - FFS on the value(s) of CW Tx power - Consider following aspects to determine backscattering loss to define requirements: - modulation loss - antenna efficiency - backscattering circuitry loss - impairment margin - minimum distance to the reader - differential radar cross section in two different modulation states - differential reflection coefficient in two different modulation states - the spectral components of the backscatter signal to be included in the power measurement, e.g. only the 1st Lower SB and 1st Upper SB, excluding the reflected carrier itself - Other factors not excluded

Discussion

Backscattering loss

  - Modulation loss

Modulation loss depend on chip and antenna parameter. It can be written as.

Kmodulation loss, dB=10*log10ρ1-ρ224

ρ=Z-Za*  Z+Za

ρ:reflection coefficient

The modulation coefficient is a measure of how effectively a tag can modulate the backscattered signal that it reflects back to the reader. It essentially quantifies how strong the change in the reflected signal is when the tag switches between two impedance states. ρ1, ρ2is refection coefficient for two state. One of the state is matched(Absorb), the other state is open.

| Z1 | Matched |
| --- | --- |
| Z2 | Open |
| ρ1 | 0 |
| ρ2 | 1 |
| K | 6dB |

Proposal 1: Use 6dB as Modulation loss for Ambient IoT device1

  - Antenna efficiency

Antenna efficiency can express as follow:

Antenna efficiency(dB)= Gain(dB)-Directivity(dBi)

Antenna type of device 1 assume dipole antenna. In general, usual small antenna for ambient IoT have Gain 0dBi.  And also TR also specify 0dBi for antenna gain. And general dipole antenna directivity is 2.15dBi. through above equation, we can calculate antenna efficiency as -2.15dB(0dB-2.15dBi). Since it enters and exits the antenna, it has a value that is doubled, so it has an antenna efficiency of -4.3 dB.

Proposal 2:  Use -4.3 dB as antenna efficiency for Ambient IoT device 1

  - backscattering circuitry loss

Backscattering circuitry loss refers to the power loss inside tag circuitry during the backscatter modulation process. In device 1, the tag modulates and reflect the incoming RF signal from the reader by switching its load impedance. This loss arises from two kind of things. First one is impedance mismatch between the antenna and the chip. Second one is internal losses in the switching/modulation circuit of the chip. In many RFID paper and models backscattering circuitry loss is treated as equivalent to modulation loss.

Observation 1: Backscattering circuitry loss is treated as equivalent to modulation loss.

Then in case of backscattering circuitry loss could be considered with internal loss like ohmic loss and transmission loss. We can assume internal loss as about 1dB.

Proposal 3: Use 1dB as backscattering circuitry loss only for internal loss.

  - Summary

| Modulation loss | Antenna efficiency | Backscattering circuitry loss | Impairment loss | Backscattering loss |
| --- | --- | --- | --- | --- |
| 6dB | -4.3dB | 1dB | 3dB | 14.3dB |

Proposal 4: Ambient IoT device 1 backscattering loss used to define requirement is 14.3 dB.

- 2. Conclusion

In this contribution, we want to share some views on Ambient IoT device1 for RF requirements and proposals are made as following:

Proposal 1: Use 6dB as Modulation loss for Ambient IoT device1

Proposal 2:  Use -4.3dB as antenna efficiency for Ambient IoT device 1

Observation 1: Backscattering circuitry loss is treated as equivalent to modulation loss.

Proposal 3: Use 1dB as backscattering circuitry loss only for internal loss.

Proposal 4: Ambient IoT device 1 backscattering loss used to define requirement is 14.3 dB.

- 3. Reference

[1] R4-2505230, “WF on RF requirements for A-IoT”