3GPP TSG-RAN WG4 Meeting #115	R4-2507633

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.13.3.2

Source:	Nokia

Title:	On OTA test enhancements

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#114 regarding OTA test enhancements. This contribution further elaborates Nokia’s views on the items related to OTA test enhancements.

# Discussion

## Horn antenna aspects

The following was agreed in the WF [1] of RAN4#114bis:

Way forward: Companies to investigate applicable frequency range for horn antenna studies (i.e., where the reference CLTA may not be available).

We have been studying extensively on evaluating the performance of wideband horn antennas as CLTA’s and our previous measurement results can be found in our contributions [2], [3] and [4]. From those measurements, it could be seen that SH400 performs well for frequencies up to ~2GHz. As it has been difficult to compare the performance of wideband horn antennas and CLTA’s with co-location measurements on higher frequencies, we further studied the measured response of wideband horn antenna and CLTA with known power level using signal generator and signal analyser and antenna panels of 3.5GHz and 7GHz. Measurements were repeated with using EMI receiver to replace signal analyzer.

SH400 and LB-2080 horn antennas were used in measuring the 3.5GHz antenna panel response. Both +/- 45° polarities were measured and sum results are presented. The horn antennas have a considerable size difference that is captured in the setup figures. This most likely affects the measurement results.

SH400 and LB-2080 have a significant size difference.

Figure 1. SH400 measurement setup.

Figure 2. LB-2080 measurement setup.

Figure 3. CLTA measurement setup.

Four closest sub-arrays were measured from the antenna panel with SH400, LB-2080 and CLTA for 3.5GHz and results are shown in Table 2.

Table 1. Measurement results for 3.5GHz.

|  | CLTA [dBm] | CLTA [dBuV] | SH400 [dBm] | SH400 [dBuV] | LB-2080 [dBm] | LB-2080 [dBuV] |
| --- | --- | --- | --- | --- | --- | --- |
| SA1 | -37.0 | 67.7 | -48.9 | 51.0 | -47.7 | 53.4 |
| SA2 | -35.0 | 69.1 | -45.4 | 58.1 | -38.0 | 67.0 |
| SA3 | -33.5 | 69.8 | -45.5 | 58.0 | -46.7 | 69.0 |
| SA4 | -35.7 | 65.7 | -49.5 | 56.5 | -62.0 | 59.4 |

It can be seen from the results that SH400 is more consistent with comparison with CLTA, while keeping in mind that there is an offset in the result. This is most likely due to the beam width of the antenna and structure of the antenna. The horn antennas used are structured to have metallic sides on two sides of the antenna as they are intended to be used directly pointing to the DUT. Also wideband horn antennas receive different frequency bands in different parts of the antenna. The higher frequencies are received in the back part of the antenna as shown for SH400 in Figure 4, and it might be required to move the horn antenna forward according to measured frequency band to get similar results. It must be noted that SH400 is almost a half a meter long antenna so this distance most likely has an effect. Also, it can be seen that as the LB-2080 is much smaller in size than SH400, there is more variation in the results depending on what subarrays are measured. The size of the horn antenna plays a major role.

Wideband horn antenna receives different frequency bands in different part of the antenna.

Wideband horn antenna results might be more comprehensive if antenna was moved forward to reflect the received surface to the radome of the DUT.

During the measurements, the front part of the horn antenna was aligned with the radome of the DUT, which might not give correct results. LB-2080 differences might be explained by the small size of the antenna compared to the size of the DUT. SH400 is not that far from the size of the DUT.

Figure 4. SH400 structure view.

For the 7GHz measurements, LB-2080 was used again in similar setup as for 3.5GHz.

Results are shown in Table 2.

Table 2. Measurement results for 7GHz.

|  | CLTA [dBm] | CLTA [dBuV] | LB-2080 [dBm] | LB-2080 [dBuV] |
| --- | --- | --- | --- | --- |
| SA1 | -57.4 | 53.6 | -50.2 | 53.3 |
| SA2 | -51.3 | 49.5 | -54.8 | 54.7 |
| SA3 | -54.1 | 51.1 | -50.8 | 54.4 |
| SA4 | -58.0 | 46.1 | -58.4 | 49.8 |

For 7GHz, the LB-2080 performs much better than for 3.5GHz. Still there are some differences in the results, which can be explained by the horn antenna structure and size.

One thing that can be noticed from the measurements is that the measurements give similar results when measured either with signal analyzer or EMI receiver. Only major difference is the measurement unit. As we are working in the near-field with distance of 0.1m between the DUT and the measurement antenna, things get tricky. Table 10.6.2.1-1 in TS 38.104 defines the OTA out-of-band blocking performance requirement as:

The equation in NOTE 2 is correct when the distance is 30m, which is far-field conditions but might not be true anymore when the distance is 0.1m in near-field conditions. Therefore, a new method for the co-location tests might be needed.

## Capturing conclusions on Core aspects

The following was agreed in RAN4#114bis:

Way forward: Previous WF (R4-2502279, RAN4#114) on the timeline for BS-to-BS isolation data collection (i.e., ”conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting”) is amended as follows: - Close BS-to-BS isolation data collection by RAN4#115 (May 2025) to allow drawing initial conclusions. NOTE: Further BS-to-BS isolation data provision during RAN4#116 (August 2025) is not precluded. - For RAN4#115 (May 2025) provide Draft CRs to TR 37.941 with the initial summary of the findings on the following areas (work-split proposed below): - Background and motivation for CLTA-alternatives discussion (Huawei), - BS-to-BS isolation analysis, including related assumptions (Ericsson), - Impact of BS-to-BS isolation analysis on the legacy co-location requirements (ZTE), - CLTA alternatives: horn antenna (Nokia).

As indicated in the WF, we will provide draftCR for CLTA alternatives: horn antenna. A draftCR will be available in RAN4#115. We have been providing wideband horn antenna measurements and analysis for several meetings with different configurations. Wideband horn antennas are extremely versatile to be used as alternative for CLTA. CLTA’s have a big problem when going higher in frequency. CLTA’s are not commercially available, which might lead to a situation where custom-made CLTA’s can be designed to work specifically well with certain DUT. These test antennas are not available for third party testing laboratories. One big problem also regarding CLTA’s is the vast number of needed CLTA’s for different frequencies. This is where wideband horn antennas would be very beneficial to be used as CLTA’s. One wideband horn antenna could cover up to five different CLTA’s.

Wideband horn antennas have a wide range of supported frequency and are available up to 110GHz. There are some basic differences compared to CLTAs:

- Wideband horn antennas are intended to be used directed towards the DUT.

- Wideband horn antennas are different size than CLTA.

- Wideband horn antennas are receiving different frequencies in different depths of the horn.

- Wideband horn antenna structure is semi-open from two sides and closed from two sides or closed from all four sides.

There are many aspects to be considered using wideband horn antennas as CLTAs. Current CLTA arrangements might not be optimal for wideband horn antennas, e.g. same plane orientation, 10cm offset enclosure to enclosure etc. According to the measurements we have done, wideband horn antennas provide very similar results than CLTA for certain frequency ranges but might give too pessimistic results for other frequencies.

Summary of measurements:

2 wideband horn antennas were measured and results were compared to CLTA for the FR1 frequencies, namely SH400 and LB-2080.

- SH400 performs better for lower frequency bands, where the LB-2080 performs better in the higher frequency bands.

- The size of the wideband horn antenna has a significant effect on the results.

- There is no immediate correlation between the coupling loss in the near-field, increasing distance from 10cm to 20cm or even to 50cm doesn’t guarantee higher coupling loss.

- CLTA arrangement as such is not optimal for wideband horn antennas due to the structure of the antenna.

- There is no straightforward one to one translation to use wideband horn antenna as CLTA, with the current CLTA arrangement.

Single column passive antennas are not readily available for all of the required frequency bands. They typically meet the CLTA requirements only at relatively narrow frequency ranges. Currently a lot of custom-made antennas are needed in many cases. This makes testing especially in higher and lower frequencies become highly complex and time-consuming. Therefore, a new method for the co-location tests might be needed.

## BS-to-BS isolation analysis

Regarding the BS-to-BS isolation analysis, the following was agreed in RAN4#114bis:

Way forward: Current baseline for BS-to-BS isolation analysis is the measurement and simulation data as captured in the previous WF (i.e., R4-2502279, RAN4#114).

The following was agreed in RAN4#114:

Way forward: BS-to-BS isolation data collection to prioritize the following scenarios: - Horizontal domain prioritized (vertical BS stacking analyses not precluded); - Same-band analysis (inter-band analysis not precluded), - Array-to-array case (array to sub-array not precluded); - Same BS type for aggressor and victim (mixed cases of various BS types not precluded). Guidance for collection of future results: Companies to clarify on the statistical distribution of the results provided (i.e., worst case, CDF, etc.) For the derivation of the conclusion, [95-99th] percentile of CDF is expected to be used for simulations, or the worst-case results from measurements. BS-to-BS isolation data (simulations, or measurements) to be collected on top of the data already collected in the following table formats: - No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 7 GHz. Isolation in vertical domain (dB); ZTE (M, R4-2501875): -54; ZTE (R4-2501875): -52; ZTE (M, R4-2501875): -56. Isolation in horizontal domain (dB); ZTE (M, R4-2501875): -31; ZTE (M, R4-2501875): -34 Huawei (M, R4-2502135): -46; ZTE (M, R4-2501875): -46. - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling. - Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated): 2.6 GHz; 3.5 GHz; 4.9 GHz; 6.7 GHz --> 7 GHz. Isolation in vertical domain (dB). Isolation in horizontal domain (dB); S: Ericsson (R4-2418398): A-to-SA: -55±20 dB; S: Ericsson (R4-2502160): A-to-A: -77±25; A-to-SA: -70±20. For the deprioritized scenarios, reuse table templates as above. - Timeline for BS-to-BS isolation data collection: conclusion on the BS-to-BS isolation analysis to be reached no later than August 2025 meeting (RAN4#116). - Data collection for other carrier frequencies is not precluded. - Any future conclusion on isolation numbers is subject to availability of sufficient amount of data from multiple companies and can be decided case-by-case.

Below is our analysis for RAN4#114bis (R4-2504191). It is introduced again for capturing our BS-to-BS isolation results.

CLTA was used to measure coupling loss between an antenna array and CLTA for 3.5GHz and 7GHz. CLTA arrangement was used with the same plane orientation, center alignment in vertical domain and 10cm distance between CLTA and DUT. Both +/- 45° polarizations were measured. Beamforming was not applied for this measurement. There were four different sub-arrays measured as shown in Figure 4.

Figure 5. Setup for the coupling measurements

Four sub-arrays (SA1…SA4) of the DUT were measured separately with keeping the CLTA arrangement constant. There are differences between sub-arrays as depicted in Table 3.

Table 3. Coupling results for 3.5GHz and 7GHz.

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in horizontal domain (dB) |  | 37.0 (SA1) 35 (SA2) 33.5 (SA3) 35.7 (SA4) |  | 57.4 (SA1) 51.3 (SA2) 54.1 (SA3) 58.0 (SA4) |

The results show that the coupling loss for 3.5GHz frequency without considering beam steering was ranging from 33.5dB to 37.0dB. For the 7GHz frequency results were ranging from 51.3dB to 58dB, respectively. The worst case value for 3.5GHz would be 33.5dB and the average would be 35.3dB. Similar results for 7GHz are 51.3dB and 55.2dB, respectively.

Coupling results for 3.5GHz without beam steering are in the neighborhood of 35dB.

Coupling results for 7GHz without beam steering are in the neighborhood of 55dB.

# Conclusion

This document shares Nokia’s views on OTA test enhancements. Following observations and proposals were introduced:

- SH400 and LB-2080 have a significant size difference.

- Wideband horn antenna receives different frequency bands in different part of the antenna.

- Wideband horn antenna results might be more comprehensive if antenna was moved forward to reflect the received surface to the radome of the DUT.

- Coupling results for 3.5GHz without beam steering are in the neighborhood of 35dB.

- Coupling results for 7GHz without beam steering are in the neighborhood of 55dB.

- When collecting data for beam steering scenarions, it makes sense to reuse CLTA arrangements.

- If specifications for AAS are loosened for frequencies >2.5GHz, there might be mismatch between the RRH requirements.

# References

- R4-2504701 “Way Forward for CLTA”, RAN4#114bis, Wuhan, China, 7th-11th April, 2025.

- R4-2415674 ”On OTA test enhancements - CLTA”, RAN4#112bis, Hefei, China, 14th – 18th October, 2024.

- R4-2418018 ”On OTA test enhancements”, RAN4#113, Orlando, USA, 18th – 22nd November, 2024.

- R4-2501323 “On OTA test enhancements”, RAN4#114, Athens, Greece, 17th – 21st February, 2025.