3GPP TSG-RAN WG4 Meeting #115	R4-2507636

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.25.2

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#114bis [1] regarding evolution of NR duplex operation. There were many agreements made regarding SBFD general aspects and system parameters. This contribution displays Nokia’s view on SBFD general aspects and system parameters.

# Discussion

## CLI handling impact on RF requirement

The following was agreed in RAN4#112 and remained open for RAN4#112bis and RAN4#113:

- Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

Regarding the FFS, our preference is to define the BS RF requirements without considering the potential impact of gNB-to-gNB CLI handling mechanisms, i.e., the FFS should not be considered. First, for the sake of progress we should not risk slowing down the work in RAN4 with revisiting some requirements pending on RAN1 progress. Secondly, the CLI handling schemes discussed in RAN1 are mainly applicable to co-channel CLI and their applicability to adjacent channel CLI is questionable. In fact, RAN1 discussions on gNB-to-gNB CLI handling currently focuses on enabling time/frequency coordination and spatial coordination (beam nulling and beam pairing). All the discussed mechanisms require, at least,

i) exchange of CLI-RS configurations across gNBs,

ii) identification of aggressor gNBs via measurements,

and iii) exchange across gNBs of measurement results and additional information to enable the aggressor-victim coordination.

All the above rely on signaling between gNBs over the Xn interface (intra-operator signaling). However, there is no standardized inter-operator interface and therefore exchange of information between operators is not possible. Moreover, the discussed CLI handling schemes are only optional features that are not expected to be supported by all the base stations in the field. Lastly, the impact of each CLI handling on the RF requirements is quite challenging to measure due to the fact that it is quite dependent on the specific deployment, load conditions, frequency of the information exchange across nodes, etc. The arguments above support the following proposal:

RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

## Declaration and flexibility for SBFD operation

### Declaration and flexibility for SBFD operation

The following was agreed in RAN4#114bis:

Agreement: - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

Our view has always been that the restrictions should also be applied to FR1 MR BS. After discussion in the last two RAN4 meetings, we are willing to compromise to the proposal of using a power level threshold for FR1 MR SBFD-capable BS. We feel that the justification of choosing the power level of 33dBm as proposed in the way forward makes sense.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

### Restriction on the number of UL subband sizes supported

The following was agreed in RAN4#114bis:

Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

The restriction on the number of UL subband sizes supported for FR1 MR SBFD capable BSs is closely related to the discussion on 2.2.1. The same method of approach should be followed on both. Therefore, with the same justification as above, we propose to choose the power level of 33dBm as the power level threshold for FR1 MR SBFD-capable BS.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to UL subband sizes supported.

### Restriction on the number of DL subband sizes supported

The following was agreed in RAN4#114bis:

Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains.

The restriction on the number of DL subband sizes supported for FR1 MR SBFD capable BSs is closely related to the discussion on 2.2.1. The same method of approach should be followed on both. Therefore, with the same justification as above, we propose to choose the power level of 33dBm as the power level threshold for FR1 MR SBFD-capable BS.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to DL subband sizes supported.

### Restriction on guardband size for SBFD

Agreement: - Restriction on guardband size for SBFD: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guardband. - The declaration of guardband size should ensure the benefits from SBFD operation. - In the RAN4 specification, the clause “It is expected that guardband size should be minimized to ensure the benefits from SBFD operation.” could be captured, and the detailed wording can be further refined.

In RAN4#114bis, there was a discussion about restricting the maximum number of RBs for guardband. We have been supporting to limit the GB size from the beginning of the WI but as there was no agreement reached and specification needed to support different sizes of GB for different implementations, there was no consensus on limiting the GB size. As we agreed no restriction on GB size in RAN4#114bis, there was an agreement to add clause stating that “GB size should be minimized to ensure the benefits from SBFD operation”. This sentence as such is misleading as the guardband plays a big role in terms of avoiding inter-subband interference, so smaller guardband does not always translate into better performance. If such note is needed, we think a more accurate formulation would be: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead”.

The guardband plays a big role in terms of avoiding inter-subband interference. Smaller guardband does not always translate into better performance.

If it is decided that a clause is introduced to the RAN4 specification related to guardband size, the text should say: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.”

### Restriction on DL subband size

Agreement: - If the SSB symbol is not overlapped with SBFD symbol, - There is no restriction on DL subband size. - If the SSB symbol is overlapped with SBFD symbol, - FFS the impact from RAN4 perspective.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. The following proposals were listed:

- The guardband plays a big role in terms of avoiding inter-subband interference. Smaller guardband does not always translate into better performance.

- RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to UL subband sizes supported.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to DL subband sizes supported.

If it is decided that a clause is introduced to the RAN4 specification related to guardband size, the text should say: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.”

# References

- R4-2504751 “Way Forward for [114bis][306] NR_duplex_evo_General”, RAN4#114bis, Wuhan, China, 7th-11th April, 2025