3GPP TSG-RAN WG4 Meeting #115	R4-2507637

Malta, MT, 19th – 23rd May, 2025

Agenda item:		7.25.3.1

Source:	Nokia

Title:	On potentially new BS RF requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#114bis [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the potentially new BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the potentially new BS RF requirements for SBFD operation.

# Discussion

## In-channel adjacent subband leakage ratio

The following was agreed in RAN4#114bis with the accompanying way forward:

Agreement: - The requirement (In-channel adjacent subband leakage ratio) is renamed as “in-channel adjacent subband leakage requirement” (by removing “ratio”).

Way forward: - FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage.

Way forward: - Absolute limit for in-channel adjacent subband leakage: - FFS by considering the following exemplary cases by performing analysis including measurement or simulation: - FR1: - DU/UD: 80MHz (DL) – 20MHz (UL) - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL) - Other cases are not precluded - FR2-1: - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL) - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL) - Other cases are not precluded - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

It was agreed in RAN4#114bis that RAN4 will still try to use a unified requirement for absolute limit. The unified limit should be applied in our view and the existing ACLR absolute basic limit values can be used for in-channel adjacent subband leakage requirements as described in Tables 6.6.3.2-2 and Table 9.7.3.3-2 in TS 38.104:

Table 6.6.3.2-2: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 9.7.3.3-2: BS type 2-O ACLR absolute limit

| BS class | ACLR absolute limit |
| --- | --- |
| Wide area BS | -13 dBm/MHz |
| Medium range BS | -20 dBm/MHz |
| Local area BS | -20 dBm/MHz |

Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage requirements.

Honestly, we feel it will be a mess if separate requirements will be introduced for all different configurations. Therefore, the most straightforward approach is to follow the existing absolute basic limits. RAN4 is also running out of time with the core part of the WI as the core part will be finished already in RAN4#116.

## In-channel adjacent subband blocking

The following way forwards were agreed in RAN4#114bis:

Way forward: - New requirement for in-channel adjacent subband blocking: - The requirement can be derived by considering co-site inter-sector case and/or inter-site case - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification. - The range of [-35dBm to -27dBm] can be used as the starting point for further study.

Way forward: - FFS the assumption of gNB-to-gNB CLI handling for two cases: - Inter-site case: - Note: for the blocking requirement, RX digital beamforming cannot be considered. - Co-site inter-sector case: - Note: Inter-sector is near field interaction.

In-channel adjacent subband blocking will have two interference cases, co-site inter-sector and inter-site. Both will have effects and need to be considered. Depending on the assumption of gNB-to-gNB CLI handling, either one could be more damaging than another. We feel that as there are many limitations on applying CLI mitigation (quite dependent on the specific deployment, load conditions, frequency of the information exchange across nodes, etc.) the requirement should be defined without gNB-to-gNB CLI mitigation. This would make the requirement always cover the worst-case scenario.

Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes.

The co-site inter-sector case has been discussed and the current WF presents a range of [-35dBm to -27dBm] that can be used as the starting point for further study. If there would be an agreed assumed isolation value between sectors, it should be noted that to ensure performance, this isolation value must be met and it should be mentioned in the specification. In our view, this isolation value could be agreed as 75dB. This would then be reflected in a blocking level of 54dBm – 75dB = -21dBm. This would be the case when only one sector is taken into account. As there are three sectors in the sectorized setup, the blocking level would actually be 3dB higher.

Introduce the in-channel adjacent subband blocking level of -18dBm.

# Conclusion

In this contribution, our views on the potentially new BS RF requirements for SBFD operation are presented. The following proposals were listed:

- Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage requirements.

- Starting point for the new requirement should be without any gNB-to-gNB CLI handling schemes.

Introduce the in-channel adjacent subband blocking level of -18dBm.

# References

- R4-2504751 “Way Forward for [114bis][306] NR_duplex_evo_General”, RAN4#114bis, Wuhan, China, 7th-11th April, 2025