3GPP TSG-RAN4 Meeting #115	R4-2507650

# St Julians, Malta, 19th – 23rd May, 2025

Title:	Cross Link Interference (CLI) handling impact on MPR for SBFD operation

Source:	Qualcomm Incorporated

Agenda item:	7.23.1

WI code:	NR_duplex_evo-Core

Release:	Rel-19

Lead WG: 	RAN1

Document for:	Discussion

# 1.	Introduction

In the RAN plenary #104 meeting, RAN-P approved some schemes for CLI handling under the WI of evolution of NR duplex operation [1]. One of the schemes for handing inter-gNB CLI is based on UL resource muting of the PUSCH for both CP-OFDM and DFT-s-OFDM waveform. In RAN1 #119 meeting, an LS was sent out to RAN4 to check the impact on transmit signal quality/MPR requirement due to UL resource muting [2].

RAN1 has made the following agreements on UL resource muting:

Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

# 2. 	Discussion on PUSCH with RE muting

In this section, we describe how UL resource muting (RM) is applied to PUSCH waveform. The muting pattern in frequency domain is based on comb-2 structure, where every other RE is reserved and not used for PUSCH resource mapping. The UL-RM is applied only to two OFDM symbol within the PUSCH.

Figure 0-1: UL resource muting for PUSCH

In CP-OFDM, the muting is applied in frequency domain (pre IFFT) in the muted PUSCH OFDM symbols. Since, half of the REs are muted, the amplitude of the remaining REs are multiplied by sqrt (2), to keep the same power. This will lead to higher PSD in the muted symbols that may affect emissions which could in turn impact MPR

Observation 1: Resource muting changes the waveform characteristics and could have an impact on MPR.

For DFT-s-OFDM waveform, the muting can be implemented by using one of the followings methods.

- Method #1: The DFT size on the muted symbol is half compared to the non-muted OFDM symbols. Then, the complex valued symbols after DFT are oversampled by factor of 2 (i.e. mapped to every other RE) at the input of the IFFT.

- Method #2: The DFT size is the same, however, the input samples are repeated to generate a comb-2 RE muting post DFT.

From the recommendation above of resource muting, it can be observed that in time domain muting happens only for two symbols and not for the entire slot. This raises the question of how should the emissions be calculated now for SBFD operation i.e still at a slot level or a symbol level?

Observation 2: With symbol level resource muting, emissions calculation (like ACLR, EVM, IBE, etc.)  might need to be revisited.

# 3. 	Analysis

Based on the waveform description in section 2, simulations are performed for a power class 2 (PC2) transmitter with following assumptions.

Waveforms : DFT-s-OFDM and CP-OFDM

Modulation : QPSK and 256QAM

Channel BW = 100 MHz with SCS = 30 kHz

UL Sub-Band Bandwidth = 20 MHz

Start RB index = 0

Use Case 1 : # RBs = 1:1:10  (SEM limited)

Use Case 2 : # RB = 51 (ACLR limited)

# of slots = 7

RE Muting = Disabled/Enabled

Muting Pattern : On symbol 3 and 7 of every slot

For the generated waveforms, linearity metrics namely ACLR, IBE, EVM are analyzed.

Difference in MPR due to RE muting in both the use cases is studied and presented.

Use Case 1: SEM limited

QPSK :

The change in MPR due to resource muting for DFT-s-OFDM and CP-OFDM QPSK waveforms is shown in Table 1 and Table 2 respectively. For the waveforms starting at the left edge of channel bandwidth which are limited by SEM, it can be observed that there is less than 0.5 dB power loss for resource muted waveforms.

Table 1 : Delta MPR for DFT-s-OFDM QPSK Waveform

| Start Index, # of RBs | Delta MPR (dB) |
| --- | --- |
| 0, 1 | 0.04 |
| 0, 2 | 0.04 |
| 0, 3 | -0.12 |
| 0, 4 | -0.12 |
| 0, 5 | -0.04 |
| 0, 6 | 0 |
| 0, 7 | 0 |
| 0, 8 | 0.08 |
| 0, 9 | 0.04 |
| 0, 10 | 0 |

Table 2 : Delta MPR for CP-OFDM QPSK Waveform

| Start Index, # of RBs | Delta MPR (dB) |
| --- | --- |
| 0, 1 | 0.36 |
| 0, 2 | 0 |
| 0, 3 | -0.17 |
| 0, 4 | 0.13 |
| 0, 5 | -0.12 |
| 0, 6 | -0.01 |
| 0, 7 | -0.37 |
| 0, 8 | -0.25 |
| 0, 9 | -0.31 |
| 0, 10 | -0.21 |

256 QAM : Results to be added

Observation 1: Resource muting for the studied QPSK waveforms at the left edge of the channel have less than 0.5 dB impact on MPR.

Proposal 1 : Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with allocations at the edges of the channel have minimal impact on the transmit waveform quality.

Use Case 2: ACLR limited

Similarly, the change in MPR due to resource muting for DFT-s-OFDM and CP-OFDM QPSK waveforms with full uplink allocations is shown in Table 3. For these ACLR limited waveforms, it can be observed that there is almost no difference in MPR due to resource muting.

Table 3 : Delta MPR Comparison for QPSK Waveforms with full allocation

| Start Index, # of RBs | DFT-s-OFDM : Delta MPR (dB) | CP-OFDM : Delta MPR (dB) |
| --- | --- | --- |
| 0, 51 | 0 | 0.03 |

Observation 2: Resource muting for the studied QPSK waveforms with full allocation has negligible impact on MPR.

Proposal 2 : Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with full allocations have no impact on the transmit waveform quality.

Proposal 3: Resource muting should be studied for the remainder of QPSK allocations along with other modulations to gauge its impact on transmit waveform quality.

# Conclusion

We discussed an approach to construct a waveform to combat CLI in SBFD operation with resource muting and its impact on UE Tx performance.

Observation 1: Resource muting for the studied QPSK waveforms at the left edge of the channel have less than 0.5 dB impact on MPR.

Proposal 1 : Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with allocations at the edges of the channel have minimal impact on the transmit waveform quality.

Observation 2: Resource muting for the studied QPSK waveforms with full allocation has negligible impact on MPR.

Proposal 2 : Resource muted QPSK DFT-s-OFDM and CP-OFDM waveforms with full allocations have no impact on the transmit waveform quality.

Proposal 3: Resource muting should be studied for the remainder of QPSK allocations along with other modulations to gauge its impact on transmit waveform quality.

# Reference

- 3GPP TSG RAN Meeting #104, RP-241614 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”

- R1-2410889, LS on impact on transmit signal quality/MPR requirement