3GPP TSG-WG4 Meeting # 115	R4-2507655

# Malta, MT, 19 - 24 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5691 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on corrections to general aspects of gapless positioning measurements and core requirements for RedCap positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR-Pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To remove square brackets around the IE MUSIM-GapConfig-17 in Clause 9.9.1.2. - IEs in core requirements for RedCap positioning measurements is not aligned with IEs in LPP signaling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Square brackets around the IE MUSIM-GapConfig-17 in Clause 9.9.1.2 is removed. - IEs in core requirements for RedCap positioning are aligned with the IEs in LPP signaling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Core requirement for gapless positioning measurement for UE capable of supporting MUSIM-GapConfig-17 is unclear. - IEs in core requirements for RedCap positioning do not map to IEs used in LPP signaling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.6, 4.6.3.6, 9.9.1.2, 9.9A.2.6.1, 9.9A.3.6.1, and 9.9A.4.8. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## START OF CHANGE # 1

#### 4.6.2.6	Measurement Period Requirements with RX FH

The requirements in this clause apply when a RedCap UE is requested by the LMF to perform measurements with FH, and UE reports measurement based on multiple hops.

The requirements in clause 4.5.2.5 shall apply with the following modifications.

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single time window.

-	Lavailable_PRS,i=j=1JNhop,i,j*Lper-hop,i,j, where

-	Nhop,i,j is the number of hops that UE can do within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 4.6.2.6-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetuneTimeFR1 or rf-RxRetuneTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop within a time window instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the time window instance considering the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 4.6.2.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Thop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15 kHz, 30 kHz, 60 kHz in FR2, 120 kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i ,

where

-	Nhop,max,i is the maximum number of Rx hops signaled as reported by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i,

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

where NrepPRS is the number of PRS repetitions within the time window occasion, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap and Nhopsslot is the applicable number of hops per slot as defined in table 4.6.2.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap ,

where

BWPRS is the minimum among:

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34],

Nhop,i,j is number of hops within a single time window as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

## END OF CHANGE # 1

## START OF CHANGE # 2

#### 4.6.3.6	Measurement Period Requirements with RX FH

When physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], and UE is configured to perform PRS-RSRP measurement with RX FH via nr-DL-PRS-RxHoppingRequest-r18 as defined in TS 37.355 [34], the UE shall be able to measure multiple (up to the UE capability specified in clause 4.6.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], during the measurement period TPRS-RSRP,Total defined in clause 4.6.3.5 with using the following definition for Nsample and Lavailable_PRS,i.

Nsample is the number of PRS-RSRP measurement samples, where

-	Nsample= 2 if the RedCap UE supports reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

A measurement sample under RX FH is defined as a PRS measurement over multiple hops.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,j ,

where,

-	Nhop,i,j is the number of hops that UE can perform within a time window occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in time window occasion j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered;

-	J is the number of time window occasions during Tavailable_PRS,i.

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 4.6.3.6-1, and RRTFH is the retuning time between Rx hops indicated by UE in rf-RxRetuneTimeFR1 or rf-RxRetuneTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the measurement time window, defined in clause 4.6.1, taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 4.6.3.6-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15 kHz, 30 kHz in FR1, and 60 kHz, 120 kHz in FR2 | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single time window Nhop is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i	,

where

-	Nhop,max,i is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability for positioning frequency layer i as specified in [34].

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i.

-	Nhops,effect,i,j is the effective number of Rx hops within a single time window occasion j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within a single time window, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in table 4.6.3.6-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within a time window instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop,i,j-1*BWoverlap	,

where

BWPRS is the minimum among:

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34].

Nhop,i,j is number of hops within a single time window as defined above.

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34].

BWoverlap is the UE capability on BW of the overlapping PRB indicated via numOfOverlappingPRB-r18 as specified in [34].

## END OF CHANGE # 2

## START OF CHANGE # 3

#### 9.9.1.2	General Aspects of Gapless Measurement

The requirements for RSTD, PRS-RSRP, UE Rx-Tx time difference, and PRS-RSRPP measurement without measurement gaps specified in clauses 9.9.2.7, 9.9.3.6, 9.9.4.6 and 9.9.6.6 shall apply provided that:

Positioning frequency layer to be measured is within an active BWP,

UE is configured with PPW, and the PPW for the active BWP containing the positioning frequency layer to be measured is activated,

No active BWP switching occurs during the measurement period specified in clauses 9.9.2.7, 9.9.3.6, 9.9.4.6, and 9.9.6.6,

PRS is within PPW and does not overlap with other signals/channels of higher priority,

-	for PPW type 1A/1B, the PPW does not overlap with any symbol for SSB-based RLM/BFD/CBD/L1-RSRP/L1-SINR measurement on any CC or for SSB based RRM measurement on any MOs that are measured outside measurement gaps,

-	for PPW type 2, PRS does not overlap with any symbol for SSB-based RLM/BFD/CBD/L1-RSRP/L1-SINR measurement on any CC or for SSB based RRM measurement on any MOs that are measured outside measurement gaps,

max∣ΔT∣≤ THR, where

∆T is the time difference between the start of a slot containing PRS from the neighbor cell/TRP and the start of the closest slot from the serving cell;

the range of ∆T is determined by the expected RSTD and expected RSTD uncertainty in the assistance data;

THR is the threshold as reported in UE capability prs-MeasurementWithoutMG-r17.

SCS of PRS within PPW and SCS of DL active BWP are the same.

All measurement requirements specified in clauses 9.9.2.7, 9.9.3.6, 9.9.4.6 and 9.9.6.6 shall apply without DRX as well as for any DRX configuration specified in TS 38.331 [2].

The UE is not required to perform additional SSB measurement for the SSB configured as QCL source of PRS resources.

The UE is only required to measure PRS resources that are unmuted and fully or partially overlapped with PPW, and the requirements in clauses 9.9.2.7, 9.9.3.6, 9.9.4.6 and 9.9.6.6 are applicable to PRS resources that are unmuted and fully or partially overlapped with PPW.

A PRS resource is considered to be fully (partially) overlapped with PPW if all (some) of its instances are overlapped with a PPW occasion. A PRS resource instance is considered to be overlapped with PPW occasion if the minimum number of unmuted repetitions of the instance taking into account Rx time difference between serving and non-serving cell is fully covered by the PPW , where the minimum number is given in the accuracy requirements in clause 10.1.23, 10.1.24, 10.1.25 and 10.1.38 for RSTD, PRS-RSRP, UE Rx-Tx time difference and PRS-RSRPP, respectively.

When UE is configured with measurement for more than one positioning requests, the measurement period for each request may be longer than measurement period when UE is configured with measurement for single positioning request.

When UE supports MUSIM-GapConfig-17 and the PPW occasion of the target positioning frequency layer is overlapping with the configured aperiodic MUSIM gap, longer period for the PRS measurement is expected than the requirements as defined in clause 9.9.2.7, 9.9.3.6, 9.9.4.6 and 9.9.6.6.

## END OF CHANGE #3

## START OF CHANGE # 4

##### 9.9A.2.6.1	Measurements Period Requirements with FH with MG

When UE physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure DL RSTD measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in clause 9.9A.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in clause 9.9A.2.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

-	Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

-	Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,j

where,

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 9.9A.2.6.1-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetuneTimeFR1 or rf-RxRetuneTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.2.6.1-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Thop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15 kHz, 30 kHz, 60 kHz in FR2, 120 kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within the MG occasion, excluding the gap retuning times, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in table 9.9A.2.6.1-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap

where

BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18 as specified in [34], and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18.as specified in [34],

Nhop,i,j is number of hops within a single MG occasion as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34],

BWoverlap BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

## END OF CHANGE # 4

## START OF CHANGE # 5

##### 9.9A.3.6.1	Measurements Period Requirements with FH with MG

When the UE the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure PRS-RSRP measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCapUE capability specified in clause 9.9A.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms as defined clause 9.9A.3.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

-	Nsample is the number of PRS-RSRP measurement samples, where

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

-	Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i= j=1JNhop,i,j*Lper-hop,i,j

where,

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 9.9A.3.6.1-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetuneTimeFR1 or rf-RxRetuneTimeFR2 via NR-DL-PRS-ProcessingCapability [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.3.6.1-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15 kHz, 30 kHz, 60 kHz in FR2, 120 kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within the MG occasion, excluding the gap retuning times, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in table 9.9A.3.6.1-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop*BWper-hop-Nhop-1*BWoverlap

where

-	BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18, and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in [34],

Nhop,i,j is number of hops within a single MG occasion as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in [34],

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in [34].

## END OF CHANGE # 5

## START OF CHANGE # 6

#### 9.9A.4.8	Measurements Period Requirements with FH

The requirements in this clause apply when a RedCap UE is requested by the LMF to perform measurements with FH, and UE reports measurement based on multiple hops.

The requirements in clause 9.9A.4.5 shall apply with the following modifications.

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-CONNECTED specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples. Otherwise, Nsample= 4.

-	A measurement sample with FH is defined as a PRS measurement over multiple hops within a single measurement gap.

-	Lavailable_PRS,i=j=1JNhop,i,j*Lper-hop,i,j, where

-	Nhop,i,j is the number of hops that UE can do in an MG occasion j as defined in the following, and

-	Lper-hop,i,j is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured in MG instance j during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i,j, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

-	J is the number of MG instance during Tavailable_PRS,i.

The sampling window per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in table 9.9A.4.8-1, and RRTFH is the retuning time between Rx hops as reported by UE in rf-RxRetuneTimeFR1 or rf-RxRetuneTimeFR2 via NR-DL-PRS-ProcessingCapability specified in TS 37.355 [34]. The first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource fully or partially overlapped with the MG instance taking into account the expectedRSTD and expectedRSTD-uncertainty in the PRS assistance data.

Table 9.9A.4.8-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Thop) in number of symbols |
| --- | --- | --- | --- |
| RRTFH≤2 symbols | (2, 12) with SCS 15 kHz, 30 kHz, 60 kHz in FR2, 120 kHz | 2 | 7 |
|  | All others | 1 | 14 |
| 2 symbols <RRTFH≤6 symbols | (≤ 6, any) | 1 | 14 |
|  | (12, 12) | ½ | 28 |
| RRTFH>6 symbols | Any combination | ½ | 28 |

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in TS 37.355 [34] for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of inter-slot PRS repetitions within the MG occasion excluding the gap retuning times, which are unmuted and fully or partially overlapped with the sampling duration within each hop. MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap and Nhopsslot is the applicable number of hops per slot as defined in table 9.9A.4.8-1.

The measurement period requirement with the above modifications applies provided all PRS resources in each PFL have the same number of PRS repetitions NrepPRS within an MG instance and the same PRS repetition interval MrepPRS, otherwise the measurement period can be longer.

UE shall be able to measure PRS resources with multiple hops at least over the BW of BWtotal defined as

BWtotal=minBWPRS,Nhop,i,j*BWper-hop-Nhop-1*BWoverlap

where

-	BWPRS is the minimum among

-	configured PRS BW, and

-	UE capability of maximum PRS BW across all hops indicated via maximumPRS-BandwidthAcrossAllHopsFR1-r18 or maximumPRS-BandwidthAcrossAllHopsFR2-r18, and

-	total BW of all hops requested by LMF via nr-DL-PRS-RxHoppingTotalBandwidth-r18 as specified in TS 37.355 [34],

Nhop,i,j is number of hops within a single MG occasion as define above,

BWper-hop is the UE capability on PRS BW per hop indicated via supportedBandwidthPRS-r16 as specified in TS 37.355 [34],

BWoverlap is the UE capability on BW of the overlapping RB indicated via numOfOverlappingPRB-r18 as specified in TS 37.355 [34].

## END OF CHANGE # 6