3GPP TSG-RAN WG4 Meeting #115	    R4-2507666

Malta, MT, 19 – 24 May 2025

Agenda Item:	7.23.2

Source: 	Ericsson

Title:	General discussion on AI/ML for mobility

Document for:	Approval

Introduction

This contribution discusses generic issues related to AI/ML aided mobility being studied in Rel. 19 in RAN4. The discussions presented in this paper addresses impact of measurement error on prediction accuracy, data collection for network sided model, and view on scenarios to be captured in the TR of the study item.

Discussion

Impact of measurement error on prediction accuracy

To reduce the measurement overhead and measurement delay for network triggered L3-based mobility, prediction of RRM measurements in time domain, frequency domain, and spatial domain are being studied in Rel. 19. To predict the RRM measurement, the UE needs to perform measurements for model input. Based on the evaluations done by RAN2, the legacy measurements (L3 filtered RSRP measurements) can be used as an input to the AI/ML model by the UE for RRM measurement prediction. The results from the evaluation highlight that the RRM measurement prediction can provide gain in terms of the measurement overhead and delay reductions in comparison to the legacy approach. These gains should however not come at a cost of degradation in the accuracy of the measurement reported to the network for the L3-based mobility when the RRM measurements reported by the UE to the network are the predicted measurements.

The accuracy of measurements predicted using an AI/ML model depends on the accuracy of the measurements provided as an input to the AI/ML model. This has been acknowledged by RAN4 in previous meetings where the following agreement was reached.

Agreement from RAN4#114 [1]:

- For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

To acquire a holistic view of the aspects that have an impact on the performance of AI/ML aided mobility, at least the following two issues needs to be investigated by RAN4:

- Issue #1: Relates to the impact of error in the measurements used as input(s) to the AI/ML model on the accuracy of the predicted measurement.

- Issue #2: Relates to the accuracy of the measurement(s) used as input(s) to the AI/ML model to guarantee that the UE predicted measurement meets the requirements for L3-based mobility.

The agreement reached in RAN4#114 and the WF from RAN4#114bis concerns the issue #1 above. Due to the limited TU allocation, RAN4 may not have time to invest on running evaluations to resolve issue #1 and therefore it is likely that issue #1 remains unresolved by the end of the study item phase. This should however not limit RAN4 to acknowledge issue #2 that needs to be studied to ensure that the UE predicted measurement meets the requirements for L3-based mobility.

Proposal 1: RAN4 to identify required accuracy for the measurement(s) used as input(s) to the AI/ML model to guarantee the UE predicted measurement meets the requirements for L3-based mobility.

To streamline the discussion on handling the impact of error in input measurement on the accuracy of the predicted measurement, it is important to establish an understanding in terms of the performance requirement RAN4 anticipates to be met by AI/ML aided mobility measurement reporting. The fact that the network needs to make consistent mobility decisions for the UEs reporting the predicted measurements, and the UEs reporting the measured measurements, it is important that the predicted and measured measurements are confined within the same accuracy threshold. One way to achieve this is by ensuring that the predicted measurements at least meet the accuracy requirements that are defined for non-AI/ML based RRM measurements for mobility purposes.

Proposal 2: Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes.

In the last meeting statistical model to characterize the error in the measurement used as model input for RRM measurement prediction was discussed. No agreement on the error model could be reached, and therefore the following was captured in the WF document [1].

WF from RAN4#114bis [2]

- Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

The measurements performed by the UE are typically impacted by both RF impairment and the baseband error. To evaluate the impact of error in measurements used as input to the AI/ML model on accuracy of the predicted measurement both factors need to be considered in the simulations. Implementing RF impairment in a simulation setting is not straightforward as RF impairment can be due to many factors relating to hardware implementation. Exploiting a statistical model is therefore a reasonable approach to simplify the implementation of RF impairment in the simulation setting. To characterize RF impairment error in RAN4 simulation a truncated Gaussian distribution with zero mean and variance corresponding to the RF calibration error (±2.5dB for FR1 and ±4dB for FR2) for RSRP measurement can be considered. This is not only in line with the RAN4 assumptions for RF calibration error but is also aligned with the approach taken to evaluate the performance of AI/ML aided beam management [3].

Baseband error on the other hand refers to the estimation error. For example, error made by the UE when estimating the RSRP of the SSB signal received from a cell/beam. Baseband error depends on the signal strength, reference signal bandwidth, noise, interference, propagation condition etc. that can be easily implemented and evaluated in a simulation setting. It is therefore our proposal that a statistical model to characterize baseband error is not needed.

Proposal 3: Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements.

- RF error in dB for FR1 is characterized by Gaussian distribution with μ=0,  σ2 = 2.5 truncated at ±2.5.

- RF error in dB for FR2 is characterized by Gaussian distribution with μ=0,  σ2 = 4 truncated at ±4.

Requirements for LCM

The framework for LCM can be found in TR 38.843 [4]. The LCM framework consists of procedures for which requirements may need to be defined by RAN4. For example, activation/deactivation of the model, fallback to non-AI/ML operation, and performance monitoring are some of the procedures that will be executed by the UE for the case where the AI/ML model(s) for mobility is hosted by the UE. These procedures are being discussed in RAN4 to identify the potential requirements to be specified in the context of the Rel. 19 AI/ML for air interface work item. The agreements reached for the use cases supported in AI/ML for air interface can either be potentially applied to the AI/ML based mobility or can be used as a starting point for the discussion to take aspects that are specific to AI/ML based mobility into account.

For AI/ML aided mobility, AI/ML model can also be deployed at the network side. The network in this scenario may configure UE to report the measurements for training and or inference purposes commonly addressed by data collection procedure within the LCM framework in [4]. As the performance of the AI/ML model at the network side relies on the data reported by the UE, RAN4 should discuss to identify relevant requirements for the data collection procedure for the network sided model.

Proposal 4: RAN4 to identify requirements for UE reporting measurements for data collection for network sided model for AI/ML based mobility.

View on scenarios to be captured in TR

For AI/ML aided mobility, RAN2 identified 6 scenarios in Table 1 and prioritized scenarios 2, 3, and 4 for evaluation purpose only. Following the RAN2 prioritization, RAN4 agreed in RAN4#112bis meeting to also prioritize scenarios 2, 3, and 4 to study the impact of AI/ML aided mobility on RAN4 requirements with an objective to align RAN4 effort with RAN2 [5]. Where the agreement to prioritize scenarios allowed RAN4 to make an efficient use of the TU allocated for the study item, the agreement should however not be interpreted to limit frequency prediction use case to FR1 and temporal domain prediction to FR2 for example. Though RAN4 may not investigate the impact of AI/ML aided mobility in scenarios 1, 5, and 6 in the study item phase, the discussion on requirement for these scenarios shall not be precluded in the normative phase based on the scenario’s prioritization agreement reached in the study item phase. There may be a risk of misinterpretation of the agreement for scenario prioritization given that the v0.0.4 of TR38.744 contains a note (copied below for reference purpose) for Table 5.2.1-1 that says the table of scenarios will be removed in the final version of the TR. It is therefore our proposal to either make an agreement in RAN4 to ensure that the agreement on scenario prioritization in the study item phase should not limit the discussion on impact on RAN4 requirement for scenario 1, 5, and 6 in the normative phase provided the reporting framework for AI/ML aided mobility supports them or capture all scenarios at least in the RAN4 section with a note to clarify that scenarios 2, 3, and 4 were prioritized in study item but requirements for scenarios 1, 5, and 6 are not precluded to be specified.

Note from v0.0.4 of TR38.744 [6]:

- Editor Note 3: The table 5.2.1-1 is captured for study purpose. It may be removed in the final TR.

Table 1. Scenarios for AI/ML aided mobility.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Proposal 5: RAN4 to either make an agreement to ensure that the agreement on scenario prioritization in the study item phase does not limit discussion on impact on RAN4 requirement for scenario 1, 5, and 6 in the normative phase provided the reporting framework for AI/ML aided mobility supports them or capture all scenarios in section for RAN4 in TR38.744 with a note to clarify that discussion on scenarios 2, 3, and 4 were prioritized in study item but requirements for scenarios 1, 5, and 6 are not precluded to be specified.

Summary

In this contribution generic issues related to AI/ML aided mobility are discussed. The following proposals summarize the discussions on the impact of measurement error on prediction accuracy, data collection for network sided model, and view on scenarios to be captured in the TR of the study item, presented in this paper.

Proposal 1: RAN4 to identify required accuracy for the measurement(s) used as input(s) to the AI/ML model to guarantee the UE predicted measurement meets the requirements for L3-based mobility.

Proposal 2: Predicted measurements should at least meet the accuracy requirements that are defined for the non-AI/ML based RRM measurement reporting for mobility purposes.

Proposal 3: Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements.

- RF error in dB for FR1 is characterized by Gaussian distribution with μ=0,  σ2 = 2.5 truncated at ±2.5.

- RF error in dB for FR2 is characterized by Gaussian distribution with μ=0,  σ2 = 4 truncated at ±4.

Proposal 4: RAN4 to identify requirements for UE reporting measurements for data collection for network sided model for AI/ML based mobility.

Proposal 5: RAN4 to either make an agreement to ensure that the agreement on scenario prioritization in the study item phase does not limit discussion on impact on RAN4 requirement for scenario 1, 5, and 6 in the normative phase provided the reporting framework for AI/ML aided mobility supports them or capture all scenarios in section for RAN4 in TR38.744 with a note to clarify that discussion on scenarios 2, 3, and 4 were prioritized in study item but requirements for scenarios 1, 5, and 6 are not precluded to be specified.

References

R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).

R4-2504901, WF on AI/ML Mobility, Moderator (Nokia).

R4-2505152, Updated simulation assumptions for AI/ML BM, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, HiSilicon, MediaTek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT.

TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 3GPP.

R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).

TR 38.744 V0.0.4, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, (Release 19).