# 3GPP TSG-RAN WG4 Meeting #115	R4-2507673

St. Julian’s, Malta, 19th to 23rd May 2025

Agenda Item:	7.11.3

Source:	Ericsson

Title:	On co-existence simulation results for NR NTN Ku band

Document for:	Information

# 1	Introduction

In 3GPP RAN#104, a new Work Item (WI) was approved for Introduction of Ku band for NR NTN. The WI was further updated in RAN#106 [1]. One of the objectives of the WI is to –

- Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services.

WF [2] captured the agreements on co-existence simulation assumptions in the last meeting.

This contribution provides initial co-existence simulation results with VSAT parabolic antenna assumptions.

# 2	Discussion

In the following sections, we present our simulation results based on the agreed simulation parameters in WF [2]. This section is further divided based on the NTN scenarios – GEO and LEO600.

## 2.1 	GEO

In this sub-section, we share the simulation results for Case 1-8 for GEO NTN scenario. The simulation assumptions have been captured in the Annex of our companion contribution [3].

Some other assumptions to be considered are listed below –

- Antenna elevation angle is 90 degrees and 35 degrees.

- Mobile VSAT with UE height 1.5m (corresponding to L-ESIM)

- VSAT NF is 2.5 dB

- VSAT antenna pattern is parabolic.

- TN BS with AAS

The highlighted scenarios are the ones that have been agreed to be prioritized.

|  |  |  |  |  | ACIR at 5% tile |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 90 degrees | 35 degrees |
|  |  |  |  |  | Mobile VSAT - Parabolic | Mobile VSAT - Parabolic |
| Scenario | Aggressor | Victim | Contributing | Frequency | 2.5 dB NF | 2.5 dB NF |
| 1 | NTN GEO UL | TN UL | NTN UE ACLR | 14 GHz | 2 | 3 |
| 2 | TN UL | NTN GEO UL | NTN SAN GEO ACS | 14 GHz | 9 | 10 (avg) |
| 3 | NTN GEO UL | TN DL | NTN UE ACLR | 14 GHz | ~0 | ~0 |
| 4 | TN DL | NTN GEO UL | NTN SAN GEO ACS | 14 GHz | 0 | 16 (avg) |
| 5 | TN DL | NTN GEO DL | NTN UE ACS | 11 GHz | 35 | 30 (avg) |
| 6 | NTN GEO DL | TN DL | NTN SAN GEO ACLR | 11 GHz | ~0 | ~0 |
| 7 | NTN GEO DL | TN UL | NTN SAN GEO ACLR | 11 GHz | ~0 | ~0 |
| 8 | TN UL | NTN GEO DL | NTN UE ACS | 11 GHz | 26 | 18 (avg) |

## 2.2	LEO600

In this sub-section, we share the simulation results for Case 1-8 for LEO600 NTN scenario. The simulation assumptions have been captured in the Annex of our companion contribution [3].

Some other assumptions to be considered are listed below –

- Antenna elevation angle is 90 degrees and 35 degrees.

- Mobile VSAT with UE height 1.5m (corresponding to L-ESIM)

- VSAT NF is 2.5 dB.

- VSAT antenna pattern is parabolic.

- TN BS with AAS

The highlighted scenarios are the ones that have been agreed to be prioritized.

|  |  |  |  |  | ACIR at 5% tile |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 90 degrees | 35 degrees |
|  |  |  |  |  | Mobile VSAT - Parabolic | Mobile VSAT - Parabolic |
| Scenario | Aggressor | Victim | Contributing | Frequency | 2.5 dB NF | 2.5 dB NF |
| 1 | NTN LEO600 UL | TN UL | NTN UE ACLR | 14 GHz | ~0 | 8 |
| 2 | TN UL | NTN LEO600UL | NTN SAN LEO600 ACS | 14 GHz | 16 | 14 |
| 3 | NTN LEO600 UL | TN DL | NTN UE ACLR | 14 GHz | ~0 | ~0 |
| 4 | TN DL | NTN LEO600 UL | NTN SAN LEO600 ACS | 14 GHz | 9 | 22 |
| 5 | TN DL | NTN LEO600 DL | NTN UE ACS | 11 GHz | 35 | 30 (avg) |
| 6 | NTN LEO600 DL | TN DL | NTN SAN LEO600 ACLR | 11 GHz | ~0 | ~0 |
| 7 | NTN LEO600 DL | TN UL | NTN SAN LEO600 ACLR | 11 GHz | ~0 | ~0 |
| 8 | TN UL | NTN LEO600 DL | NTN UE ACS | 11 GHz | 26 | 20 (avg) |

# References

- RP-243314, “Revised WID: Introduction of Ku bands for NR NTN”, 3GPP RAN#105

- R4-2504723, “WF for Ku band”, 3GPP RAN4#114bis

- R4-2507672, “On co-existence simulation assumptions for NR NTN Ku band”, 3GPP RAN4#115, Ericsson