3GPP TSG-RAN WG4 Meeting #115 	R4-2507680

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	OTA test case reduction

Agenda Item:	7.13.4.2.1

Document for:	Approval

1 Introduction

New WID on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at RAN#103 meeting. The work item concentrates on 2 work areas where the current BS specification may be optimised, i.e. Expected EIRP mask for upper 6GHz and OTA test enhancement. One of major identified RF requirements with the aim to shorten the test time is TX IM.

The considerations on test case reduction for TX IM test reduction was discussed in previous meetings and the latest WF is captured in [5]. In this contribution we provide further discussion and proposals for the objective.

2 Test scope reduction for Tx requirement

At RAN4#114bis meeting, for TX test case reduction it is agreed to focus on Tx IMD requirement and some agreements on the applicable interferer offsets and test configurations.

Issue 1-1-1: test scope reduction for Tx requirement

- Agreement:

  - Further discuss how to reduce the test simplification for OBUE limits.

    - Option 1: 1st frequency offset

    - Option 2: 3rd frequency offset to check OBUE limits

  - Further discuss how to reduce the test simplification for spurious limits.

    - Option 1: 3rd frequency offset

    - Option 2: For bands with operating band < 100 MHz to use 1st interferer offset and for bands with operating band >= 100MHz to use 3rd interferer offset.

Issue 1-1-2: Reduction of  test configuration for Tx requirement

- Agreement:

  - Proposal 1: 	For the reduction of test configurations for TX IM requirement, regardless of the test configuration selected (ATCRx or ANTCRx), the condition of maximum output power – Rated total output power (Prated,t,AC, or Prated,t,TABC) – should be met.

2.1	Interferer offsets

6 interferer offsets are tested for each and 12 if AAS supports NC operation, these offsets date back to the UTRA requirements and have been carried through for each subsequent RAT however not they are generating such excessive test time so it is appropriate to see if they are all still necessary or can be reduced.

Table 6.8.5.1-1 in 38.141-2: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

At last meeting, the applicable interference freq offset for ACLR is agreed, and it is FFS for OBUE and spurious emission.

OBUE

LTE/NR OBUE is generally considered that in the 1st adjacent channel the ALCR requirement dominates and as the emission moves away from the carrier the OBUE is tougher. They have a tougher requirement beyond 10 MHz so considering 5MHz channels the 3x CBW offset. As we are testing at M only, the band filter can be ignored.

Proposal 1: it is proposed to use 3xCBW offset to check OBUE limits.

Spurious emissions

Spurious emissions are applied 10MHz outside the operating band. The only test case which can generate 3rd order products in SE domain is the 3xCBW case, and this is only if the wanted signal is places on the band edge. As we only test TX IM at middle there are very few bands where we will ever see 3rd order products in the SE domain. If so only the 3xCBW offset seems worth testing.

Proposal 2: it is proposed to use 3xCBW offset to check Spurious emissions limits.

3 Out of band blocking

For RX test case reduction, it is agreed to focus on out of band blocking and focus on the measurement step size for Rx OOBB requirement.

Issue 1-2-1: Out of band blocking

- Agreement on step size of OOBB requirement for further discussion:

    - Option 1: 1/3 of the minimum supported bandwidth

    - Option 2: Option 2: Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

    - Other options are not precluded.

RAN4 had similar discussion for FR2 in Rel-15 and finally agreed to reduce the amount of time by increasing the interferer step size. The less than 1/3 BW approach is adopted when define the measurement step.

Table 7.6.4.2.3-1: Interferer signal step size for FR2-1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| 30 to 6000 | 50, 100, 200, 400 | 1 |
| 6000 to 60000 | 50 | 15 |
|  | 100 | 30 |
|  | 200 | 60 |
|  | 400 | 60 |

For FR1 similar approach can be considered, e.g. option 1. We propose to set the measurement step size according to the supported BS channel bandwidth declared by the manufacture.

Table 3-1: Interferer signal step size for FR1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

For option 2 is may be feasible for some BS receiver architectures but it will miss some frequency points which may cause worse blocking cases for some implementations, e.g. superheterodyne receiver.

Proposal 3: It is proposed to set the measurement step size according to the minimum supported BS channel bandwidth declared by the manufacture, as shown in Table 3-1.

4	Summary

Proposal 1: it is proposed to use 3xCBW offset to check OBUE limits.

Proposal 2: it is proposed to use 3xCBW offset to check Spurious emissions limits.

Proposal 3: It is proposed to set the measurement step size according to the minimum supported BS channel bandwidth declared by the manufacture, as shown in Table 3-1

Table 3-1: Interferer signal step size for FR1

| Frequency range (MHz) | Minimum supported BS channel bandwidth (MHz) | Measurement step size (MHz) |
| --- | --- | --- |
| FR1 | 5 | 1 |
|  | 10 | 3 |
|  | 20 | 6 |

3	Reference

[1] R4-2419243, Test case reduction for TX IM, Huawei

[2] R4-2419828, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation

[3] R4-2501854, OTA test case reduction, Huawei

[4] R4-2502375, Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation

[5] R4-2504740, Way Forward for [114bis][304] NR_BS_RF_Part3_OTA_TRP