3GPP TSG-RAN WG4 Meeting #115 	R4-25

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom, Deutsche Telekom

Title: 	Deployment scenarios for FR1 SBFD macro BS

Agenda Item:	7.25.2

Document for:	Approval

Introduction

After the comprehensive study carried out by the Rel-18 SI for SBFD, the BS RF requirements for SBFD have been extensively discussed with good progress since the first meeting of the Rel-18 WI for SBFD. However, there are some disagreements on FR1 WA SBFD deployment scenario and corresponding RX in-band blocking and TX unwanted emission requirements. To facilitate RAN4 discussion, we provide our views in this contribution.

Discussion

At last meeting, the following way forward is approved on the Deployment scenarios for FR1 SBFD macro BS. In this contribution we discuss on 1st bullet on whether SBFD-SBFD co-ex case should be in Rel-19 scope.

- Issue 1-1: Deployment scenarios for FR1 SBFD macro BS

- Way forward

  - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope.

  - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include:

    - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.:

      - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios

      - Option 2: Co-existence simulation and potentially analysis of co-located scenarios

    - Expected spectrum allocation and sub-band sizes

    - Expected feasible filtering performances considering the above two aspects

We think the potential SBFD deployment scenarios especially for FR1 WA could be quite limited if RAN4 only focus on SBFD-TDD co-existence and define the RF requirements accordingly. Following analyses are mainly based on the Rel-18 SBFD SI outcomes.

- Per the conclusions for Case 3 SBFD UL co-existence evaluation in TR 38.858 clause 11.3, the throughput degradation is increased with grid-shift lower than 100%.

Case 3 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 | FR1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput, and minor degradation but acceptable to some companies for average throughput. With other assumptions (higher BS Tx power and lower grid-shifts), the degradation is increased for cell edge throughput and average throughput. |

But it should be noted that the 100% grid-shift might be too ideal for macro BS comparing with real deployment. Operators in adjacent channels in the same band have been collocating macro BS since the beginning of cellular wireless to minimize the near-far effect on ACIR. As a result, we assumed lower grid-shift, e.g., 20% considering macro BS deployment today. Thus the inevitably increased CLI due to such deployment-dependent site distance would be worthy of attention from this point of view.

- Per those findings in TR 38.858 clause 11.2, the exercises on enhancing ACIR had been conducted by RAN4. With the principle that no change on ACLR/ACS from legacy TDD BS or UE can be expected, the ACIR gain respective to each concerned case can be found as below.

ACIRvictim=11ACLRaggressor+1ACSvictim

  - Case 2 (Legacy BS as a victim and SBFD network as aggressor): The BS-to-BS ACIR enhancement cannot exceed 6.2 dB in FR1 and 1.5 dB in FR2-1 based on the limitation of the legacy BS ACS simulation assumption.

  - Case 3 (SBFD-capable BS as a victim and legacy BS as aggressor): The BS-to-BS ACIR enhancement cannot exceed 1.2 dB in FR1 and 5.5 dB in FR2-1 based on the limitation of the legacy BS ACLR simulation assumption.

Obviously, The ACIR gain would be subject to the legacy RF requirements and has limited help for CLI mitigation.

However, for a new wide TDD band without existing deployment of legacy TDD base stations, it would be more beneficial to deploy SBFD BS to increase UL coverage and reduce latency to realize the full potential of SBFD technology In this case improved adjacent channel co-existence between an SBFD BS with another SBFD BS would be feasible by enhancing the ACIR, i.e. defining additional requirements both transmitter side and receiver side. As shown in below figure, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. It would not impose additional implementation difficulty for taking the advantage of per operator filter.

More specifically, we reran the RAN4 co-ex simulation with 10%/20%/50% grid-shift to see how much UL throughputs degradation can be reduced with the help from the increased ACIR. Note that even for 50% grid-shift the baseline UL throughput loss i.e. applying the existing 42.5dB ACIR determined by 45dB BS ACLR and 46dB BS ACS, is 100% at cell-edge and 16.72% at cell-average, respectively.

|  | 10% grid shift UL throughput loss |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR(dB) UL Thr. Loss @X%-tile |  | 42.5 (baseline) | 52.5 | 62.5 | 65 | 67.5 | 70 | 72.5 | 75 | 77.5 | 80 |
| @5% (cell-edge) |  | 100% | 100% | 100% | 100% | 100% | 100% | 12.98% | 8.24% | 5.22% | 2.71% |
| @50% (cell-average) |  | 33.2% | 13.17% | 4.51% | 3.39% | 2.52% | 1.99% | 1.54% | 1.04% | 0.67% | 0.49% |

|  | 20% grid shift UL throughput loss |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR(dB) UL Thr. Loss @X%-tile |  | 42.5 (baseline) | 52.5 | 62.5 | 65 | 67.5 | 70 | 72.5 |
| @5% (cell-edge) |  | 100% | 100% | 22.18% | 18.31% | 9.96% | 4.29% | 2.23% |
| @50% (cell-average) |  | 22.88% | 10.09% | 2.64% | 1.91% | 1.40% | 0.98% | 0.64% |

|  |  |  | 50% grid shift UL throughput loss |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACIR(dB) UL Thr. Loss @X%-tile |  | 42.5 (baseline) |  | 50 | 52.5 | 55 | 57.5 | 60 |
| @5% (cell-edge) |  | 100% |  | 100% | 100% | 9.08% | 5.34% | 1.96% |
| @50% (cell-average) |  | 16.72% |  | 3.51% | 2.61% | 1.94% | 1.27% | 0.77% |

From the above table, it is found that,

If the grid shift is set to 10%, around 36 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

If the grid shift is set to 20%, around 28 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

If the grid shift is set to 50%, around 16 dB improvement on ACIR can be enough to guarantee <5% UL throughput loss.

Observation 1: For TDD-SBFD co-ex, ACIR enhancement is limited by ACLR of TDD BS. Limited ACIR enhancement cannot help much for better co-existence performance. Co-existence in only feasible for 100% grid-shift, which is a quite limited deployment scenario.

Observation 2: For lower grid shift, e.g. 10%/20%/50%, for co-existence the required ACIR enhancement is far exceed the legacy BS ACIR, whose enhancement cannot exceed 1.2 dB in FR1 based on the limitation of the legacy BS ACLR simulation assumption.

In conclusion, we think SBFD-SBFD co-existence for FR1 macro BS is an important deployment scenario and should be considered in Rel-19. Although Rel-19 may still be an early experiment of flexible duplexing, it is reasonable to expect the new SBFD BS hardware come with enhanced ACIR for better co-existence and performance with an adjacent SBFD BS to come a later. This way we can continuously evolve flexible duplexing technologies in 5G Adv. and 6G for better performance and deployment flexibility. The enhanced requirements in the analysis above are only for SBFD-capable BS w/o any legacy TDD impact, the final requirement should be based on RAN4 co-ex simulation practise and realistic consideration regarding grid-shift and other parameters if necessary.

Observation 3: Operators' interest in SBFD-SBFD inter-operator co-existence is identified.

Proposal 1: At least for FR1 macro BS, the deployment scenario of SBFD-SBFD co-existence in adjacent channels should be considered for defining RF requirements for inter-operator co-existence.

Conclusion

In this contribution we discussed the deployment scenarios for FR1 SBFD macro BS.

Proposal 1: At least for FR1 macro BS, the deployment scenario of SBFD-SBFD co-existence in adjacent channels should be considered for defining RF requirements for inter-operator co-existence.

- References

- TR 38.858.