3GPP TSG-RAN WG4 Meeting #115	R4-2507698

Malta, MT, May 19 – 23, 2025

# Title:	DC_(n)3CA MSD

# Agenda Item:	6.2.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution proposes a single one-uplink (1 UL) MSD test point for downlink (DL) DC_(n)3CA.

- Discussion

  - MSD Test Point Selection

It is proposed to evaluate the MSD for the configuration captured in Table 1 in which

- the DL LTE 5 MHz CBW component carrier (CC) is affected by one uplink (1 UL) IMD5,

- the DL LTE 20 MHz CBW CC is affected by 1 UL IMD7.

Table 1: DC_(n)3CA MSD Test Point.

| EN-DC configuration | E-UTRA/ NR band | FC (UL) | Channel bandwidth | UL allocation | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (LCRB) | (MHz) | (dB) |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | [FFS] | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | [FFS] |  |
|  | n3 | 1772.5 | 25 | 5 (RBstart = 128) | 1867.5 | N/A |  |

  - MSD Evaluation

For Power Amplifier (PA) transmitter (TX) noise emission measurements in the receiver (RX) band, we assume:

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB maximum power reduction (MPR)

- Power Class 3 (PC3) operation and 4 dB post-PA loss

- Local Oscillator (LO) leakage: –28 dBc; IQ image rejection: –28 dB

- C-IM3: –60 dBc, C-IM5: –70 dBc

- RF transceiver (Trx) noise PSD in RX band: –125 dBm/Hz.

For REFSENS/MSD estimations, we assume:

- 50 dB TX to RX and 45 dB TX to antenna duplexer rejection in RX band

- 10 dB primary antenna to diversity antenna isolation

- 4 dB post-PA loss and 4 dB antenna-to-LNA front-end losses

- 3 dB antenna diversity gain and –1 dB SNR

In Table 2, the B3 5 MHz/20 MHz MSD is evaluated at 1.3 dB and 0.3 dB based on the measurement results below:

- PA+Trx noise affecting B3 5 MHz CC at Fc = 1832.5 MHz: –50.6 dBm/4.5 MHz

- PA+Trx noise noise affecting B3 20 MHz CC at Fc = 1845MHz: –52.0 dBm/18 MHz

Table 2: DC_(n)3CA MSD Evaluation Based on Measurement Results.

MSD less than 0.5 dB are not usually specified at RAN4. Since it is preferable to avoid specifying 0 dB MSD, it is proposed to capture the B3 20 MHz DL CC MSD as “N/A” with a footnote indicating that the MSD is less than 0.5 dB.

Proposal: Consider adopting the following PC3 MSD requirement for DC_(n)3CA:

| EN-DC configuration | E-UTRA/ NR band | FC (UL) | Channel bandwidth | UL allocation | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (LCRB) | (MHz) | (dB) |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | 1.3 | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | N/AX |  |
|  | n3 | 1772.5 | 25 | 5 (RBstart = 128) | 1867.5 | N/A |  |
| Note x: The MSD is less than 0.5dB. |  |  |  |  |  |  |  |

- Conclusion

This paper presents PA measurements to evaluate the DC_(n)3CA MSD requirements.

Proposal: Consider adopting the following PC3 MSD requirement for DC (n)3CA:

| EN-DC configuration | E-UTRA/ NR band | FC (UL) | Channel bandwidth | UL allocation | FC (DL) | MSD | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (LCRB) | (MHz) | (dB) |  |
| DC_(n)3CA | 3 | N/A | 5 | N/A | 1832.5 | 1.3 | FDD |
|  | 3 | N/A | 20 | N/A | 1845 | N/AX |  |
|  | n3 | 1772.5 | 25 | 5 (RBstart = 128) | 1867.5 | N/A |  |
| Note x: The MSD is less than 0.5dB. |  |  |  |  |  |  |  |

- References