3GPP TSG-RAN WG4 Meeting #115	R4-2507700

Malta, MT, May 19 – 23, 2025

# Title:	Impact of Rel-19 BCS4_5 new greatest-smallest CBW on MSD

# Agenda Item:	6.12.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution captures our views on handling the Rel-19 optional channel bandwidth (CBW) as captured in the agreed upon way forward (WF) [1]. The impact on MSD requirements due to BCS4/5 enabling support for new greatest/smallest CBW is also discussed.

- Discussion

  - Background on handling optional CBW carried from Rel-18 into Rel-19

At RAN4 meeting #114-bis, the following agreements 1 & 2 were reached [1] whereby interested companies are invited to share their views on whether the following CBW should remain optional or if they should become mandatory support: 3MHz, 5MHz for band n41/n90 and 35MHz for band n71.

# Beyond the “optional/mandatory aspects, we are primarily interested in reaching consensus on which of these new CBW should be considered for MSD analysis when these CBW become “automatically” supported due to request for BCS4/5.

  - Which CBW to consider for MSD analysis in Rel-19 for BCS4/5 combos?

Case of 3MHz CBW: In our view, this new CBW should not be considered as new smallest CBW for MSD analysis except when band n106 is one of the constituent bands. We therefore do not have any strong views on WF option 1 vs option 2.

Case of 7MHz CBW: This new CBW should not be considered as new smallest CBW for MSD analysis.

Case of 5MHz CBW for band n41/n90: This CBW can only configured with SCS 15kHz. It is our understanding that only SCS 30kHz is currently supported and commercially deployed around the various regions of the world where these bands have been allocated. We therefore propose to consider the WF option 3.

Case of band n71 35MHz CBW: RAN4 has already agreed an optional 1UL MSD test points with this new greatest UL CBW. We therefore believe that this CBW could become mandatory support in Rel-19, i.e. WF option 2.

Special case of smallest CBW with mandatory support inherited from Rel-18 but for which operation is restricted to Scell in an inter-band CA / DC band combination: These cases correspond to

- 5MHz CBW for band n40, n48, n50.

- 10MHz CBW for band n46.

To the best of our knowledge, RAN4 has never discussed any rule on whether these smallest CBW should be systematically specified for MSD requirements. A quick parsing of TS 38.101-1 shows that in some cases these CBW are specified (Table 1), while in other cases they are ignored (Table 2). This situation is rather unfortunate as it introduces inconsistencies across MSD requirements. However, considering these are rare exceptions, and for the sake of avoiding having to revisit once again some MSD test points, we exceptionally propose to avoid ruling on these “smallest” CBW and let each proponent decide on a case-by-case basis whether these “special” smallest CBW should be specified.

Table 1: Example of harmonic MSD requirements where the band n48 and band n50 smallest CBW of 5MHz restricted to Scell operation is specified (Table 7.3A.4-1).

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n25 | n48 | 5 | 15 | 25 | 5 | 8.1 | NOTE 6 | UL2/DL1 near-miss |
| n28 | n50 | 5 | 15 | 12 | 5 | 28.1 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 | 5 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |

Table 2: Counterexample of harmonic mixing MSD requirements where the band n40 smallest CBW of 5MHz restricted to Scell operation is not specified (Table 7.3A.4-4).

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n40 | 10 | 15 | 12 | 10 | 11.7 | NOTE 2 | UL2/DL3 |

Proposal 1: Consider adopting the following rules for Rel-19 MSD requirements and reflect them into the RAN4 PRD:

- 3MHz CBW: At the exception of the case when band n106 is the DL affected band, do not consider this CBW for MSD requirements.

- 7MHz CBW: do not consider this CBW for MSD requirements.

- 5MHz CBW for band n41/n90: WF option 3, i.e. do not consider this CBW for MSD requirements and

  - remove it from the TS 38.101-1

  - replace this CBW with 10MHz whenever 5MHz was agreed and specified.

- 35MHz CBW for band n71: WF option 2, i.e. consider this new greatest CBW for MSD requirements as mandatory CBW. Revisit or introduce MSD requirements whenever applicable.

- Case of smallest CBW restricted to Scell operation: exceptionally, do not mandate to specify MSD requirements with these CBW, i.e. proponents to decide on a case-by-case basis if these CBW are appropriate for MSD requirements.

  - Corrections to Rel-19 MSD requirements

Several Rel-19 MSD requirements need to be re-evaluated based on the above proposal 1 and on the fact that many BCS4/5 requests were approved without checking if MSD requirements were impacted.

    - Corrections due to new “greatest” UL CBW

PC3 CA_n5A-n71A:

A new MSD test point is proposed to account for proposal 1 band n71 35MHz CBW. The DL band n5 is affected by “flat noise PSD” due to large frequency separation distance between UL Band n71 and DL band n5. No measurements are needed, and the 2dB MSD applies to this new test point.

Proposal 2: Consider introducing the following PC3 CA_n5A-n71A MSD requirements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 680.5 | 35 | 15 | 20 (RBstart=168) | 871.5 | 5 | 2.0 | >ACLR2 |

PC3, PC2 CA_n71A-n85A: Consider adopting the following changes to reflect mandatory support of 35MHz UL CBW for band n71.

Proposal 3: For PC3 CA_n71A-n85A, void footnotes 6 and 7 from Table 7.3A.6-1.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2 | >ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (RBstart=168) | 730.5 | 5 | 23 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL Band 3rd order IMD product falls into the affected DL channels. NOTE 6: Void NOTE 7: Void |  |  |  |  |  |  |  |  |  |

For PC2 CA_n71A-n85A, void footnotes 4 and 5 from Table 7.3A.6-1a-1.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 10.96 15.97 | >ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (RBstart=168) | 730.5 | 5 | 266 32.37 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: Void NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

PC3 CA_n78A-n102A:

Based on WF [1] agreement 2, new MSD requirements are needed to account for band n102 100MHz CBW.

Requirements from CA_n78-n104A are proposed for CA_n78A-n102A.

Proposal 4: Consider introducing the following PC3 CA_n78A-n102A MSD requirements.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n102 | 3750 | 100 | 30 | 270 (RBstart=3) | 5935 | 20 | 14.4 | >ACLR2 |
| n102 | n78 | 5975 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 15.8 | >ACLR2 |

    - Corrections due to BCS request omissions

PC3 CA_n3A-n74A and CA_n3A-n75A:

Issues for these cross-band isolation MSD requirements:

We make the following observations based on summary of MSD requirements summarized in Table 3.

Table 3: PC3 Band n74, n75, B11 5MHz CBW MSD due to cross-band isolation interference from band n3.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 2.6 | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 4.3 | >ACLR2 |
| n3 | 11 | 1735 | 50 | 15 | 50 (RBstart=0) | 1493.4 | 5 | 6.4 | >ACLR2 |
| n3 | 11 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1493.4 | 5 | 6.4 | >ACLR2 |

- The band n3 UL CBW is not specified using the greatest UL CBW of 40MHz for band n74A and 50MHz for band n75A. Instead, the smallest UL CBW of 5MHz is specified for band n3.

- The band n75 DL carrier frequency is wrong. It should be specified at 1514.5MHz.

- Considering the minimum frequency separation between the affected DL bands and the band n3 lowermost UL channel is at least 200MHz, the cross-band isolation MSD should be dominated by the “flat-band” noise PSD, not by any PA IMD product. MSD requirements for n74, n75 and B11 should only reflect each band’s REFSENS for 5MHz CBW of -100dBm for Band n74 and Band 11 and -99.5dBm for Band n74.

- The CA_n3A-n75A MSD analysis produced in TR 38718-02-01 refers to MSD analysis of R4-2304218 which itself mentions n3 PA output noise of 50MHz UL CBW, yet the reported MSD in this TP is 1.1dB.

- To add more confusion, the agreed LTE Band 11 5MHz CBW MSD due to band n3 50MHz CBW is specified at 6.4dB based for DC_11_n3.

Observation: Based on the list of reported issues, it can be concluded that the MSD requirements due to cross-band isolation for CA_n3-n74, CA_n3-n75 and DC_11_n3 are inconsistent and need to be corrected.

To resolve the above inconsistencies, we propose to adopt the MSD requirements from DC_11_n3 based on MSD a

Proposal 5: Consider introducing the following PC3 corrections for CA_n3A-n74A and CA_n3A-n75A.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1735 | 50 | 15 | 50 (RBstart=0) | 1515.5 | 5 | 6.4 | >ACLR2 |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 6.4 | >ACLR2 |
| n3 | n75 | 1712.5 | 50 | 15 | 50 (RBstart=0) | 1514.5 | 5 | 6.4 | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1514.5 | 5 | 6.4 | >ACLR2 |

PC2 CA_n2A-n66A

Proposal 6: Consider adopting the following PC2 MSD corrections for CA_n2-n66 based on CA_n25-n66 agreements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1890 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.76 0.97 | >ACLR2 |

- Conclusion

This paper presents our views on handling the new smallest/greatest CBW in Rel-19 and the impact of these CBW on MSD due to omissions related to BCS requests.

Proposal 1: Consider adopting the following rules for Rel-19 MSD requirements and reflect them into the RAN4 PRD:

- 3MHz CBW: At the exception of the case when band n106 is the DL affected band, do not consider this CBW for MSD requirements.

- 7MHz CBW: do not consider this CBW for MSD requirements.

- 5MHz CBW for band n41/n90: WF option 3, i.e. do not consider this CBW for MSD requirements and

  - remove it from the TS 38.101-1

  - replace this CBW with 10MHz whenever 5MHz was agreed and specified.

- 35MHz CBW for band n71: WF option 2, ie. consider this new greatest CBW for MSD requirements as mandatory CBW. Revisit or introduce MSD requirements whenever applicable.

- Case of smallest CBW restricted to Scell operation: exceptionally, do not mandate to specify MSD requirements with these CBW, i.e. proponents to decide on a case-by-case basis if these CBW are appropriate for MSD requirements.

Proposal 2: Consider introducing the following PC3 CA_n5A-n71A MSD requirements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 680.5 | 35 | 15 | 20 (RBstart=168) | 871.5 | 5 | 2.0 | >ACLR2 |

Proposal 3: For PC3 CA_n71A-n85A, void footnotes 6 and 7 from Table 7.3A.6-1.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2 | >ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (RBstart=168) | 730.5 | 5 | 23 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL Band 3rd order IMD product falls into the affected DL channels. NOTE 6: Void NOTE 7: Void |  |  |  |  |  |  |  |  |  |

For PC2 CA_n71A-n85A, void footnotes 4 and 5 from Table 7.3A.6-1a-1.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Source of interference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 10.96 15.97 | >ACLR2 |
| n71 | n85 | 680.5 | 35 | 15 | 20 (RBstart=168) | 730.5 | 5 | 266 32.37 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-Band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting Band n78 with a n77 implementation. NOTE 4: Void NOTE 5: Void NOTE 6: Applicable to UE’s supporting PC2 with 1Tx NOTE 7: Applicable to UE’s supporting PC2 with 2Tx |  |  |  |  |  |  |  |  |  |

Proposal 4: Consider introducing the following PC3 CA_n78A-n102A MSD requirements.

| UL band | DL band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n102 | 3750 | 100 | 30 | 270 (RBstart=3) | 5935 | 20 | 14.4 | >ACLR2 |
| n102 | n78 | 5975 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 15.8 | >ACLR2 |

Observation: Based on the list of reported issues, it can be concluded that the MSD requirements due to cross-band isolation for CA_n3-n74, CA_n3-n75 and DC_11_n3 are inconsistent and need to be corrected.

Proposal 5: Consider introducing the following PC3 corrections for CA_n3A-n74A and CA_n3A-n75A.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n74 | 1735 | 50 | 15 | 50 (RBstart=0) | 1515.5 | 5 | 6.4 | >ACLR2 |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 6.4 | >ACLR2 |
| n3 | n75 | 1712.5 | 50 | 15 | 50 (RBstart=0) | 1514.5 | 5 | 6.4 | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1514.5 | 5 | 6.4 | >ACLR2 |

Proposal 6: Consider adopting the following PC2 MSD corrections for CA_n2-n66 based on CA_n25-n66 agreements.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1890 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | 0.76 0.97 | >ACLR2 |

- References

- R4-2505214 WF on handling optional channel bandwidth across releases, 3GPP TSG-RAN WG4 Meeting #114-bis, Wuhan, China, Skyworks Solutions, Inc.

- R4-2005122 Correction TP for TR 37.716-11-11: DC_B28_n50, DC_11_n3, DC_28_n5, 3GPP TSG-RAN WG4 Meeting #94-bis-e, Electronic Meeting, Mediatek Inc.