3GPP TSG-RAN WG4 Meeting #115	R4- 2507703

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.10.3.3.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN HPUE TX RF requirements

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable high power UE (HPUE) with defining RF requirements targeting at support of IoT-NTN UEs for FR1 single carrier scenario.

In this contribution, our views about HPUE TX RF requirements for IoT-NTN UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

2.2		Previous Meeting Agreements

In RAN4#114bis meeting, the agreed WF [2] provides the following works for specifying TX requirements for IoT-NTN under HPUE operation. The partial contents are shown as follows.

Tx requirements Issue 1-1-8: Basic MOP tolerance (Including eMTC and NB-IoT based) for PC2 - Proposals - For PC2: - Option 1: +2 / -3dB - Aligned with NR-NTN - Option 2: +2 / -2dB - Aligned with LTE, and more adaptable for narrowband - WF (Ad-hoc agreement): - Use Option 2 as basic MOP tolerance for PC2, including eMTC and NB-IoT based NTN. - Foe PC2 NB-IoT based Iot-NTN, further boosting discussed in issue 1-1-10 may have an impact on tolerance based on the basic MOP tolerance Issue 1-1-10: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation. - Proposals - Option 1: Relax TX MOP tolerance upper limitation for NTN HD-FDD (MTK) EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). 256; 26; +2x, -2; 23; +2x, -2. 255; 26; +2x, -2; 23; +2x, -2. NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx].. - Option 2: Additional NW signal to control relaxation of TX MOP upper limitation for NTN HD-FDD. (MTK) EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). 256; 26x; +2, -2; 23x; +2, -2. 255; 26x; +2, -2; 23x; +2, -2. NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx].. - Option 3: Power tolerances are applied uniformly to HD-FDD and FD-FDD. (Qualcomm) - WF (Ad-hoc agreement): - Restrict the boosting condition to PC2 NB-IoT based IoT-NTN - To down-select in next meeting between: - Option 1: Increase [1] dB MOP upper tolerance - Option 2: Increase [1] dB MOP nominal power (optional with capability signalling)

2.3		UE TX RF requirements

Proposal 1: Regarding “On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation”, consider option 1 of increasing MOP upper tolerance first. If option 1 is not acceptable, consider option2.

- Option 1: Regarding IoT-NTN HD-FDD with IL improvement compared to FD-FDD, increasing TX MOP upper tolerance is used to accommodate power boost under HD-FDD operation.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26 | +2x, -2 | 23 | +2x, -2 |
| 255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: Increasing 0.8dB MOP upper power tolerance is used to accommodate power boost under HD-FDD operation. |  |  |  |  |

- Option 2: As for IoT-NTN HD-FDD with IL improvement compared to FD-FDD, increasing 0.8 dB MOP nominal power is controlled by optional capability signalling.

Proposal 2: Basic nominal MOP and MOP tolerance are still applicable to FD-FDD.

3 Conclusion

In this paper, our views on HPUE TX RF requirements for IoT-NTN UE are provided as follows:

Proposal 1: Regarding “On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation”, consider option 1 of increasing MOP upper tolerance first. If option 1 is not acceptable, consider option2.

- Option 1: Regarding IoT-NTN HD-FDD with IL improvement compared to FD-FDD, increasing TX MOP upper tolerance is used to accommodate power boost under HD-FDD operation.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26 | +2x, -2 | 23 | +2x, -2 |
| 255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: Increasing 0.8dB MOP upper power tolerance is used to accommodate power boost under HD-FDD operation. |  |  |  |  |

- Option 2: As for IoT-NTN HD-FDD with IL improvement compared to FD-FDD, increasing 0.8 dB MOP nominal power is controlled by optional capability signalling.

Proposal 2: Basic nominal MOP and MOP tolerance are still applicable to FD-FDD.

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2505114, “WF on HPUE RF requirements for IoT-NTN”, Vivo