3GPP TSG-RAN WG4 Meeting #115	R4-2507716

Malta, MT, 19th – 23th May, 2025

Source:	Nokia

Title:	Discussion on NR NTN UE requirement for Ku bands

Agenda item:	7.11.5

Document for:	Approval

Introduction

For WI on the introduction of NTN Ku Band [1], RAN4#114-bis has agreed the Way-forward [5] regarding the UE and SAN RF requirement for FR1-NTN and FR2-NTN Ku bands. In this contribution, we discuss some of remaining open issues regarding UE RF requirements.

Discussion

In RAN4#114-bis meeting, radiated transmitter requirement is agreed in the following.

#### Issue 1-1-4: General RF requirements impacts on numerologies dependency - The following aspects are considered when defining the general RF requirements for the Ku bands. - No update needed on 9.3.1 Minimum output power, 9.3.2 transmit off power, 9.3.4 Power control, and 9.4 Transmitter signal quality for supporting Ku bands. - For 9.3.3 Transmit ON/OFF time mask, - The time masks and transient period times will be re-used for FR2-NTN Ku bands - FFS on FR1-NTN Ku bands

Transmit ON/OFF time mask requirements are specified differently for FR1-NTN and FR2-NTN for the existing NTN bands, i.e., conducted requirement is specified for FR1-NTN and radiated requirement is specified for FR2-NTN. The time mask is shorter for FR2-NTN while larger subcarrier spacing and shorter symbol duration are utilized.

For Ku bands, on the other hand, only the radiated requirement will be specified for both FR1-NTN Ku bands and FR2-NTN Ku bands.

There has been a request from some satellite operators to align the requirement between FR1-NTN and FR2-NTN as they use the same frequency range but only with different numerologies. The radio implementation likely shares the common hardware design for FR1-NTN and FR2-NTN. This would result in the same transient characteristic of the radio transmitter regardless of the numerologies. Therefore, it is reasonable to assume that the same requirement is applied to both FR1-NTN Ku bands and FR2-NTN Ku bands regarding transmit on/off time mask.

Transmit ON/OFF time mask for FR1-NTN Ku bands (n248 and n247) are the same as FR2-NTN Ku bands (n509 and n508).

In RAN4#114-bis meeting, handling of Network Signalling was discussed in the following.

#### Issue 1-1-5: Handling of FCC, ISED, CEPT regulation related requirement - New NSs will be defined for the Ku band concerning different regulations - FFS on whether different NS numbers will be used for FR1-NTN Ku bands and FR2-NTN Ku bands - Different NS numbers will be used to handle FCC and CEPT regulation at least - FFS on how to handle FCC and ISED requirements in TS 38.101-5 - Option 1: capture the most stringent requirement with same NSs - Option 2: separate NSs

In the NR TN specifications, we reuse the same NS number among multiple NR bands in case the same regulatory requirement and the same A-MPR values are applied. It is proposed to follow the same principle also in NTN specifications.

The same NS number can be reused for the FR1-NTN and FR2-NTN bands for the same regulatory requirement with the same A-MPR values.

For FCC and ISED requirements, it is proposed to evaluate A-MPR values first and decide if NS can be combined or not, depending on how different A-MPR values are for these two requirements.

It is proposed to evaluate A-MPR values for FCC and ISED requirements and to decide whether NS number should be combined or separated.

In RAN4#114, Maximum EIRP requirement was agreed in the following.

Issue 1-2-1: Maximum EIRP requirement - The maximum EIRP requirement for the Rel.19 Ku bands VSAT is set at 88.2 dBm - Additional limitations for VSATs in CEPT regions will be captured, including the following - 54.5 dBW (or captured as 84.5 dBm) for land ESIM operating in CEPT regions

In RAN4#114-bis, Minimum EIRP requirements are agreed in the following.

#### Issue 1-2-1: Minimum EIRP requirements - Consider the following as the starting point Operating band; UE Type; Min peak EIRP (dBm). [When communicating with LEO]; [When communicating with GSO]. n248, n247, n509, n508; 4; [64]; [70]. 5; [64]; [70]. - FFS on whether further clarification is needed

Type 4 UE is Mobile VSAT communicating with GSO and LEO with mechanical steering antenna.

Type 5 UE is Mobile VSAT communicating with GSO and LEO with electronic steering antenna.

The above table can be understood that both Type 4 and Type 5 are required to support both LEO and GSO as mandatory feature, unless no further clarification is present. There may be a VSAT that is only capable of communicating with LEO in the market. It may be understood that such VSAT implementation may not be allowed in the current table. If that is the intention of the agreement, no further clarification may be needed.

It is to further clarify whether LEO-only or GSO-only capable VSAT is allowed or not.

However, if such single mode implementation is intended to be allowed, the requirement (at least the minimum peak EIRP) is not the same and should be differentiated. Thus, it would be necessary to distinguish VSAT implementations in other UE types or by other means (such as another UE capability or power class, for example).

If the intention is that all VSAT UE is mandated to support both LEO and GSO, then, UE anyway must support 70 dBm for GSO. In that case, it is not so crucial to specify minimum EIRP 64 dBm for LEO, because the minimum EIRP requirement in the core requirement is to check the peak EIRP at bare minimum. If we know UE can reach 70 dBm, we already know it can exceed 64 dBm.

It is to further clarify whether two minimum peak EIRP levels shall be specified for the same UE type, or only one minimum peak EIRP level shall be specified for the same UE type.

If there is a need to reduce the power for LEO, it should not be specified by reducing the minimum peak EIRP. It should be specified by reducing the maximum EIRP for LEO.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Transmit ON/OFF time mask for FR1-NTN Ku bands (n248 and n247) are the same as FR2-NTN Ku bands (n509 and n508).

Proposal 2: The same NS number can be reused for the FR1-NTN and FR2-NTN bands for the same regulatory requirement with the same A-MPR values.

Proposal 3: It is proposed to evaluate A-MPR values for FCC and ISED requirements and to decide whether NS number should be combined or separated.

Proposal 4: It is to further clarify whether LEO-only or GSO-only capable VSAT is allowed or not.

Proposal 5: It is to further clarify whether two minimum peak EIRP levels shall be specified for the same UE type, or only one minimum peak EIRP level shall be specified for the same UE type.

- References

- RP-250799 Revised WID: Introduction of Ku bands for NR NTN, RAN#107

- R4-2502288 WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#114

- R4-2502290 Way Forward for [114][313] NR_NTN_Ku_Band_General, RAN4#114

- R4-2504720 Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#114-bis

- R4-2504761 Way Forward for [114bis][313] NR_NTN_Ku_Band_General, RAN4#114-bis

- R4-2504721 Draft CR for Operating bands and system parameters, RAN4#114-bis