# 3GPP TSG-RAN WG4 Meeting #115		        R4-2507719

# St Julian’s, Malta, 19th – 23th May, 2025

Agenda Item:	6.5.3

Source: 	Huawei, HiSilicon

Title:	On PC2 HPUE requirements

Document for:	Approval

# 1	Introduction

A WF [1] was agreed in the last meeting to progress the PC2 requirements for band n26 and n5 as well as merge the 1Tx and 2Tx PC2 A-MPR for NS_48 and NS_49.

Issue 1: n26 PC2

PC2 requirements in draft CR R4-2504186 for band n26 could be set as baseline for further evaluation until RAN4#115.

- Encourage companies to provide further results, TPs or CRs at next meeting.

Issue 2: n5 PC2

Further discuss option 1 of Issue 2-1 in the topic summary R4-2504663.

Issue 3: A-MPR for NS_48 NS_49

Aim to clean up the NS_48 and NS_49 A-MPR at next meeting.

# 2	Discussion

Regarding issue 1 and issue 3, we maintain our views presented in the last meeting [4].

Issue 1: n26 PC2

- It’s feasible to adopt the PC2 requirements in draft CR R4-2503019 for band n26.

Issue 3: A-MPR for NS_48 and NS_49

- It’s feasible to merge the 1Tx and 2Tx PC2 A-MPR requirements for NS_48 and NS_49 by adding 0.5dB relaxation to the 1Tx A-MPR table.

Table 6.2.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
|  | QPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
|  | 16 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
|  | 64 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
|  | 256 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
| CP-OFDM | QPSK | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 | ≤5 |
|  | 16 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 | ≤5 |
|  | 64 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 | ≤5 |
|  | 256 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 | ≤5 |

Table 6.2.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | QPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 16 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 64 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 256 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |

On the other hand, Note 7 in Table 6.2.3.1-1 also applies to NS_05 and NS_05U, for which new A-MPR requirements are under discussion in a separate thread. To avoid confusion, it’s better not to void Note 7 before work on NS_05(U) is completed.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |

| … |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NS_48 | 6.5.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.26-1, Table 6.2.3.26-3 | Table 6.2.3.26-2, Table 6.2.3.26-4 |
| NS_49 | 6.5.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.27-1, Table 6.2.3.27-3 | Table 6.2.3.27-2, Table 6.2.3.27-4 |
| … |  |  |  |  |  |

- Remove Note 7 for NS_48 and NS_49 from Table 6.2.3.1-1 when updating the corresponding PC2 A-MPR requirements. Do not void Note 7 before the work on NS_05(U) is completed.

Issue 2: n5 PC2

Upon the requests from US operators, NS_14 and NS_15 have been proposed to add to band n5 for the sake of protecting public safety frequencies [5].

The PC3 A-MPR requirements for NS_14 and NS_15 have been specified for band n26, with PC2 requirements to be completed soon [2]. Since the frequency range of band n5 is a subset of band n26, it’s possible to reuse the same A-MPR requirements.

Figure 1: Operating frequencies for band n5 and n26

- The same A-MPR requirements for NS_14 and NS_15 for band n26 can be reused for band n5, provided that the two bands share the same duplexer in a UE implementation.

Table 6.2.3.24-4: A-MPR for NS_15 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 12 | ≤ 6.5 | ≤ 7 | ≤ 10.5 | ≤ 3 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 12 | ≤ 6.5 | ≤ 9 | ≤ 13.5 | ≤ 4.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 13.5 | ≤ 8 | ≤ 7 | ≤ 12 | ≤ 4.5 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 13.5 | ≤ 8 | ≤ 9 | ≤ 13.5 | ≤ 6.5 | ≤ 7 |

Table 6.2.3.23-4: A-MPR for NS_14 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 3.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 3.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5 |  | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 5 |  | ≤ 5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 5 | ≤ 7 | ≤ 3 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 5 | ≤ 7 | ≤ 3 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 5.0 | ≤ 7 | ≤ 3.5 |
| CP-OFDM 256 QAM | ≤ 8 |  | ≤ 8 | ≤ 6.5 |

As per the solution described in [5], the network should broadcast NS_01 in addition to NS_14 or NS_15 in SIB1.

The possible configuration of the NR-NS-PmaxList is {p-Max=23dBm, NS_01} + {p-Max=26dBm, NS_14/NS_15}. For PC3 UEs that do not support NS_14 or NS_15, such as legacy UEs, they can connect to the cell via NS_01 with P-max limited to 23dBm. For PC2 UEs introduced after Rel-19, they can support NS_14/NS_15 with p-Max=26dBm.

- Due to large A-MPR (>3dB), a PC2 UE that accesses a cell via NS_14/NS_15 could perform worse than a PC3 UE that connects to the cell via NS_01.

Given the access mechanism in the RRC protocol, it’s also possible for a PC2 UE to access the cell via NS_01 with p-Max limited to 23dBm. The advantage is that the UE does not need to apply large A-MPR to meet NS_14 or NS_15 emission requirement.

- Both PC2 and PC3 UEs can use NR-NS-PmaxValue of {p-Max=23dBm, NS_01} to access the cell according to the RRC protocol if it’s signaled.

- If NS_14 and NS_15 are optional for PC3 UEs, they can also be optional for PC2 UEs.

- Consider that NS_14 and NS_15 on band n5 are optional for a UE regardless of its power class. The support by the UE can be indicated via modifiedMPR-behaviour.

# 3	Conclusion

Regarding the PC2 requirements for band n26, n1 and n5, the following proposals are made:

Proposal 1: It’s feasible to adopt the PC2 requirements in draft CR R4-2503019 for band n26.

Proposal 2:It’s feasible to merge the 1Tx and 2Tx PC2 A-MPR requirements for NS_48 and NS_49 by adding 0.5dB relaxation to the 1Tx A-MPR table.

Proposal 3:Remove Note 7 for NS_48 and NS_49 from Table 6.2.3.1-1 when updating the corresponding PC2 A-MPR requirements. Do not void Note 7 before the work on NS_05(U) is completed.

Observation 1:The same A-MPR requirements for NS_14 and NS_15 for band n26 can be reused for band n5, provided that the two bands share the same duplexer in a UE implementation.

Observation 2:Due to large A-MPR (>3dB), a PC2 UE that accesses a cell via NS_14/NS_15 could perform worse than a PC3 UE that connects to the cell via NS_01.

Observation 3:Both PC2 and PC3 UEs can use NR-NS-PmaxValue of {p-Max=23dBm, NS_01} to access the cell according to the RRC protocol if it’s signaled.

Observation 4:If NS_14 and NS_15 are optional for PC3 UEs, they can also be optional for PC2 UEs.

Proposal 4:Consider that NS_14 and NS_15 on band n5 are optional for a UE regardless of its power class. The support by the UE can be indicated via modifiedMPR-behaviour.

# References

- R4-2505117 WF on HPUE for NR bands, CMCC, RAN4#114bis

- R4-2503019 Draft CR to 38.101-1: Introduction of requirements for PC2 operation in band n26, Ericsson, RAN4#114

- R4-2501984 NS_05 NS_48 NS_49 PC2 A-MPR clean-up, Skyworks, RAN4#114

- R4-2504555 On HPUE PC2 requirements, Huawei, HiSilicon, RAN4#114bis

- R4-2504185 Specification of PC2 and NS values for n5 and n26, Ericsson, RAN4#114bis