3GPP TSG-RAN WG4 Meeting #115	R4-2507725

St Julian's, Malta, May 19th – 23rd 2025

Agenda Item:	7.25.3.1

Source:	Ericsson

Title:	New SBFD BS RF requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of new BS RF requirements for SBFD. Discussion on impact on existing BS RF TX and RX requirements for SBFD and SBFD general aspects are presented in separate submissions [1-3].

# 2	Discussion

In-channel adjacent sub-band leakage ratio

##### Issue 4-3-2: Assumption for in-channel adjacent subband leakage requirement Way forward: - FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage. ##### Issue 4-3-3: Absolute limit for in-channel adjacent subband leakage Way forward: - Absolute limit for in-channel adjacent subband leakage: - FFS by considering the following exemplary cases by performing analysis including measurement or simulation: - FR1: - DU/UD: 80MHz (DL) – 20MHz (UL) - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL) - Other cases are not precluded - FR2-1: - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL) - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL) - Other cases are not precluded - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit.

During the discussion in RAN4#114bis, one question was raised on the assumption for in-channel adjacent subband leakage requirement. For example, is it possible that for one operator some sites deploy SBFD with a subband configuration of 80 MHz - 20 MHz (DU), while other sites deploy with another subband configuration of 60 MHz - 40 MHz (DU).

In our view, to ensure network performance and avoid inter-site interference and receiver saturation, operators have to deploy SBFD uniformly across all sites, rather than

- selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD.

- nor with varying subband configurations across sites.

Without this uniformity, even small configuration mismatches can lead to undesirable effects. When SBFD is inconsistently implemented, the overlapping frequency portions of UL and DL transmission between adjacent cells can lead to significant interference which saturates the receiver, degrading overall system performance.

Therefore, it is proposed to clarify it in the specification indicating that SBFD subband configurations should remain consistent across all sites.

- Operators cannot deploy SBFD selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD.

- Operators cannot deploy SBFD with varying subband configurations across sites.

Add a note in TS 38.104 where appropriate that “Operators shall deploy SBFD uniformly across all sites with exactly the same subband configuration.”

As to the requirement level, the proposal is to take the ACLR absolute value as defined in Table 6.6.3.2-2 in TS 38.104 [4] as a starting point.

Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Specify -13 dBm/MHz as the requirement level for the SBFD in-channel adjacent sub-band leakage.

In-channel adjacent sub-band Blocking and adjacent sub-band selectivity

### Sub-topic 4-4: In-channel adjacent subband blocking ##### Issue 4-4-1: In-channel adjacent subband blocking requirement Way forward: - New requirement for in-channel adjacent subband blocking: - The requirement can be derived by considering co-site inter-sector case and/or inter-site case - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification. - The range of [-35dBm to -27dBm] can be used as the starting point for further study. ##### Issue 4-4-2: Assumption of gNB-to-gNB CLI handling Way forward: - FFS the assumption of gNB-to-gNB CLI handling for two cases: - Inter-site case: - Note: for the blocking requirement, RX digital beamforming cannot be considered. - Co-site inter-sector case: - Note: Inter-sector is near field interaction.

In RAN4#113 meeting, it was agreed to define a new requirement of in-channel adjacent subband blocking requirement for SBFD capable BS, and further study the requirement level.

When defining the requirement level of in-channel adjacent subband blocking requirement, spatial isolation is an important factor to consider. In SI, spatial isolation was widely discussed and was summarized in TR 38.858, where the spatial isolation values evaluated by different companies were captured in Table 9.2.2.1-1 (partly copied below for reading purpose). Considering the values provided from all companies comprehensively, a compromise value of 80 dB spatial isolation between sectors is selected.

For a SBFD capable BS with 53 dBm TRP, -27 dBm is expected at the UL subband, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

During the discussion at the RAN4#114bis meeting, some companies propose that CLI should be considered when defining in-channel adjacent subband blocking. In our view, the inter-sector interference is not about avoiding the beam direction from one site to another. It is about avoiding beam directions that cause unintended radiation, which radiates backward from one sector to another in the near field, such as sidelobes radiating backward across the near field. The CLI handling schemes for interaction in the near field at the same site differ from what is currently being studied in RAN1 and has not been studied so far.

When considering inter-site interference, it is important to acknowledge that while CLI handling schemes can suppress a portion of the interference, they cannot completely eliminate it. On one hand, RX digital beamforming cannot be considered in the CLI handling scheme, because receiver blocking occurs at each sub-array, and once a sub-array is blocked, digital mitigation is no longer possible because the receiver and ADC is already saturate. On the other hand, the CLI handling scheme is not always feasible. In practical deployments, especially in dense multi-site TDD networks, various factors limit the effectiveness of these schemes. Therefore, CLI handling scheme should not be considered when defining in-channel adjacent subband blocking requirement.

- In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far.

- CLI handling schemes for the case of inter-site interference could suppress a portion of the interference, but they cannot completely eliminate it, because it is not always feasible, and RX digital beamforming cannot be considered.

Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

Table 9.2.2.1-1: FR1 WA BS Co-site Inter-sector Co-channel Interference Analysis Summary (Partial copy)

| FR1 |  |  | Samsung | Samsung | Ericsson | Huawei | Nokia |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  | Wide Area BS (subband filter-1) | Wide Area BS (subband filter-2 and EM conjugated structure) | Wide Area BS | Wide Area BS | Wide Area BS |  |
| BS TX Power dBPTX = ① dBm |  |  | 49 dBm | 49 dBm | 53 dBm | 53 dBm | 54 dBm |  |
| Number of co-site co-channel sectors considered |  |  | 1 | 1 | 2 | 2 | 2 (plus 1 sector for self-interference) |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc | 45 dBc |  |
|  |  | Frequency isolation techniques used | DPD utilized | DPD utilized | DPD, CFR | DPD | DPD, digital filtering |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc | 75 dBc | 100 dBc | 75-90 dBc (Varies depending on scheduled beam directions) | 85 dBc | 60-80 dBc |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used | Based on 75dB for typical spatial isolation | Based on 75dB for typical spatial isolation and additional 25dB by installing EM conjugated structure between sectors | Typical site layout with around 400mm between sectors | Spatial separation between TX panel with absorbing material and choke structure. | Spatial separation between TX/RX panel; cross polarization |  |

Transient period between SBFD and non-SBFD

Figure 6.4.2.1-1 in TS 38.104 currently illustrates the transition between DL and UL transmission, providing a timing model for the switch between these two modes in legacy TDD. However, with the introduction of SBFD in Rel-19, this figure no longer fully captures the transition between SBFD and non-SBFD, and an updated figure dedicated for the transient period between SBFD and non-SBFD is needed. Our proposal is as shown in Figure 1 below.

Transmitter output power in UL subband/DL subband(s)

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

NOTE: The figure illustrates the transition between SBFD (either DUD or DU/UD patterns) and non-SBFD. The switching between downlink and uplink depends on the switching subband, i.e., UL subband for Case A or Case C, DL subband(s) for Case B or Case D.

Figure 1: Example of transient period between SBFD and non-SBFD for transmitter in SBFD UL subband/DL subband(s)

Terminology “non-SBFD” need to be introduced.

Use Figure 1 for the illustration of Transient period between SBFD and non-SBFD in TS 38.104.

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

Observation 1	Operators cannot deploy SBFD selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD.

Observation 2	Operators cannot deploy SBFD with varying subband configurations across sites.

Observation 3	In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far.

Observation 4	CLI handling schemes for the case of inter-site interference could suppress a portion of the interference, but they cannot completely eliminate it, because it is not always feasible, and RX digital beamforming cannot be considered.

Proposal 1	Add a note in TS 38.104 where appropriate that “Operators shall deploy SBFD uniformly across all sites with exactly the same subband configuration.”

Proposal 2	Specify -13 dBm/MHz as the requirement level for the SBFD in-channel adjacent sub-band leakage.

Proposal 3	Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

Proposal 4	For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

Proposal 5	Terminology “non-SBFD” need to be introduced.

Proposal 6	Use Figure 1 for the illustration of Transient period between SBFD and non-SBFD in TS 38.104.

# References

R4-2507724, SBFD general aspects, Ericsson

R4-2507726, Impact on SBFD BS RF TX requirements, Ericsson

R4-2507727, Impact on SBFD BS RF RX requirements, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP