3GPP TSG-RAN WG4 Meeting #115	R4-2507726

St Julian's, Malta, May 19th – 23rd 2025

Agenda Item:	7.25.3.2

Source:	Ericsson

Title:	Impact on SBFD BS RF TX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for SBFD.

Discussion on existing BS RF RX requirements for SBFD, new requirements for SBFD, and SBFD general aspects are detailed in separate submissions [1-3].

# 2	Discussion

Unwanted emissions

## Issue 1-1: Deployment scenarios for FR1 SBFD macro BS - Way forward - Encourage companies to provide more inputs on whether SBFD-SBFD co-ex case should be in Rel-19 scope. - If it is in the scope of Rel-19, companies are also encouraged to provide input on whether/how to derive the additional requirements. FFS aspects that may need to be taken into account include: - Expected emissions and blocking levels, which could be estimated by means of considering, e.g.: - Option 1: Potential expected coupling loss and total receiver desensitization (considering self-interference, co-channel interference and adjacent operator interference) considering relevant site deployment scenarios - Option 2: Co-existence simulation and potentially analysis of co-located scenarios - Expected spectrum allocation and sub-band sizes - Expected feasible filtering performances considering the above two aspects

General conditions and limitations for adjacent operator co-existence

The scenario under consideration for the SBFD-SBFD co-existence requirement is the one in which operators are protected from transmitter emissions leakage and receiver blocking requirements to/from other operators based on the involved operators (i) Deploying SBFD and (ii) meeting especially designed requirements on in band emissions and blocking that consider the scenario of adjacent operator SBFD operation.

The reason for deploying SBFD is that the existence of sub-bands allows for a significant frequency range within which a filter can roll-off. Examples for two operators and three operators are provided in figures 1-2.

Figure 1: SBFD-SBFD co-existence arrangement (three operators)

Figure 2: SBFD-SBFD co-existence arrangement (two operators)

Operator should provide suppression of unwanted emissions towards one another. For example, operator 1 must attenuate emissions from their downlink sub-band towards operator 2. The filter can roll-off within the DL sub-band of operator 2. Similar ranges exist for suppression of DL sub-band emissions between operators.

Operator 2 must protect their receiver from the DL sub-band of operator 1. The attenuation can similarly be provided within the frequency range of the operator 2 DL sub-band. Similar ranges exist for supressing UL blocking towards the receiver for other operators.

By this means, SBFD operators can provide filtering to avoid interference between one another.

There exist some general constraints to enable this kind of co-existence between operators. Firstly, each operator must be protected from unwanted emissions from any other operator within the band. This means that every operator must supress emissions in all of the band. This will not be guaranteed if there is any operator that has a deployment that is TDD and does not meet the additional requirements. Thus, all operators in the band need to deploy SBFD filtering.

- All operators in the band must deploy SBFD filtering.

- The proposed scenario will only suppress the inter-operator interference, meanwhile, the challenges on other types of interference still exist.

From the receiver point of view, an SBFD operator can protect their receiver as long as the blocker is supressed by a filter within their DL sub-bands. However, the more operators there are in the band, the greater will need to be the suppression in some cases; for example, if there are multiple operators that have BSs close to one another or on the same tower then each operator will experience the combined power from the other operators.

Although not directly part of operator-operator co-existence, it is important to note that SBFD can only function if an operator deploys SBFD at all sites on the same carrier. Consider, for example that FSPL (similar to the pathloss for a rooftop deployment) for 500m is around 100 dB. If an operator would have SBFD deployed at one site, but not at the next then the SBFD uplink sub-band could experience interference of around -70 dBm/MHz (plus/minus antenna gains), which is 40dB above the noise floor. Between sectors of the same site, the interference towards the UL sub-band if another sector would not deploy SBFD would be even larger, although it may be expected that SBFD would be deployed for all sectors of a site. Inter-site CLI would not be useful here if the other site would not be SBFD capable.

Taking these issues into account, in order for the paradigm under which the additional new emissions and blocking requirements for SBFD-SBFD would be designed to work, all operators would need to deploy SBFD BSs at every site.

- Operators must deploy SBFD at every site to avoid sites at which SBFD does not work due to interference.

In our understanding, for operators to mutually invest in deploying SBFD at every site within a band, a strong commercial agreement would be required between the operators. Simply being interested would not be sufficient; it would not be justifiable for one operator to deploy SBFD BSs across their network without certainty that all of their neighbours would do, since to do so would risk an investment in a network that would fail due to interference, as long as any operator in the same band decides to deploy TDD without investing in analogue filters to suppress emissions to protect other operators.

As is discussed further below, the BS design to meet SBFD-SBFD co-existence requirements would need to incorporate analogue filters based on the sub-band frequencies of the own and other operators. Thus, the SBFD BSs deployed by the operators would need to assume a carrier and sub-band allocation between operators that would not always be adjustable without hardware changes by all involved operators.

- The flexibility to configure carrier and sub-band allocations within a band without hardware changes would be restricted due to filtering, and may need coordination between operators.

Operator deployment scenarios

The purpose of operators collaborating in order to deploy SBFD would be badly undermined if there would be constraints on the operators also having to co-ordinate their BS sites in order to avoid being close to one another. In case there would be three operators within a band, co-ordination would become even more complex, since every operator would need to maintain at least a certain distance from every other operator.

In our view, if additional requirements for SBFD-SBFD deployments would be considered, then they should guarantee full flexibility for operator site deployment. This should, for example enable SBFD to function when operators are within 10m of one another, and also when they are sharing the same tower.

We remind that there is a general goal to be able to use existing site grids for e.g. 4GHz or upper 6GHz. If the new SBFD-SBFD requirements would require a complex redesign of the network layout with operators needing to co-ordinate and negotiate on site positions then the deployment scenario would not be useful.

- If new types of emissions and blocking requirements requiring operator collaboration are created then they should ensure that all kinds of deployment scenarios can work, including re-use of existing sites, close and co-located BS.

Spectrum and sub-band considerations

An SBFD-SBFD co-existence requirement would require analogue filtering to supress emissions and protect the SBFD receiver. The analogue filter characteristics would be designed based on the bandwidth of the DL and UL sub-bands, and those of neighbour operator sub-bands. Since BS design that meets the requirements would need to take the widths of the sub-bands into account, the specification would need to fix sub-band and carrier sizes.

The sub-band sizes (and associated filter feasibility) would depend on the specific band and sub-band sizes. It should be noted that once operators have deployed SBFD, as noted above, hardware upgrades might be needed by all operators to change carrier or sub-band allocations.

The bands for which SBFD-SBFD co-existence requirements have been speculated include the upper 6GHz and a potential future band of around 200MHz in the US. Without concrete details of the bands, the operator allocations and the operator sub-band sizes, specific requirements cannot be designed.

- Further details on band allocations, operator spectrum allocations and plans for sub-band sizes are needed to determine requirements and filtering feasibility.

Filtering feasibility considerations

As discussed above, for an SBFD-SBFD co-existence scenario to be workable, it should not place strong constraints on operators to maintain mutual distance between one another (especially not in the case of three operators), and should not prevent re-use of existing site grids. To achieve this, inter-operator BS distances of down to 10m, and also co-location should not be precluded.

Existing co-location requirements assume 30dB isolation between operators. For higher frequencies, this may be somewhat pessimistic, and a discussion on co-location requirements is taking place in the context of the BS RF enhancements WI.

Much more investigation is needed into the impact of SBFD interference between operators, considering beamforming and deployment scenarios. It should be noted that for isolation levels of 30-70dB, the emissions would need to be supressed to somewhere between -45 to -85 dBm. Blocking levels of between -20 to +20dBm would need to be dealt with at the receiver. This implies filtering suppression of up to 50-70dB; the exact filtering requirement needs further analysis

- Further analysis is needed on the filtering suppression and requirement levels.

Existing band- or sub-band filters need to supress emissions down to spurious emissions levels and out of band blocking within delta_f_OBUE and delta_f_OOB. It is quite likely that SBFD-SBFD scenario to work properly a larger amount of filtering is required than from sub-band filters. If the filtering requirements are stricter, this implies much higher Q factor, a large volume for the filters and potentially higher insertion loss and associated coverage compromise.

Although without extensive investigation it is not possible to determine how much emissions and suppression levels are needed, clearly there should also be a careful consideration of the feasibility of the filtering solution, considering both emissions and blocking suppression.

Use the two-operator case (see Figure 2) as an example, to protect operator 1’s UL subband, suppression of inter-operator interference from operator 2 is needed. This necessitates both a transmitter analogue filter for the whole carrier on operator 2’s side and a receiver analogue filter for the UL subband on operator 1’s side. The two analogue filters (one for transmission and one for reception) have different frequency positions, different past bands and different roll-offs, and are required per polarization per subarray.

- Both transmitter and receiver analogue filters are required per polarization per subarray to suppress inter-operator interference. The two types of analogue filters have different frequency positions, different pass bands and different roll-offs.

- An operator’s transmitter analogue filter is customized based on other operators’ subband configurations.

The theoretical UL coverage gain is 4.7 dB for SBFD compared with regular TDD (DDDDU pattern). It is reduced by self-interference, inter-sector and inter-site interference, and the fact that UE cannot transmit continuously. Therefore, it is important to avoid further losses. Adding TX and RX analogue filters will bring insertion loss, which should be carefully considered to maintain the rest of the gain.

- The insertion loss of both the TX and RX analogue filters should be carefully considered, as it can further reduce the theoretical UL coverage gain.

- Filter feasibility in terms of roll-off, size, shape needs to be studied.

RAN4 do not consider SBFD-SBFD adjacent operator scenario in Rel-19. If regulators decide or there is a strong commercial agreement among operators to require SBFD on adjacent operators in a band, a future WI will address necessary emission requirements.

RAN4 do not introduce the new emission requirements for FR1 WA BS in Rel-19.

Measurement of Averaged EVM

## Issue 1-2: Joint measurement for normal DL symbols/slots and SBFD symbols/slots - Way forward - Further discuss on the following aspects in the performance WI - Whether the following two slots configured with flexible symbols could be used for EVM measurement. - Flexible symbols per slot is configured with both SBFD symbols and guard period symbols or - Flexible symbols per slot is configured with SBFD symbols, guard period and also uplink symbols. - Separately measure the averaged EVM for SBFD and non-SBFD, and the averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. - Use the following text proposal as starting point. - --------------------------------------------------------Start of Text Proposal ------------------------------------------------------- # B.7 Averaged EVM EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval. For SBFD, only resource blocks allocated to downlink subbands shall be measured. For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step. - Where Ni is the number of resource blocks with the considered modulation scheme in slot i. - The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements: - Thus is calculated using in the expressions above and is calculated using in the calculation. - Thus we get: EVM=maxEVMframe,l,EVMframe,h For TDD or SBFD, let NdlTDD be the number of slots with downlink symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval. - EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS). EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2 - Where Ni is the number of resource blocks with the considered modulation scheme in slot i. - The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2). EVMframe=maxEVMframe,l,EVMframe,h - In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by. Nframe=10×NslotNdlTDD and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP. - Unite by RMS. EVM=1Nframek=1NframeEVMframe,k2 - ---------------------------------------------------------End of Text Proposal---------------------------------------------------------.

The averaged EVM for SBFD slots shall be calculated based only on the data samples where DLs are allocated within the SBFD slots. The proposed solution is to classify all symbols as either SBFD symbols or non-SBFD symbols and then spread non-SBFD symbols across multiple frames. For example, for the case of three SBFD symbols and one non-SBFD symbol, four frames should be used to average the non-SBFD symbols, rather than using just one. This approach ensures that there are the same number of symbols for non-SBFD as today.

Regarding the highlighted issue in the WF, in our view, the testing process should be static and based on a well-defined test model. Then the question is whether flexible symbols should be considered when defining the test model, which we believe should not be considered.

Regarding the averaged EVM testing, the same number of symbols is required for SBFD as today.

The testing process should be static and based on a well-defined test model, in which flexible symbols should not be considered.

# Conclusion

In the previous sections we made the following observations:

Observation 1	All operators in the band must deploy SBFD filtering.

Observation 2	The proposed scenario will only suppress the inter-operator interference, meanwhile, the challenges on other types of interference still exist.

Observation 3	Operators must deploy SBFD at every site to avoid sites at which SBFD does not work due to interference.

Observation 4	The flexibility to configure carrier and sub-band allocations within a band without hardware changes would be restricted due to filtering, and may need coordination between operators.

Observation 5	If new types of emissions and blocking requirements requiring operator collaboration are created then they should ensure that all kinds of deployment scenarios can work, including re-use of existing sites, close and co-located BS.

Observation 6	Further details on band allocations, operator spectrum allocations and plans for sub-band sizes are needed to determine requirements and filtering feasibility.

Observation 7	Further analysis is needed on the filtering suppression and requirement levels.

Observation 8	Both transmitter and receiver analogue filters are required per polarization per subarray to suppress inter-operator interference. The two types of analogue filters have different frequency positions, different pass bands and different roll-offs.

Observation 9	An operator’s transmitter analogue filter is customized based on other operators’ subband configurations.

Observation 10	The insertion loss of both the TX and RX analogue filters should be carefully considered, as it can further reduce the theoretical UL coverage gain.

Observation 11	Filter feasibility in terms of roll-off, size, shape needs to be studied.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 do not consider SBFD-SBFD adjacent operator scenario in Rel-19. If regulators decide or there is a strong commercial agreement among operators to require SBFD on adjacent operators in a band, a future WI will address necessary emission requirements.

Proposal 2	RAN4 do not introduce the new emission requirements for FR1 WA BS in Rel-19.

Proposal 3	Regarding the averaged EVM testing, the same number of symbols is required for SBFD as today.

Proposal 4	The testing process should be static and based on a well-defined test model, in which flexible symbols should not be considered.

# References

R4-2507727, Impact on SBFD BS RF RX requirements, Ericsson

R4-2507725, New SBFD BS RF requirements, Ericsson

R4-2507724, SBFD general aspects, Ericsson