3GPP TSG-RAN WG4 Meeting #115		R4-2507733

St Julian’s, Malta, 19th – 25th May

Title:	LS Reply on precompensation for NB-IoT NTN TDD mode

Response to:	LS R1-2503142 on precompensation for NB-IoT NTN TDD mode from RAN1

Release:	Release 19

Work Item:	Introduction of IoT-NTN TDD mode (IoT_NTN_TDD-Core)

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact person:	Ville Vintola

vvintola@qti dot qualcomm dot com

+1858 three six one 8110

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN4 would like to thank RAN1 for informing RAN4 about the agreement regarding UE behaviour in NTN TDD system regarding the pre-compensation. RAN4 do not see any issues with the agreement and has agreed to incorporate the agreement in TS 36.102 clause 6.4B.1 as follows:

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3] or except for band 249 UE is allowed to perform pre-compensation before the beginning of the uplink burst.

The highlighted part as the new text for this part.

# 2	Actions

To RAN1

ACTION: 	RAN4 asks RAN1 if the included TS change is in accordance with the RAN1 agreement, if it is, no action is needed from RAN1. If it is not, RAN4 asks for further guidance from RAN1

# 3	Dates of next WG RAN4 meetings

RAN4#116	2025-08-25- 2025-08-29	TBC, India

RAN4#116-bis	 2025-10-13- 2025-10-17	 Prague, CZ