3GPP TSG-RAN WG4 Meeting #115 	R4-2507738

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR 38.133 Inter-frequency measurement requirements with XR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inter-frequency measurements with XR are not supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement requirements are introduced for inter-frequency measurements with measurement gaps which are cancelled due to the UE XR operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-frequency measurements with XR are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2; 9.3.1, 9.3.4, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### --- start of Change 1 ---

### 9.1.2	Measurement gap

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE does not support independent measurement gap patterns for different frequency ranges as specified in table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply the network must provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers.

If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports independent measurement gap patterns for different frequency ranges as specified in table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply the network must provide either per-FR measurement gap patterns for frequency range where UE requires per-FR measurement gap for concurrent monitoring of all frequency layers of each frequency range independently, or a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers of all frequency ranges.

If the UE is configured via LPP [34] to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference measurement, PRS-RSRPP measurement, RSCP and RSCPD measurement defined in TS 38.215 [4], in order for the requirements in clauses 9.9.2, 9.9.3, 9.9.4, 9.9.6, 9.9.7 and 9.9.8 to apply, the network must provide

-	a single per-UE measurement gap pattern for concurrent monitoring of all positioning frequency layers and intra-frequency, inter-frequency and/or inter-RAT frequency layers of all frequency ranges, or

-	if UE supports independent measurement gap patterns for different frequency ranges for PRS measurement, i.e. supporting independentGapConfigPRS-r17, per-FR measurement gap pattern for the frequency range for concurrent monitoring of all positioning frequency layers and intra-frequency, inter-frequency cells and/or inter-RAT frequency layers in the corresponding frequency range.

During the per-UE measurement gaps the UE:

-	is not required to conduct reception/transmission from/to the corresponding E-UTRAN PCell, E-UTRAN SCell(s) and NR serving cells for E-UTRA-NR dual connectivity except the reception of signals used for RRM measurement(s) and the signals used for random access procedure according to TS 38.321 [7].

-	is not required to conduct reception/transmission from/to the corresponding NR serving cells for SA (with single carrier or CA configured) except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7].

-	is not required to conduct reception/transmission from/to the corresponding PCell, SCell(s) and E-UTRAN serving cells for NR-E-UTRA dual connectivity except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7].

-	is not required to conduct reception/transmission from/to the corresponding NR serving cells for NR-DC except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7].

During the per-FR measurement gaps the UE:

-	is not required to conduct reception/transmission from/to the corresponding E-UTRAN PCell, E-UTRAN SCell(s) and NR serving cells in the corresponding frequency range for E-UTRA-NR dual connectivity except the reception of signals used for RRM measurement(s) and the signals used for random access procedure according to TS 38.321 [7].

-	is not required to conduct reception/transmission from/to the corresponding NR serving cells in the corresponding frequency range for SA (with single carrier or CA configured) except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to TS 38.321 [7].

-	is not required to conduct reception/transmission from/to the corresponding PCell, SCell(s) and E-UTRAN serving cells in the corresponding frequency range for NR-E-UTRA dual connectivity except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to TS 38.321 [7].

-	is not required to conduct reception/transmission from/to the corresponding NR serving cells in the corresponding frequency range for NR-DC except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to TS 38.321 [7].

UEs shall support the measurement gap patterns listed in table 9.1.2-1 based on the applicability specified in tables 9.1.2-2 and 9.1.2-3. UE determines measurement gap timing based on gap offset configuration and measurement gap timing advance configuration provided by higher layer signalling as specified in TS 38.331 [2] and TS 36.331 [16].

Table 9.1.2-1: Gap Pattern Configurations

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 5.5 | 20 |
| 13 | 5.5 | 40 |
| 14 | 5.5 | 80 |
| 15 | 5.5 | 160 |
| 16 | 3.5 | 20 |
| 17 | 3.5 | 40 |
| 18 | 3.5 | 80 |
| 19 | 3.5 | 160 |
| 20 | 1.5 | 20 |
| 21 | 1.5 | 40 |
| 22 | 1.5 | 80 |
| 23 | 1.5 | 160 |
| 24 | 10 | 80 |
| 25 | 20 | 160 |

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE or NR-E-UTRA dual connectivity UE

| Measurement gap pattern configuration | Serving cell | Measurement PurposeNote 5 | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE | E-UTRA + FR1, or | non-NR RAT Note1,2 | 0,1,2,3 |
| Measurement gap | E-UTRA + FR2, or E-UTRA + FR1 + FR2 | FR1 and/or FR2 Note 6,7 | 0-11, 24, 25 |
|  |  | non-NR RATNote1,2 and FR1 and/or FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10, 24 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 | 0,1,2,3 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 only Note 6,7 | 0-11,24,25 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR2 only Note 6,7 | No gap |
| Per-FR | FR2 if configured |  | 12-25 |
| measurement gap | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 and FR2 Note 6,7 | 0-11,24,25 |
|  | FR2 if configured |  | 12-25 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | 12-25 |
|  | E-UTRA and, FR1 if configured | Non-NR RAT Note1,2 and FR1 and FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | 12-25 |
| NOTE: In E-UTRA-NR dual connectivity mode, if GSM or UTRA TDD or UTRA FDD inter-RAT frequency layer is configured to be monitored, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. In NR-E-UTRA dual connectivity mode, if UTRA FDD inter-RAT frequency layer is configured to be monitored for SRVCC, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 1: In E-UTRA-NR dual connectivity mode, non-NR RAT includes E-UTRA, UTRA and/or GSM. In NR-E-UTRA dual connectivity mode, non-NR RAT means E-UTRA, and UTRA for SRVCC. NOTE 2: Void NOTE 3: When E-UTRA inter-frequency RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. NOTE 4: For UE supporting supportedGapPattern-NRonly-NEDC or measGapPatterns-NRonly-ENDC-r16 but not supporting supportedGapPattern for the corresponding gap patterns among GP2-11, the corresponding gap patterns are not applicable to measurement of non-NR RATs as defined in NOTE 1. NOTE 5: Inclusion of positioning measurements: Measurement purpose which includes E-UTRA measurements includes also E-UTRA RSRP and E-UTRA RSRQ measurements for E-CID. NOTE 6: Measurement gap patterns #24 and #25 can be requested [2] only when the UE is configured at least with any of RSTD, UE Rx-Tx, PRS-RSRP, PRS-RSRPP, RSCP or RSCPD measurements requiring such gaps and can only be used during the corresponding positioning measurement period NOTE 7: Inclusion of positioning measurements for measurement gaps: Measurement purpose which includes any of FR1 and FR2 measurements includes also RSTD, UE Rx-Tx, PRS-RSRP, PRS-RSRPP, RSCP and RSCPD measurements. |  |  |  |

In E-UTRA-NR dual connectivity mode,

-	if per-UE measurement gap is configured with MG timing advance of TMG ms, the measurement gap starts at time TMG ms advanced to the end of the latest E-UTRA subframe occurring immediately before the configured measurement gap among MCG serving cells subframes.

-	if per-FR measurement gap for FR1 is configured with MG timing advance of TMG ms, the measurement gap for FR1 starts at time TMG ms advanced to the end of the latest E-UTRA subframe occurring immediately before the configured measurement gap among MCG serving cells subframes.

-	if per-FR measurement gap for FR2 is configured with MG timing advance of TMG ms, the measurement gap for FR2 starts at time TMG ms advanced to the end of the latest NR subframe occurring immediately before the configured measurement gap among SCG serving cells subframes in FR2.

In NR-E-UTRA dual connectivity mode,

-	if per-UE measurement gap is configured with MG timing advance of TMG ms, the measurement gap starts at time TMG ms advanced to the end of the latest NR subframe occurring immediately before the configured measurement gap among MCG serving cells subframes.

-	if per-FR measurement gap for FR1 is configured with MG timing advance of TMG ms and UE has NR serving cell in FR1, the measurement gap for FR1 starts at time TMG ms advanced to the end of the latest NR subframe occurring immediately before the configured measurement gap among MCG serving cells subframes in FR1.

-	if per-FR measurement gap for FR1 is configured with MG timing advance of TMG ms and UE does not have NR serving cell in FR1, the measurement gap for FR1 starts at time TMG ms advanced to the end of the latest E-UTRA subframe occurring immediately before the configured measurement gap among SCG serving cells subframes.

-	if per-FR measurement gap for FR2 is configured with MG timing advance of TMG ms, the measurement gap for FR2 starts at time TMG ms advanced to the end of the latest NR subframe occurring immediately before the configured measurement gap among MCG serving cells subframes in FR2.

In NR-NR dual connectivity mode,

-	If per-UE measurement gap is configured with MG timing advance of TMG ms, the measurement gap starts at time TMG ms advanced to the end of the latest MCG subframe occurring immediately before the configured measurement gap among MCG serving cells subframes.

-	If per-FR measurement gap for FR1 is configured with MG timing advance of TMG ms, the measurement gap for FR1 starts at time TMG ms advanced to the end of the latest MCG subframe occurring immediately before the configured measurement gap among MCG serving cells subframes.

-	If per-FR measurement gap for FR2 is configured with MG timing advance of TMG ms, the measurement gap for FR2 starts at time TMG ms advanced to the end of the latest SCG subframe occurring immediately before the configured measurement gap among SCG serving cells subframes in FR2.

TMG is the MG timing advance value provided in mgta according to TS 38.331 [2].

In determining the measurement gap starting point, UE shall use the DL timing of the latest E-UTRA or NR subframe occurring immediately before the configured measurement gap among E-UTRA or NR serving cells.

For per-FR measurement gap capable UE configured with E-UTRA-NR dual connectivity or NR-E-UTRA dual connectivity, when serving cells are in E-UTRA and FR1, measurement objects are in both E-UTRA/FR1 and FR2,

-	If MN indicates UE that the measurement gap from MN applies to E-UTRA/FR1/FR2 serving cells, UE fulfils the per-UE measurement requirements for both E-UTRA/FR1 and FR2 measurement objects based on the measurement gap pattern configured by MN;

-	If MN indicates UE that the measurement gap from MN applies to only LTE/FR1 serving cell(s),

-	UE fulfils the measurement requirements for FR1/LTE measurement objects based on the configured measurement gap pattern;

-	UE fulfils the requirements for FR2 measurement objects based on effective MGRP=20 ms;

For per-FR measurement gap capable UE configured with E-UTRA-NR dual connectivity, NR-E-UTRA dual connectivity or NR-NR dual connectivity, when serving cells are in E-UTRA, FR1 and FR2, or in E-UTRA and FR2, or in FR1 and FR2, measurement objects are in both E-UTRA /FR1 and FR2,

-	If MN indicates UE that the measurement gap from MN applies to E-UTRA/FR1/FR2 serving cells, UE fulfils the per-UE measurement requirements for both E-UTRA/FR1 and FR2 measurement objects based on the measurement gap pattern configured by MN.

Table 9.1.2-3: Applicability for Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA and NR-DC configuration)

| Measurement gap pattern configuration | Serving cell | Measurement Purpose NOTE 2 | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
|  | FR1 NOTE5, or FR1 + FR2 | non-NR RAT NOTE3,6 | 0,1,2,3 |
|  |  | FR1 and/or FR2 NOTE 8,9 | 0-11, 24, 25 |
|  |  | non-NR RAT and FR1 and/or FR2 NOTE3,6,8,9 | 0, 1, 2, 3, 4, 6, 7, 8,10, 24 |
| Per-UE measurement | FR2 NOTE5 | non-NR RAT only NOTE3,6 | 0,1,2,3 |
| gap |  | FR1 only NOTE 8.9 | 0-11, 24, 25 |
|  |  | FR1 and FR2 NOTE 8,9 | 0-11, 24, 25 |
|  |  | non-NR RAT and FR1 and/or FR2 NOTE3,6,8,9 | 0, 1, 2, 3, 4, 6, 7, 8,10, 24 |
|  |  | FR2 only NOTE 8,9 | 12-25 |
|  | FR1 if configured | non-NR RAT only | 0,1,2,3 |
|  | FR2 if configured | NOTE3,6 | No gap |
|  | FR1 if configured | FR1 only NOTE 8.9 | 0-11,24,25 |
|  | FR2 if configured |  | No gap |
|  | FR1 if configured | FR2 only NOTE 8.9 | No gap |
| Per-FR | FR2 if configured |  | 12-25 |
| measurement | FR1 if configured | non-NR RAT and | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
| gap | FR2 if configured | FR1 NOTE3,6,8,9 | No gap |
|  | FR1 if configured | FR1 and FR2 NOTE 8.9 | 0-11, 24,25 |
|  | FR2 if configured |  | 12-25 |
|  | FR1 if configured | non-NR RAT and | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured | FR2 NOTE3,6,8,9 | 12-25 |
|  | FR1 if configured | non-NR RAT and FR1 and FR2 NOTE3,6,8,9 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | 12-25 |
| NOTE 1: When E-UTRA inter-RAT RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. NOTE 2: Measurement purpose which includes E-UTRA measurements includes also inter-RAT E-UTRA RSRP and RSRQ measurements for E-CID; measurement purpose which includes E-UTRA measurements includes also E-UTRA RSRP and E-UTRA RSRQ measurements for E-CID. NOTE 3: Void NOTE4: If per-UE measurement gap is configured with MG timing advance of TMG ms, the measurement gap starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among all serving cells subframes. If per-FR measurement gap for FR1 is configured with MG timing advance of TMG ms, the measurement gap for FR1 starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among serving cells subframes in FR1. If per-FR measurement gap for FR2 is configured with MG timing advance of TMG ms, the measurement gap for FR2 starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among serving cells subframes in FR2. TMG is the MG timing advance value provided in mgta according to [2]. In determining the measurement gap starting point, UE shall use the DL timing of the latest subframe occurring immediately before the configured measurement gap among serving cells. NOTE 5: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. NOTE 6: In NR single carrier, NR CA, and NR-DC mode, non-NR RAT means E-UTRA, and UTRA for SRVCC. In NR single carrier, NR CA, and NR-DC mode, if UTRA FDD inter-RAT frequency layer is configured to be monitored for SRVCC, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 7: For UE only supporting supportedGapPattern-NRonly for any gap patterns among GP2-11, the corresponding gap patterns are not applicable to measurement of non-NR RATs as defined in NOTE 6. NOTE 8: Measurement gap patterns #24 and #25 can be requested [2] only when the UE is configured with any of RSTD, UE Rx-Tx time difference, PRS-RSRP, PRS-RSRPP, RSCP or RSCPD measurements requiring such gaps and can only be used during the corresponding positioning measurement period. NOTE 9: Inclusion of positioning measurements for measurement gaps: Measurement purpose which includes any of FR1 and FR2 measurements includes also RSTD, UE Rx-Tx time difference, PRS-RSRP, PRS-RSRPP , RSCP and RSCPD measurements. |  |  |  |

For per-FR measurement gap capable UE in NR standalone operation (with single carrier, NR CA and NR-DC configuration), for per-FR gap based measurement, when there is no serving cell in a particular FR, where measurement objects are configured, regardless if explicit per-FR measurement gap is configured in this FR, the effective MGRP in this FR is used to determine requirements;

-	20 ms for FR2 NR measurements

-	40 ms for FR1 NR measurements

-	40 ms for LTE measurements

-	40 ms for FR1+LTE measurements

For per-FR measurement gap capable UE in NR standalone operation (with single carrier, NR CA and NR-DC configuration), when serving cells are in FR1 or FR2, measurement objects are in both E-UTRA /FR1 and FR2,

-	If MN indicates UE that the measurement gap from MN applies to E-UTRA/FR1/FR2 serving cells, UE fulfils the per-UE measurement requirements for both E-UTRA/FR1 and FR2 measurement objects based on the measurement gap pattern configured by MN;

If measurement gap is configured in one FR but measurement object is not configured in the FR, the scheduling opportunity in the FR depends on the configured measurement gap pattern.

For single carrier or CA with aligned frame boundaries,

For E-UTRA-NR dual connectivity (with NR single carrier, NR CA configuration), if UE is not capable of per-FR-gap, total interruption time on SCG during MGL is defined only when MGL(N) = 20 ms, 10 ms, 6 ms, 4 ms and 3 ms. And if UE is capable of per-FR-gap, total interruption time on FR1 serving cells in SCG during MGL is defined only when MGL(N) = 20 ms, 10 ms, 6 ms, 4 ms and 3 ms, and total interruption time on FR2 serving cells in SCG during MGL is defined only when MGL(N) = 20 ms, 10 ms, 5.5 ms, 3.5 ms and 1.5 ms.

For NR standalone operation (with single carrier, NR CA and NR-DC configuration), if UE is not capable of per-FR-gap, total interruption time on a serving cell during MGL is defined when MGL(N) = 20 ms, 10 ms, 6 ms, 5.5 ms, 4 ms, 3.5 ms, 3 ms, and 1.5 ms. And if UE is capable of per-FR-gap, total interruption time on FR1 serving cells during MGL is defined only when MGL(N) = 20 ms, 10 ms, 6 ms, 4 ms, and 3 ms, and total interruption time on FR2 serving cells during MGL is defined only when MGL(N) = 20 ms, 10 ms, 5.5 ms, 3.5 ms, and 1.5 ms.

For NR-E-UTRA dual connectivity (with NR single carrier, NR CA configuration), if UE is not capable of per-FR-gap, total interruption time on MCG during MGL is defined only when MGL(N) = 20 ms, 10 ms, 6 ms, 4 ms, and 3 ms. And if UE is capable of per-FR-gap, total interruption time on FR1 serving cells in MCG during MGL is defined only when MGL(N) = 20 ms, 10 ms, 6 ms, 4 ms, and 3 ms, and total interruption time on FR2 serving cells in MCG during MGL is defined only when MGL(N) = 20 ms, 10 ms, 5.5 ms, 3.5 ms, and 1.5 ms.

For CA with non-aligned frame boundaries,

-	The total interruption time on an SCC is the same as the case CA with aligned frame boundaries, if no SCC slots are partially overlapped with the measurement gap.

-	The total interruption time on an SCC will be additionally extended by one SCC slot, if there exist SCC slots partially overlapped with the measurement gap.

(a)	Measurement gap with MGL = N(ms) with MG timing advance of 0 ms for all serving cells in synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and synchronous NE-DC, and for serving cells in MCG in NR standalone operation (with asynchronous NR-DC configuration)

(b)	Measurement gap with MGL = N(ms) with MG timing advance of 0.5 ms for all serving cells in synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and synchronous NE-DC, and for serving cells in MCG in NR standalone operation (with asynchronous NR-DC configuration)

(c)	Measurement gap with MGL = N(ms) with MG timing advance of 0 ms for all serving cells in asynchronous EN-DC and asynchronous NE-DC, and for serving cells in SCG in NR standalone operation (with asynchronous NR-DC configuration)

(d)	Measurement gap with MGL = N(ms) with MG timing advance of 0.5 ms for all serving cells in asynchronous EN-DC and asynchronous NE-DC, and for serving cells in SCG in NR standalone operation (with asynchronous NR-DC configuration)

Figure 9.1.2-1: Measurement GAP and total interruption time on serving cells for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC

The corresponding total number of interrupted slots on serving cells is listed in table 9.1.2-4 for all serving cells in synchronous EN-DC, NR standalone (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and for serving cells in MCG in NR standalone operation (with asynchronous NR-DC configuration). The corresponding total number of interrupted slots on serving cells is listed in table 9.1.2-4a for asynchronous EN-DC, and for serving cells in SCG in NR standalone operation (with asynchronous NR-DC configuration).

Table 9.1.2-4: Total number of interrupted slots on all serving cells during MGL for Synchronous EN-DC, NR standalone operation (with single carrier, NR CA and synchronous NR-DC configuration) and NE-DC, and on all serving cells in MCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0 ms is applied |  |  |  |  | When MG timing advance of 0.5 ms is applied |  |  |  |  |
| (kHz) | MGL=20 ms | MGL=10 ms | MGL=6 ms | MGL=4 ms | MGL=3 ms | MGL=20 ms | MGL=10 ms | MGL=6 ms | MGL=4 ms | MGL=3 ms |
| 15 | 20 | 10 | 6 | 4 | 3 | 21Note3 | 11Note3 | 7Note3 | 5Note3 | 4Note3 |
| 30 | 40 | 20 | 12 | 8 | 6 | 40 | 20 | 12 | 8 | 6 |
| 60 | 80 | 40 | 24 | 16 | 12 | 80 | 40 | 24 | 16 | 12 |
| 120 | 160 | 80 | 48 | 32 | 24 | 160 | 80 | 48 | 32 | 24 |
| 480 Note4 | 640 | 320 | 192 | 128 | 96 | 640 | 320 | 192 | 128 | 96 |
| 480 Note5 | 641 | 321 | 193 | 129 | 97 | 641 | 321 | 193 | 129 | 97 |
| 960 Note4 | 1280 | 640 | 384 | 256 | 192 | 1280 | 640 | 384 | 256 | 192 |
| 960 Note5 | 1281 | 641 | 385 | 257 | 193 | 1281 | 641 | 385 | 257 | 193 |
| NOTE 1: Void NOTE 2: NR SCSs of 120 kHz, 480 kHz and 960 kHz are only applicable to the case with per-UE measurement gap. NOTE 3: Non-overlapped half-slots occur before and after the measurement gap. Whether a Rel-15 UE can receive and/or transmit in those half-slots is up to UE implementation. NOTE 4: For NR SCSs of 480 kHz and 960 kHz, total number of interrupted slots on all serving cells during MGL for single carrier, intra-band NR CA with per-UE measurement gap. NOTE 5: For NR SCSs of 480 kHz and 960 kHz, total number of interrupted slots on all serving cells in SCG for inter-band NR-CA and synchronous NR-DC with per-UE measurement gap. |  |  |  |  |  |  |  |  |  |  |

Table 9.1.2-4a: Total number of interrupted slots on serving cells during MGL for Asynchronous EN-DC, and on all serving cells in SCG for NR standalone operation (with asynchronous NR-DC configuration) with per-UE measurement gap or per-FR measurement gap for FR1

| NR | Total number of interrupted slots on serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0 ms is applied |  |  |  |  | When MG timing advance of 0.5 ms is applied |  |  |  |  |
| (kHz) | MGL=20 ms | MGL=10 ms | MGL=6 ms | MGL=4 ms | MGL=3 ms | MGL=20 ms | MGL=10 ms | MGL=6 ms | MGL=4 ms | MGL=3 ms |
| 15 | 21 | 11 | 7 | 5 | 4 | 21 | 11 | 7 | 5 | 4 |
| 30 | 41 | 21 | 13 | 9 | 7 | 41 | 21 | 13 | 9 | 7 |
| 60 | 81 | 41 | 25 | 17 | 13 | 81 | 41 | 25 | 17 | 13 |
| 120 | 161 | 81 | 49 | 33 | 25 | 161 | 81 | 49 | 33 | 25 |
| 480 Note3 | 641 | 321 | 193 | 129 | 97 | 641 | 321 | 193 | 129 | 97 |
| 960 Note3 | 1281 | 641 | 385 | 257 | 193 | 1281 | 641 | 385 | 257 | 193 |
| NOTE 1: Void NOTE 2: NR SCSs of 120 kHz, 480 kHz and 960 kHz are only applicable to the case with per-UE measurement gap. NOTE 3: For NR SCSs of 480 kHz and 960 kHz, total number of interrupted slots on all serving cells in SCG for asynchronous NR-DC with per-UE measurement gap. |  |  |  |  |  |  |  |  |  |  |

In case that UE capable of per-FR measurement gap is configured with per-FR measurement gap for FR2 serving cells, total number of interrupted slots on FR2 serving cells during MGL is listed in table 9.1.2-4b.

Table 9.1.2-4b: Total number of interrupted slots on FR2 serving cells during MGL for EN-DC, NR standalone operation (with single carrier, NR CA and NR-DC configuration) and NE-DC with per-UE measurement gap or per-FR measurement gap for FR2

| NR | Total number of interrupted slots on FR2 serving cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS | When MG timing advance of 0 ms is applied |  |  |  |  | When MG timing advance of 0.25 ms is applied |  |  |  |  |
| (kHz) | MGL= 20 ms | MGL= 10 ms | MGL= 5.5 ms | MGL= 3.5 ms | MGL= 1.5 ms | MGL= 20 ms | MGL= 10 ms | MGL= 5.5 ms | MGL= 3.5 ms | MGL= 1.5 ms |
| 60 | 80 | 40 | 22 | 14 | 6 | 80 | 40 | 22 | 14 | 6 |
| 120 | 160 | 80 | 44 | 28 | 12 | 160 | 80 | 44 | 28 | 12 |
| 480 Note3 | 640 | 320 | 176 | 112 | 48 | 640 | 320 | 176 | 112 | 48 |
| 960 Note3 | 1280 | 640 | 352 | 224 | 96 | 1280 | 640 | 352 | 224 | 96 |
| NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell. NOTE 3: For NR SCSs of 480 kHz and 960 kHz, total number of interrupted slots on FR2-2 serving cells during MGL for NR standalone operation (single carrier, NR CA and NR-DC) with per-UE measurement gap or per-FR measurement gap for FR2-2. NOTE 4: If UE is configured with per-UE measurement gap, the MG timing advance of 0.25 is applicable only for FR2 NR SA (with both single carrier and NR CA configuration). |  |  |  |  |  |  |  |  |  |  |

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s),

-	when MGTA is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap

-	when MGTA is applied and the SCS of the UL carrier is other than 15 kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap

-	when MGTA is applied and the SCS of the UL carrier is 15 kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the slot partially overlapped with measurement gap

where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if  for the UL transmission is less than the length of one slot; L=2 otherwise.

Note: Network is supposed to take into account the possible difference between the estimated TA at network and actual TA at UE when scheduling UE in the above slot(s).

Table 9.1.2-5: (Void)

If a positioning measurement gap is configured via PosGapConfig and activated by MAC CE, the measurement requirements do not apply for RRM measurement that requires measurement gaps.

A measurement gap occasion is considered as cancelled measurement gap occasion, when the following conditions are met:

- an explicit indication is received via DCI (TBD: RAN1 reference) to skip this particular measurement gap occasion,

- the UE is not configured with positioning measurements requiring measurement gaps,

- the UE is not configured with any MUSIM measurement gap pattern via MUSIM-GapConfig [2],

- the UE is not configured with concurrent measurement gaps,

- FFS: other conditions.

For any cancelled measurement gap occasion:

- The UE shall not apply the measurement gap RF switching time in the beginning and in the end of the cancelled measurement gap and shall be ready to transmit or receive the XR traffic scheduled by the DCI during the time of the entire cancelled measurement gap occasion on the corresponding carrier frequency;

- For transmitting UL XR data during the cancelled measurement gap occasion, the guard period for DL-to-UL switching shall be within the duration of the cancelled measurement gap occasion;

- For transmitting UL XR data during the cancelled measurement gap occasion, the time of RF switching from DL to UL and back, shall be within the duration of the cancelled measurement gap occasion.

### --- end of Change 1 ---

### --- start of Change 2 ---

### 9.3.4	Inter-frequency measurement with measurement gaps

When measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured for the FR1 and FR2-1 target frequency layers and UE supporting deriveSSB-IndexFromCellInterNon-NCSG-r17. Otherwise UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter-frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.4-1, table 9.3.4-2, table 9.3.4-5 when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17 and table 9.3.4-9 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.4-7 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.4-2 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.4-2 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TPSS/SSS_sync_inter is given in table 9.3.4-9; otherwise, TPSS/SSS_sync_inter is given in table 9.3.4-2.

-	For UE with at least one non-available SMTC occasion within a measurement gap occasion cancelled due to the UE XR operation during TPSS/SSS_sync_inter, the TPSS/SSS_sync_inter is given in tables 9.3.4-12 and 9.3.4-13, for FR1 and FR2-1, respectively, for XPSS/SSS,gaps ≤ XPSS/SSS,gaps,max. Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.4-3, table 9.3.4-6 when highSpeedMeasInterFreq is configured and UE supports measurementEnhancementInterFreq-r17, and table 9.3.4-10 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.4-8 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.4-4 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.4-4 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, if SMTC ≤ 40ms, TSSB_measurement_period_inter is given in table 9.3.5-5; otherwise, TSSB_measurement_period_inter is given in table 9.3.5-2.

-	For UE with at least one non-available SMTC occasion within a measurement gap occasion cancelled due to the UE XR operation during TSSB_time_index_inter, the TSSB_time_index_inter is given in tables 9.3.4-14 and 9.3.4-15, for FR1 and FR2-1, respectively, for Xind,gaps ≤ Xind,gaps,max. Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.5-1, table 9.3.5-2, table 9.3.5-3 when highSpeedMeasInterFreq is configured and UE supports measurementEnhancementInterFreq-r17, and in table 9.3.5-5 when highSpeedMeasFlagFR2-r17 is configured and UE supports measEnhCAInterFreqFR2-r18. When the SCG is deactivated, table 9.3.5-4 applies for an inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.5-2 applies for an inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.5-2 applies for an inter-frequency carrier configured only by MCG.

-	For UE supporting power class 6 and measEnhCAInterFreqFR2-r18 with highSpeedMeasFlagFR2-r17 configured, TSSB_measurement_period_inter is given in table 9.3.5-5; otherwise, TSSB_measurement_period_inter is given in table 9.3.5-2.

-	For UE with at least one non-available SMTC occasion within a measurement gap occasion cancelled due to the UE XR operation during TSSB_measurement_period_inter, the TSSB_measurement_period_inter is given in section 9.3.5, tables 9.3.5-6 and 9.3.5-7, for FR1 and FR2-1, respectively, for Xmeas ≤ Xmeas,max. Upon exceeding Xmeas,max the UE shall restart the measurement.

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 96. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 60.

MSSB_index_inter: For a UE supporting FR2-1 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 4, MSSB_index_inter = 24 samples. For a UE supporting FR2-2 power class 2 or 3, MSSB_index_inter = 48 samples. For a UE supporting FR2 power class 1, MSSB_index_inter = 72 samples.

Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter =64. For a UE supporting FR2-1 power class 2, Mmeas_period_inter=40. For a UE supporting FR2-1 power class 3, Mmeas_period_inter =40. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 40. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 96. For a UE supporting FR2-2 power class 2, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 60.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent GAPs or MUSIM gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and/or per-FR measurement GAPs within the same FR, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other GAP occasions and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for the GAP and MUSIM gap in clauses 9.1.8.3, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Kgap is only applicable for UE supporting concurrent GAPs or MUSIM gaps. When concurrent GAPs or MUSIM gaps are configured, requirements in this clause do not apply if Navailable =0.

When UE supports [musim-GapPreference-r17] and if the configured aperiodic MUSIM gap collides with the measurement gap associated with the target frequency layer, where MUSIM gap collision rule in clause 9.1.10.4 is applied, longer cell identification period for the target inter-frequency is expected.

Table 9.3.4-1: Time period for PSS/SSS detection (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(8 * Kgap) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(8*1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-2: Time period for PSS/SSS detection, (FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(Kgap Mpss/sss_sync_inter x KFR) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter x KFR) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap Mpss/sss_sync_inter x KFR) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 4: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Table 9.3.4-3: Time period for time index detection (FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120 ms, Ceil(3 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil(3 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(3 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-4: Time period for time index detection (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap MSSB_index_inter) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-5: Time period for PSS/SSS detection when highSpeedMeasInterFreq-r17 is configured (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600 ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160 ms | max(600 ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160 ms < DRX cycle ≤ 320 ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320 ms | N4 x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: N4=6 if SMTC periodicity > 40 ms, otherwise N4=5 |  |

Table 9.3.4-6: Time period for time index detection when highSpeedMeasInterFreq-r17 is configured (FR1)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120 ms, 3 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil(3 M2 NOTE3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | 3 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

Table 9.3.4-7: Time period for PSS/SSS detection when the inter-frequency carrier is configured only by SCG and the SCG is deactivated (FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600 ms, Ceil(Kgap Mpss/sss_sync_inter) Max(MGRP , measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 * Kgap Mpss/sss_sync_inter) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap Mpss/sss_sync_inter) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-8: Time period for time index detection when inter-frequency carrier is configured only by SCG and the SCG is deactivated (FR2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap MSSB_index_inter) Max(MGRP, measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.4-9: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (FR2-1) when SMTC period ≤40ms

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600 ms, M1Note 3 x Kgap x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle≤ 80 ms | max(600 ms, ceil(M1Note 3 x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| 80 ms< DRX cycle≤ 320 ms | max(600 ms, ceil(Mpss/sss_sync_with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle>320 ms | Ceil( Mpss/sss_sync_with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 3: For UE supporting power class 6 and measEnhCAInterFreqFR2-r18, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.3.4-10: Time period for time index detection when when highSpeedMeasFlagFR2-r17 is configured (Frequency range FR2-1) when SMTC period <= 40 ms

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(Kgap , M1Note 3) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle≤ 80 ms | Max(200 ms, Ceil(1.5 * Kgap M1Note 3) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| 80 ms< DRX cycle≤ 320 ms | Max(200 ms, Ceil(1.5 * Kgap MSSB_index_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap MSSB_index_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 3: For UE supporting power class 6 and measEnhCAInterFreqFR2-r18, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.3.4-11: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_inter_less_than_5Mhz |
| --- | --- |
| No DRX | Max(120 ms, Ceil(6 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120ms, Ceil(6 M2 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(6* Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1. |  |

Table 9.3.4-12: Time period for PSS/SSS detection (FR1) for UE configured with MG cancellation due to its XR operation

| Condition NOTE1 | TPSS/SSS_sync_inter NOTE2 |
| --- | --- |
| No DRX | Max(600 ms, (8+XPSS/SSS,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8+XPSS/SSS,gaps) DRX cycle CSSFinter |
| NOTE 1: DRX or non-DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: When DRX is not configured, XPSS/SSS,gaps is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_inter for PSS/SSS detection, where XPSS/SSS,gaps ≤ XPSS/SSS,gaps,max. When DRX is configured, XPSS/SSS,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_inter for PSS/SSS detection, where XPSS/SSS,gaps ≤ XPSS/SSS,gaps,max. NOTE 3: XPSS/SSS,gaps,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, XPSS/SSS,gaps,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and XPSS/SSS,gaps,max=5, when max(DRX cycle, SMTC period, MGRP) > 320 ms. NOTE 4: The requirements in this table apply for UE power class 3 only and provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. |  |

Table 9.3.4-13: Time period for PSS/SSS detection (FR2-1) for UE configured with MG cancellation due to its XR operation

| Condition NOTE1 | TPSS/SSS_sync_inter NOTE2 |
| --- | --- |
| No DRX | Max(600 ms, (Mpss/sss_sync_inter + XPSS/SSS,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(600 ms, Ceil(1.5 (Mpss/sss_sync_inter + XPSS/SSS,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mpss/sss_sync_inter+ XPSS/SSS,gaps) DRX cycle CSSFinter |
| NOTE 1: DRX or non-DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: When DRX is not configured, XPSS/SSS,gaps is the number of SMTC occasions not available at the UE during TPSS/SSS_sync_inter for PSS/SSS detection, where XPSS/SSS,gaps ≤ XPSS/SSS,gaps,max. When DRX is configured, XPSS/SSS,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TPSS/SSS_sync_inter for PSS/SSS detection, where XPSS/SSS,gaps ≤ XPSS/SSS,gaps,max. NOTE 3: XPSS/SSS,gaps,max=1.5Mpss/sss_sync_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, XPSS/SSS,gaps,max=Mpss/sss_sync_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and XPSS/SSS,gaps,max=0.5 Mpss/sss_sync_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms. NOTE 4: The requirements in this table apply for UE power class 3 only and provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. |  |

Table 9.3.4-14: Time period for time index detection (FR1) for UE configured with MG cancellation due to its XR operation

| Condition NOTE1 | TSSB_time_index_inter NOTE2 |
| --- | --- |
| No DRX | Max(120 ms, (3+Xind,gaps) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(120 ms, Ceil((3+Xind,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (3+Xind,gaps) DRX cycle CSSFinter |
| NOTE 1: DRX or non-DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: When DRX is not configured, Xind,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_inter for time index identification, where Xind,gaps ≤ Xind,gaps,max. When DRX is configured, Xind,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_inter for time index identification, where Xind,gaps ≤ Xind,gaps,max. NOTE 3: Xind,gaps,max=5 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xind,gaps,max=3 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and Xind,gaps,max=2 when max(DRX cycle, SMTC period, MGRP) > 320 ms. NOTE 4: The requirements in this table apply for UE power class 3 only and provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. |  |

Table 9.3.4-15: Time period for time index detection (FR2-1) for UE configured with MG cancellation due to its XR operation

| Condition NOTE1 | TSSB_time_index_inter NOTE2 |
| --- | --- |
| No DRX | Max(200 ms, (MSSB_index_inter + Xind,gaps) Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(1.5 (MSSB_index_inter + Xind,gaps)) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (MSSB_index_inter + Xind,gaps) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: When DRX is not configured, Xind,gaps is the number of SMTC occasions not available at the UE during TSSB_time_index_inter for time index identification, where Xind,gaps ≤ Xind,gaps,max. When DRX is configured, Xind,gaps is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_time_index_inter for time index identification, where Xind,gaps ≤ Xind,gaps,max. NOTE 3: Xind,gaps,max=1.5 MSSB_index_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xind,gaps,max=MSSB_index_inter, when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and Xind,gaps,max=0.5 MSSB_index_inter, when max(DRX cycle, SMTC period, MGRP) > 320 ms. NOTE 4: The requirements in this table apply for UE power class 3 only and provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. |  |

#### 9.3.4.1	Void

#### 9.3.4.2	Void

### --- end of Change 2 ---

### --- start of Change 3 ---

### 9.3.5	Inter-frequency measurements

When measurement gaps are provided for inter frequency measurements, or the UE supports capability of conducting such measurements without gaps, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 9.3.5-1 and 9.3.5-2. When highSpeedMeasInterFreq-r17 is configured, and UE supports measurementEnhancementInterFreq-r17, TSSB_measurement_period_inter is specified in table 9.3.5-3. When SCG is deactivated, TSSB_measurement_period_inter is specified in table 9.3.5-4 applies for inter-frequency carrier configured by SCG and not configured by MCG and table 9.3.5-2 applies for inter-frequency carrier configured by both SCG and MCG. Regardless of whether the SCG is activated or deactivated, table 9.3.5-2 applies for an inter-frequency carrier configured only by MCG.

Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (FR1)

| Condition NOTE1,2 | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200 ms, Ceil(8 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil(8 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(8 * Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.5-2: Measurement period for inter-frequency measurements with gaps (FR2)

| Condition NOTE1,2 | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400 ms, Ceil(Kgap Mmeas_period_inter) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 * Kgap Mmeas_period_inter) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap Mmeas_period_inter) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.5-3: Measurement period for inter-frequency measurements with gaps when highSpeedMeasInterFreq-r17 is configured (FR1)

| Condition NOTE1,2 | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200 ms, 7 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 160 ms | max(200 ms, ceil(7 x M2 NOTE3) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| 160 ms < DRX cycle ≤ 320 ms | ceil(7 x M2 NOTE3) x DRX cycle x CSSFinter |
| DRX cycle>320 ms | 4 x M2 NOTE3 x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

Table 9.3.5-4: Measurement period for inter-frequency measurements with gaps when the inter-frequency carrier is configured only by SCG and the SCG is deactivated (FR2)

| Condition NOTE1,2 | TSSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400 ms, Ceil(Kgap Mmeas_period_inter) Max(MGRP, measCyclePSCell)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 * Kgap Mmeas_period_inter) Max(MGRP, measCyclePSCell, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | Ceil(Kgap Mmeas_period_inter) Max(measCyclePSCell, DRX cycle) CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. |  |

Table 9.3.5-5: Measurement period for inter-frequency measurements with gaps when highSpeedMeasFlagFR2-r17 is configured (FR2-1) when SMTC period≤40ms

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(400 ms, ceil(M1Note 3 x Kgap ) x max(MGRP, SMTC period)) x CSSFinter |
| DRX cycle≤ 80 ms | max(400 ms, ceil(M1Note 3 x Kgap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFinter |
| 80 ms< DRX cycle≤ 320 ms | max(400 ms, ceil(Mmeas_period with_gaps x Kgap) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFinter |
| DRX cycle>320 ms | Ceil( Mmeas_period with_gaps x Kgap ) x max(MGRP, DRX cycle) x CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured. NOTE 3: For UE supporting power class 6 and measEnhCAInterFreqFR2-r18, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 |  |

Table 9.3.5-6: Measurement period for inter-frequency measurements with gaps (FR1) for UE configured with MG cancellation due to its XR operation

| Condition NOTE1 | TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(200 ms, (8 + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(200 ms, Ceil((8 + Xmeas) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (8 + Xmeas) DRX cycle CSSFinter |
| NOTE 1: DRX or non-DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: When DRX is not configured, Xmeas is the number of SMTC occasions not available at the UE during T SSB_measurement_period_inter for inter-frequency measurements with gaps, where Xmeas ≤ Xmeas,max. When DRX is configured, Xmeas is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_measurement_period_inter for inter-frequency measurements with gaps, where Xmeas ≤ Xmeas,max. NOTE 3: Xmes,max=12 for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xmes,max=8 when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and Xmes,max=5 when max(DRX cycle, SMTC period, MGRP) > 320 ms. NOTE 4: The requirements in this table apply for UE power class 3 only and provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. |  |

Table 9.3.5-7: Measurement period for inter-frequency measurements with gaps (FR2-1) for UE configured with MG cancellation due to its XR operation

| Condition NOTE1 | TSSB_measurement_period_inter NOTE2 |
| --- | --- |
| No DRX | Max(400 ms, (Mmeas_period_inter + Xmeas) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320 ms | Max(400 ms, Ceil(1.5 (Mmeas_period_inter + Xmeas) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320 ms | (Mmeas_period_inter + Xmeas) DRX cycle CSSFinter |
| NOTE 1: DRX or non-DRX requirements apply according to the conditions described in clause 3.6.1. NOTE 2: When DRX is not configured, Xmeas is the number of SMTC occasions not available at the UE during T SSB_measurement_period_inter for inter-frequency measurements with gaps, where Xmeas ≤ Xmeas,max. When DRX is configured, Xmeas is the number of DRX cycles in which at least one SMTC occasion is not available at the UE during TSSB_measurement_period_inter for inter-frequency measurements with gaps, where Xmeas ≤ Xmeas,max. NOTE 3: Xmes,max=1.5 Mmeas_period_inter for non-DRX or when max(DRX cycle, SMTC period, MGRP)≤40 ms, Xmes,max= Mmeas_period_inter when 40ms<max(DRX cycle, SMTC period, MGRP)≤320 ms, and Xmes,max=0.5 Mmeas_period_inter when max(DRX cycle, SMTC period, MGRP) > 320 ms. NOTE 4: The requirements in this table apply for UE power class 3 only and provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known. |  |

#### 9.3.5.1	Void

#### 9.3.5.2	Void

#### 9.3.5.3	Void

### --- end of Change 3 ---