3GPP TSG-RAN WG4 Meeting #115	R4-2507760

# St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		7.31.3.1

Source: 		Qualcomm Inc.

Title: 		NR NTN UE RF requirements

Document for:		Approval

## 1	Introduction

RAN4 has received a reply LS from RAN2 [1] on previous LS from RAN4 [2]. This contribution discusses the reply back to RAN2. Secondly, previous meeting concluded on further evaluation on power boosting, and this contribution provides our views.

## 2	Discussion

Simultaneous operation between NR NTN and GNSS

Since release 17, 3GPP has assumed that NR NTN is able to simultaneously operate with GNSS. This is needed for example for accurate Tx frequency pre-compensation. However, no studies were done on this. The current work item explicitly mentions that “GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.”

Simultaneous GNSS and NR-NTN operation is especially challenging for operating bands n255 and n254 due to the narrow frequency separation between the NR-NTN UL and GNSS. This results in both Tx emissions leaking on top of GNSS Rx, as well as the NR-NTN UL acting as jammer to GNSS. Both of these effects can result in UE not being able to get a location fix, potentially leading to field issues.

There are multiple options on improving the likelihood of successful GNSS reception. For example, UE could take advantage of a more complex dual-frequency GNSS receiver. This would increase UE complexity receiving signals at both lower and upper L-band would need more complicated RF front-end and antennas, as well as more advanced digital receiver. All UEs do not have these capabilities.

Alternatively, UE could use more advanced RF filters which would reject Tx leakage to GNSS. At the same time, GNSS band would also need to use similarly advanced filter to avoid NR NTN UL from jamming the receiver. However, filtering solutions are not always viable. For example, n254 operates immediately adjacent to GLONASS frequencies, so one would need a perfect “brickwall filter” to fully avoid interference scenarios. This kind of perfect filter does not exists. In some cases filters can work, but they do add to cost and size of the RF solution.

UE could also try to rely on natural gaps in UL transmissions. The satellite beam covers a large area and could serve a large number of users, and all of these users cannot transmit at the same time. However, there can also be situations when the network load is low, and a single user has a long continuous data transmission. Even half-duplex operation cannot guarantee long transmission gaps for UL heavy traffic.

As such, there is no fool-proof solution which in all scenarios for all UEs could guarantee simultaneous GNSS and NR-NTN operation, and standards solution is needed.

Generally, GNSS needs a sufficiently long and sufficiently interference free time period to get a reliable location fix. As different UE implementations have different RF filtering, antenna performance, emission performance and jammer tolerance, the definition on “sufficient” will vary between implementations. To facilitate the co-existence between GNSS and NR-NTN, a standardized TDM-solution which can guarantee gaps in NR UL transmissions would be needed. Wide configurability would be needed to address different UE implementations, while it is recognized some advanced UEs may not need this solution at all.

Based on our studies, as an example of one implementation, a TDM pattern which can guarantee maximum 50% duty cycle for few seconds could work Within these few seconds, the UL transmissions cannot be continuous for a long time. This kind of gap would be needed regularly, multiple times in a minute. Still, signaling should accommodate a wide range of parameters.

Proposal 1: Reply to RAN2 that to cover wide range of UE implementations periodicity of the IDC solution needs to range from few seconds to few tens of seconds. Duration of the solution can range from a second to ten seconds, and within this period Tx cannot exceed 50% duty cycle. Draft LS is provided in appendix.

Proposal 2: Emphasize to RAN2 that network needs to support the solution in real deployments.

RAN2 specifically mentioned autonomous denials in their reply. Current signaling for autonomous denials have spare values available.

AutonomousDenialParameters-r18 ::=  SEQUENCE {

autonomousDenialSlots-r18           ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1},

autonomousDenialValidity-r18        ENUMERATED {n200, n500, n1000, n2000}

}

There are different options to update autonomous denials parameters to enable up to 50% duty cycles, for example, extending autonomousDenialSlots-r18 to n500 or n1000 and taking use of the two spare values are available in autonomousDenialSlots-r18. Another approach is to extend the set of values in autotonousDenialValidity-r18 to include e.g. 20ms, 50ms and 100ms options. This would enable applying duty cycle of 50% (10ms out of 20ms).

Proposal 3: Ask RAN2 to consider taking the spare values in autonomousDenialSlots-r18 into use and add values n500 and n1000.

Proposal 4: Ask RAN2 to consider taking the spare values in autonomousDenialValidity-r18 into use and add values n20, n50 and n100.

It should be noted that it is not only the needs of NR connectivity that need to be considered. There are applications for location tracking which require more frequent locations. These updates are more frequent than the ones need for keeping NR connection and e.g. Tx-precompensation functional.

Observation 1: Applications like location tracking may set more stringent IDC requirements and require more frequent GNSS updates than what is required to keep NR connection functional.

Power boosting for NR NTN

Agreements:

- If defined, additional output power enhancement will be optional for UE to support.

  - FFS on need of new UE capability

- Evaluate potential to further enhance output power for RAN4#115:

  - Amount of output power enhancement [0.5 to 1 dB]

    - Less than 0.5 dB improvement is not specified

  - Initially consider all RB allocations and modulation orders

    - Consider both MPR reduction and MOP increase

      - FFS how to capture MOP increase in the specification

    - Choice between MPR reduction and MOP increase can be different for inner/outer allocations and modulation orders

  - Evaluate both HD-FDD and FD-FDD

    - Consider at least insertion loss impact

    - FFS whether to apply same enhancement to both HD-FDD and FD-FDD

  - Evaluate the option to support both rel-18 and rel-19 output power enhancement schemes simultaneously

    - FFS whether simultaneous support is specified

    - Consider whether PC3 or PC2 ACLR requirement should apply

  - FFS Applicability to PC3 non-RedCap NR NTN

In RAN4#114bis a WF [3] was agreed outlining the study for power boosting:

PC3 power boosting was evaluated in detail in rel-18, and in the end up to 1 dB power boosting was specified for pi/2 BPSK and QPSK modulation orders for inner allocations. Furthermore, boost is mandatory within so called enhanced inner region and optional within the rest of the inner allocations. Evaluations in rel-18 assumed that the transmitter is calibrated as usual to just meet the minimum performance requirements. There could be further potential for power boosting for cases where transmitter was margin to 3GPP minimum requirements.

Observation 2: Power boosting potential beyond rel-18 power boosting can be available if UE has margins to 3GPP minimum requirements.

Given that it has been agreed that the boosting is optional, the most straightforward approach to enable the boosting would be similar to used for allocations outside the enhanced inner region in rel-18:

- Reference power is increased by the amount of power boosting

- Allowed MPR is increased by the amount of power boosting

However, the downside of this approach is that there is no guaranteed power boost, only an allowance to UE. As such a more preferable approach would be to have a set of RB allocations where boosting is mandatory, if supported. To differentiate the UEs which support power boosting, new UE capabilities are needed. Similar to rel-18 power boosting, the boost could be enable by network indication. This also enables RAN5 to define test cases for UEs indicating support for the feature and require improvement for the performance.

Proposal 5: Define new UE capabilities to

- For UE to indicate support for the additional power boosting

- For network to enable the boosting

Proposal 6: Define a set of RB allocations/modulation orders for which power boosting is mandatory when supported and enabled by the network. For rest of the allocations in scope, MPR is increased by the amount of power boosting.

The specification impacts expected from this approach are similar to rel-18 power boosting: MPR clause needs to describe the impact to reference power and MPR, and configured max output power can further detail the conditions when power boost applies and include the boost to Pcmax formulas. Speaking of the conditions, there are many for the rel-18 power boosting:

ΔPPowerBoost is defined as 1dB for power class 3 and 0.5dB for power class 2, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, and if IE powerBoostPi2BPSK-r18 and/or powerBoostQPSK-r18 is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2 in a TDD band or power class 3

-	IfΔPPowerClass is 0dB

-	If scheduled UL transmission is DFT-s-OFDM with either PI/2 BPSK modulation (with either Rel-15 DMRS or with pi/2 BPSK DMRS) or QPSK modulation

-	If the RB allocation belongs to the inner region defined in clause 6.2.2.

-	If UE indicates power class 3, the percentage of uplink symbols transmitted in a certain evaluation period is less than 80%

-	If UE indicates power class 2, when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*50%; or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is less than 0.9*maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame)

-	0 dB otherwise;

It is necessary to examine which additional conditions the additional power boosting requires. If the two power boosting schemes are simultaneously supported (boost on top of boost), the conditions also should not be conflicting. If the two schemes consider different modulation orders and RB allocations and as such are not simultaneously active, the conditions can be different.

Proposal 7: Exact conditions for power boosting to apply needs to be defined, and if simultaneous rel-18 and rel-19 power boosting is considered the interplay of the conditions needs to be considered

As specification changes which are different from TN specification are needed for MPR and configured maximum output power, at least the necessary specification sections should be written in 38.101-5. Moving the full content of MPR and configured maximum output power should be considered. However, this comes with a risk that a maintenance change is made only to TN specification, and is forgotten to be carried over to NTN specification.

Proposal 8: As the changes are different from TN specification, at least necessary MPR and configured maximum output power changes need to be captured in TS 38.101-5.

For impact to minimum requirements, a number of new co-existence evaluations were recently made for HPUE ACLR. In those evaluations it was observed the PC3 ACLR could be applied also to higher power classes, however it was decided to align with TN ACLR requirements for simplicity and alignment between specifications. Most importantly, no co-existence concerns were found.

Proposal 9: Based on co-existence evaluations PC3 ACLR requirement can apply to power boosted transmissions.

As the new power boosting scheme is already agreed to be optional, it can be defined for both FD-FDD and HD-FDD. It is expected that there is no impact to Rx requirements, but it is up to RAN5 whether to define test cases for this.

Proposal 10: New boosting scheme applies for both FD-FDD and HD-FDD

Generally RedCap implementation is considered as simplified from non-RedCap, implicit already from the name REDuced CAPability. As such, we see that if RedCap device can enable power boosting, it is reasonable to apply the same option for the “more capable” non-RedCap NR NTN UEs.

Proposal 11: New boosting scheme applies for both PC3 RedCap and non-RedCap PC3 NTN UEs.

## 3	Conclusion

In this contribution reply to RAN2 on simulataneous operation between GNSS and NR NTN was discussed. Following observations and proposals were made.

Simultaneous operation between NR NTN and GNSS

Proposal 1: Reply to RAN2 that to cover wide range of UE implementations periodicity of the IDC solution needs to range from few seconds to few tens of seconds. Duration of the solution can range from a second to ten seconds, and within this period Tx cannot exceed 50% duty cycle. Draft LS is provided in appendix.

Proposal 2: Emphasize to RAN2 that network needs to support the solution in real deployments.

Proposal 3: Ask RAN2 to consider taking the spare values in autonomousDenialSlots-r18 into use and add values n500 and n1000.

Proposal 4: Ask RAN2 to consider taking the spare values in autonomousDenialValidity-r18 into use and add values n20, n50 and n100.

Observation 1: Applications like location tracking may set more stringent IDC requirements and require more frequent GNSS updates than what is required to keep NR connection functional.

Power boosting for NR NTN

Observation 2: Power boosting potential beyond rel-18 power boosting can be available if UE has margins to 3GPP minimum requirements.

Proposal 5: Define new UE capabilities for

- UE to indicate support for the additional power boosting

- network to enable the boosting

Proposal 6: Define a set of RB allocations/modulation orders for which power boosting is mandatory when supported and enabled by the network. For rest of the allocations in scope, MPR is increased by the amount of power boosting.

Proposal 7: Exact conditions for power boosting to apply needs to be defined, and if simultaneous rel-18 and rel-19 power boosting is considered the interplay of the conditions needs to be considered

Proposal 8: As the changes are different from TN specification, at least necessary MPR and configured maximum output power changes need to be captured in TS 38.101-5.

Proposal 9: Based on co-existence evaluations PC3 ACLR requirement can apply to power boosted transmissions.

Proposal 10: New boosting scheme applies for both FD-FDD and HD-FDD

Proposal 11: New boosting scheme applies for both PC3 RedCap and PC3 non-RedCap NTN UEs.

## 4	References

[1]	R4-2503119/R2-2501568, Response LS on simultaneous operation between GNSS and NR NTN, RAN2

[2]	R4-2419898, LS on simultaneous operation between GNSS and NR NTN, RAN4

[3]	R4-2504708, Way Forward for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm Inc.

## Appendix

Title:	draft Reply LS on simultaneous operation between GNSS and NR NTN

Response to:	R4-2503119/R2-2501568

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact person:	Toni Lähteensuo

tlaehtee@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 would like to thank RAN2 for the LS reply in R2-2501568. RAN4 has discussed possible values for both TDM solution and autonomous denial solution.

According to RAN4, in worst case UE may need IDC free duration of up to 10s with a periodicity of approximately 2s to 40s for NR connection purposes.

There are also use cases like location tracking which require constant location updates, and for which autonomous denial solution is more appropriate. For autonomous denials, for some UEs up to 50% of duty cycle is possible provided long mute times are avoided. As an example, a UE can avoid transmitting 10ms out of every 20ms. It is recommended to add values n20, n50 and n100 to autonomousDenialValidity-r18 and values n500 and n1000 to autonomousDenialSlots-r18.

RAN4 would like to ask RAN2 to take the above information into account in developing a IDC solution which UE can rely to be supported in real deployments.

# Actions

ACTIONS:

To RAN WG2 and WG1: RAN4 respectfully asks RAN2 and RAN1 to ensure that NR NTN UEs can rely on an in-device co-existence solution for GNSS co-existence with sufficient gaps in NR UL transmissions, as described above.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #115	May 19 - 23, 2025	Malta, MT

TSG-RAN4 Meeting #116	Aug 25 - 29, 2025	Bangalore, IN