3GPP TSG-RAN WG4 Meeting #115                                          R4-2507767

# Malta, 19th – 23rd of May 2025

Title: 	SSB extension impact on UE cell search delay

Agenda Item:	7.31.2 – General aspects

Release:	Rel-19

Source: 	THALES

Document for:	Discussion

# Information

The SS/PBCH Block is utilized by the UE in the initial cell search process. A crucial aspect of this procedure is to reduce delays. This can be accomplished by establishing a sparse synchronization raster, which restricts the number of frequency domain positions that the UE needs to examine for the SS/PBCH Block.

Initial cell search/cell selection does not rely on any stored data. The UE scans all RF channels within its supported frequency bands. This scanning follows the synchronization raster, which is relatively coarse to minimize the number of potential carrier frequencies and, consequently, the delay caused by band scanning. At each candidate carrier frequency, the UE looks for one or more SS/PBCH Blocks, with each frequency assigned a Global Synchronization Channel Number (GSCN).

As per RAN1#118bis agreement, for cell search procedure a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 160ms. Due to the longer period between consecutive SS blocks, compared to the existing 20ms default periodicity, a device searching for NR carriers must dwell on each possible frequency for a longer time. In our view this additional delay when considering the new default value for SSB periodicity during the initial cell search will not cause none acceptable extra delay.

In principle, UE is allowed to use stored information to support the cell selection procedure. This can include carrier frequencies which the UE has previously camped on. It can also include cell parameters from previously received measurement control information or previously detected cells. In the case of cell selection after leaving RRC Connected mode, the Base Station can use the RRC Release message to direct the UE towards a specific carrier. A UE uses the initial cell selection procedure if cell selection based upon stored information is unsuccessful. If a new default value of SSB periodicity is introduced on top of existing default one of 20ms, the default value which is actually in use in the network could be stored at the UE as part of stored information to support the cell selection procedure.

The issue related to extra delay discussed above is only valid in case of a “cold start”. That’s is to say, the case UE of a UE without any a priori knowledge/information is stored at the UE to ease cell search and cell selection procedure. To mitigate the delay, as an optimization task, RAN1 may further discuss potential solutions to speed-up initial cell search procedure, especially for UE without any a priori knowledge. To reduce the overall search time while keeping the device complexity comparable to legacy design, NR NTN may support a sparse frequency raster for the SS block with 160ms periodicity. This implies that the possible frequency-domain positions of the SS block could be significantly sparser compared to the possible positions of an NR carrier (the carrier raster).

# Discussion

Moreover, in RAN1, the following agreement has been made (see also R1-2503702, RAN1#120-bis):

Agreement

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

- The additional default value assumed by UE during initial access (apart from the existing 20ms value) is 160 ms.

*** Unchanged parts are omitted *** A UE can be provided per serving cell by ssb-periodicityServingCell a periodicity of the half frames for reception of the SS/PBCH blocks for the serving cell. If the UE is not configured a periodicity of the half frames for receptions of the SS/PBCH blocks, the UE assumes a periodicity of a half frame. A UE assumes that the periodicity is same for all SS/PBCH blocks in the serving cell. For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames. If initial cell selection is for NTN, the UE may additionally assume that half frames with SS/PBCH blocks occur with a periodicity of 16 frames. For operation without shared spectrum channel access, an SS/PBCH block index is same as a candidate SS/PBCH block index. *** Unchanged parts are omitted ***

Proposal 1: For an FR1-NTN configuration under NR_NTN_Ph3 (Rel-19), when the NR NTN cell supports SSB periodicity of 160 ms for initial access, RAN4 to consider reducing the synchronization raster frequency points for reduced (overall) FR1-NTN UE cell search time.