3GPP TSG-RAN WG4 Meeting #115							         		R4-2507775

# Malta, MT, 19th – 23rd May, 2025

Agenda Item:	7.10.3.2.1 Tx requirements [NR_IoT_NTN_req_test_enh-Core]

Title: 	On power boosting options for PC2 NR-NTN

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# 1	Background

In R4#114b we demonstrated power boosting options based on a 1Tx PC2 PA in [1]. A way forward [2] captured a number of agreements on NR-NTN HPUE emissions and MPR while leaving power boosting for Release 19 open. In this contribution, we propose a way forwards on NTN PC2 power boosting in Release 19 and Release20.

# Discussion

## 2.1 Way forward input

The way forwards [2] captured an agreement on NR-NTN HPUE topics:

Issue 1-1-2: Power boosting option for PC2

- FFS on the power boosting option for PC2, including

  - Whether or not and how to define power boosting for PC2 as proposed above.

- The power boosting discussion not to impact the requirements discussion for PC2 without power boosting, e.g. MPR/AMPR, for PC2.

Issue 1-5: MOP and tolerance: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC1 and PC1.5 for NR-NTN Non-Redcap HPUE.

Issue 1-6: SEM

- Use agreed baseline approach for SEM.

  - Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline.

- No need to further discuss the FCC 47 CFR part 25 specification in clause 202 in this work item in this release.

Issue 1-7-1: PC2 and PC1.5 ACLR

- Agreement:

- Re-use existing TN ACLR:

  - For 2Tx PC1.5: Reuse TN PC1.5 ACLR

  - For 1Tx and 2Tx PC2: Reuse TN PC2 ACLR

Issue 1-8-1: PC2/1.5 MPR

- Agree to re-use TS 38.101-1 MPR for PC2 and PC1.5

  - TS 38.101-1 Table 6.2.2-2 applies to NR-NTN PC2 HPUE MPR with 1Tx

  - TS 38.101-1 Table 6.2D.2-1 applies to NR-NTN PC2 HPUE MPR with 2Tx

  - TS 38.101-1 Table 6.2D.2-3 applies to NR-NTN PC1.5 HPUE MPR with 2Tx

Issue 1-9: A-MPR Discuss the A-MPR in the draft CRs in next meeting.

Based on the above agreements, the MPR/SEM/ACLR is decided to be equivalent to the TN cases for PC2 and PC1.5. Thus, the only open aspect is whether a power boosting option for PC2 1Tx can be pursued and specified.

Since our measurement in [1] were already performed with the agreed ACLR and SEM, they stay valid after the outcome of the way forward [2]. They are reproduced in the next section as a technical background for the following discussion. Being based on 1Tx PC2 PA measurements, the outcomes are also partially valid for PC1.5 2Tx implementation which uses two PC2 PAs.

However, we recognize our bandwidth limitations to further study boosting capability for more inner third allocations cases and CBW and their potential in-band limitations like EVM or IBE. We also recognize that there is little time left in Release 19 and without the effort of other companies there is practically no chance to conclude in time for Release 19.

However, we strongly believe that improved power boosting for inner third allocation based on existing TN PC2 PA is a very good alternative to a 1Tx PC1.5 that is not practically available. It can deliver a very similar performance for the key narrow allocations used in NTN and stay within the thermal/power and DC current envelope of smartphones while enabling the reuse of existing hardware in the UE.

The inner third are especially promising allocations as shown in our measurements for LCRB up to NRB/3 and located at least LCRB/3 away from the edge.

Proposal:

- In Release 19, allow further PC2 NTN power boosting option evaluation based on a PC2 1Tx PA using existing TN impairment and targeting inner third allocations (LCRB up to NRB/3 located at least LCRB/3 away from the edge).

- The goal of this investigation would not be to define power boosting in Release 19, but to provide a case for this approach against or as a complement to a new 1Tx PC1.5 power class in Release 20

- This would be applicable to both TN and NTN

- Applicability of such boosting to multi-Tx based on PC2 PAs is FFS.

## 2.2 Measurement Results

We performed power boosting measurements on a PC2 1Tx Mid-band PA calibrated for 31dB ACLR for 20MHz QPSK DFT-s-OFDM 100RB0 waveform. Further investigations are necessary as the measurement was made with external current and thermal limitations based on NR waveforms which did not allow the maximum boosting, and more power should be achievable.

The measurement results are for a limited number of NR waveforms:

- 20MHz QPSK CP and DFT waveforms covering inner, outer, and inner third waveforms as discussed in [1].

- Transceiver impairments were set at the nominal NR 28dB and the 34dB 256QAM image and carrier leakage level.

- The power levels are at the antenna output assuming 4dB post PA losses as the PA is designed for PC2 1Tx NR power with up to 4dB post PA losses (30dBm 0dB MPR level at the PA output).

Table 1 provides the NR 31dB PC2 ACLR and SEM margins of the measured waveforms per output power, current increase versus MPR0 and for 28dB and 34dB levels of impairments:

- SEM0/1/2/3/4 stands for SEM regions with respective limits of -13dBm/1%CBW/-10dBm/MHz/-13dBm/MHz/-25dBm/MHz/-30dBm/MHz.

- Dark green corresponds to pass, light green equates to a just pass, orange is a just failed and red is failed.

- In the boost column, dark green is an all pass with margin and highest power. Light green is a pass but with little margin or lower power.

Table 1: UTRA ACLR margin versus PA output power for various NB-IoT waveforms

Observations:

- All inner third waveforms (in blue) pass with good margin regardless of the impairment level.

- Small allocations like 4RB2 also passe with margin and have improved margin with improved impairments.

- Even 1RB edge allocation passes with improved impairments but it may not be the case with relaxed windowing filters.

- Even with normal impairment levels, significant power boosting is achieved

- The power was limited due to external current limitation on the PA and it should be feasible to reach higher power once those limitations are controlled.

- Provided that no other in-band limitations like EVM and IBE are not dominating, up to 2.5dB boosting would be feasible

- Since these power boosting options are based on a 1Tx PC2 PA they are partially applicable to a 2Tx PC1.5 implementation.

# Conclusion

In this contribution, based on the set of results presented in last meeting, we make the following proposals on NTN PC2 power boosting based on a PC2 1Tx NR PA.

Proposal:

- In Release 19, allow further PC2 NTN power boosting option evaluation based on a PC2 1Tx PA using existing TN impairment and targeting inner third allocations (LCRB up to NRB/3 located at least LCRB/3 away from the edge).

- The goal of this investigation would not be to define power boosting in Release 19, but to provide a case for this approach against or as a complement to a new 1Tx PC1.5 power class in Release 20

- This would be applicable to both TN and NTN

- Applicability of such boosting to multi-Tx based on PC2 PAs is FFS.

# References

[1] R4-2504610 Initial power boosting result for NR-NTN for a PC2 NR PA, Skyworks Solutions Inc., R4#114b

[2] R4-2505113 WF on HPUE RF requirements for NR-NTN, Samsung, R4#114b