3GPP TSG-RAN WG4 Meeting #115	R4-2507813

Malta, MT, May 19 – 23, 2025

Agenda Item:	7.28.5.2

Source:	MediaTek Inc.

Title:	Discussion on the RRM core requirements for LP-WUS

Document for:	Discussion

# Introduction

The RRM core requirements for Rel-19 LP-WUS/WUR was further discussed in the last RAN4 meeting, and the achieved agreements are captured in the WF [1]. In this paper, we continue the discussion on the open issues in the RRM core part and provide our view. The WID [2] is also provided as below for reference.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

# Discussion

## Cases/states of RRM relaxation for LP-WUS capable UE

In RAN4#111 meeting, RAN4 agreed on Case#1 (RRM offloading case) first and then in RAN4#112 meeting, RAN4 agreed on Case#3 (RRM relaxation case). Regarding whether to discuss other cases for LP-WUS (as shown below in Issue 1-1-1), in our view the current agreed cases in RAN4, Case#1 and Case#3, are sufficient to consider both scenarios of MR RRM offloading and MR RRM relaxation of serving and neighbour cell measurements, which align with the WID objective in [2]. Therefore, RAN4 should focus on defining the core requirements for the current agreed cases, and if some issues are detected later to consider other cases, it can be revisited.

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Recommendations: Postpone the discussion until more RAN2 progress

Proposal 1: No need to discuss further cases for LP-WUS in RAN4.

In the last meeting, RAN4 also agreed to support relaxed higher priority frequency layer search at case 1 as shown below from the WF [1]:

Issue 1-1-10: Considerations on higher priority frequency layer Agreement: - Support relaxed higher priority frequency layer search at case 1

Form the above agreement, it is not clear which relaxation requirements will be applied for the measurements of higher priority frequency layer search at case 1. In the current spec 38.133, the relaxed higher priority frequency layer measurements are specified for different relaxation criterion (low mobility and not-at-cell edge criterion in section 4.2.2.10).

- It is noticed that for low mobility criteria, the relaxation of higher priority frequency layer is 60 times Thigher_priority_search described in clause 4.2.2.7.

When UE is only configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, - if the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60 - otherwise, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

- On the other hand, for not-at-cell edge criteria, the relaxation of higher priority frequency layer is same as no relaxation, i.e., it is same as Thigher_priority_search described in clause 4.2.2.7.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, then regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Therefore, RAN4 need to clarify which relaxation requirements will be applied for the measurements of higher priority frequency layer search at case 1, either based on low mobility relaxation criteria or not-at-cell edge criteria.

Proposal 2: Clarify which relaxation requirements is applied for the measurements of higher priority frequency layer search at case 1. The relaxation can be either based on

- low mobility criteria (i.e., 60 times), or

- not-at-cell edge criteria (i.e., 1 time)

## Entry/Exit conditions

Following the progress in RAN4, LP-WUS capable UE may operate under three scenarios (see Figure 1):

- RRM offloading (Case#1): when MR is deactivated and LR is activated.

- RRM relaxation (Case#3): when both MR and LR are activated.

- Legacy UE: when MR is activated and LR is deactivated.

Figure 1: Different cases of LP-WUR UE

For both cases (Case#1 and Case#3) RAN4 can define same evaluation requirements for exit conditions based on LR measurements only. On the other hand, the entry conditions can be evaluated by MR against MR threshold for both Case#1 and Case#3, however, if LR threshold is also configured then the LR evaluation can also be considered. Note that the evaluation of MR is based on DRX cycle (similar to legacy serving cell measurements Nserv), while LR evaluation should be based on LO cycle regardless of whether LP-SS is configured or not (Note: DRX cycle and LO periodicity are the same based on RAN1 agreement).

| Cases | Entry condition | Exit condition |
| --- | --- | --- |
| Case#1 (only LR) | - Evaluation based on MR using DRX cycle - Optional evaluation based on LR using LO cycle | - Evaluation based on LR using LO cycle |
| Case#3 (LR+MR) | - Evaluation based on MR using DRX cycle - Optional evaluation based on LR using LO cycle | - Evaluation based on LR using LO cycle |

Another issue needs to be discussed is that how to avoid the ping pong behaviour when transitioning between different cases. Activating and deactivating MR can consume more power and takes longer time compared to LR. Therefore, it is important to ensure UE can confidently switch to any case without having ping pong impact.

Proposal 3: Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

Since in Case#1 RRM measurements are fully offloaded to LR, UE need to be confident enough to enter Case#1 before deactivating MR. Therefore, we propose that RAN4 to study allowing the UE entering Case#3 first before entering Case#1 after a period of time (i.e., even when signal quality is higher than the threshold). This will help UE switching to Case#1 for RRM fully offloading more confidently after ensuring that signal quality condition does not change during Case#3 for a given period of time.

Proposal 4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1.

Regarding the evaluation requirement, RAN4 previous agreement:

RAN4#114-bis Issue 1-2-5-2: On MR evaluation requirements Agreement: - Reuse legacy Nserv - Note: the legacy values are Nserv = 4 when DRX = 0.32s and 0.64s, Nserv = 2 when DRX = 1.28 and 2.56s RAN4#114 Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

RAN4 need to discuss the number of samples required to define the evaluation duration requirements for LR for both types OOK LR and OFDM LR. Given LR perform worse than MR, a higher number of samples is required to achieve reliable evaluation.

Proposal 5: For LR evaluation requirements a higher number of samples is required compared to MR evaluation (where MR reuses legacy Nserv) to achieve reliable evaluation.

Also, regarding the applicability of MR and LR evaluation, in our view MR evaluation apply for the entry to case#1 and case#3, while LR evaluation apply to exit cases from case#1/3 to legacy case; and (optional) switching between case3# and case#1.

Proposal 6: On the applicability of MR and LR evaluation:

- MR evaluation applies to the entry to case#1 and case#3

- LR evaluation applies to exiting from case#1 and case#3 to legacy case; and (optional) switching between case3# to case#1

## On RAN1 LS

In RAN1#120 meeting, the LS R1-2501624 was sent to RAN4 as below:

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120:

Agreement

For UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

  - FFS: different sets of 3 candidate value for different SSB periodicities

RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

In the last meeting, RAN4 discussed the LS and agreed that the same number of SSBs will apply for different SSB periodicities and a lower bound periodicity of 20ms will be used for MR wake up delay as captured below.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR and RAN1 LS R1-2501624 Issue 1-2-6-1: On the number of SSBs Agreement: Aim to conclude the issue by Tuesday in RAN4 #115. Issue 1-2-6-2: On SSBs number for different SSB periodicities Agreement Same SSB number for different SSB periodicity Agreement When SSB periodicity is 5 ms or 10ms, a lower bound 20 ms will be used for MR wake up delay. Note: This above agreement based on the assumption that the wake up duration is defined as the ramping time + SSB number * SSB periodicity

However, in this meeting, RAN4 need to discuss on whether the number SSBs (3 and 5 samples) assumed by RAN1 is sufficient for MR wake up delay. The number of SSB samples required for synchronization will depend on the SNR condition. In our view,

- For deep sleep case:

  - For SNR condition ≥ -3dB, 3 samples are required (2 SSBs for AGC + 1 SSBs for time/freq tracking).

  - For SNR conditions <-3dB 5 samples are required (2 SSBs for AGC + 3 SSBs for time/freq tracking).

- For ultra deep sleep case, where UE can be out-of-sync:

  - For SNR condition ≥ -3dB, 5 samples are required (2 SSBs for AGC + 3 SSBs for time/freq tracking).

  - For SNR conditions <-3dB, 6 samples are required (2 SSBs for AGC + 4 SSBs for time/freq tracking).

Proposal 7: On the number of SSB samples used for MR wake up delay, RAN4 to consider the MR SNR condition with respect to the type of sleep as below:

| Scenario | #SSBs samples SNR ≥ -3dB | #SSBs samples SNR <-3dB | Notes |
| --- | --- | --- | --- |
| Deep sleep | 3 | 5 | 2 for AGC + 1~3 for time/frequency tracking |
| Ultra deep sleep | 5 | 6 | Additional samples for out-of-sync during ultra deep sleep |

# Summary

In this contribution, the following conclusions are provided:

Proposal 1: No need to discuss further cases for LP-WUS in RAN4.

Proposal 2: Clarify which relaxation requirements is applied for the measurements of higher priority frequency layer search at case 1. The relaxation can be either based on

- low mobility criteria (i.e., 60 times), or

- not-at-cell edge criteria (i.e., 1 time)

Proposal 3: Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

Proposal 4: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1.

Proposal 5: For LR evaluation requirements a higher number of samples is required compared to MR evaluation (where MR reuses legacy Nserv) to achieve reliable evaluation.

Proposal 6: On the applicability of MR and LR evaluation:

- MR evaluation applies to the entry to case#1 and case#3

- LR evaluation applies to exiting from case#1 and case#3 to legacy case; and (optional) switching between case3# to case#1

Proposal 7: On the number of SSB samples used for MR wake up delay, RAN4 to consider the MR SNR condition with respect to the type of sleep as below:

| Scenario | #SSBs samples SNR ≥ -3dB | #SSBs samples SNR <-3dB | Notes |
| --- | --- | --- | --- |
| Deep sleep | 3 | 5 | 2 for AGC + 1~3 for time/frequency tracking |
| Ultra deep sleep | 5 | 6 | Additional samples for out-of-sync during ultra deep sleep |

# References

- R4-2504908, WF on RRM requirements for NR_LPWUS, vivo.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO.

- R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD, CATT