3GPP TSG-RAN WG4 Meeting # 115	R4-2507816

Malta, MT, May 19 – May 23, 2025

Agenda Item: 7.24.4.1

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for UE-initiated/event-driven beam management

Document for: Discussion

# Introduction

According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. The WID [1] is divided into two parts to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “UE-initiated/event-driven beam management”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- RRM requirement on UE-initiated/event-driven beam management

- Capability on UE-initiated/event-driven beam management

## 2.1 RRM requirement on UE-initiated/event-driven beam management

In the last meeting, RAN4 reached the following agreements as captured in the WF [2]:

Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - For T_RS (TSSB or TCSI-RS) in L1_measurement periods - T_RS (TSSB or TCSI-RS): - the value of current beam and new beams are the same. - For M in L1_measurement periods: - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam - If timeRestrictionForChannelMeasurement is not configured, - Further discuss on: - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used - Option 2: No requirements even the NW can configure it. - Option 3: FFS - For P (sharing factor) in L1_measurement periods: - For SSB based or CSI-RS based, - The definition is the same as legacy - The value can be the same or different for the current beam and new beam

In legacy, the number of samples (M) is decided by whether RAN1 would have time domain restriction for channel measurements. For example, M=1 if higher layer parameter of time domain restriction (timeRestrictionForChannelMeasurement) is configured, and M=3 otherwise. In RAN4 we can reuse same values as legacy, it can be revisited if RAN1 make a different agreement otherwise.

Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. If timeRestrictionForChannelMeasurement is not configured, measurement period is based on 3 sample.

Since the L1-RSRP measurement for event trigger beam reporting has not much difference than legacy, we could reuse the P (sharing factor) in legacy for SSB based or CSI-RS based. We provided our view that whether the same or different P used in L1-RSRP measurement for current beam and new beam.

P value is same for intra-cell SSB resources included both current and new beam,

P value is different for inter-cell SSB resources included both current and new beam, defined as

- P = PL1 sharing * Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

- P = PL1 sharing * Ntotal / Navailable in FR2 with Navailable > 0

  - Ntotal is the total number of SSB resource occasions including those overlapped with GAP occasions or SMTC occasions, and

  - Navailable is the number of SSB resource occasions that are not overlapped with any gap occasion nor any SMTC occasion.

  - Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside gap is not overlapped SSB symbols for L3 measurement. Otherwise, Psharing factor = 3.

  - PL1_sharing = 2, if SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

P value is different for CSI-RS resources included both current and new beam, defined as

- P = PL1 sharing * Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

- P = PL1 sharing * Ntotal / Navailable in FR2 with Navailable > 0

  - Ntotal is the total number of CSI-RS resource occasions including those overlapped with GAP occasions or SMTC occasions, and

  - Navailable is the number of CSI-RS resource occasions that are not overlapped with any gap occasion nor any SMTC occasion.

  - Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside gap is not overlapped SSB symbols for L3 measurement. Otherwise, Psharing factor = 3.

  - PL1_sharing = 2, if CSI-RS resources from serving cell is configured for L1-RSRP measurement, and any symbol of the CSI-RSs from serving cell are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

Proposal 2: RAN4 to consider the legacy sharing factor (P) for L1-RSRP measurement as baseline:

- P value is same for intra-cell SSB resources included both current and new beam

- P value is different for inter-cell SSB resources included both current and new beam

- P value is different for CSI-RS resources included both current and new beam

Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: The event triggered measurement reporting delay includes the Tfirst UL channel - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH Further discuss the two options and select one option in the next meeting: The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK) - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC) RAN1 LS [3] Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting. Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? <Way forward > Option 1: (Qualcomm, Xiaomi, Apple, LGE) - Do not define requirements for eventInstanceCount-r19 > 1 Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured. Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19 Option 3: (Nokia, Ericsson) - Wait for further RAN1 conclusion For how to specify the requirements in option 2 above: - Option 2a: - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where, - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - C: event instances threshold is configured by NW. - Ttime_window: time window is configured by NW. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. - Other options are not precluded

Until the latest RAN1 progress related to RAN1 LS [3], there’s no RAN1 conclusion to support different periodicity of current and new beam. Therefore, RAN4 does not need to discuss something has not been supported in RAN1. Even if RAN1 later will support different periodicities, in our understanding, we could consider the maximum period of current beam and new beam. For example, if the SSB period of current beam and new beam is different, we could define the maximum measurement period as Max{TSSB_current, TSSB_new} for TL1_measurement period.

Proposal 3: RAN4 not to discuss the scenario of different periodicities for the current and new beam since RAN1 has no further agreement whether to support it or not.

- Consider the maximum periodicity regardless of same or different periodicities of current and new beam:

  - Max {TSSB_current, TSSB_new} for TL1_measurement period_SSB

  - Max {TCSI-RS_current, TCSI-RS_new} for TL1_measurement period_CSI-RS

In the case of “no time window”, this is the basic feature (M instance = 1), could discussed in issue 1-1. In the case of “time window with configurable M”, UE initiate beam reporting only if the number of instances for event 2 (i.e., the quality of new beam becomes a threshold value better than current beam) is larger than or equal to event instances threshold. Also, the evaluation should be done within time window. Otherwise, UE should re-evaluate whether the event condition is met since the time window is changed.

For the case of no time window, RAN4 can define the requirements for reporting delay based on the L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS as agreed in the previous meeting in Issue 1-2. However, whether to consider the uncertainty time of event condition is met to be part of event triggered reporting delay, we can either follow legacy L3 event triggered reporting principle (i.e., to exclude this uncertainty time) or to include the uncertainty time for UL reporting.

For the case of when time window is configured, during the configured time window there might be N number of instances to check for event 2, where N can be ≥ M event instances threshold. Based on RAN1 design, UE initiate beam reporting only if at least M number of event instances satisfy event 2. Therefore, if we want to define the requirements for this case, RAN4 need to consider the worst-case scenario. For example, when only M out of N instances meet event 2 condition, for which these M instances could occur anywhere within the configured time window as shown the figure below. There can be two ways to define the requirements for this case. One way is that the reporting delay with configured time window can reuse the same principle of Time To Trigger (TTT) in the legacy requirements. Another way is that the reporting delay can be defined based on the multiple (M) times of L1-RSRP measurement period within the time window.

Figure 1: the case of time window with configurable M

Proposal 4: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”:

Proposal 5: For the case of no time window:

- Measurement reporting delay is TL1_measurement period + Tfirst UL channel where,

- TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

- Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Proposal 6: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window.

Proposal 7: For the case of time window with configurable M (event instances threshold):

- Option 1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window.

- Option 2: Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

  - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

  - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW.

  - Ttime_window: time window is configured by NW.

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM <Way forward > Option 1: - Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management Option 2: - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Since there’s no L1-RSRP measurement for CSI-RS on inter-cell in legacy. To save time, we prefer to postpone the discussion until the enhancement of existing requirements are finished.

Proposal 8: Do not consider Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management.

RAN1 Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded Issue 1-6: Whether to specify ? <Way forward > For MAC-CE based DL/UL TCI state switch delay - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson) - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management - Option 2: (ZTE) - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc - Option 3: (MediaTek) - FFS on whether to remove Tfirst-SSB+TSSB-proc For DCI based DL/UL TCI state switch delay - Option 1: - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management - Option 2: - Whether to further reduce the delay, wait for further RAN1 progress

There’s RAN1 agreement of reducing additional SSB reception if new beam(s) reported or QCLed to SSB in the UEI/ED beam report. In our understanding, after UE send event beam reporting to NW, it can be expected that there will be TCI state switching command to target new beam from NW.

Follow the legacy known condition of TCI state, if NW send TCI state switching command within 1280ms upon the last transmission of the RS resource for beam reporting or measurement. Then UE will treat this target TCI state as known TCI state. For known TCI state, we could discuss whether to reduce additional SSB reception. However, there’s no RAN1 conclusion for several alternatives on this issue. If RAN1 agree to reduce additional SSB reception for TCI state of UE-initiated/event-driven beam, then RAN4 could consider skipping the fine timing sync (additional SSB reception) for known TCI state on MAC-CE based TCI state switching. Otherwise, reuse the existing unified TCI state delay requirement.

Proposal 9: RAN4 to consider skipping the fine timing sync (additional SSB reception) for known TCI state if RAN1 has further agreement on the reduction of additional SSB reception for TCI state of UE-initiated/event-driven beam. Otherwise, reuse the existing unified TCI state delay requirement.

Issue 1-7: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? <Way forward > Option 1: (Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. Option 2: - For such case, No RAN4 requirements. Option 3: (Apple) - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

In our understanding, all the events evaluation is at least included indicated TCI state which is current beam. Therefore, all the requirements should be in the scope of indicated TCI state. If indicated TCI state is changed, then no requirement is applied.

Proposal 10: No requirement is applied if indicated TCI states is changed during the L1 measurement time.

## 2.2 Capability on UE-initiated/event-driven beam management

Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)? <Way forward > Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities. Option 1: - For event trigger L1 reporting, do not define such capability in parallel per category like L3. Option 2: - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3. Issue 1-11: Compatibility of capabilities with LTM <Way forward > Option 1: (Apple, Ericsson) - Discuss it later. Option 2: (CMCC, Huawei, Nokia) - Not preferred to mix Rel-19 mobility and Rel-19 MIMO. The capability for supported number of events triggered L1 measurement reporting is defined separately for Rel-19 mobility and Rel-19 MIMO Option 3: (MediaTek) - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. TS 38.133 Table 9.1.4.2-1: Requirements for reporting criteria per measurement category Measurement category; Ecat; Note. Intra-frequency Note 1,2,3,4,5; 9; Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NGRAN intra-frequency cells. Inter-frequency Note 2,3,4,5; 10; Events for any one or a combination of inter-frequencyvSS-RSRP, SS-RSRQ, SS-SINR, CSIRSRP, CSI-RSRQ, and CSI-SINR for NG-RANvinter-frequency cells. …; …; …. NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode.

In legacy, capabilities for support of event triggered L3 measurement reporting had been specified in TS 38.133 section 9.1.4. For event-based reporting, each instance of event, with the same or different event identities, is counted as separate reporting criterion in Table 9.1.4.2-1.

In our understanding, we could follow the similar principle to define the number of events supported for UE-initiated beam reporting in MIMO.

Proposal 11: Define UE reported capabilities for numbers of event refer to legacy L3 (like legacy L3 in section 9.1.4).

- For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: Legacy number of samples (M) for L1-RSRP measurement can be reused. If timeRestrictionForChannelMeasurement is not configured, measurement period is based on 3 sample.

Proposal 2: RAN4 to consider the legacy sharing factor (P) for L1-RSRP measurement as baseline:

- P value is same for intra-cell SSB resources included both current and new beam

- P value is different for inter-cell SSB resources included both current and new beam

- P value is different for CSI-RS resources included both current and new beam

Proposal 3: RAN4 not to discuss the scenario of different periodicities for the current and new beam since RAN1 has no further agreement whether to support it or not.

- Consider the maximum periodicity regardless of same or different periodicities of current and new beam:

  - Max {TSSB_current, TSSB_new} for TL1_measurement period_SSB

  - Max {TCSI-RS_current, TCSI-RS_new} for TL1_measurement period_CSI-RS

Proposal 4: RAN4 to consider delay requirement of event 2 for both “no time window” and “with time window with configurable M”:

Proposal 5: For the case of no time window:

- Measurement reporting delay is TL1_measurement period + Tfirst UL channel where,

- TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

- Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Proposal 6: RAN4 to discuss how to define the delay requirement when only M out of N instances meet event 2 condition within the configured time window.

Proposal 7: For the case of time window with configurable M (event instances threshold):

- Option 1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window.

- Option 2: Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

  - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

  - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW.

  - Ttime_window: time window is configured by NW.

  - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

Proposal 8: Do not consider Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management.

Proposal 9: RAN4 to consider skipping the fine timing sync (additional SSB reception) for known TCI state if RAN1 has further agreement on the reduction of additional SSB reception for TCI state of UE-initiated/event-driven beam. Otherwise, reuse the existing unified TCI state delay requirement.

Proposal 10: No requirement is applied if indicated TCI states is changed during the L1 measurement time.

Proposal 11: Define UE reported capabilities for numbers of event refer to legacy L3 (like legacy L3 in section 9.1.4).

- For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

# Reference

[1] RP-233090 New WID - Rel-19 MIMO

[2] R4-2504902, WF on RRM requirements for NR_MIMO_Ph5_Part1(Samsung)

[3] R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management