# 3GPP TSG-RAN WG4 Meeting #115	R4-2507831

# Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 		Follow-up on SAN RF way forward agreements

Agenda Item:	7.9.5

Document for:	Discussion

# Introduction

On top of the WF from last RAN4#114bis meeting, we provide brief feedback to SAN RF requirements, as input to further discussion, and justification to the Draft CR on the output power dynamics and Tx signal quality in a companion contribution.

# Discussion

Issue 4-1-1: Satellite Access Node output power

- WF: No specification impact.

- SAN output power requirement does apply to NTN SAN (TS 38.108), as well as to IoT NTN SAN (TS 36.108). Therefore, it is suggested to consider this requirement. As WID mentions high power SAN, it would be beneficial for the discussion to know more details of the planned deployments.

Issue 4-1-2: RE power control dynamic range

- WF: No specification impact.

- As RE power control dynamic range is not applicable to PMCH, it does not have to be defined for 5G_Broadcast_GSO_NTN_band.

Issue 4-1-3: Total power dynamic range

- WF: FFS on whether total power dynamic range requirement applies for LTE based 5G broadcast over Satellite

- Total power dynamic range requirement does apply to NTN SAN (TS 38.108), as well as to IoT NTN SAN (TS 36.108). Even if TPDR does not apply to terrestrial broadcast, it is seen valid to reassure that the SAN performance is not worse than existing NTN SAN spec. Therefore, it is suggested to consider TPDR requirement for 10MHz channel (15kHz SCS).

Issue 4-1-4: Transmit ON/OFF power

- WF: No specification impact.

- This requirement is not applicable to SAN.

Issue 4-1-5: Transmitted signal quality

- WF: No specification impact.

- While TAE is not applicable, frequency error shall be considered. EVM is captured below.

Issue 4-1-6: Occupied bandwidth

- WF: No specification impact.

- This requirement shall be applicable.

Issue 4-1-7: Adjacent Channel Leakage Power Ratio

- WF: Reuse the legacy requirements for GEO

- Agree, as it shall not be worse than existing SAN spec.

Issue 4-1-8: EVM

  - WF: To specify the EVM requirements for LTE based 5G broadcast over satellite based on the following table.

Table 6.5.2-1: EVM requirements for E-UTRA carrier

| Modulation scheme | Required EVM (%) |
| --- | --- |
| QPSK | 17.5 |
| 16QAM | 12.5 |

- Agree, as it shall not be worse than existing SAN spec.

Issue 4-1-9: Out-of-band emissions

- WF: Reuse the legacy requirements.

- Reuse SAN specification.

Issue 4-1-10: Transmitter spurious emissions

- WF: Reuse the legacy requirements.

- Follow ITU.

Issue 4-1-11: Transmitter intermodulation

- WF: No specification impact, not applicable to SAN.

- Not applicable to SAN, but there may be some impact on spec due to suffix structure.

Issue 4-1-12: Window length

- WF: To reuse EVM window length for 10MHz defined in 36.104.

- Good starting point; may be discussed during Perf part.

### Sub-topic 4-2: SAN RF conformance requirements

  - WF: As per WID: RAN4 to further discuss whether SAN RF conformance requirements and tests will be defined.

  - To be discussed after Core completion.