3GPP TSG-RAN WG4 Meeting #115                                          R4-2507873

# Malta, 19th – 23rd of May 2025

Title: 	On VSAT UE RF requirements for Ku-band

Agenda Item:	7.11.5 – UE RF requirements

Release:	Rel-19

Source: 	THALES

Document for:	Discussion

# Information

The intention of this document is to provide some information and some proposals for the computation of essential requirements for VSAT UE RF in Ku-band (under NR_NTN_Ku_bands WID).

# Discussion

According to the offline discussions we would like to make the following proposals:

Proposal 1: EIRPmax, off-axis EIRP mask and ACLR requirements resulted from coexistence studies should be considered as independent requirements in TS 38.101-5 (one specific requirement will not be used to drive the other(s)).

Proposal 2: FCC and CEPT requirement shall be considered independently. Different NS values can be defined for different regions and TS 38.101-5 shall not capture only one single (more stringent) requirement.

Proposal 3: RAN4 to further discuss if EIRPmax value at lower elevation (e.g. 35°) with respect to FR1-NTN/FR2-NTN numerologies and the respective CBW being used.

Proposal 4: If companies propose to decrease EIRPmax for higher antenna size (e.g. 78 x 78 antenna element) because off-axis EIRP mask is not met at certain elevation angles, it should be clarified if FR1-NTN and/or FR2-NTN configuration.

Proposal 5: VSAT EIRPmin should be based on the following equation (or similar):

EIRPmin=-174dBm/Hz+10log10BW+NF+SNR+IM-SANAntennaGain+PathlossFreeSpace+AtmosphereLoss

Where:

- NF is the GEO/LEO Noise Figure in Ku-band (assumption with same noise figure 3dB for all orbits)

- BandWidth

  - can be assumed as 100 MHz, since 100 MHz is defined for both FR1-NTN and FR2-NTN

  - could be differentiated between FR1-NTN (e.g. 50 MHz) and FR2-NTN (e.g. 200 MHz)

- SNR is assumed as -1dB which is applicable for QPSK with 1/3 coding rate;

- IM is the implementation margin for low modulation order, 2dB could be considered;

- SANAntennaGain is for instance:

  - 44-[3/0]dB; where 44dBi is the GEO Rx antenna gain in Ku-band

  - 38.5-[3/0]dB; where 38.5dBi is the LEO Rx antenna gain in Ku-band

    - 3dB margin is considered for VSAT is located at the beamprint edge which is corresponding to 3dB beamwidth of radiation beam.

    - If VSAT is located at the center of beamprint, 0dB margin could be considered;

- An AtmosphereLoss of 2dB could be considered

- PathlossFreeSpace=32.44+20*log10(D2)+20*log10(fMHz);

- fMHz is assumed as 14.5 GHz for UL direction (worst case for Ku-band);

- D2 is slant distance between VSAT and SAN and could be derived as following:

D2=(B*sinθ)2+r2-B2-B*sinθ

Where:

- h is assumed as

  - 35786km for GEO,

  - 1200km for LEO@1200km

  - 600km for LEO@600km

- B is assumed as 6378km for Earth radius

- r is assumed as h+B;

- θ is the elevation angle

Proposal 6: Companies to further discuss the elevation angle θ to be used to derive the requirement. θ could be e.g. 35° elevation angle, 75° or higher, but 90° elevation angle should be avoided.