3GPP TSG-RAN WG4 Meeting #115                                                                                         R4-2507930

Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.1.1

Source:	Moderator (Huawei, HiSilicon)

Title:	WF on NR_ENDC_RF_Ph4_part1

Document for:	Approval

- Topic #1: Power domain enhancements for single carrier

- Sub-topic 1-1: Approaches to enable MPR reduction

Issue 1-1-1: Extension values/ratios

- Agreement in main session:

- At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases

- It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides.

- It is FFS on details of the single side case.

Issue 1-1-2: Guard band for extended CBW

- WF

- GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB

- Extended CBW is not limited to the existing UE CBW

- RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests

Issue 1-1-3: Derivation of new inner RB region in extended CBW

- WF

- The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well

  - For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

    - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

    - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

    - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

    - RBshift_low, RBshift_high ≤ NRB/2

- Other approach is not precluded

- The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios

- Sub-topic 1-2: Applicable requirements

Issue 1-2-1: Derivation of the extended IBE

- Agreement in main session:

- Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed)

  - The details and the related test cases can be discussed and decided later.

  - If significant issue is identified, other options can be further considered.

- WF

Further discuss whether to apply the extended IBE in the GB of extended CBW.

- Option1: Yes

- Option2: No

Issue 1-2-2: Relaxation of the extended IBE

- Options

  - Option 1: using BPSK EVM in the IBE formula for all extended RBs

  - Option 2: no relaxation is needed.

- WF

  - If there is no obvious system performance impact observed, option 1 is adopted

- Sub-topic 1-3: Signaling

Issue 1-3-1: NW signaling

- Agreement in main session:

- The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities,

  - The details of the signaling including the contents are FFS

- WF

- It is up to the NW to judge the gap width on both sides of the UE

- NW sends info of extension ratio (at least 1/2) per side

Issue 1-3-2: UE capability

- Agreement in main session:

- No separate UE capabilities for scenario 1 and scenario 2

- Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only

  - Both dual-sided and single sided cases are supported.

- Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension

  - Both dual-sided symmetric and single sided cases are supported.

  - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced.

- Note: the other ratio number depends on discussion of issue 1-1-1

- Topic #2: MPR applicability for FR1 intra-band UL CA

- Sub-topic 2-1: Intra-band contiguous UL CA

Issue 2-1-1: Clarification on channel BW on MPR applicability for FR1 intra-band UL CA

- Proposals

  - Proposal: RAN4 to update the specs wording on MPR applicability for FR1 intra-band contiguous UL CA as: “For UE indicating [MPRenhancement-r19] supported and if single CC is activated for intra-band contiguous CA, the allowed MPR is specified in Table 6.2.2-1 based on aggregatd channel bandwidth for PC3 CA bandwidth classes B and C, Table 6.2D.2-1 for PC2 CA bandwidth classes B and C when TxD is indicated, Table 6.2.2-2 for PC2 CA bandwidth classes B and C when TxD is absent.”.

- WF

  - No need to have the clarification.