3GPP TSG-WG4 Meeting #115	R4-2507991

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:			6.3.3

Source:	ZTE, Skyworks

Title:	WF on UL CA_n41C IMD MSD test point

Document for:	Approval

Introduction

The UL CA_n41C MSD test point and IMD range definition for UL CA_n40A-n41C and CA_n41C-n79A are approved in the WF[1].

Before RAN4 started to discuss the UL configurations of UL CA_n40A-n41C and CA_n41C-n79A, RAN4 only define one IMD test point with partial RB configurations for each CC (most of them are 1RB+1RB configurations) for IMD MSD source, no matter for FDD-TDD CA band configuration or TDD-TDD CA band configuration supporting non-simultaneous Rx/Tx operation.

When UL CA_n40A-n41C and CA_n41C-n79A start to be discussed, operator request to study the IMD MSD based on practical MSD test point, i.e. full +full RB.

Except for these two UL configurations of CA_n40A-n41C and CA_n41C-n79A, there are no other CA configurations falls into the similar discussion.

This WF capture the clarification of the source of IMD MSD test point for inter-band UL CA with two contiguous CC full allocated RB in each CC based on the IMD order dominating the interference power

WF on UL CA_n41C IMD MSD test point

- The mandatory 1RB+1RB test point and optional full+full RB test points are captured in the current MSD table

- Include as a note for the optional full+full RB test points in IMD MSD table in the related band combination 	TP to TR and TS38.101-1.

- The IMD order captured in the table is the lowest IMD order of the two UL CCs that dominates the interfering power in the victim channel.

- This source of IMD are the same for the mandatory 1RB+1RB test point and the optional full+full RB test points.

- The optional full+full RB test point for the inter-band UL CA configurations with two contiguous CC in one band relies on the operator request.

Reference

[1] R4-2502955, WF on ULCA_n41C MSD test point and IMD range definition, Skyworks, Qualcomm, ZTE, Huawei

[2] R4-2506392	Clarification on UL CA_n41C IMD MSD test point	ZTE Corporation, Sanechips

[3] R4-2507791	PRD input on IMD range definition for inter-band MSD due to 1UL 2CC UL configuration	Skyworks Solutions Inc.