3GPP TSG-RAN WG4 Meeting #115                                                    	R4-2508009

Malta, MT, May 19 – 23, 2025

Source:	ZTE Corporation, Sanechips

Title:	TP for TR 37.719-11-11 to introduce intra-band non-contiguous DC_7A_n7A_BCS1

Agenda item:			6.2.2 (DC_R19_1BLTE_1BNR_2DL2UL)

Document for:	Approval

# 1. Introduction

The intra-band non-contiguous DC configuration DC_7A_n7A_BCS1 was requested in Rel-19 basket WID [1]. Note that DC_7A_n7A_BCS0 has already been captured in TR 37.716-11-11 [2] and specified in TS 38.101-3. This TP is to introduce DC_7A_n7A_BCS1 to TR 37.719-11-11.

Proposal 1.  It is suggested to approve the following TP for DC_7A_n7A_BCS1 to TR 37.719-11-11.

2. Reference

[1] RP-250143, Revised WID: Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia, CHTTL, LG Electronics, Samsung.

[2] TR 37.716-11-11, Rel-16 E-UTRA (Evolved Universal Terrestrial Radio Access) - NR Dual Connectivity (EN-DC) of 1 band LTE (1 Down Link (DL) / 1 Up Link (UL)) and 1 NR band (1 DL / 1 UL)(1DL/1UL).

[3] TR 37.719-11-11, Rel-19 Dual Connectivity of EN-DC and NE-DC configurations consisting of 2 different bands downlink (DL) with 2 different bands uplink (UL) (1 LTE band and 1 NR band).

# 3. Text Proposal

<Start of Text Proposal>

6.4.x	DC_7_n7

#### 6.4.x.1	Channel bandwidths per operating band for DC

Table 6.4.x.1-1: Supported channel bandwidths per DC configuration

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_7A_n7A3 | DC_7A_n7A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 1 |
|  |  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |
| NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. |  |  |  |  |  |  |

NOTE: 	BCS0 is introduced from Rel.16.

#### 6.4.x.2	Configuration for DC

Table 6.4.x.2-1:  Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_7A_n7A6 | DC_7A_n7A5 | Yes5 |
| NOTE 5: Only single switched UL is supported. NOTE 6: Requirements in this specification apply for NR SCS of 15 kHz only. |  |  |

#### 6.4.x.3	Maximum output power for DC

Table 6.4.x.3-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n7A4 | 23 | +2/-3 |
| NOTE 4: Only single switched UL is supported. |  |  |

#### 6.4.x.4	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, no spurious emission band UE co-existence requirement is needed.

6.4.x.5	Analysis of MSD requirements

Note that only Single Tx Switched UL mode is supported for this combination, no MSD requirement is needed.

6.4.x.6	∆RIBNC values

The ∆RIBNC values for intra-band non-contiguous EN-DC DC_7_n7 are same as for CA_7A-7A in TS 36.101 and are shown in Table 6.4.x.6-1 for Table 7.3B.3.2-1 in TS 38.101-3 with one uplink on E-UTRA which is closer to the NR downlink carrier than to the E-UTRA downlink carrier.

Note that the requirements for one uplink on NR and closer to E-UTRA downlink carrier than NR downlink carrier have already been introduced for DC_7_n7_BCS0 in Table 7.3B.3.2-2 from Rel-16. The requirements in Table 6.4.x.6-1 are exactly the same as in Table 7.3B.3.2-2 in TS 38.101-3.

Table 6.4.x.6-1: Intra-band non-contiguous EN-DC with one uplink configuration on E-UTRA for reference sensitivity (E-UTRA uplink carrier is closer to the NR downlink carrier than it is to the E-UTRA downlink carrier)

| DC configuration | Aggregated channel bandwidth |  | Wgap / (MHz) |  |  | UL E-UTRA allocation (LCRB) |  |  | ΔRIBNC (dB) |  |  | Duplex mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA | NR |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A_n7A | 5MHz | 5MHz |  | 0< Wgap ≤ 60 | 25 |  |  | 0.0 |  |  | FDD |  |  |
|  | 5MHz | 10MHz |  | 0 < Wgap ≤ 55 | 25 |  |  | 0.0 |  |  |  |  |  |
|  | 5MHz | 15MHz |  | 0 < Wgap ≤ 50 | 25 |  |  | 0.0 |  |  |  |  |  |
|  | 5MHz | 20MHz |  | 0 < Wgap ≤ 45 | 25 |  |  | 0.0 |  |  |  |  |  |
|  | 10MHz | 5MHz |  | 30 < Wgap ≤ 55 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0 < Wgap ≤ 30 | 50 |  |  | 0.0 |  |  |  |  |  |
|  | 10MHz | 10MHz |  | 25.0 < Wgap ≤ 50.0 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0.0 < Wgap ≤ 25.0 | 50 |  |  | 0.0 |  |  |  |  |  |
|  | 10MHz | 15MHz |  | 20 < Wgap ≤ 45 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0 < Wgap ≤ 20 | 50 |  |  | 0.0 |  |  |  |  |  |
|  | 10MHz | 20MHz |  | 15 < Wgap ≤ 40 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0 < Wgap ≤ 15 | 50 |  |  | 0.0 |  |  |  |  |  |
|  | 15MHz | 5MHz |  | 20.0 < Wgap ≤ 50.0 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0.0 < Wgap ≤ 20.0 | 501 |  |  | 0.0 |  |  |  |  |  |
|  | 15MHz | 10MHz |  | 20.0 < Wgap ≤ 45.0 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0.0 < Wgap ≤ 20.0 | 501 |  |  | 0.0 |  |  |  |  |  |
|  | 15MHz | 15MHz |  | 15.0 < Wgap ≤ 40.0 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0.0 < Wgap ≤ 15.0 | 501 |  |  | 0.0 |  |  |  |  |  |
|  | 15MHz | 20MHz |  | 10 < Wgap ≤ 35 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0 < Wgap ≤ 10 | 501 |  |  | 0.0 |  |  |  |  |  |
|  | 20MHz | 5MHz |  | 25 < Wgap ≤ 45 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0 < Wgap ≤ 25 | 451 |  |  | 0.0 |  |  |  |  |  |
|  | 20MHz | 10MHz |  | 20 < Wgap ≤ 40 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0 < Wgap ≤ 20 | 451 |  |  | 0.0 |  |  |  |  |  |
|  | 20MHz | 15MHz |  | 15.0 < Wgap ≤ 35.0 | 361 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0.0 < Wgap ≤ 15.0 | 501 |  |  | 0.0 |  |  |  |  |  |
|  | 20MHz | 20MHz |  | 15.0 < Wgap ≤ 30.0 | 321 |  |  | 0.0 |  |  |  |  |  |
|  |  |  |  | 0.0 < Wgap ≤ 15.0 | 451 |  |  | 0.0 |  |  |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. |  |  |  |  |  |  |

#### 6.4.x.7	MPR, A-MPR requirements

Note that only Single Tx Switched UL mode is supported for this combination, no MPR or A-MPR requirement is needed.

<End of Text Proposal>