3GPP TSG-RAN4 Meeting #115 	R4-2508010

# St. Julian's, Malta,  19 – 23 May 2025									   (Revision of R4-2507711)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0028 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.863: Introduction of NR-NTN band n252 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Boost Mobile Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Sband-Core |  |  |  |  |  | Date: |  |  | 2025-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Capturing RAN4 agreements for introduction/definition of NTN band n252 in TR 38.863. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Regulatory requirement for n252 - Channel bandwidth, asymmetric channel bandwidth, channel raster, sync raster - RF requirements including MOP, Additional spurious emission for UE-to-UE coexistence, NS values and A-MRP, Tx-Rx frequency separation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN4 agreements and necessary changes for n252 definition are not captured in TR 38.863 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 7.2.2, 7.2.3, 7.2.4, 7.4.2, 7.4.3, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r0: initial version r1: including company C AMPR simulation result |  |  |  |  |  |  |  |  |

--- Start of Changes ---

<not affected part ommited>

# 5	Regulatory aspects

<not affected part ommited>

## 5.2	Regulatory aspects for NTN satellites

<not affected part ommited>

### 5.2.4	NTN Satellite band: UL 2000–2020 MHz / DL 2180–2200 MHz

Table 5.2.4-1 shows the service allocation for the frequency range in the ITU-R Radio Regulations [3] and Table 5.2.4-2 shows the FCC frequency allocation table for the frequency range [20].  From Table 5.2.4-1 and Table 5.2.4-2, there is some deviation in how MSS S-band is allocated in region 2 versus in all other regions. Per ITU, the Region 2 MSS S-Band had been allocated to UL 1980-2025 MHZ and DL 2160-2200 MHz. In the case of North America region (US, Canada, Mexico) the MSS band is allocated to UL 2000 MHz -2020 MHz and DL 2180 MHz -2200 MHz which is not exactly aligned to n256 definition, i.e., UL 1980 MHz – 2010 MHz, DL 2170 MHz – 2200 MHz. The 3GPP NTN  S-band n252 (UL 2000 MHz -2020 MHz and DL 2180 MHz -2200) is defined for consistency across North America and Region 2 as a whole.

Table 5.2.4-1: Allocation of 2000-2020 MHz and 2180–2200 MHz in the ITU-R Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  |
| 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 | 2 010-2 025 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 |
| 2 170-2 200 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A 5.388 5.389A 5.389F |  |  |
| 5.388: The frequency bands 1 885-2 025 MHz and 2 110-2 200 MHz are intended for use, on a worldwide basis, by administrations wishing to implement International Mobile Telecommunications (IMT). Such use does not preclude the use of these frequency bands by other services to which they are allocated. The frequency bands should be made available for IMT in accordance with Resolution 212 (Rev.WRC-15)* (see also Resolution 223 (Rev.WRC-15)*). 5.389A: The use of the bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service is subject to coordination under No. 9.11A and to the provisions of Resolution 716 (Rev.WRC-2000)** 5.389B: The use of the frequency band 1 980-1 990 MHz by the mobile-satellite service shall not cause harmful interference to or constrain the development of the fixed and mobile services in Argentina, Brazil, Canada, Chile, Ecuador, the United States, Honduras, Jamaica, Mexico, Paraguay, Peru, Suriname, Trinidad and Tobago, Uruguay and Venezuela. 5.389F: In Algeria, Cape Verde, Egypt, Iran (Islamic Republic of), Mali, Syrian Arab Republic and Tunisia, the use of the frequency bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service shall neither cause harmful interference to the fixed and mobile services, nor hamper the development of those services prior to 1 January 2005, nor shall the former service request protection from the latter services. |  |  |

Table 5.2.4-2: FCC frequency allocation for 2000-2020 MHz and 2180–2200 MHz

The following are the applicable FCC regulatory requirements:

#### 47 CFR § 25.202

According to paragraph (a)(4)(ii), the following frequencies are available for use by the 2 GHz Mobile-Satellite Service:

- 2000-2020 MHz: User-to-Satellite Link;

- 2180-2200 MHz: Satellite-to-User Link.

According to paragraph (f) on emission limitations, except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section:

- In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB;

(2)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB;

(3)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts;

(4)  In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

#### 47 CFR § 25.216

According to paragraph (e) on limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service, the e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

It should be noted that for the TN Band 23, which is studied and documented in [xx], the FCC regulation part 25.252 on ATC (Ancillary Terrestrial Component) for satellite service was referred to derive the RF requirements. However, since n252 is defined as an NTN band, ATC related requirements are not applicable for this band.

The following figure 5.2.4-1 shows an overview of the NR TN bands adjacent to the NTN n252 band.

Figure 5.2.4-1: Adjacent TN bands to NTN band n252 in North America

As shown in figure 5.2.4-1, the n252 is adjacent to NR bands n25 DL and n2 DL, is overlap with n70 DL and n66 DL. Section 7.4.2.2.10.2 describes the RAN4 agreement on UE-to-UE coexistence requirement to protect the n25 DL, n2 DL, and n66 DL.  The same section also describes the agreement for n70 DL.

## 5.3	Regulatory aspects for HAPS

ITU–R began to study HAPS in the 1990s initially for fixed services. The telecommunications ecosystem and technology enablers for HAPS have evolved significantly since then. At WRC-2000, the bands 1885 – 1980 MHz, 2010 – 2025 MHz and 2110 – 2170 MHz were identified for HAPS operating as IMT Base Stations, in accordance with Resolution 221 (Rev.WRC-07) [3]. With increasing interest in HAPS to offer mobile services, the WRC-19 agreed to study certain frequency bands which have been already identified for IMT below 2.7 GHz for HAPS as IMT Base Stations (HIBS). ITU-R Working Party 5D is currently studying co-existence requirements for HIBS in the additional three bands listed in Table 5.4-1 to support spectrum identification decisions in WRC-23 in accordance with Resolution 247 (WRC-19) [3].

NOTE:	In the current ITU terminology, the use of HAPS to implement IMT (i.e. offer mobile wireless services) is referred to as “HAPS as IMT Base Stations” or HIBS.

Table 5.4-1: Frequencies for HAPS as IMT Base Stations (HIBS).

| Region | Spectrum | Remarks |
| --- | --- | --- |
| Region 1 and 3 | 1885 – 1980 MHz 2010 – 2025 MHz 2110 – 2170 MHz | Identified HIBS designations at WRC-2000 |
| Region 2 | 1885 – 1980 MHz3 2110 – 2160 MHz4 |  |
| Global | 694 – 960 MHz 1710 – 1885 MHz1 2500 – 2690 MHz2 | Under study, for decision at WRC-23 |
| NOTE 1: 1710-1815 MHz to be used for uplink only in Region 3. NOTE 2: 2500-2535 MHz to be used for uplink only in Region 3, except 2655--2690 MHz in Region 3). NOTE 3: In most of Region 2 1885-1910 is used for Mobile uplink and 1930-1980 is used for Mobile downlink. In the United States, 1910-1915 and 1915-1920 are also used for Mobile uplink. NOTE 4: In Region 2 2110-2160 is used for Mobile downlink. |  |  |

<not affected part ommited>

--- Next Change ---

# 7	RF requirements

<not affected part ommited>

## 7.2	Common issues for satellite access node and NTN UE

### 7.2.1	General

This section aims to present the agreed definitions for the newly introduced NTN operating bands. The possible configurations in terms of channel bandwidth, SCS, channel and synchronization raster for NTN operations are introduced as well.

### 7.2.2	Operating bands

The following bands in Table 7.2.2-1 are agreed NTN satellite bands. Regarding the band numbering for NTN satellite bands, it is agreed to start from the largest band number in FR1 range for NTN satellite bands which fully within FR1 frequency ranges to better differentiate NTN satellite band and TN bands and ensure contiguous band number allocation for NTN satellite bands, the number can be taken in a decreased order with first come, first service.

Table 7.2.2-1: NTN satellite bands in FR1

| NTN satellite band # | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n252 | 2000 MHz – 2020 MHz | 2180 MHz – 2200 MHz | FDD |
| NOTE: NTN bands are numbered in descending order from n256. |  |  |  |

### 7.2.3	Channel bandwidth, SCS and spectral utilization

The common definition for channel bandwidth, transmission bandwidth configuration, minimum guard band, and RB alignment in TS 38.104[16] and TS 38.101-1[17] can be reused for NTN satellite system.

The supported channel bandwidths per operating band should be defined based on NTN operator input. The details are given in Table 7.2.3-1.

Table 7.2.3-1: Channel bandwidths and SCS per NTN satellite operating band in FR1

| NTN satellite band # | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 30 MHz |
| --- | --- | --- | --- | --- | --- | --- |
|  | 15 | Yes | Yes | Yes | Yes |  |
| n256 | 30 |  | Yes | Yes | Yes |  |
|  | 60 |  | Yes | Yes | Yes |  |
|  | 15 | Yes | Yes | Yes | Yes |  |
| n255 | 30 |  | Yes | Yes | Yes |  |
|  | 60 |  | Yes | Yes | Yes |  |
| n252 | 15 | Yes | Yes | Yes | Yes |  |
|  | 30 |  | Yes | Yes | Yes |  |
|  | 60 |  | Yes | Yes | Yes |  |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |

### 7.2.3.1		Asymmetric channel bandwidth

The UE channel bandwidth can be asymmetric in downlink and uplink. In asymmetric channel bandwidth operation, the narrower carrier shall be confined within the frequency range of the wider channel bandwidth.

In FDD, the confinement is defined as a maximum deviation to the Tx-Rx carrier center frequency separation (defined in table 7.4.1.1-1) as following:

ΔFTX-RX = | (BWDL – BWUL)/2 |

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 7.2.3-1.

Table 7.2.3-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 |  |
|  | 15 | 20 |  |

### 7.2.4	Channel raster and sync raster

#### 7.2.4.1	Channel raster

In order to be compatible with current design of sync raster below 3 GHz, the ARFCN parameters below 3000 MHz will be reused and 100 kHz was chosen as the channel raster for NR NTN satellite bands below 3 GHz.

The applicable ARFCNs per operating and for satellite access node are defined in table 7.2.4.1-1.

Table 7.2.4.1-1: ARFCNs per operating band satellite access node

| NTN satellite band # | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n252 | 100 | 400000 – <20> – 404000 | 436000 – <20> – 440000 |

In addition to the 100 kHz channel raster, Enhanced channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 7.2.4.1-2 is given as <2>.

Table 7.2.4.1-2: ARFCNs per operating band satellite access node

| NTN satellite band # | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 |
| n252 | 10 | 400000 – <2> – 404000 | 436000 – <2> – 440000 |

#### 7.2.4.2	Sync raster

The applicable SS raster entries per operating band are defined in Table 7.2.4.2-1

Table 7.2.4.2-1: Applicable SS raster entries per operating band

| NTN satellite band # | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n252 | 15 kHz | Case A | 5456 – <1> – 5494 |

<not affected part ommited>

--- Next Change ---

## 7.4	NTN UE requirements

### 7.4.1	General

In general, NTN UE products are assumed to meet TN UE requirements as specified in TS 38.101-1[17]. The structure and content of the NTN UE specification TS 38.101-5 should be based on TS 38.101-1[17]. The development of TS 38.101-5 should follow the set of conditions that were agreed at the RAN#94e meeting in RP-213618[27].

#### 7.4.1.1	Tx-RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 7.4.1.1-1 for FR1-NTN.

Table 7.4.1.1-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n252 | 180 MHz1 165 – 195 MHz2 |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. |  |

--- Next Change ---

### 7.4.2	UE Transmission characteristics for satellite access

#### 7.4.2.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0dBi is assumed. Handheld PC3 UE is assumed in Release 17 for satellite access.

#### 7.4.2.2	Conducted transmitter characteristics

##### 7.4.2.2.1	Maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 7.4.2.2.1-1: UE Power Class

| NTN satellite band # | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n256 | 23 | ±2 |
| n255 | 23 | +2 |
| n252 | 23 | +2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

##### 7.4.2.2.2	MPR/AMPR

7.4.2.2.2.1	MPR

UE is allowed to reduce the maximum output power (MPR) due to higher order modulations and transmit bandwidth configurations to meet the general transmission requirements which includes in-band emission, ACLR, out-of-band emission and general spurious emission. As it was agreed those general transmission requirements can be reused from TN, the same requirements specified in Table 6.2.2-1 in TS 38.101-1 [17] can be reused for NTN satellite access UEs.

7.4.2.2.2.2	A-MPR

A-MPR is band specific requirement which is used to meet the additional requirements indicated by network signalling (NS) value.

For n256, based on the regulation conclusion in section 5.2, the A-MPR requirements for n65 could be as starting point. From TS 38.101-1, since the maximum carrier centre frequency of NS_05 and NS_05U is lower than 1960 MHz and the maximum carrier centre frequency of NS_51 is lower than 1980 MHz, it is proposed NS_05, NS_05U and NS_51 don’t need to be considered for n256. The requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 [17] for band n65 can be reused for n256

For n255, based on regulatory requirements, a new NS is required. The following observations were made from the measurement results using an LTE modem and transceiver supporting 1626.5 – 1656.5 MHz:

-	For 5 MHz channel, PA emissions for DFT-s-OFDM are below the spurious emission limits between 1559 and 1606 MHz.

-	For 10 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits for 1 MHz MBW by 5 dB between 1604 and 1606 MHz.

-	For 15 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits for 1 MHz MBW by 9 dB between 1599 and 1606 MHz.

-	For 20 MHz channel, PA emissions for DFT-s-OFDM are exceeding the spurious emission limits by 9 dB between 1584 and 1606 MHz.

-	For 700 Hz MBW, PA emissions for all channel bandwidths are below the spurious emission limits between 1559 and 1606 MHz.

A n255 duplexer with rejection of more than 10 dB will be a sufficient to satisfy the spurious emission limits specified in Table 7.4.2.2.8-1.

Based on this measurement observations, the expected filter attenuation of more than 10 dB below 1610 MHz is going to be adequate to meet the additional emissions requirements between 1559 – 1610 MHz, and therefore, no A-MPR needs to be specified for n255.

In view of the above, the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value shall be specified in Table 7.4.2.2.2.2-1 and the mapping of NR satellite band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 7.4.2.2.2.2-1A.

For n252, the following three NSs are defined:

-	NS_06N: for protection of aeronautical radionavigation-satellite service. This requirement is based on FCC rule 47 C.F.R § 25.216 (e)

- 	NS_07N and NS_08N: for protection of n2/n25. This requirement applies based on coordination between operators and subject to regional/national regulation. See section 7.4.2.2.8 for detail on UE-to-UE coexistence additional spurious requirements. From UE perspective, UE applies this requirement based on the received NS value. For implementation flexibility, the requirement for aeronautical radionavigation-satellite service is also included in these NSs.

Table 7.4.2.2.2.2-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2.1-1 | 5, 10, 15, 20 | Table 5.3.2-1 in TS 38.101-1 | N/A |
| NS_02N | 6.5.3.3.1 | n255 | 5, 10, 15, 20 |  | N/A |
| NS_24 | 6.5.3.3.13 in TS 38.101-1 | n256 | 5, 10, 15, 20 | Table 6.2.3.15-1 in TS 38.101-1 | Clause 6.2.3.7 in TS 38.101-12 |
| NS_100 | 6.5.2.4.2 in TS 38.101-1 | n2561 |  |  | Table 6.2.3.1-2 in TS 38.101-1 |
| NS_06N | 7.4.2.2.8 | n252 | 5, 10, 15, 20 |  | N/A |
| NS_07N | 7.4.2.2.8 | n252 | 5, 10, 15, 20 |  | Table 7.4.2.2.2.2.1-2 |
| NS_08N | 7.4.2.2.8 | n252 | 5, 10, 15, 20 |  | Table 7.4.2.2.2.2.2-2 |
| NOTE 1: This NS can be signalled for NTN satellite bands that have UTRA services deployed. NOTE 2: A-MPR for the upper 5 MHz is not specified, and therefore shall be used as a guard band. NOTE 3: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |  |

Table 7.4.2.2.2.2-1A: Mapping of network signalling label

| NTN satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n252 | NS_01 | NS_06N | NS_07N | NS_08N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331. |  |  |  |  |  |  |  |  |

For n256, FFS for how to protect n2, n25, n70 and whether A-MPR required.

7.4.2.2.2.2.1	AMPR for NS_07N (UE-to-UE coexistence requirement of -40dBm/MHz)

Table 7.4.2.2.2.2.1-1: A-MPR regions for NS_07N

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | < 2007.5 | ≥ 0 | ≥ 2.16 | A1 |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.24 | ≥ 0 | A3 |
|  |  | > 3.24 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 3.24 | > 7.2 | A2 |
|  |  | > 10.8 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 4.68 | > 7.2 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

Table 7.4.2.2.2.2.1-2: A-MPR for NS_07N

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 7 | ≤ 11 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 7.5 | ≤ 11.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 8 | ≤ 12 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 8 | ≤ 12 |
| CP-OFDM QPSK | ≤ 7 | ≤ 9 | ≤ 13 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 9 | ≤ 13 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 9 | ≤ 13 |

7.4.2.2.2.2.2	AMPR for NS_08N (UE-to-UE coexistence requirement of -30dBm/MHz)

Table 7.4.2.2.2.2.2-1: A-MPR regions for NS_08N

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | - | - | - | - |
| 10MHz | < 2010 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | > 2.7, < 4.5 | A2 |
|  |  | ≤ 1.44 | ≥ 4.5 | A1 |
|  |  | ≥ 1.44 | ≥ 5.4 | A2 |
| 15MHz | < 2012.5 | ≤ 3.24 | ≤ 2.7 | A1 |
|  |  | ≤ 3.24 | > 2.7, < 4.5 | A2 |
|  |  | ≤ 3.24 | ≥ 4.5 | A1 |
|  |  | ≥ 3.24 | ≥ 5.4 | A2 |
|  | 2012.5 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | > 2.7, < 5.4 | A2 |
|  |  | ≤ 1.44 | ≥ 5.4 | A1 |
|  |  | ≥ 1.44 | ≥ 7.2 | A2 |
| 20 MHz | 2010 | ≤ 4.68 | ≤ 3.6 | A1 |
|  |  | ≤ 4.68 | > 3.6, ≤ 5.4 | A2 |
|  |  | ≤ 4.68 | > 5.4 | A1 |
|  |  | > 4.68 | ≥ 7.2 | A2 |

Table 7.4.2.2.2.2.2-2: A-MPR for NS_08N

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 2.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 4 |
| CP-OFDM QPSK | ≤ 7 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 4.5 |

<not affected part ommited>

--- Next Change ---

##### 7.4.2.2.8 	Spurious emissions

The spurious emissions limits specified in TS 38.101-1[17] sub-clause 6.5.3 would also be applicable to NTN satellite access UEs referring to the ERC Recommendation 74-01[28] Annex 3 item 3.1.4 for Mobile Earth Stations (MES) (see Note 1bis) transmitting in the Mobile Satellite Service between 1 GHz and 3 GHz.

Considering FCC rule 47 C.F.R § 25.216 (h) [23], TN UE’s general spurious emission requirements will be inadequate for L-band NTN satellite UE operation in the US. A new NS value and a subclause with Table 7.4.2.2.8-1 needs to be specified for n255 for NTN satellite operations in the US.

Table 7.4.2.2.8-1: Additional requirements for NS_02N

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBW) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f ≤ 1605 | -80 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -80 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -70 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -70 + 24/5 (f-1605) | 1MHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

For n252, to protect n2 and n25, the following two additional spurious emission requirements are defined, each is associated to an NS value, and a corresponding A-MPR (See section 7.4.2.2.2).

-	-40 dBm/MHz (NS_07N)

-	-30 dBm/MHz (NS_08N)

The above requirements apply based on coordination between operators and subject to regional/national regulation. From UE perspective, UE applies this requirement based on the received NS value.

For n252, as described in section 5.2.4, FCC rule 47 C.F.R § 25.216 (e) shall be applied to protect aeronautical radionavigation-satellite service. This requirement is included in all three NSs (NS_06N, NS_07N, NS_08N) for n252 for implementation flexibility purpose.

Table 7.4.2.2.8-2: Additional out-of-band requirements for "NS_06N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |

Table 7.4.2.2.8-3: Additional out-of-band requirements for "NS_07N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz |  |

Table 7.4.2.2.8-4: Additional out-of-band requirements for "NS_08N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -30 | 1MHz |  |

##### 7.4.2.2.9 	Transmit intermodulation

The transmit intermodulation limits specified in TS 38.101-1[17] sub-clause 6.5.4 (Table 7.4.2.2.9-1) would also be applicable to NTN satellite access UEs.

The requirement of transmit intermodulation is specified in Table 7.4.2.2.9-1.

Table 7.4.2.2.9-1: Transmit Intermodulation

| Wanted signal channel bandwidth | BWChannel |  |
| --- | --- | --- |
| Interference signal frequency offset from channel center | BWChannel | 2*BWChannel |
| Interference CW signal level | -40 dBc |  |
| Intermodulation product | < -29 dBc | < -35 dBc |
| Measurement bandwidth | The maximum transmission bandwidth configuration among the different SCS's for the channel BW as defined in Table 6.5.2.4.1-1 from TS 38.101-1[17] |  |
| Measurement offset from channel center | BWChannel and 2*BWChannel | 2*BWChannel and 4*BWChannel |

##### 7.4.2.2.10	UE to UE coexistence issues between NTN and TN

###### 7.4.2.2.10.1	UE-to-UE coexistence for n256

In recent RAN4#102 and RAN4#103 meetings, the spurious emissions requirements (-50dBm/MHz) for protected bands UE co-existence (especially for band n256 UL: 1980~2010MHz DL: 2170~2200MHz) were widely discussed. The adjacent/overlapping TN bands to NTN band n256 are shown in Figure 5.2.1-1.

1)	Coexistence protection for band n34 DL

The NS_24 including associated A-MPR value as specified in TS 38.101-1[17] is reused to solve this issue in Release 17. However, other companies pointed out the n256 UL frequency range can’t be leveraged due to 5MHz guard band at the upper of n256 UL frequency range to protect band n34.If NS_24 is used, n256 will be reduced due to the 5 MHz guard band that has been considered with NS_24, frequency range 2005-2010 MHz can not be used anymore. Besides, when NS_24 is indicated to protect the DL frequency range of band n34, satellite UE has to reduce the out power in band n256. It will lead to UL throughput loss and even UL radio link failure for some UL RB configurations.

2)	Coexistence protection for band n39 DL

-50dBm/MHz co-existence requirement can be met considering 20MHz channel bandwidth for band n256 without A-MPR requirement, even if a band n65 duplexer is reused for band n256.

Based on extrapolating the SEM mask to the ACLR4 region which overlaps n39 for the lowest n256 20MHz channel it is feasible to specify -50dBm/MHz protection level for n39 by n256 even when using n65 duplexer (no filter help assumed). This assumes the maximum n256 bandwidth is 20MHz.

Figure 7.4.2.2.10.1-1: Coexistence analysis based on SEM mask extension for band n256

3)	Coexistence protection for band n2, n25, n70 DL

From RAN4 perspective, UE co-existence requirements can’t be fulfilled (-50dBm/MHz) for the TN bands (i.e., n2, n25 and n70) which are overlapping n256. In Release 17, it is RAN4 understanding that the co-existence issue between n256 and those TN bands shall rely on regional or national regulation.

Other proposals:

Other proposals were submitted to solve the UE to UE coexistence issues between NTN and TN especially for the protected DL frequency range which is overlapping with UL frequency range of band n256 or next to the edge of band n256 UL filter.

For example:

-	Isolation regions could be defined between NTN and TN coverage. And a corresponding signaling could be designed by RAN2.

-	Before transmitting UL band n256 signals, satellite UE should determine (or be informed) whether the IMT services exist for the protected DL frequency range in the vicinity.

-	Other potential mechanisms/solutions are not precluded.

Considering these proposals will have impacts on RAN1/RAN2 specifications, these proposals were not further discussed in Release 17.

###### 7.4.2.2.10.2	UE-to-UE coexistence for n252

The adjacent/overlapping TN bands to NTN band n252 are shown in Figure 5.2.4-1. Band n252 UL has 5MHz frequency separation from TN band n25, and 10 MHz separation from TN band n2. Furthermore, band n252 UL is overlapped with TN band n70 DL, and band 252 DL is overlapped with TN band n66 DL. The following are RAN4 agreements regarding UE-to-UE coexistence between band n252 and the adjacent/overlapping TN bands.

- Coexistence protection for band n2, n25 DL

It is very challenging to meet the traditional -50 dBm/MHz UE-to-UE coexistence spurious emission requirement for n2, n25 without significant amount of power reduction (A-MPR) to n252. RAN4 agrees to define a more relaxed UE-to-UE coexistence spurious emission requirement compared to -50 dBm/MHz, that allows the NTN service to work effectively, across the full n252 bands, whilst still provide protection to n2/n25 DL UE from n252 UL UE.

The following two (2) different additional spurious emission requirements, each with the associated NS value and A-MPR, are agreed.

-	-30 dBm/MHz

-	-40 dBm/MHz

The above requirement applies based on coordination between operators and subject to regional/national regulation

To allow operation flexibility, RAN4 also agreed to define asymmetric channel bandwidth with 5, 10 and 15 MHz UL channel bandwidth for band n252, with BCS0 including all permutation based on UL channel bandwidth. See Table 7.2.3-1 for asymmetric channel bandwidth definition.

- Coexistence protection for band n70 DL

From RAN4 perspective, there is no 3GPP solution to address coexistence between overlapping bands. Coexistence between n252 and band n70 is subject to regional/national regulation.

- Coexistence protection for band n66 DL

UE-to-UE coexistence requirements of -50dBm/MHz between n252 UL and TN band n66 DL can be met without additional power reduction (A-MPR).

<not affected part ommited>

--- Next Change ---

### 7.4.3	UE Receiver characteristics for satellite access

<not affected part ommited>

--- Next Change ---

#### 7.4.3.2	Conducted receiver characteristics

<not affected part ommited>

--- Next Change ---

##### 7.4.3.2.2	Reference sensitivity

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For below 6GHz, the REFSENS level can be calculated by the equation below in TR 38.817-01[32]:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

It is noted that the Rx BW is identical to the transmission bandwidth configuration, which is determined by the spectrum utilization. The RB values in the analysis of this contribution are based on the agreed SU for NR.

SNR in Nagoya meeting was agreed as -1dB for NR.

The REFSENS for E-UTRA assumes that the receiver is equipped with two Rx port as a baseline. If 2Rx is considered, the diversity gain is 3dB and the Implementation Margin (IM) uses 2.5dB. In the following calculation, same assumptions are used for NR NTN satellite bands.

For band n255, 9dB noise figure is assumed which is aligned with band n24.

Both dedicate 30MHz duplexer and n65 duplexer are allowed to be used for band n256. Finally, 9dB noise figure is assumed for band n256 as well.

For n252, RAN4 agreed that as a starting point the UE REFSENS value for n252 is aligned with the values for n256.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 from TS 38.101-1 [17] (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 from TS 38.101-1 [17])  with parameters specified in Table 7.4.3.2.2-1 and Table 7.4.3.2.2-2.

Table 7.4.3.2.2-1: Two antenna port reference sensitivity QPSK REFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 | FDD |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |
| n256 | 30 |  | -97.1 | -95.1 | -94.0 | FDD |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |
| n252 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |
|  | 30 |  | -96.6 | -94.6 | -93.5 | FDD |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |

Table 7.4.3.2.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NTN satellite band # | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n255 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
| n252 | 15 | 25 | 50 | 75 | 100 | FDD |
|  | 30 |  | 24 | 36 | 50 |  |
|  | 60 |  | 10 | 18 | 24 |  |

<not affected part ommited>

--- Next Change ---

##### 7.4.3.2.5	Blocking characteristics

For blocking characteristics, it is agreed, the same requirements as for NR TN UEs are applicable, and therefore reused. For NR satellite band n255, OOBB requirements of n24 are reused. For n256, to keep UE implementation freedom with both options (i.e., reusing n65 duplexer or dedicated 30MHz duplexer) is agreed. FFS for n256 OOBB requirements if the same hardware of n65 is expected to be reused.

For n252, the agreed Out-of-band blocking requirement does not preclude any implementation of UE duplexer architecture.

Table 7.4.3.2.5-1 describes the agreed out-of-band blocking requirement for n252.

Table 7.4.3.2.5-1: Out-of-band blocking for n252

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |

<not affected part ommited>

--- Next Change ---

Annex C:
Summary of NR-NTN co-existence study

All NR-NTN co-existence study results have been captured in the list. Please see attachment Summary of NR-NTN co-existence study.

[Editor note: see attachment Summary of NR-NTN coexistence study_r1]

# C1:		Summary of n252 UE-to-UE co-existence study/simulations

## C.1.1 	UE-to-UE separation distance and UE-to-UE protection limit (R4-2416459)

The origins of the -50 dBm/MHz limit can be tracked back to 2008 [R4-080710]. The analysis back then was largely based on approximately 1m separation between UEs and assumptions on human body and antenna losses. For terrestrial networks, this scenario can realize e.g. in indoor environments or in a bus where multiple people are using data intensive applications simultaneously. Given the penetration loss of walls, it is also possible that indoors UEs both would need to use high transmit power to reach the gNB outdoors, as well as operate near sensitivity.

For NTN, outdoor environment is the typical condition. One could also expect outdoor rural conditions to be more appropriate, as one would expect urban environments to have TN coverage always available. As rural environment is very different from indoor environment, we did evaluations on how different UE-to-UE distances and what would need to be the emission level at UE antenna to provide same protection as -50 dBm/MHz requirement does for 1m separation. We looked at different distances and two different propagation (path loss) models, i.e. free space and urban micro. Urban micro was included as TR 38.863 has documented it to be used for UE-to-UE interference in Ka-band studies.

The summary of the results is provided in Table C.1.1.1-1.

Table C.1.1.1-1: impact of UE-to-UE separation distance to required UE-to-UE protection limit

2GHz center frequency was used in the path loss calculation. It can be seen that with the assumed parameters 1m separation distance sets the emission power to -49.7 dBm/MHz. At 10 m distance there is 20 dB more power allowed assuming free space propagation. Then every doubling of the distance allows 6 dB more emissions. For example, if 20 m would be used in deriving the coex-limit, -24 dBm/MHz could be used as the limit.

## C.1.2:		AMPR study for n252

### C.1.2.1 	Power back-off simulation from company A (R4-2503176)

C.1.2.1.1	 	Assumption for power-back off simulation

The following are common assumptions for the power back-off simulation results for the NTN band n252:

-	Tx power: PC3 (+23dBm)

-	LO placement: always in the centre of the carrier

-	Additional requirements: -40dBm/MHz and -30dBm/MHz protection criterion for band n25

C.1.2.1.2			Power back-off simulations for -40dBm/MHz

Figure C.1.2.1.2-1 and Figure C.1.2.1.2-2 below present simulation results for the 5MHz channel, which is placed in the following locations within the band: left edge of the band (Fc=2002.5MHz) and when the center frequency shifted further away from the left edge (Fc=2007.5MHz).  For the sake of brevity, we present only DFT-s-OFDM and CP-OFDM results (DFT-s-OFDM PI/2 BPSK results are usually close to DFT-s-OFDM QPSK and sometimes even outperform the latter). So, referring to Figure C.1.2.1.2-1 and Figure C.1.2.1.2-2 the following general observations could be made for the 5MHz channel:

-	If the 5MHz channel is right at the left edge of the band, then large UL allocations may need large power back-off of up to 5dB even for DFT-s-OFDM. On the other hand, if we consider small UL allocations, which are likely to be the case for the NTN channels, then no power back-off is needed. In fact, even medium size UL allocations of up to 10RBs are not likely to have any power back-off.

-	If the 5MHz channel is shifted farther away from the band lower edge, then required power back-off reduces dramatically. Only large UL allocations may need power back-off.

Figure C.1.2.1.2-1: Power back-off values for the 5MHz channel (Fc=2002.5MHz)

Figure C.1.2.1.2-2: Power back-off values for the 5MHz channel (Fc=2007.5MHz)

Figure C.1.2.1.2-3 and Figure C.1.2.1.2-4 present power back-off values for the 10MHz channel considering the following locations of the channel: right at the left edge of the band (Fc=2005) and shifted to the right so that the offset from the band edge is 5MHz (Fc=2010). The following general observations can be made:

-	For the first case, i.e. when the 10MHz channel resides at the left edge of the band, large power back-off is needed even for small UL allocations.

-	If the 10MHz channel is offset by at least the 5MHz gap from the left edge, then small UL allocations will require no power back-off backoff.

Figure C.1.2.1.2-3: Power back-off values for the 10MHz channel (Fc=2005MHz)

Figure C.1.2.1.2-4: Power back-off values for the 10MHz channel (Fc=2010MHz)

Next, Figure C.1.2.1.2-5 and C.1.2.1.2-6 present power back-off values for the 15MHz channel at the following locations: right at the left edge of the band (Fc=2007.5) and shifted to the right so that the offset from the band edge is 5MHz (Fc=2012.5). As can be seen from the figures, irrespective of the15MHz channel location, edge allocations, even small ones, will require large power back-off.

Figure C.1.2.1.2-5: Power back-off values for the 15MHz channel (Fc=2007.5MHz)

Figure C.1.2.1.2-6: Power back-off values for the 15MHz channel (Fc=2012.5MHz)

Figure C.1.2.1.2-7 presents power back-off results for the 20MHz channel, which as can be seen has even large back-off values comparing to the 15MHz channel bandwidth.

Figure C.1.2.1.2-7: Power back-off values for the 20MHz channel (Fc=2010MHz)

C.1.2.1.3			Power back-off simulations for -30dBm/MHz

In this section we present UE power back-off values for the same channel sizes and center frequencies as in the previous sub-section, but a more relaxed -30dBm/MHz protection criterion is used.

Figure C.1.2.1.3-1 below presents power back-off values for the 5MHz channel, which is right at the left edge of the band. As can be seen, only large allocations might need power back-off, which should be even within the existing MPR margins.

Figure C.1.2.1.3-1: Power back-off values for the 5MHz channel (Fc=2002.5MHz)

Figure C.1.2.1.3-2 and Figure C.1.2.1.3-3 present power back-off values for the 10MHz channel. General observations are the same as for the -40dBm/MHz protection limit, with the only noticeable difference that for the edge 10MHz channel bandwidth only the leftmost edge allocations will need high power back-off values.

Figure C.1.2.1.3-2: Power back-off values for the 10MHz channel (Fc=2005MHz)

Figure C.1.2.1.3-3: Power back-off values for the 10MHz channel (Fc=2010MHz)

Figure C.1.2.1.3-4 and Figure C.1.2.1.3-5 present power back-off values for the 15MHz channel. Like in the previous simulation case of the -40dBm/MHz protection limit, the edge 15MHz channel will require large back-off values for a wide range of UL allocations. Once shifted to the right, large power back-off values are mostly constrained to the leftmost edge RBs.

Figure C.1.2.1.3-4: Power back-off values for the 15MHz channel (Fc=2007.5MHz)

Figure C.1.2.1.3-5: Power back-off values for the 15MHz channel (Fc=2012.5MHz)

Figure C.1.2.1.3-6 presents power back-off results for the 20MHz channel, which follows the trend of large channels resulting in higher power back-off values for large allocations and allocations residing at the left edge of the channel.

Figure C.1.2.1.3-6: Power back-off values for the 20MHz channel (Fc=2010MHz)

### C.1.2.2 	Power back-off simulation from company B (R4-2504544)

The need for A-MPR was studied using the following assumptions:

- PA calibrated to 30 dB ACLR with 100 RB DFT-s-OFDM QPSK waveform with 22 dBm at antenna connector

- -28 dBc IQ-image and DC-leakage

- -60 dBc CIM3

The proposed A-MPR is summarized in Tables C.1.2.2-1 to C.1.2.2-4.

Table C.1.2.2-1: A-MPR regions for -40 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | < 2007.5 | ≥ 0 | ≥ 2.7 | A1 |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≥ 0 | A3 |
|  |  | > 3.06 | 4.5 ≤ LCRB*12*SCS ≤ 9 | A1 |
|  |  | > 3.06 | > 9 | A2 |
|  |  | > 10.98 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | 4.5 ≤ LCRB*12*SCS ≤ 9 | A1 |
|  |  | > 4.68 | > 9 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

Table C.1.2.2-2: A-MPR for -40 dBm/MHz

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 6 | ≤ 7 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 6.5 | ≤ 7.5 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 7.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 7 | ≤ 8 |
| CP-OFDM QPSK | ≤ 6 | ≤ 8.5 | ≤ 9 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 8.5 | ≤ 9 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 8.5 | ≤ 9 |

Table C.1.2.2-3: A-MPR regions for -30 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5 MHz | - | - | - | - |
| 10MHz | < 2010 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | 2.7 < LCRB*12*SCS < 4.5 | A2 |
|  |  | ≤ 1.44 | ≥ 4.5 | A1 |
|  |  | ≥ 1.44 | ≥ 5.4 | A2 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≤ 2.7 | A1 |
|  |  | ≤ 3.06 | 2.7 < LCRB*12*SCS < 4.5 | A2 |
|  |  | ≤ 3.06 | ≥ 4.5 | A1 |
|  |  | ≥ 3.06 | ≥ 5.4 | A2 |
|  | 2012.5 | ≤ 1.44 | ≤ 2.7 | A1 |
|  |  | ≤ 1.44 | 2.7 < LCRB*12*SCS < 5.4 | A2 |
|  |  | ≤ 1.44 | ≥ 5.4 | A1 |
|  |  | ≥ 1.44 | ≥ 7.2 | A2 |
| 20 MHz | 2010 | ≤ 4.68 | ≤ 2.7 | A1 |
|  |  | ≤ 4.68 | 2.7 ≤ LCRB*12*SCS ≤ 5.4 | A2 |
|  |  | ≤ 4.68 | > 5.4 | A1 |
|  |  | > 4.68 | ≥ 7.2 | A2 |

Table C.1.2.2-4: A-MPR for -30 dBm/MHz

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 2.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 4 |
| CP-OFDM QPSK | ≤ 6 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 4.5 |

### C.1.2.3 	Power back-off simulation from company C (R4-2506521)

C.1.2.3.1 	Simulation parameters

The following are the parameters used for this simulation:

- Operating band n252 (UL 2000 MHz – 2020 MHz)

- 10 MHz, 15 MHz, and 20 MHz CBWs

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Only additional spurious emission mask from NS_07N is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

C.1.2.3.2 	NS_07N mask

Table C.1.2.3.2-1: Additional out-of-band requirements for "NS_07N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1559 ≤ f < 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1605 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz |  |

Table C.1.2.3.2-1 shows the NS_07N mask. This table is aligned with the agreed emission mask as described in the Table 7.4.2.2.8-3. Only the 1930 MHz ≤ f ≤ 1995 MHz segment is considered because the other segments are very far from the channel.

C.1.2.3.3 	Simulation result

Figure. C.1.2.3.3-1  and C.1.2.3.3-2 present the examples of the A-MPR triangles for 10 MHz and 20 MHz CBWs, respectively. The maximum A-MPR at 20 MHz CBW is close to the average between the two results in C.1.2.1 and C.1.2.2. Details results of the A-MPR triangles can be seen from the appendix of R4-2506521.

Fc = 2010 MHz

Fc = 2005 MHz

Figure C.1.2.3.3-1: Examples of the simulated A-MPR to satisfy the -40dBm/MHz additional spurious emission limit for 10 MHz channel at Fc = 2005 MHz (upper row) and Fc = 2010 MHz (bottom row).

Figure C.1.2.3.3-2: Examples of the simulated A-MPR to satisfy the -40dBm/MHz additional spurious emission limit for 20 MHz channel at Fc = 2010 MHz.

C.1.2.3.4 	A-MPR proposal

Using with the result in C.1.2.1 and C.1.2.2 as a comparison, the result that differ to either of the two proposals are highlighted with yellow color. In this simulations the maximum A-MPR values at 10 MHz do not differ much from the corresponding ones at higher CBWs, therefore the A2 region at 10 MHz is changed to A3 to better optimize the remaining A2 values. If the region at 10 MHz CBW is kept as A2, then the A2 values need to be increased.

Table C.1.2.3.4-1: A-MPR regions for -40 dBm/MHz

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 10MHz | < 2010 | ≤ 1.44 | ≥ 0 | A3 |
|  |  | > 1.44 | ≥ 3.6 | A1 |
|  |  | > 7.74 | ≤ 0.72 | A1 |
|  | 2010 ≤ Fc ≤ 2015 | ≥ 0 | ≥ 4.5 | A1 |
| 15MHz | < 2012.5 | ≤ 3.06 | ≥ 0 | A3 |
|  |  | > 3.06 | ≥ 4.5, ≤ 7.2 | A1 |
|  |  | > 3.06 | > 7.2 | A2 |
|  |  | > 10.98 | ≤ 0.72 | A1 |
|  | 2012.5 | ≤ 1.44 | ≥ 0 | A2 |
|  |  | > 1.44 | ≥ 6.48 | A1 |
|  |  | > 12.6 | ≤ 0.72 | A1 |
| 20 MHz | 2010 | ≤ 4.68 | ≥ 0 | A3 |
|  |  | > 4.68 | ≥ 4.5, ≤ 9 | A1 |
|  |  | > 4.68 | > 9 | A2 |
|  |  | > 12.6 | ≤ 0.72 | A1 |

Table C.1.2.3.4-2: A-MPR for -40 dBm/MHz

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 4 | ≤ 8 | ≤ 10.5 |
| DFT-s-OFDM QPSK | ≤ 4.5 | ≤ 8 | ≤ 11 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 8 | ≤ 11.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 8 | ≤ 11.5 |
| DFT-s-OFDM 256 QAM | ≤ 5.5 | ≤ 8 | ≤ 11.5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 8.5 | ≤ 12.5 |

<not affected part not shown>

End of Changes