3GPP TSG-RAN WG4 Meeting #115	R4-2508051

Malta, 19th – 23rd May, 2025

Title: 	WF on AI/ML part 2

Agenda Item:	7.22.1

Source: 	Ericsson

Document for:	Approval

# Two-sided CSI compression

## Simulation result observations

Agreements:

During the ad-hoc, it was agreed that the moderator should propose some general observation that the study has focussed on SGCS convergence, not absolute SGCS. The following text is provided:

The focus in this study has been on convergence between SGCS reported for different company UE encoders when operating with test decoders, in order to understand whether it is feasible to specify a test decoder (or, for option 4, information used to create a test decoder) that can be used for consistent performance testing across multiple UE vendor implementations.

Absolute SGCS has not been considered, since the assumed model structure is somewhat arbitrary and the purpose of the study has been to consider testing.

For option 3 track 1, the endorsed observations are as follows:

- It is possible for UE vendors to train an encoder based on another company’s frozen test decoder that is functional

- The performance of the encoder is very dependent on the dataset used for training the encoder and the dataset used for training the frozen decoder, and the testset

  - If the encoder is trained using a dataset of the UE vendor, the UE vendor dataset should contain the [features] captured in the dataset used to develop the test decoder.

- The variation in the performance depending on the training datasets and test datasets can be as much as 30-50% in SGCS. Further details are available in the results spreadsheet.

- For option 3, track 1 to be useful, more careful attention would be needed to the alignment of datasets

- Only one encoder structure was considered for option 3 track 1. Other lower complexity encoder structures were not examined.

For option 3 track 2, the endorsed observations are as follows:

- There is reasonable SGCS convergence when companies jointly train an encoder-decoder pair using the mixed dataset and test using the mixed testset

  - Around +-2% variation in SGCS; see results spreadsheet for more details

- It is possible for UE vendors to train an encoder based on a frozen test decoder that is functional

- When training “own” encoder with the frozen decoder, with all training (frozen decoder and encoder) and testing using the mixed dataset, there is good convergence in average SGCS results

  - Around - 2% to +1.4% variation in SGCS from the average

- When training “own” encoder with the frozen decoder, with training of the frozen decoder using the mixed dataset, own encoder using own dataset and testing using the mixed dataset, there is convergence in average SGCS results from results presented so far

  - Around -+-4% variation in SGCS from the average (+-1.5% with outlier performance excluded)

- When training “own” encoder with the frozen decoder, with training of the frozen decoder using the mixed dataset, own encoder using own dataset and testing using the own dataset, there is much less convergence in average SGCS results from results presented so far

  - Around -40% to 13% variation in SGCS from the average (+-8% to 5% with outlier performance excluded)

- The results considering “own” datasets demonstrate that there is divergence in the datasets, and attention must be paid to alignment of the training datasets.

  - Note: Alignment of datasets does not refer to aligning UE algorithms such as channel estimation, CSI estimation etc.

For option 4, the endorsed observations are as follows:

- For option 4b: It is feasible to train a functional “own” decoder based on the frozen encoder, and then a functional “own” encoder based on the “own” decoder when the mixed dataset is used

- For option 4a: For the mixed dataset, a similar observation to option 4b can be expected (in this case, the labelled dataset would be the same latent space as the selected encoder (created using joint training) acting on the mixed dataset, and so option 4a converges to option 4b)

- If the frozen encoder is created using the mixed dataset, TE/UE vendors create decoders using the mixed dataset, UE vendors create an encoder using the mixed dataset and testing uses the mixed testset then good convergence is seen in SGCS

  - Around +- 1.5% from average SGCS

  - UE encoders were shown to be interoperable with all companies own decoders

  - This represents the situation where there is a well defined dataset, and UE vendors use the same dataset to train their encoder models

- Some companies observed that

  - if the frozen encoder is created using the mixed dataset,

  - TE/UE vendors create decoders using the mixed dataset,

  - UE vendors create an encoder using their own dataset and

  - testing uses the mixed testset

then [good] convergence is seen in SGCS, but there is more variation than when mixed dataset is used for training the encoder

  - Around [+- 3%] from average SGCS

    - [Further SGCS results may be provided in the next meeting]

  - [UE encoders were shown to be interoperable with all companies own decoders]

  - This represents the situation where there is a well defined dataset that is used for all 3GPP responsibilities (Creating the frozen decoder, providing information for the TE vendor to train the decoder, providing test data), but the UE vendor uses their own dataset to create the encoder

- Some companies observed that if the frozen encoder is created using the mixed dataset,

  - TE/UE vendors create decoders using the mixed dataset,

  - UE vendors create an encoder using their own dataset and

  - testing uses the own testset

then [convergence is seen in SGCS,] but there is more variation than when mixed dataset is used for training the encoder. The variation is similar between using the mixed dataset and own dataset for testing.

  - Around [+- 1.6 to +4.7%] from average SGCS

  - [Further SGCS results may be provided in the next meeting]

  - [UE encoders were shown to be interoperable with all companies own decoders]

  - This represents the situation where there is a well defined dataset that is used for all 3GPP responsibilities (Creating the frozen decoder, providing information for the TE vendor to train the decoder, providing test data), but the UE vendor uses their own simulation to create the encoder

- Two lower complexity encoders were considered and tested based on the frozen decoder.

  - A transformer with 3.63 MFlops

  - A CNN encoder with 4.14 MFlops

- When the lower complexity encoders were examined using the mixed dataset for all training and the mixed test set for testing, SGCS convergence (- 1 to 3.5%) and interoperability with all companies “own” decoders was observed for the considered structures.

Further simulations (not agreement):

In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtained aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures.

- Performance assuming training and testing with the Samsung frozen decoder

- Performance achieved with a jointly trained low complexity encoder and decoder

- Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity)

- Consider transformer, CNN, MLP

## What should be captured or specified for each of the options

Agreements:

The following agreements were reached for option 3. For clarity, the agreement is added alongside the agreement from RAN4#114bis.

Agreements from RAN4#114bis

- The encoder that corresponds to the standardized test decoder is stored within 3GPP

  - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS)

  - FFS whether several encoders would be captured (e.g. due to different complexity)

New agreement from RAN4#115:

The training dataset used to train the test decoder in 3GPP should be captured

- FFS whether channel, Eigenvectors or something else

The following agreements were reached for option 4a. For clarity, the agreement is added alongside the agreement from RAN4#114bis.

Agreements from RAN4#114bis

- Dataset is specified for option 4a.

- Assume reference decoder is captured.

New agreements from RAN4#115:

- Reference encoder should be captured

- FFS whether the labelled dataset is based on Eigenvector, channel estimation or something else

- The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure.

  - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters.

  - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

The following agreements were reached for option 4b. For clarity, the agreement is added alongside the agreement from RAN4#114bis.

Agreements from RAN4#114bis

- Dataset is specified for option 4a.

- FFS on dataset is captured for option 4b.

- Assume reference decoder is captured.

- FFS on reference encoder for option 4a

- Test decoder structure is (i) fully specified or (ii) partially specified

- NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase.

New agreements from RAN4#115:

- Specify dataset to train frozen encoder and decoder

    - TE vendor shall use the dataset to train the test decoder

    - This is the same dataset as used to train the frozen encoder

    - FFS whether the datasets consist of Eigenvector, channel or something else

- The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure.

  - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters.

  - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

## Other agreements

Agreements:

If RAN1 direction C agreed, RAN4 and RAN1 should co-ordinate on model structure etc. for direction C due to the similarities between option 3/4 and direction C

- Duplicated work is to be avoided

At the current moment in time, neither option 3 track 2, nor option 4a or 4b should be ruled out for a WI. Option 3 track 1 can be ruled out.

Option 3 and 4 are similar:

- An encoder, decoder and training dataset are always captured

- The difference is that for option 3, the TE vendor is mandated to use the standardized test decoder. In option 4 they could choose to use the reference test decoder or to create their own based on the standardized information.

# LCM requirements

Agreements:

Activation delay:

- Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction

  - It is FFS for CSI prediction

- For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism.

- For semi-persistent CSI reporting

  - Activation delay starts at the reception of the MAC-CE/DCI

- For aperiodic CSI reporting

  - Activation delay starts at the reception of the DCI

Switching delay and Fallback delay:

- Deprioritize the discussion on switching and fallback delay requirements and focus on activation/deactivation delay requirement, including both AI/nonAI, discussion.

Monitoring reporting delay and accuracy:

Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric:

- BM-Case1 and possibly BM-Case 2

o	Performance monitoring type 1 option 2

- If it proves to be not possible to define, then reverse the agreement

- Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed.

CSI prediction:

- CSI prediction, the details are subject to RAN1 decision

- RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

Way forward (not agreements)

Deactivation delay for CSI prediction:

- The following questions were noted for further discussion

  - Can NW differentiate between the predicted CSI reporting and the measured CSI reporting?

  - In case of switch between AI, is there existing reporting to differentiate the reports before and after the deactivation.

# Generalization

Agreements:

The high level and general guidelines for the generalization test are provided as follow. The exact decisions are to be made case by case.

- Study and , if feasible, define requirements for each AI/ML functionality

- Define one test per UE capability as a minimum

  - FFS >1 test per UE capability

- Define a minimum set of test configurations (including NW sided conditions if any) as mandatory for testing of AI/ML-enabled Feature

Potential areas to consider for generalization testing:

- gNB array parameters

  - port layouts, array size, antenna virtualization

- propagation conditions

- Deployment scenarios

  - Carrier frequencies

  - Speeds

  - Indoor/outdoor

  - Bandwidths

- SNR

# Post-deployment

Agreements:

Option 2 (Post-deployment post-activation functionality testing based on performance monitoring) is prioritized.

- it should be further investigated/established that reliable and testable monitoring procedures can be established for all use cases