3GPP TSG-RAN WG4 Meeting #115															R4-2508082

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:		7.21.1

Source: 	Qualcomm Incorporated

Title: 	AI/ML ad-hoc meeting minutes

Document for:	Approval

# Introduction

The discussion on the AI/ML study is organized in two threads [131] and [132] in RAN4#115. The ad-hoc meeting will discuss some of the topics from the moderator summary in [1].

# Discussion

## Beam management

### Sub-topic 2-1

Beam ID prediction KPIs

The KPI was agreed in the last meeting, however, some of the parameters N, x are still FFS.

- Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x values

Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Issue 2-1: Metrics/KPIs for beam ID prediction

Proposals

  - Option 1: adopt N

    - N=2

    - Further study the actual value

    - N>2

    - N= K2+1

  - Option 2: Do not use N, remove the FFS on top-N

  - Option 3: x<3dB

  - Option 4: further study x based on simulations

  - Option 5: Others (e.g. different distributions, etc)

- Recommended WF

  - Option 1/2 and 3/4 to be discussed separately

Discussion:

CMCC: what is N? N is used to refer to a subset of K. max of ground truth is chosen from N. other interpretation means that N is the percentage of top K/N

NTU: this is from Nokia’s proposal. N=2 or 3, we would check top-K against the 2 best or 3 best beams based on ground truth with X as tolerance. When we have K>1, the result of top K approach will contradict topK/N. there will be a contradiction so we support Option 2. With K=2, we compare top 2 predicted and compared with top 2 given by ground truth.

Nokia: we should not only verify against the top -1 genie added, we should compare also the 2nd predicted against the 2nd best genie added.

Nokia: our understanding of N, for performance monitoring there is a similar concept. The best beams is withing the K predicted beams.

Qualcomm: let’s assume X=0. Top-K predicted beams will contain the top genie added. Nokia’s proposals is basically saying that the top-K has to contain the N genie added beams. RAN1 says that one of top K should contain one of the top N. from our perspective the necessity of N is not there. With X, within the top-K the predicted bems should not be that far off from the ground truth.

Oppo: we share similar view as QC. Intention to report K is that best beam is within top K.

Nokia: if we do not consider N, we go back to option 3.

### Sub-topic 2-2

Measurement period for inference

Some agreements were made in the previous meeting on the measurement period, however, some details are still to be agreed.

Agreement:

- For measurement delay component of the prediction delay:

  - Use “legacy” L1-RSRP measurement as the baseline for Case 1

    - FFS whether same values for M,N,P should be reused

  - FFS for case 2

Issue 2-2: Measurement period for inference

- Proposals

  - Option 1: use the same values for M,N,P for Case 1

    - Option 1a:

- For M and N, reuse the same values defined in existing L1-RSRP measurement period requirements

- For P, RAN4 to discuss which following alternatives to adopted:

  - Alt 1: The definition of P will be based on the R15 spec version and the enhanced features introduced in previous releases are not supported by default

  - Alt 2: The definition of P needs to take these enhancements introduced after R15 into account, such as MUSIM gap, NCSG, etc

  - Option 2: use the legacy L1-RSRP also for case 2

    - Further discuss what value to use for M

  - Option 3: Others

- Recommended WF

  - Option:  1 and Option 2

Discussion:

Qualcomm: we agree with Option 1a. UE does not need to support MUSIM, NGCSG, etc. For Option 2, RAN1 is discussing how many samples the UE needs to measure for a prediction, we should not discuss option 2.

Vivo: we support option 1a. we have different definition of P. it is more complicated in later releases.

Xiaomi: for case 1, for the Rx beam sweeping UE, RAN1 is using different beam sweeping assumptions. For case 2 , we support QC.

Apple: we would like to keep N=8. If a UE supports enhancements, it can always a smaller N.

Samsung: we prefere to only have minimum reqs. We would like Option 1a.

E///: for Option 1a, we are fine to keep legacy M, N. for P, maybe we need more time.

Agreement:

Case 1: reuse existing M, N, P (reference to an existing clause)

Case 2: postpone discussion until RAN1 concludes

### Sub-topic 2-3

Rx beam knowledge and TCI state handling

The issue of UE Rx beam knowledge for a predicted Tx beam and how this relates to whether a TCI state is known or unknown has been constantly brought up but hasn’t been discussed in the previous meetings. This influence the handling of the TCI state switching and would be part of the core requirements.

Issue 2-3: Rx beam knowledge and TCI handling

- Proposals

  - Option 1: UE always knows the Rx beam to receive a predicted Tx beam. TCI state known/unknown should follow the legacy requirements (predicted beam should be handled as a measured/reported beam)

  - Option 2: UE knows the Rx beam for the case when predicted RSRP is reported. TCI state known/unknown should follow the legacy requirements (predicted beam should be handled as a measured/reported beam)

  - Option 3: UE Rx beam knowledge is optional, introduce a capability. TCI state known/unknown is based on this capability

  - Option 4: Split into different cases:

- Set A and Set B are different (Set B is NOT a subset of Set A). E.g., use known SSB beams to predict CSI-RS beams:

  - if the predicted Tx beam which is not measured is QCL Type-D to a known Tx beam in Set B, the corresponding Rx beam is known.

- Set B is a subset of Set A.

  - Use part of SSB beams to predict all the SSB beams: Whether Rx beam of the predicted Tx beam which is not measured can be known can be up to UE capability.

  - Use part of CSI-RS beams with repetition off to predict more CSI-RS beams with repetition off: If Rx beam of the QCL-ed SSB is known, then Rx beam of the predicted CSI-RS beam is also known.

  - Option 5: The TCI state is known when the following is met:

    - Target TCI state is based on RS from set A beams with QCL relationship configured to a set B beam

    - The time between TCI state switch command and transmission of the QCLed RS from set B shall not exceed X ms

      - X = 1280 ms (legacy)

    - UE shall send measurement report for the QCLed RS from Set B between transmission and TCI state switch

    - TCI state and SSB of TCI state remain detectable during TCI state switching period

    - Otherwise, it is unknown.

  - Option 6: The TCI state is known when the following is met:

    - The TCI state switch command is received within a delay X*1280 ms upon the last transmission of the RS resource for beam reporting or measurement or prediction.

    - FFS: Further analysis on defining X values

  - Option 7: other

- Recommended WF

  - To be discussed

Agreement in main session:

  - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known.

  - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown.

  - Known Rx beam means no additional Rx beam sweeping is needed

  - Unknown Rx beam means additional Rx beam sweeping is needed.

  - Rx beam is known mean TCI state is known if conditions below are also met

- The target TCI state is known when the following is met:

  - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1  RSRP report of the target TCI state (clause 8.10.2 of 38.133)

  - Applies for case 1

  - FFS for case 2

  - Whether RS resource is only from set B should be clarified

  - FFS on whether prediction also impacts if a TCI state is known or unknown

    - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown

    - FFS on the time delay if prediction is used as a condition for a known TCI state

Discussion:

Apple: why is X there? There should be no X.

Qualcomm: we already agreed that TCI state can be known if it was predicted.

Xiaomi: this only applies for case 2.

Intel: we should clarify that if Rx beam is known this implies that TCI state is known

E///: Xiaomi’s comments is aligned with us, for Case 2 the measurement will have to be within 1.28s so the prediction window has to be limited to this amount of time. Known/unknown will depend on the prediction window length.

NTU: we should define the known/unknown also based on prediction so that we can save overhead when doing prediction. If UE still has ot do measurement, the advantage of doing prediction is very limited

### Sub-topic 2-4

Relative RSRP accuracy

A tentative definition for the relative RSRP accuracy was agreed in the last meeting, however, some details are still to be discussed/agreed

Agreement:

- RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined

  - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value].

    - Reported L1-RSRP can be predicted,

    - FFS whether reported L1-RSRP can be measured RSRP

  - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Issue 2-4: Relative RSRP accuracy

- Proposals

  - Option 1: Relative RSRP accuracy requirement could be applied to other predicted beams with differential RSRP reporting in RAN4 tests.

    - Should apply to both Case 1 and Case 2

  - Option 2: Reported L1-RSRP is the predicted L1-RSRP

  - Option 3: clarify that the prediction is for time instance m for Case 2

    - The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance

  - Option 4: For BM case 2, Relative accuracy can happen in two dimensions:

    - Intra-time beam comparison (same time, different beams)

    - Inter-time beam comparison (same beam, different times)

- Recommended WF

  - Option 1 and Option 2

Discussion:

### Sub-topic 2-5

Simulation results

Issue 2-5:	Simulation results

- Proposals

  - Option 1: Discuss the simulation results based on summary – to be prepared by vivo

    - Simulation results, any insights on how to define the requirements?

    - Refinement of simulation assumptions (e.g. error modelling, etc)

    - Next steps for simulations

- Recommended WF

  - To be discussed

Discussion:

Vivo: after summarizing the results, even with the case not considering the errors, there is a misalignment in results. Companies are encouraged to share the dataset for the simulations.

Qualcomm: we should start simulating SSB to CSI-RS. This is another subcase of case 1. Others are encouraged to bring results. In order to define K or X. the test will be link level so we have to discuss how to come up with a CDL type channel for testing.

NTU: based on my observations, none of the K and x are aligned. SSB to CSI-RS is important but without the alignment of CSI-RS to CSI-Rs case, SSB to CSI-RS will be even more difficult to align.

Oppo: maybe we can think about how to handle the dataset. If some companies are willing to do that, companies can think about how to use the dataset to check the alignment/result.

Apple: QC brought up that we are trying to align the results based on system level channels but we need link level channels. We should focus on doing something based on link level. For CSI prediction where we seem to be more aligned.

### Sub-topic 2-6

Impact to other requirements

Use of beam prediction could impact other RRM requirements which rely on measurement reports. Possible impact to CBD was brought up in one of the contributions. RAN4 could discuss how to handle such cases

Issue 2-6:	Simulation results

- Proposals

  - Option 1: RAN4 to further analyze which requirements could be impacted. Companies to bring analysis until next meeting on impacted requirements and potential changes.

  - Option 2: RAN4 to only discuss the impact to CBD evaluation period:

    - RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams

  - Option 3: RAN4 does need to discuss such impact, the changes would implicitly apply by assume a predicted beam as measured/reported

  - Option 4: There is no impact, this is not in scope

  - Option 5: others

- Recommended WF

  - To be discussed

Discussion:

### Sub-topic 2-7

LCM

Issue 2-7:	LCM Requirements

- Proposals

  - Option 1: the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

  - Option 2: For performance monitoring for BM, the corresponding delay is defined as

    - Tmonitoring_period_BM = Nmonitor* TL1-RSRP_Prediction_Period_CSI-RS,

      - Where Nmonitor is the configured number of the latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

  - Option 3:postpone discussion until RAN1/2 monitoring framework is clear

- Recommended WF

  - To be discussed

Discussion:

### Sub-topic 2-8

Test system basic setup

The basic test setup has been discussed for several meetings without much progress.

Issue 2-8: Test system setup

- Proposals

  - Option 1: Single AoA with UE antenna pattern knowledge

  - Option 2: Single AoA without any UE antenna pattern knowledge

  - Option 3: Multi AoA based on MPAC

  - Option 4: Multi AoA based on RRM testing setup

- Recommended WF

  - To be discussed

### Sub-topic 2-9

Test system channel model

In the previous meeting it was agreed that a CDL-based channel model will be taken as starting point.  Further details need to be discussed

Issue 2-9: Test system setup

- Proposals

  - Option 1: “full” CDL model to be emulated in the test

  - Option 2: simplified CDL should be used

  - Option 3: others

- Recommended WF

  - To be discussed

If Option 2 is chosen, simplification methods should be discussed.

### Sub-topic 2-10

Absolute RSRP accuracy applicability

In the previous meeting it was agreed that absolute RSRP would apply to the top predicted beam but applicability to the other predicted beams is still FFS

Agreement:

- Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least

  - FFS whether to apply to other beams

Issue 2-10: Absolute RSRP accuracy applicability

- Proposals

  - Option 1: applicable only to top predicted beam

  - Option 2: should apply to all K predicted beams

  - Option 3: postpone this discussion to the performance part as it relates to accuracy

  - Option 4: others

- Recommended WF

  - Option 3

### Sub-topic 2-11

Draft CR

Draft of a CR to introduce requirements in 38.133 was presented in R4-2506464. Some high level structure of the specs should be discussed.

Issue 2-11: Beam prediction CR

- To be discussed

  - High level structure: in which clauses should the beam prediction specs go

    - LCM

    - Prediction(inference) requirements

    - Performance requirements (to be finalized later but initial agreement on structure would be desirable)

  - Use “legacy” formatting or try new “pseudocode” based option?

  - Other suggestions

Discussion:

## Positioning

### Sub-topic 3-1

Requirements for case 1

It has been discussed for several meetings whether to define requirements for case 1 without any progress

Issue 3-1: Requirements for case 1

Proposals

  - Option 1: RAN4 will define new positioning accuracy requirements

    - Accuracy should be the difference between ground truth and reported position

    - See R4-2507533 for possible testing solution

  - Option 2: existing requirements for legacy positioning techniques(RSTD, PRS-RSRP, etc) should be reused for Case 1

  - Option 3: RAN4 to define only reporting delay(including inference delay) requirements

  - Option 4: RAN4 does not define any requirements for case 1

- Recommended WF

  - Option 2

Defining new requirements(tighter than existing ones will be a very long and complicated process)

Discussion:

E///: Option 2 is not relevant for case 1 because the reports just assist the positioning determination. It’s not UE computing the position

CMCC: we would prefer to have some requirements but we understand it is difficult to define accuracy but we should have some requirements for the delay.

Agreement:

- Discuss how to define a measurement delay requirement(reporting delay)

- Do not define any positioning accuracy requirement

### Sub-topic 3-2

Report mapping for sample based timing measurement reporting

A proposal for the reporting range and report mapping is provided in R4-2507664.

Issue 3-2: Report mapping for sample based timing measurement reporting

Proposals

  - Option 1: Agree Proposals 2, 3, 4,5,6,7,8

  - Option 2: Others

- Recommended WF

  - Option 1

Discussion:

QC: the proposed tables are similar to the legacy tables of the 1st path, there is nothing related to the additional path. It was proposed to report additional paths through bitmaps. We should use the existing mapping for both 1st path and additional path.

Agreement:

  - Use legacy UL RTOA mapping for 1st sample

  - FFS for the additional sample reporting

### Sub-topic 3-3

Case 3b requirements

Issue 3-3: Requirements for case 3b

- Proposals

  - Option 1:

    - RAN4 to introduce gNB Rx-Tx time difference reporting accuracy requirements (i.e., path-based measurements) by reusing the legacy accuracy requirements in Case 3b.

    - RAN4 to study feasibility of reusing the legacy performance requirements for the sample-based measurement, and how to specify separate performance requirements for sample-based measurement if reusing is not feasible in Case 3b.

  - Option 2: Others

- Recommended WF

  - Option 1

Discussion:

E///: Rx-Tx time difference is only accuracy for 3a which we already agreed. 2nd bullet for Option 1 is that there is no requirement for sample based.

CMCC: legacy Rx-Tx requirements for path based should apply.

Nokia: our intention was to extent the case 3a agreement to 3b path based

E///: we cannot reuse the legacy requirements because it is only for the 1st path.

Agreement:

- Reuse legacy path based Rx-Tx accuracy requirements for case 3b

- Requirement applies only for the 1st path

# References

[1] R4-2507570, “Topic summary for [115][131] NR_AIML_air_part1”, Moderator(Qualcomm Incorporated), RAN4#115