3GPP TSG-RAN WG4 Meeting #115	R4-2508089

Malta, MT, May 19th – May 23rd, 2025

Source:	Nokia

Title:	TP for TR 38.774 on LP-WUS

Agenda item:	7.28.2

Document for:	Discussion

Introduction

In this text proposal for TR 38.774, we provide our inputs on section 7.1. These inputs are based on the agreements achieved in the previous meetings.

- References

- R4-2505234, WF for [114bis][131] NR_LPWUS_UERF, Vivo, RAN4#114bis, Wuhan, China, 7th – 11th April, 2025.

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.

- R4-2420528, WF on UE RF requirements for LP-WUS, Vivo, RAN4#113, Orlando, US, 18th – 22nd Nov, 2024.

- R4-2417112, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112bis, Hefei, China, 14th – 18th Oct, 2024.

- R4-2414309, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112, Maastricht, Netherlands, 19th – 23rd August, 2024.

****************************************** Start of TP ******************************************

### 7.1.1	General

The minimum requirements will be determined assuming there is only one receiver. A 1% MDR criterion will be used for verification of all core RF requirements. Following LP-WUS parameters will be used

Table 7.1.1-1. Common reference channel parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 15/30kHz |
| RM coding | Bits | 16 |
| CRC |  | No CRC |
| Chip rate |  | M=4 (4 chips in an OFDM symbol) |
| Overlaid OFDM sequence |  | Length 33: generated by 31-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 4 |
| WUS duration for OOK |  | 8 OFDM symbols |
| WUS duration for OFDM |  | 2 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits | Bits | 5 |

In the specification, tables will be defined corresponding to 15 KHz and 30 KHz SCS.

************************* UNCHANGED CLAUSES HAVE BEEN OMITTED *************************

### 7.1.4	REFSENS requirements

It was agreed to derive the REFSENS for LP-WUR in the typical RF manner with AWGN being the channel for which SNR is derived. Thus, the REFSENS for LR is defined as

REFSENSLR (dBm) = -174 + 10log10(BWLR)+ (NFLR + IMLR) + SNRLR.

Here, BWLR corresponds to the bandwidth of the LP-WUS signal in hertz. RAN1 has decided to have 11 RBs for LP-WUS irrespective of SCS, thus the LP-WUS bandwidth will be 1.98 MHz and 3.96 MHz, for 15 and 30 KHz SCS, respectively. Further, there are two set of values for NFLR + IMLR, i.e. 18.0dB and 13.5dB for Type 1 and Type UEs, respectively. There is only single receiver so there is no diversity gain included in the REFSENS.

In the REFSENS test, only the RBs allocated for LP-WUS are populated with the LP-WUS and the rest of the RBs for a given channel bandwidth are left empty. This configuration, however, does not correspond to the real-life case where NR signals in other RBs are generally present and thus can create a coverage shortfall. Separately, the ASCS requirement allows a maximum degradation of 0.5 dB and the side conditions incorporate populated adjacent RBs.

Given the small value of the ASCS degradation factor (0.5 dB), it was agreed not to include this factor in the REFSENS.

For FR1, a SNR value of -4.5 dB for both Type 1 and Type 2 UEs was selected by RAN4 to be used for the REFSENS equation.

****************************************** End of TP ******************************************