3GPP TSG-RAN WG4 Meeting #115                                           R4-2508096

Malta, MT, 19th – 23rd May, 2025

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.774: Blocking requirements for LP-WUR

Agenda Item:	7.28.3.3

Document for:	Approval

Background

This contribution provides the TP for TR 38.774 on blocking requirements for LP-WUR.

References

[1] R4-2503830, TR 38.774 V0.3.0

Text Proposal

<Start of Text Poposal>

- 7.1.7	IBB requirements

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit.

In the specification, the absolute interferer levels are adopted for both in-band blocking (IBB) and out-of-band blocking (OBB). IBB refers to interference that occurs within the second adjacent and next adjacent channels, while OBB occurs outside a frequency range 15 MHz below or above the UE receive band. The interference of IBB and OBB are higher than ACS. The interferers relevant to the wanted signal are illustrated in Figure 7.1.7-1:

Figure 7.1.7-1: Interference of ACS, IBB and OBB

The main challenges for receiver design in presence of strong interferer are the Rx chain dynamic range as well as the filter suppression capability, which could be the channel filters to alleviate interfering by ACS or IBB, or could be the band filter/duplexer to suppress OBB.

Regarding the dynamic range, the main determining components are ADC and AGC. The more ADC bits, the larger dynamic range for the receiver. However, the ADC bits cannot be increased without limitation. The relationship between ADC bit depth and power consumption is typically inversely proportional. Higher bit-depth ADCs generally offer greater precision and resolution but tend to consume more power due to the increased complexity of circuitry required for finer quantization and digitalization. Conversely, lower bit-depth ADCs consume less power but sacrifice resolution and dynamic range.

In the context of 5G smartphones, ADCs with 12 bits or higher are commonly employed. These higher precision ADCs are favoured to accommodate the demanding requirements of 5G communication, such as higher data rates and broader spectrum ranges. While some 5G smartphones may opt for even higher bit-depth ADCs like 14 or 16 bits for enhanced performance, the general trend revolves around 12 bits.

However, there's a trade-off to consider. LP-WUR which aims to minimize power consumption in smartphones, often utilize lower bit-depth ADCs to reduce power consumption. While this approach yields power-saving benefits, it may compromise the dynamic range and overall performance of the ADC. During the SI evaluation, 4 bits, 6 bits and 8 bits are considered for LLS simulation. Apparently, if low power consumption is the main target for LP-WUR design, it may not be possible that a LP-WUR could adopt the same ADC as MR, which means the dynamic range for LP-WUR could be sacrificed to some extent.

Apart from ADC, the dynamic range of AGC is another main factor which could have impact on the possibly large interferer handled by LP-WUR. On the other hand, due to the limited suppression capability of the LP-WUR filters, it may not guarantee the same level of resistance against IBB and OBB interference as the main receiver (MR). Or if the same interference levels are kept as MR, the wanted signal for blocking requirements should be relaxed more instead.

Similar to ACS, different cases are considered for IBB. The power difference between Case 2 and Case 1 is 12 dB, reflecting filtering effects with further frequency offsets and the dynamic range of the receiver.

Keeping the same Pinterferer as MR, the REFSENS should be relaxed as in Table 7.1.7-1 and Table 7.1.7-2 for type 1 and type 2 respectively with the interferer level defined in Table 7.1.7-3.

Table 7.1.7-1: In-band blocking parameters for Type 1 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENS + [TBD] dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 7.1.7-2: In-band blocking parameters for Type 2 LP-WUR

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- | --- |
|  |  | 11 |
| Power in LP-WUS transmission bandwidth configuration | dBm | REFSENS + [TBD] dB |
| BWinterferer | MHz | 5 |
| FIoffset, case 1 | MHz | 7.5 |
| FIoffset, case 2 | MHz | 12.5 |
| NOTE 1: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 7.1.7-3: In-band blocking for LP-WUR

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Pinterferer | dBm | [-56] | [-44] |
| Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |

- 7.1.8	OBB requirements

For out-of-band blocking (OBB), with suppression of the out-of-band interferer by band filter and the analog filter before ADC, the blocking could be suppressed to relatively low level even with less dynamic range of LR, unless band filter is not considered for better coverage with less insertion loss for LP-WUR design. According to the evaluation, at least 6dB relaxation should be considered for OBB, either to reduce the interferer level or to increase the wanted signal level. Keeping the same Pinterferer as MR, the REFSENS should be relaxed as in Table 7.1.8-1 and Table 7.1.8-2 for type 1 and type 2 respectively with the interferer level defined in Table 7.1.8-3.

Table 7.1.8-1: Out-of-band blocking parameters Type 1 LP-WUR

| LP-WUS bandwidth (RBs) | Power in LP-WUS transmission bandwidth configuration [dBm] |
| --- | --- |
| 11 | REFSENS + [TBD] dB |

Table 7.1.8-2: Out-of-band blocking parameters Type 2 LP-WUR

| LP-WUS bandwidth (RBs) | Power in LP-WUS transmission bandwidth configuration [dBm] |
| --- | --- |
| 11 | REFSENS + [TBD] dB |

Table 7.1.8-3: Out of-band blocking for LP-WUR

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pinterferer | dBm | [-44] | [-30] | [-15] |
| Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

In addition to the REFSENS degradation level and the interferer level, the allowed exceptions for spurious responses should also be taken into account for LP-WUR. Due to the constraints of low-cost implementation, the ability to restrict harmonics may be inferior compared to MR. Consequently, the number of exceptions may need to be increased to accommodate this limitation.

- 7.1.x	NBB requirements

Narrow Band Blocking (NBB), it was initially defined to address coexistence scenarios with GSM at a 200 kHz frequency offset, inherited from LTE for NR, which is also applicable to LP-WUR. Given the close offset to the desired signal, NBB can significantly degrade REFSENS, particularly if the phase noise of the interferer is poor. If keeping the same Puw (CW) level as MR, the relaxed sensitivity level for Type 1 and Type 2 UE are considered in Table 7.1.x-1 and Table 7.1.x-2 respectively.

Table 7.1.x-1: Narrow Band Blocking or Type 1 LP-WUR

| Parameter | Unit | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + [TBD] |
| Puw (CW) | dBm | [-55] |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

Table 7.1.x-2: Narrow Band Blocking or Type 2 LP-WUR

| Parameter | Unit | LP-WUS transmission bandwidth configuration (RB) |
| --- | --- | --- |
| Pw | dBm | PREFSENS + [TBD] |
| Puw (CW) | dBm | [-55] |
| Fuw (offset SCS= 15 kHz) | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |
| NOTE 1: Fuw shall be rounded to half of SCS NOTE 2: LP-WUS is configured at the edge of the NR carrier with minimum guardband specified in Table 5.3.3-1 for NR channel bandwidth ≥ 5 MHz. |  |  |

<End of Text Poposal>