3GPP TSG-RAN WG4 Meeting # 115	R4-2508122

Malta, MT, 19th – 23rd May, 2025

Source: 	vivo

Title: 	WF on IoT-NTN HPUE

Agenda Item:	7.10.1

Document for:	Approval

- Tx Requirements

- Tx requirements

Issue 1-1-1: MPR requirements definitions for NB-Iot based Iot-NTN

- Updated information and tentative conclusions in previous offline discussion

  - Modulation and SCS restriction in physical spec

    - Single Tone allocation

      - Supported Modulation: pi/2 BPSK, pi/4 QPSK

      - Supported SCS: 15KHz, 3.75KHz

    - Multi-Tone allocation

      - Supported Modulation: QPSK

      - Supported SCS: 15KHz

  - An allowed configuration without requirement is likely deemed as 0dB for MPR

Agreement:

  - MPR requirements need to cover all supported modulation and SCS

Issue 1-1-2: MPR requirements for NB-Iot based Iot-NTN PC2

- Agreement:

  - Unified requirements for pi/2 bpsk and pi/4 QPSK for single Tone

  - The following structure and requirements considered as starting point. Further refinements on tables and wording can be made later

- Table 6.2B.2-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |
| --- | --- | --- |
| SCS | 15KHz | 3.75kHz |
| Tone positions for 1 Tone allocation | [0-11] | [0-47] |
| MPR | [0] dB | TBD dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |

- Table 6.2B.2-3: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ TBD dB | TBD dB |  | ≤ TBD dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ TBD dB |  | ≤ TBD dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [3] dB |  |  |  |
| Note 1: For multi-tone allocation, only QPSK and 15KHz SCS can be supported |  |  |  |  |

Issue 1-1-3: MPR requirements for NB-Iot based Iot-NTN PC1

- Agreement:

  - Try to agree the following parameters and tentative requirements as following (as high lightened in the summary table, with already two sets of measurements and one set of simulation):

    - Tone positions for 1 Tone allocation for 15KHz SCS:

      - Using option 1, which divided to  three region in square brackets [0~2], [3~8], [9~11]

    - MPR requirements for 1 Tone allocation for 15KHz SCS

      - [1.5~2dB] for edge regions, and [0.5dB] for center region

      - Unified requirement for pi/2 BPSK and pi/4 QPSK

    - MPR requirements for 12 Tone allocation for 15KHz SCS

      - [5-5.5dB]

  - These requirements are targeted for all form factors.

  - The remaining requirements (3/6 tones allocation, 1Tone allocation for 3.75kHz SCS) would be discussed in next meeting.

  - Using following structure as starting point. Further refinements on tables and wording can be made later

- Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  | 3.75kHz |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] | [8-11] | [0-TBD] | TBD | [TBD-47] |
| MPR | ≤[1.5~2] dB | [0.5] dB | ≤[1.5~2] dB | ≤ TBD dB | [0.5] dB | ≤ TBD dB |
| Note 1: For single-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |  |  |

- Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15KHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤TBD dB | TBD dB |  | ≤TBD dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤TBD dB |  | ≤ TBD dB |  |
| [Tone positions for 12 Tones allocation] | 0-11 |  |  |  |
| [MPR] | ≤ [5-5.5] dB |  |  |  |
| Note 1: For multi-tone allocation, only QPSK and 15KHz SCS can be supported |  |  |  |  |

Issue 1-1-4: Further MPR/A-MPR assumptions for NB-Iot based Iot-NTN in further evaluation;

- Agreement:

  - [3] dB post-PA insertion losses for calibration purpose

  - For PC2, PC3 NR PA can also be considered for reference architecture

    - Calibration point for PC2 (PC3 NR PA) (further update based on offline common view):

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 dBc

      - CIM3: 60 dBc

      - NR ACLR: 30dBc, other number is not precluded

    - Calibration based on NB-IOT signal is not precluded.

  - For PC1, consider adding [-34] dBc IQ-image and LO-leakage and 60 dBc CIM3 as another option for evaluation

    - Calibration points for PC1 (based on NR PC2 PA) (further update based on offline common view):

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 / [-34]dBc

      - CIM3: 60 dBc

      - NR ACLR: 31dBc, other number is not precluded

    - Calibration based on NB-IOT signal is not precluded.

Issue 1-1-5: A-MPR for NB-Iot based Iot-NTN PC2

- Agreement:

  - Wait for PC3 A-MPR baseline before concluding PC2 A-MPR for NS_04N and NS_05N (band 254)

  - No A-MPR is needed for [NS_03N] and NS_02N for NB-IoT PC2 in square brackets

  - Define [2] dB A-MPR increase for NS_24 to support NB-IoT PC2 compared to PC3

    - The exact value can be revisited once MPR is agreed.

Issue 1-1-6: ACLR for eMTC based Iot-NTN for PC1;

- Agreement:

  - [34dB]

Issue 1-1-7: MPR requirement for eMTC based Iot-NTN for PC1

- Agreement:

  - Reuse eMTC TN PC2/PC3 MPR requirements for eMTC based Iot-NTN for PC1 as starting point

  - Edge situation need to be checked

Issue 1-1-7a: General treatment for eMTC based requirements

- Agreement:

  - If there is leftover issue unsolved for a specific PC by the end of RAN4#116, MTC based solution associated with the corresponding PC will not be pursued in R19.

Issue 1-1-8: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation.

- Background (Last meeting’s WF):

  - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN

  - To down-select in next meeting between:

    - Option 1: Increase [1] dB MOP upper tolerance

    - Option 2: Increase [1] dB MOP nominal power (optional with capability signalling)

- WF(NEW):

  - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN

  - Increase [0.8] dB MOP or add a power boosting item in Pcmax equation (optional with capability signalling)

- Requirements Application

Issue 1-2-1: For NB-Iot based Iot-NTN, whether to have some limitation of numbers for the schedule of tones for PC1.

- Agreement:

  - Do not have restrictions of tone numbers

Current Status: (For Information)

Table: Current status for requirements consideration and feasibility for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | Both |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

- SAR Related issues

- SAR Related issues

Issue 2-1-1: Whether and how to define SAR scheme other than already agreed P-MPR (e.g. duty cycle related scheme):

- Background (for information):

  - P-MPR was already agreed to be the baseline SAR solution for all NTN HPUE cases.

  - The current status for duty-cycle based SAR scheme in TN

    - NR:

      - Defined in MOP section;

      - Applied in ΔPPowerClass in configured transmitted power clause;

      - whether can apply to FDD band has some different understanding.

    - eMTC:

      - Not defined in MOP section;

      - Reference general LTE configurated transmitted power clause, no separate definition

      - For general LTE, the duty cycle related power backoff explicitly defined only for TDD band whose frame configuration is 0 or 6

    - NB-Iot:

      - Not defined in MOP section;

      - Not defined in configured transmitted power clause which use dedicated equations (Not reference LTE general equation)

- Options for Selection:

  - Option 1: Keep the same status as TN for respective base RAT

    - NR-NTN:	Reuse NR TN,

      - Including ΔPPowerClass in configured transmitted power clause

    - eMTC based Iot-NTN:		Reuse eMTC

      - Not precluding ΔPPowerClass in configurated transmitted power clause

    - NB-Iot based Iot-NTN: 	Reusing NB-Iot

      - No duty cycle related scheme defined

  - Option 2: Introduce further revision / new method

    - NR-NTN sub-options:

      - A. Introduce clarification for FDD band support; (Nokia, Qualcomm)

      - B. changing the ΔPPowerClass from mandatory to optional;(Qualcomm)

      - C. UE voluntarily dropping uplink transmissions by self-assessment to control the SAR (vivo)

      - D. differentiate UE working status in terms of whether SAR is required in a time duration. (OPPO)

      - E. To have the network to notify/signal its upper bound of uplink ratio for a certain time duration (Samsung)

      - F. Energy headroom reporting (Qualcomm)

      - G. ΔPPowerClass and P-MPR reporting (Ericsson, SONY, Nokia)

      - H. Send information LS to RAN2 support PC1 NTN HPUE for FDD use cases. (Nokia)

      - I. inform the RAN that it is close to the SAR limit (ViaSet, Inmarsat)

    - Iot-NTN (including eMTC & NB-Iot based):

      - Introduce NR like scheme

      - Introduce other revision / new method as proposed for NR-NTN

  - Option3: Do not introduce for NTN

    - NR NTN	:	Explicitly preclude ΔPPowerClass and no other duty cycle related reference

    - eMTC-based Iot-NTN:		Explicitly preclude ΔPPowerClass and no other duty cycle related reference

    - NB-Iot based Iot-NTN:		No duty cycle or other new scheme defined

- Rx Requirments

- RSD (for full duplex eMTC based)

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based)

- Ad-hoc agreement:

  - For 256, PC2 1Tx RSD as 0.5 dB.

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Ad-hoc agreement:

  - Consider [0] dB RSD for PC1 non-handheld devices for the current NTN bands with the following.

    - To document 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD in the TR.

    - Revisit this value with more inputs in next meeting.