3GPP TSG-RAN WG4 Meeting #115	R4-2508263

# Malta, MT, 19th - 23rd May, 2025

Title: 	WF on RRM requirements for NR_ATG_enh

Agenda Item:	7.12.1

Source: 	CMCC

Document for:	Approval

<Topic 1> RRM core requirement

Issue 1-1-1: UE Beam Type capability signaling

Issue 1-1-1-1: The Report granularity of UE capability

Agreement: Per BC.

Issue 1-1-1-2: The Report feature

Agreement:

- Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination

- In the meanwhile, revisit the name and definition in the “components” for the FG.

Issue 1-1-1-3: The Report functionalities between FDD and TDD

Agreement: No

Issue 1-1-1-4: Note for the capability

Agreement: This UE feature is applicable only for inter-band CA band combination(s) in Table [TBD] in TS 38.101-1.

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rx beam Type | Indicates whether the UE supports one common Rx beam or two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | Per BC | No | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table [TBD] in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination |

Issue 1-2-1: Interruption requirements

Agreement:

- The existing interruption requirements caused by SCell addition / release and SCell activation / deactivation can be reused for ATG CA depending on intra-band CA and inter-band CA (i.e., no need to consider different requirement for common Rx beam ATG UE in inter-band CA).

Issue 1-2-2: SCell activation delay enhancement

Agreement:

Reuse the legacy SCell activation delay requirement for ATG

Issue 1-2-3: Active BWP switch delay of single CC requirement

Agreement:

Introduce SCell dormancy related impact for active BWP switch delay of single CC requirement. The legacy TN requirement can be reused.

Can be revert if technical issue found.

Issue 1-3-1: Sharing factor between L1 measurement and RRM measurement - Condition for Klayer1_measurement=1 and definition format

Agreement:

- For UE capable of [antennaArrayType-r18] on the measured carrier

  - Klayer1_measurement=1,

    - If inter-band carrier aggregation within FR1 is configured [and UE doesn’t support capability of case 4]

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of same serving cell

    - Otherwise,

      - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement - effective non-overlapping duration

- Proposals

  - Proposal 1: For the ‘SSB-ToMeasure and SS-RSSI-Measurement are configured’, use MRTD as the reference when calculate the margin for non-overlapping duration

    - Proposal 1-1:

      - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

        - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

    - Proposal 1-2:

      - For UE capable of [antennaArrayType-r18] and [intra-band or inter-band carrier aggregation is configured],

        - Klayer1_measurement=1,

          - -	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

          - -	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in different band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbols before each consecutive SSB symbols and the RSSI symbols, and 2 symbols after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured and the UE is capable of [common Rx beam between PCC band and SCC band], where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

  - Proposal 2:

    - Firstly align the understanding about the reference timing of SSB symbols and the RSSI symbols in the definition of Klayer1_measurement.

      - If the serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then extend the margin considering the maximum propagation delay difference between serving cell and neighbour cells.

      - If the neighbour cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then reuse the legacy 1 symbol margin

  - Proposal 3:

    - reuse the legacy specification, i.e.

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

Issue 1-3-3 Measurement enhancement for ATG UE with antenna array

Agreement:

- introduce a network signaling to indicate whether an ATG UE supporting antennaArrayType-r18 on the SCC shall perform the neighbour cell measurements or not on the frequency configured by servingcellMO for SCC, i.e. to indicate whether the UE is only required to measure the serving cell or both serving and neighbour cell on the frequency configured by servingCellMO for SCC

<Topic 2> RRM perf requirement

Issue 2-1-1: Test applicability rule

  - Proposal 1:

    - UE is only required to be tested in one of the supported test configurations

    - UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration

  - Proposal 2:

    - Test configuration for NR PCell and test configuration for NR SCell shall be chosen independently

Issue 2-1-2: Test setups - TDD pattern for SCell

  - Proposal 1: Reuse the legacy TDD pattern (CATT, LGE, CMCC, ZTE)

    - Proposal 1-1: (CATT)

      - For intra-band CA, TDD pattern for SCell need to be the same with PCell.

      - For inter-band CA, the pattern used in R18 ATG test cases can also be reused.

    - Proposal 1-2: (ZTE)

      - For TDD case, the TDD pattern for SCell could be same as PCell (ZTE)

  - Proposal 2: RAN4 to discuss whether or not to define a new TDD pattern for ATG (HW)

  - Proposal 3: Define all ‘D’ slot TDD pattern for ATG (Ericsson)

Issue 2-1-3: Test method and scaling factor for UE with antenna array

Agreement

  - Use conductive test method, apply scaling factor (including N1, Klayer1 and P) = 1 for antenna array related test requirements

  - Not to consider separate test cases for ATG UE to support common Rx beam

Issue 2-1-4: Test scope

FFS based on following summary:

  - In RAN4#115, following test cases supported by no less than 5 companies

    - Interruptions during measurements on deactivated NR SCC in FR1

    - Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot

    - Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot

    - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

    - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

    - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification)

    - Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification)

    - Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

    - Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification)

    - SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification)

  - In RAN4#115, no supporting company for following test cases

    - Direct SCell Activation at RRC Resume

    - non-simultaneous Timer based BWP switch delay on multiple CCs

  - In RAN4#115, 1-4 supporting companies for following test cases

    - Direct SCell activation at handover with known SCell in FR1

    - TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX

    - Inter-band SSB-less SCell Activation based on A-TRS

    - Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG

    - Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode

    - Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

    - Simultaneous RRC-based Active BWP Switch on multiple CCs

    - SCell dormancy switch

    - Event triggered reporting on SCC with deactivated SCell test under non-DRX

    - Test case for Pre-configured measurement gap requirement

    - Intra-frequency measurement(with scheduling restriction verified as well)

    - Inter-frequency measurement(with scheduling restriction verified as well)

    - L1-RSRP measurement

    - CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX

    - The interruption of SCell addition/release

Issue 2-1-5: Detail test configuration and test procedure

- Proposals

  - Proposal 1:

    - If the ‘deactivated SCell measurement with measCycleSCell larger than 640ms’ test case will be introduced eventually, then verify the UE interruption performance on deactivated SCC measurement together in ‘deactivated SCell measurement’ test. Otherwise, introduce a separate interruption test case

    - For the test ‘Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs’, one PCell and one SCell shall be configured, and BWP switching happens on PCell and SCell simultaneously.

  - Proposal 2:

    - Inter-band SSB-less SCell Activation test cases should be restricted to ATG UE explicitly supporting [Case 4 capability]