3GPP TSG-RAN WG4 Meeting #115										                     R4-2508274

Malta, MT, 19 – 23 May, 2025

Agenda item:			7.25.4

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR_duplex_evo

Document for:	Approval

Agreements

# Topic #1: UE-to-UE CLI handling

    - Sub-topic 1-1: General issues

Issue 1-1-1: Requirement applicability

- Agreement (Monday online session)

  - For method #2, 3 and non-basic UE operation of method #1

    - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

      - when SRS-RSRP measurement resource is configured within active DL BWP

      - when CLI-RSSI measurement resource is configured within active DL BWP

  - FFS for basic UE operation for method #1

  - Note: The assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion.

Issue 1-1-2: SCS for performing measurement

- Agreement

  - UE shall perform L1-CLI-RSSI and L1-SRS-RSRP measurements with same SCS as the active DL BWP.

  - Note: this is same as existing CSI-RS based L1-RSRP measurement.

Issue 1-1-4: Collision between L1 CLI and L3 CLI measurements

- Agreement

  - Close the issue based on following RAN1 agreement in RAN1#121

Agreement A UE does not expect to be simultaneously configured with L3 and L1 UE-to-UE CLI measurement and reporting.

Sub-topic 1-2: L1-SRS-RSRP

Issue 1-2-1: Measurement period

- Agreement (Monday online session)

  - Not define separate RRM core requirements for intra- and inter-cell scenario, and it is not needed to introduce network configuration on the intra- and inter-cell scenario from RAN4 RRM perspective.

Issue 1-2-2: Scheduling restriction

- Agreement (Tuesday AH session)

  - Do not define DL scheduling restriction on symbol after SRS resource for the switching between UL subband and DL subband.

Issue 1-2-3: Measurement restriction

- Agreement

  - Take into account following RAN1 agreement in RAN1#121, and further discuss the measurement restriction between Method #1 and Method #2.

Agreement A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol.

Sub-topic 1-3: L1-CLI-RSSI

Issue 1-3-2: Scheduling restriction

- Agreement (Tuesday AH session)

  - For Method #3

    - Do not define DL scheduling restriction on symbol after RSSI resource for the switching between UL subband and DL subband.

Issue 1-3-4: Nominal BW and SCS for reporting

- Agreement

  - UE shall scale the measured CLI-RSSI to report a nominal RSSI equivalent to 6RB measurement with 15 kHz SCS.

  - Note: this is same as existing report mapping for L3 CLI-RSSI as defined in clause 10.1.22.2.2.

# Topic #2: SBFD operation impacts

Sub-topic 2-1: RRM impacts of SBFD operation

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Agreement

  - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex.

  - Further discuss the upper bound of the measurement period due to processing time and its applicability.

Issue 2-1-1c: CSI-RS L1 measurement – different symbol types

- Agreements

  - For L1-RSRP and L1-SINR

    - Measurement period is extended by using R16 NR-U approach.

  - For RLM/BFD/CBD

    - Confirm the following for defining the requirements

      - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type.

      - Existing measurement period apply.

Issue 2-1-2: Requirements for CSI-RS based L3 measurement

- Agreements

  - For DUD case

    - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in one DL subband of SBFD symbols and/or on non-SBFD DL symbols of the cell transmitting the CSI-RS.

- Agreements (Monday online session)

  - For collision between CSI-RS resource and dynamic UL transmission

    - existing scheduling restriction apply, i.e. CSI-RS based L3 measurement is prioritized

Issue 2-1-3: Requirements for scheduling restriction

- Agreements

  - Do not define new scheduling restriction requirements for transition between UL and DL.

  - RAN4 to use the existing scheduling restriction requirements for inter-frequency SSB measurement without gap as a starting point, i.e. no new restriction is defined.

Topic #3: Performance part

Sub-topic 3-1: Work plan for the performance part

- Agreements

  - Approve the RRM part of work plan in R4-2507487.

# Potential issues for further discussion

Topic 1: UE-to-UE CLI handling

Sub-topic 1-1: General issues

Issue 1-1-3: L1 CLI measurement in non-SBFD symbols

Sub-topic 1-2: L1-SRS-RSRP

Sub-topic 1-3: L1-CLI-RSSI

Issue 1-3-1: Measurement period

Issue 1-3-3: Measurement restriction

Topic #2: SBFD operation impacts

Sub-topic 2-1: RRM impacts of SBFD operation

Issue 2-1-4: Requirements for RACH requirements

Issue 2-1-5 (new): Lmax when using NR-U approach for L1 CSI-RS measurement

Topic #3: Performance part

Sub-topic 3-2: Accuracy requirements

Issue 3-2-1: L1-SRS-RSRP – side condition on time offset

Issue 3-2-2: L1-SRS-RSRP – side condition on SRS BW

Issue 3-2-3: L1-SRS-RSRP – side condition on SRS Es/Iot

Issue 3-2-4: L1-SRS-RSRP – accuracy requirements

Issue 3-2-5: L1-CLI-RSSI – accuracy requirements

Issue 3-2-6: L1-RSRP and L1-SINR – accuracy requirements

Sub-topic 3-3: Report mapping

Issue 3-3-1: Absolute report mapping

Issue 3-3-2: Differential report mapping

Sub-topic 3-4: Test cases

Issue 3-4-1: General test setup

Issue 3-4-2: Test case list

Issue 3-4-3: Test case details and test purpose

Issue 3-4-4: Test case scenarios

Annex

Table A-1 lists the simulation assumption for L1-RSRP. It is same as in R4-1904760 (Simulation summary of L1-RSRP accuracy, Ericsson) with 96 RB removed.

Table A-1: simulation assumption for L1-RSRP

| Parameters | Value |
| --- | --- |
| SCS | 15 kHz, 30 kHz, 120 kHz |
| Side condition (SNR) | -3 dB |
| Density (D) | 3 |
| Number of samples (M) | 1 (single sample measurement) |
| Number of PRB | 24, 48 |
| Propagation condition | AWGN |

Tables A-2 to A-6 list the simulation assumption for L1-SINR. They are same as in R4-2103726 (L1-SINR simulation results summary, Samsung) with

- 24 RB added for the BW of CSI-RS as CMR/IMR

- number of samples set to 1 based on RAN4#97 agreement (in R4-2017375)

- SNR for CMR/IMR set to 0dB for NZP-IMR cases (Case 2C and 2D) based on RAN4#97 agreement (in R4-2017375)

Table A-2: simulation assumption for L1-SINR with CMR only (Case 1A)

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | N/A |
| Side condition (SNR) on CMR | -3dB |
| Density (D) | 3 |
| Number of samples (M) | 1 |
| Number of PRBs | 24, 48 |
| Propagation condition | AWGN |

Table A-3: simulation assumption for L1-SINR with SSB-based CMR + NZP-IMR (Case 2C)

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | 0dB |
| Side condition (SNR) on IMR | 0dB |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1 |
| Number of PRBs for IMR | 24, 48 |
| Propagation condition | AWGN |

Table A-4: simulation assumption for L1-SINR with SSB-based CMR + ZP-IMR (Case 2A)

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | SSB |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | -3dB |
| Side condition (SNR) on IMR | N/A (only AWGN noise) |
| Density (D) for IMR | 3 |
| Number of samples (M) for IMR/CMR | 1 |
| Number of PRBs for IMR | 24, 48 |
| Propagation condition | AWGN |

Table A-5: simulation assumption for L1-SINR with CSI-RS based CMR + NZP-IMR (Case 2D)

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | 0dB |
| Side condition (SNR) on IMR | 0dB |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1 |
| Number of PRBs for CMR/IMR | 24, 48 |
| Propagation condition | AWGN |

Table A-6: simulation assumption for L1-SINR with SSB-based CMR + ZP-IMR (Case 2B)

| Parameters | Values |
| --- | --- |
| SCS | 15kHz (FR1), 30kHz (FR1), 120kHz (FR2) |
| Channel measurement resource (CMR) | CSI-RS |
| Interference measurement resource (IMR) configuration | CSI-RS |
| Periodicity | CMR Periodicity = IMR Periodicity |
| Ideal SINR | -3dB |
| Side condition (SNR) on CMR | -3dB |
| Side condition (SNR) on IMR | N/A (only AWGN noise) |
| Density (D) for CMR/IMR | 3 |
| Number of samples (M) for CMR/IMR | 1 |
| Number of PRBs for CMR/IMR | 24, 48 |
| Propagation condition | AWGN |