# 3GPP TSG-RAN WG4 Meeting #115                                                      R4-2508275

# Malta, MT, 19th – 23rd May, 2025

Agenda item:			7.26.1

Source:	Moderator (CMCC)

Title:	WF on RRM requirements for Ambient_IoT_Solutions

Document for:	Approval

Topic #1: RRM core requirements (7.26.4)

Sub-topic 1-1: A-IoT random access procedure

Issue 1-1: A-IoT random access procedure

Agreement:

        - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec.

- CBRA procedure

- the correct behaviour when transmitting A-IoT MSG1(Random ID transmission)

- the correct behaviour when receiving A-IoT MSG2

- the correct behaviour when not receiving A-IoT MSG2

- CFRA procedure

- the correct behaviour when transmitting First D2R message

Sub-topic 1-2: timing

Issue 1-2-1: Device transmit timing

Agreement:

- RAN4 define D2R transmit timing requirements.

- Define D2R transmit timing error requirements as follows:

- For MSG1 X=1,

- The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1.

- For MSG1 X=2, follow the same principle as starting point.

- Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements.

- Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any.

Issue 1-2-2: Device receive timing

FFS:

- Option 1 (ZTE): RAN4 shall consider accuracy requirements on TD2R_min if the range of it depends on RAN1.

CR split

| Requirements (TS 38.191) | Title | Leading company |
| --- | --- | --- |
| 9.1.1 | Introduction of random access | Ericsson |
| 9.1.2.1 | CBRA: Correct behaviour when transmitting A-IoT MSG1(Random ID transmission ) | CATT |
| 9.1.2.2 | CBRA: Correct behaviour when receiving A-IoT MSG2 | ZTE |
| 9.1.2.3 | CBRA: Correct behaviour when not receiving A-IoT MSG2 | Xiaomi |
| 9.1.3.1 | CFRA: Correct behaviour when transmitting First D2R message | Huawei |
| 9.2 | D2R Timing | CMCC |